3GPP TSG-RAN5 Meeting #106	R5-251739

Athens, Greece, 17 Feb – 21 Feb, 2025

Title:							Discussion on FR2 beam correspondence for initial access

Source:			Huawei, HiSilicon

Agenda Item:

Document for:			Discussion

# 1.	Background

RAN4 introduced FR2 beam correspondence requirement for initial access in Rel-18. The requirement consists of EIRP spherical coverage requirement for PRACH in initial access. Considering the differences in UE behaviour between RRC_IDLE mode and RRC_CONNECTED mode, following working assumptions are made by RAN4 when deciding the minimum requirement [1].

- RAN4 has agreed to inform RAN5 to develop the beamlock function in initial access.

- UE locks the beam direction after requested by test equipment

- UE is kept in RRC_IDLE state to ensure at least 1ms EIRP measurement period for PRACH.

- Enable multiple PRACH transmissions in testing mode, including holding RAR.

- UE transmits with the optimal Tx beam that is autonomously selected by UE.

- UE achieves Pcmax before PRACH EIRP measurement.

- UE transmits PRACH with gap <=20ms.

- To accelerate the testing, it’s recommended to use the longest applicable PRACH preamble format and minimum gap between PRACH transmission.

In previous RAN5 meetings there was discussion on whether to introduce a beamlock function for testing this requirement. This contribution focuses on comparison of the methods with and without beamlock function.

# 2	Discussion

The purpose of introducing the beam correspondence requirements is to verify that UE could transmit the PRACH preamble using the proper UL transmission beam selected based on the DL measurement of the SSB signal level.

The SSB is transmitted by gNB every 20ms, with the maximum time duration of 5ms. Within the 5ms, multiple SSBs could be transmitted using different DL beams of the gNB. UE could perform the SSB measurement and decide the strongest SSB it could respond to. On the other hand, the UE usually also have multiple candidate Rx/Tx beams. It could switch the Rx beam every 20ms to repeat the SSB measurement, then pick one proper Rx beam and transmit the PRACH preamble using the correspondent Tx beam. The method of UE selecting Rx beam is not standardized and up to UE implementation.

Observation 1: UE measures the SSB every 20ms across multiple candidate Rx beams to decide the proper Rx beam, and transmit PRACH preamble using the correspondent Tx beam. The method of UE selecting Rx beam is not standardized and up to UE implementation.

To fasten the initial access process, UE might consider various method of selecting Rx beam. Below Figure 1 is one potential implementation discussed in RAN4.

- a) At the beginning, UE uses the wide beam to measure the SSB and decide the rough direction of DL signal. If the SSB measurement result of at least one Rx beam is above the network configured threshold rsrp-ThresholdSSB, UE could transmit the PRACH preamble in the correspondent Tx beam.

- b) If no RAR is received, UE might ramp up the Tx power, or switch to narrow beam and repeat the SSB measurement. The set of narrow beams might be selected based on the measurement of wide beam.

- c) If no RAR is received, UE might switch to a larger set of narrow beams and perform the full SSB measurement.

- d) If the number of preamble transmission attempt exceeds the network configured preambleTransMax, the UE considers the current random access procedure is unsuccessfully completed. Another random access procedure might be initiated by the higher layer later.

Figure 1. Example implementation of beam selecting

Observation 2: The Rx/Tx beam type and the set of beams might change during the initial access procedure.

During the whole random access procedure, UE will perform the PRACH attempt on the Rx/Tx beams where the SSB measurement is above rsrp-ThresholdSSB, while the selected beam might not be the optimal one depending on the sequence of beam scanning. The below Figure 2 shows an example of the beam scanning, and corresponding Tx power of each PRACH attempt. It could be told from the figure that the UE’s Tx power might ramp up on the same beam, jump up due to switching from wide beam and narrow beam, or fall down due to attempt performed in a larger beam set.

Figure 2. Example case of PRACH Tx power without beamlock

Observation 3: The Tx power of PRACH attempts might be fluctuating.

In the real deployment case, UE usually doesn’t need to experience the whole procedure in above Figure 2. The network controls the coverage area by configuring the threshold rsrp-ThresholdSSB. Once the SSB measurement result is above this threshold, the UE is within the expected coverage area, and the PRACH transmission could be successfully received by the gNB. Therefore, the initial access could be completed anywhere in Figure 2 once the RAR is returned from the gNB.

However, for conformance testing the purpose is to verify that UE could transmit enough power in the most critical case, i.e. the Tx power from the best beam needs to be above the minimum requirement during the measurement period of 1ms. The measurement has to be done across several PRACH preamble transmissions. To enforce the UE continually retransmit the PRACH preamble, the RAR needs to be hold during the whole test procedure. Therefore it’s quite possible UE would experience part of or even the whole procedure in the Figure 2. This makes the measurement and verdict very difficult since deciding the proper time slot for measurement is very hard. On top of that the minimum requirements are hard to decide when the power fluctuation needs to be taken into account. That’s why RAN4 decides that the beamlock function is needed as the baseline for deriving the Rel-18 requirement.

Observation 4: The Tx power fluctuating is a problem mainly for conformance testing due to intentionally holding the RAR.

Observation 5: Beamlock function is the basic assumption for RAN4 requirement derivation.

There was concern that using the beamlock function might hurt the purpose and the value of this test case, while this is not true. Considering the core purpose of this test case is to verify that the UE could select the best beam based on SSB measurement, the beamlock function could be carefully designed to meet the purpose. Below Figure 3 is one possible design of the beamlock function with following steps:

- UE is required to perform the SSB measurement using any set of Rx beams silently.

- When UE decides the proper beam for PRACH transmission, it starts to transmit PRACH preambles repeatedly with the Tx beam locked through the whole test procedure.

- The TE could perform the measurement on any of the PRACH preambles until the required measurement period is reached.

With this procedure, the UE’s Tx beam is selected completely based on the SSB measurement and could perfectly reflect its ability of beam correspondence. At the same time this could reduce the complexity of the measurement procedure, and the test time could also be reduced since UE doesn’t need to waste time to transmit PRACH on sub-optimal beams and wait for the RAR-window to expire.

Figure 3. Example case of PRACH Tx power with beamlock

Observation 6: Beamlock function could be carefully designed to meet the purpose of this test. At the same time the test procedure is more simplified and the test time is reduced.

The test procedure of beamlock function could be shown and below Figure 3a.

Figure 3a	Flow chart for Beam lock function in RRC_IDLE

The detailed test procedures are listed below for reference

- Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= polarization

- [SS activates the UE Beamlock Function for IDLE mode (UBF_IDLE).]

- SS transmit RRC_RELEASE message

- Rotate the UE randomly to make sure a different measurement grid is used in following steps

- Wait for a time period up to [BEAM_SELECT_WAIT_TIME_IDLE] for the UE TX beam selection to complete.

- SS transmit paging message

- When SS detects a PRACH preamble transmited by the UE, the measurement is done at the [preambles [0~9] or preambles [189~198] using pramble B4 and index 143].

- Measure the mean power Pmeas (PolMeas= PolLink=) across multiple preambles until the accumulated measurement time achieves 1ms.

- Measure the mean power Pmeas (PolMeas=, PolLink=) across multiple preambles until the accumulated measurement time achieves 1ms.

- Calculate total EIRP(PolLink=) = EIRP(PolMeas= PolLink=)  + EIRP(PolMeas= PolLink=).

- SS transmit RAR and Ensure the UE is in state RRC_CONNECTED.

- Advance to next measurement grid and repeat 1~10 until all measurement grids are completed.

- Connect the SS (System Simulator) with the DUT through the measurement antenna with PolLink= polarization.

- Repeat 1~11. Calculate total EIRP(PolLink=) for all the measurement grids

- Calculate EIRP = min (EIRP(PolLink=), EIRP(PolLink=)) for each measurement grid.

During previous meetings, the option of not introducing beamlock function was also discussed. The major problem for this option is this violates the basic assumption of RAN4 requirement derivation. RAN5 is not in the right position to revert this decision.

From technical point of view, if beamlock function is not specified, the solution of selecting the time slot for PRACH Tx power measurement needs to be figured out. Based on the offline discussion in last meeting, one possible solution is to record the Tx power of all the PRACH transmissions, and picking the highest samples as the measurement result and making verdict accordingly. This solution is workable if enough samples are recorded. If the number of samples is not enough, some sub-optimal samples might also be selected in the final measurement result. The test time will be quite long since many samples are not used in the end, but the time of UE transmitting PRACH preamble and waiting for RAR is still spent. Moreover, this method can’t distinguish the conformant UE and the non-conformant UE well. If a non-conformant UE is not able to select a proper beam by itself (NB_4 in the example), it still can pass the test if it tries to scan several Tx beams (NB_3~NB_5). This will make the testing meaningless.

In addition, the currently the power measurement needs to be the sum of power from UE’s signal of both polarizations. While due to the implementation restriction, the measurement could to be done for single polarization at one time. Without beamlock function, it’s impossible to measure UE’s both polarization under the same condition.

Figure 4. Example solution without beamlock function

Observation 7: The potential solution without beamlock function has some drawbacks:

- The method can’t distinguish the conformant UE and the non-conformant UE

- The sub-optimal beam might be included in the measurement result

- The test time is long

- UE’s both polarizations can’t be measured under same condition.

As summary of above discussion, we think the beamlock function should still be the baseline of RAN5 testing. We could further discuss the details after the

Proposal 1: RAN5 introduces beamlock function for testing of beam correspondence in initial access.

# 3.	Conclusion

In summary, following observations and proposal are for RAN5 discussion:

Observation 1: UE measures the SSB every 20ms across multiple candidate Rx beams to decide the proper Rx beam, and transmit PRACH preamble using the correspondent Tx beam. The method of UE selecting Rx beam is not standardized and up to UE implementation.

Observation 2: The Rx/Tx beam type and the set of beams might change during the initial access procedure.

Observation 3: The Tx power of PRACH attempts might be fluctuating.

Observation 4: The Tx power fluctuating is a problem mainly for conformance testing due to intentionally holding the RAR.

Observation 5: Beamlock function is the basic assumption for RAN4 requirement derivation.

Observation 6: Beamlock function could be carefully designed to meet the purpose of this test. At the same time the test procedure is more simplified and the test time is reduced.

Observation 7: The potential solution without beamlock function has some drawbacks:

- The method can’t distinguish the conformant UE and the non-conformant UE

- The sub-optimal beam might be included in the measurement result

- The test time is long

- UE’s both polarizations can’t be measured under same condition.

Proposal 1: RAN5 introduces beamlock function for testing of beam correspondence in initial access.

# 4.	Reference

[1]	R4-2317764, LS on demodulation of FR2 UE UL by TE

[2]	R5-241402, Initial discussions for Rel-18 beam correspondence for initial access, Keysight Technologies UK Ltd, RAN5#102

[3] R5-240944, Time synchronization required for UE beamlock function during initial access, Apple AB Denmark, RAN5#102

[4] R5-243823, Discussion on FR2 beam correspondence for initial access, Huawei, RAN5#103

[5] R5-245019, Discussion on FR2 beam correspondence for initial access , Huawei, RAN5#104