3GPP TSG-RAN5 Meeting #106	R5-250470r1

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1577 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to contents of 200 OK message for CEN alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Default message contents of 200 OK message clause A.3.1 needs update to align with CEN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated contents of A.3.1 - 200 OK for other requests than REGISTER or SUBSCRIBE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eCallCEN-UEConTest WI will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-250470r1: 1. Clause A.3.1 is aligned to latest specification. 2. Updated Accept header. |  |  |  |  |  |  |  |  |

<Start of modified section>

## A.3.1	200 OK for other requests than REGISTER or SUBSCRIBE

| Header/param |  | Cond |  | Value/remark |  | Rel |  | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Status-Line |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| SIP-Version |  |  |  | SIP/2.0 |  |  |  |  |  |
| Status-Code |  |  |  | 200 |  |  |  |  |  |
| Reason-Phrase |  |  |  | OK |  |  |  |  |  |
| Via |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| via-parm |  |  |  | same value as received in request |  |  |  |  |  |
| Record-Route |  |  |  | order of the parameters in this header must be like in the respective rows |  |  |  | RFC 3261 [15] |  |
| rec-route |  | A1 AND NOT A23 |  | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: protected server port of SS;lr> |  |  |  |  |  |
|  |  | A3 AND NOT A23 |  | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: unprotected server port of SS (optional);lr> |  |  |  |  |  |
|  |  | A20 AND NOT A23 |  | same value as received in the request (initial INVITE from the SS) Note: for requests other than INVITE it is not regulated if and what the UE writes into this header in a response. |  |  |  |  |  |
|  |  | A6 |  | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:protected server port of SS;lr> |  |  |  |  |  |
|  |  | A7 |  | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:unprotected server port of SS;lr> |  |  |  |  |  |
| From |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | same value as received in request |  |  |  |  |  |
| tag |  |  |  | same value as received in request |  |  |  |  |  |
| To |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | same value as received in request |  |  |  |  |  |
| tag |  |  |  | same value as received in request or any value added if missing from request |  |  |  |  |  |
| P-Asserted-Identity |  | A6 |  |  |  |  |  | RFC 3325 [89] |  |
| addr-spec |  |  |  | A tel URI that can be recognized as valid emergency numbers if dialled by the user are specified in 3GPP TS 22.101 [39]. The emergency numbers 112 and 911 are stored on the ME, in accordance with 3GPP TS 22.101 [39] |  |  |  |  |  |
| uri-parameter |  |  |  | lr |  |  |  |  |  |
| Contact |  | A1,A2 A3,A4 |  |  |  |  |  | RFC 3261 [15] RFC 5627 [61] |  |
| addr-spec |  | A1,A3 |  | MO call: px_IMS_CalleeContactUri MT call: Contact address provided by SS in dialog creating INVITE |  |  |  |  |  |
|  |  | A2,A4 |  | MO call: Contact address provided by UE in dialog creating INVITE MT call: Contact address provided by UE in 1xx response establishing the dialog according to RFC 3261 [15] clause 12. (NOTE: The 'transport' url parameter may change, url parameters are not checked) |  |  |  |  |  |
| feature-param |  | A10 |  | audio |  |  |  |  |  |
|  |  | A18 AND (A19 OR A20) |  | video |  |  |  | IR.94 [134] |  |
|  |  | A11 AND A15 AND (A16 OR A17 OR A22) |  | audio |  |  |  |  |  |
| Call-ID |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| callid |  |  |  | same value as received in request |  |  |  |  |  |
| Call-Info |  | A12 |  |  |  |  |  | RFC 8147 [149] |  |
| info |  |  |  |  |  |  |  |  |  |
| absoluteURI |  |  |  | cid:psap@3gpp.org |  | Rel-14 |  |  |  |
| info-param |  |  |  | purpose=EmergencyCallData.Control |  |  |  |  |  |
| CSeq |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | same value as received in request |  |  |  |  |  |
| P-Access-Network-Info |  | A8 |  |  |  |  |  | RFC 7315 [132] RFC 7913 [154] |  |
| access-net-spec |  | A21 |  | access network information and, if applicable, the cell ID |  |  |  |  |  |
|  |  | A22 |  | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID |  |  |  |  |  |
| Accept |  | A12,A24 |  |  |  |  |  | RFC 8147 [149] |  |
| media-range |  | A12 |  | application/sdp, application/pidf+xml, application/EmergencyCallData.Control+xml, application/emergencyCallData.eCall.MSD |  | Rel-14 |  |  |  |
|  |  | A24 |  | application/EmergencyCallData.Control+xml |  | Rel-19 |  | TS 24.229 [10] |  |
| Recv-Info |  | A12,A24 |  |  |  |  |  | RFC 8147 [149] |  |
| Info-package-type |  |  |  | emergencyCallData.eCall.MSD |  | Rel-14 |  |  |  |
| Content-Type |  | A12 |  |  |  |  |  | RFC 8147 [149] |  |
| media-type |  |  |  | multipart/mixed; boundary=boundary1 |  | Rel-14 |  |  |  |
| Content-Length |  | A10 |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | length of message body |  |  |  |  |  |
| Message-body |  | A12 |  |  |  |  |  |  |  |
|  |  | A13 |  | --boundary1 Content-Type: application/EmergencyCallData.eCall.Control+xml Content-ID: <psap@3gpp.org> Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <EmergencyCallData.control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack received="true" ref=" addr-spec of the Content-ID of MIME body part containing the MSD sent by the UE in INVITE"/> </EmergencyCallData.control> --boundary1 |  |  |  | RFC 8147 [149] |  |
|  |  | A14 |  | --boundary1 Content-Type: application/EmergencyCallData.eCall.Control+xml Content-ID: <psap@3gpp.org> Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <emergencyCallData.Control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack received="false" ref=" addr-spec of the Content-ID of MIME body part containing the MSD sent by the UE in INVITE "/> </EmergencyCallData.control> --boundary1 |  |  |  | RFC 8147 [149] |  |

| Condition | Explanation |
| --- | --- |
| A1 | Response sent by SS for INVITE/UPDATE (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A2 | Response sent by UE for INVITE/UPDATE (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A3 | Response sent by SS for INVITE/UPDATE (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A4 | Response sent by UE for INVITE/UPDATE (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A5 | Void |
| A6 | Response sent by SS for INVITE for emergency call or non-UE detectable emergency call |
| A7 | Response sent by SS for INVITE for emergency call without emergency registration |
| A8 | Any response sent by the UE within a dialog, except for CANCEL requests (NOTE 2) |
| A9 | Void |
| A10 | Response sent by SS |
| A11 | Response sent by UE |
| A12 | Response sent by SS for INVITE for eCall over IMS session with either ACK or NACK |
| A13 | Response sent by SS for INVITE for eCall over IMS session with ACK element = TRUE |
| A14 | Response sent by SS for INVITE for eCall over IMS session with ACK element = FALSE |
| A15 | UE supports audio media feature tag (A.12/56 3GPP TS 34.229-2 [5]) |
| A16 | UE uses E-UTRAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.301 [150], clauses 8.2.1 and 9.9.3.12A |
| A17 | UE uses UTRAN/GERAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.008 [12], clauses 9.4.2 and 10.5.5.23 |
| A18 | UE supports video feature tag (A.12/32 3GPP TS 34.229-2 [5]) |
| A19 | Response sent by SS for INVITE |
| A20 | Response sent by UE for INVITE |
| A21 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A22 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A23 | Response sent for re-INVITE within an established dialog |
| A24 | Response sent by UE subjected to national regulations in CEN EN 17184:2024 (A.4.5/67 3GPP TS 34.229-2 [5] or A.4.5/68 3GPP TS 34.229-2 [5]) for INVITE for PSAP Callback |

NOTE 1:	When the UE supports IMS security then condition A1 or A2 is applied unless condition A3 or A4 is specified explicitly by the test case.

NOTE 2:	As the 200 OK for SIP MESSAGE is the only case where a 200 OK for other requests than REGISTER or SUBSCRIBE is not sent within a dialog, in fact condition A8 is always applied except for the 200 OK responses for CANCEL and SIP MESSAGE.

<End of modified section>