3GPP TSG-RAN5 Meeting #106	R5-251034r1

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0790 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ICS table for RedCap UE supporting HD FDD band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FDD_bands_R18_redcap-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | R4 has introduced n105 for HD-FDD RedCap UE in R18 core WI NR_FDD_bands_R18_redcap, but there are still a few FDD band with undefined Refsens minimum Conformance for HD-FDD RedCap UE just like n31, n72, n100, n106,e,g, which may cause confusing when using the NR FDD FR1 RF baseline implementation capabilities to select the specific test band for HD-FDD RedCap UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the ICS table A.4.3.1-1a for NR HD-FDD RedCap UE FR1 RF Baseline Implementation Capabilities |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specific band for HD-FDD RedCap UE will be unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Updated the new table title to A.4.3.1-12 Updated the Mnemonic to HD_FDD |  |  |  |  |  |  |  |  |

<Start of changes>

A.4.3.1	RF Baseline Implementation Capabilities

NOTE:	The values indicated in column "Release" for bands are to be understood as the specifications release version in which a band was introduced and not as a mandate that a UE conforming to particular release shall support a particular band. For further guidance to release independent bands see TS 38.307 [19].

NOTE:	See Annex B for status of completed NR bands and power classes in this version of 3GPP UE conformance test specifications.

Table A.4.3.1-1: NR FDD FR1 RF Baseline Implementation Capabilities

| Item | NR FDD FR1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 1920-1980 MHz (UL), 2110-2170 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand1_Supp | NR FDD FR1 Band n1 |
| 2 | NR Frequency band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand2_Supp | NR FDD FR1 Band n2 |
| 3 | NR Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand3_Supp | NR FDD FR1 Band n3 |
| 4 | NR Frequency band: 824-849 MHz (UL), 869-894 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand5_Supp | NR FDD FR1 Band n5 |
| 5 | NR Frequency band: 2500-2570 MHz (UL), 2620-2690 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand7_Supp | NR FDD FR1 Band n7 |
| 6 | NR Frequency band: 880-915 MHz (UL), 925-960 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand8_Supp | NR FDD FR1 Band n8 |
| 6a to 6c | Reserved |  |  |  |  |
| 6d | NR Frequency band: 699-716 MHz (UL), 729-746 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand12_Supp | NR FDD FR1 Band n12 |
| 6e | NR Frequency band: 777-787 MHz (UL), 746-756 MHz (DL) | 38.101-1, 5.2 | Rel-17 | pc_nrBand13_Supp | NR FDD FR1 Band n13 |
| 6f | NR Frequency band: 788-798 MHz (UL), 758-768 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand14_Supp | NR FDD FR1 Band n14 |
| 6g to 6i | Reserved |  |  |  |  |
| 6j | NR Frequency band: 815-830 MHz (UL), 860-875 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand18_Supp | NR FDD FR1 Band n18 |
| 6k | Reserved |  |  |  |  |
| 7 | NR Frequency band: 832-862 MHz (UL), 791-821 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand20_Supp | NR FDD FR1 Band n20 |
| 7a to 7c | Reserved |  |  |  |  |
| 7d | NR Frequency band: 1626.5-1660.5 MHz (UL), 1525-1559 MHz (DL) | 38.101-1, 5.2 | Rel-17 | pc_nrBand24_Supp | NR FDD FR1 Band n24 |
| 7e | NR Frequency band: 1850-1915 MHz (UL), 1930- 1995 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand25_Supp | NR FDD FR1 Band n25 |
| 7f | NR Frequency band: 814-849 MHz (UL), 859-894 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand26_Supp | NR FDD FR1 Band n26 |
| 7g | Reserved |  |  |  |  |
| 8 | NR Frequency band: 703-748 MHz (UL), 758-803 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand28_Supp | NR FDD FR1 Band n28 |
| 8a | Reserved |  |  |  |  |
| 8b | NR Frequency band: 2305-2315 MHz (UL), 2350-2360 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand30_Supp | NR FDD FR1 Band n30 |
| 8c | NR Frequency band: 452.5-457.5 MHz (UL), 462.5-467.5 MHz (DL) | 38.101-1, 5.2 | Rel-18 | pc_nrBand31_Supp | NR FDD FR1 Band n31 |
| 8d | Reserved |  |  |  |  |
| 8e | NR Frequency band: 1920-2010 MHz (UL),2110-2200 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand65_Supp | NR FDD FR1 Band n65 |
| 9 | NR Frequency band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand66_Supp | NR FDD FR1 Band n66 |
| 9a to 9c | Reserved |  |  |  |  |
| 10 | NR Frequency band: 1695-1710 MHz (UL), 1995-2020 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand70_Supp | NR FDD FR1 Band n70 |
| 11 | NR Frequency band: 663-698 MHz (UL), 617-652 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand71_Supp | NR FDD FR1 Band n71 |
| 12 | NR Frequency band: 451-456 MHz (UL), 461-466 MHz (DL) | 38.101-1, 5.2 | Rel-18 | pc_nrBand72_Supp | NR FDD FR1 Band n72 |
| 13 | Reserved |  |  |  |  |
| 14 | NR Frequency band: 1427-1470 MHz (UL), 1475-1518 MHz (DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand74_Supp | NR FDD FR1 Band n74 |
| 14a | NR Frequency band: 698-716 MHz (UL), 728-746 MHz (DL) | 38.101-1, 5.2 | Rel-17 | pc_nrBand85_Supp | NR FDD FR1 Band n85 |
| 15 | NR Frequency band: 832-862 MHz (UL), 1427-1432 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand91_Supp | NR FDD FR1 Band n91 |
| 16 | NR Frequency band: 832-862 MHz (UL), 1432-1517 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand92_Supp | NR FDD FR1 Band n92 |
| 17 | NR Frequency band: 880-915 MHz (UL), 1427-1432 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand93_Supp | NR FDD FR1 Band n93 |
| 18 | NR Frequency band: 880-915 MHz (UL), 1432-1517 MHz (DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand94_Supp | NR FDD FR1 Band n94 |
| 19 | NR Frequency band: 874.4-880 MHz (UL), 919.4-925 MHz (DL) | 38.101-1, 5.2 | Rel-17 | pc_nrBand100_Supp | NR FDD FR1 Band n100 |
| 20 to 23 | Reserved |  |  |  |  |
| 24 | NR Frequency band: 663 MHz-703 MHz (UL), 612 MHz-652 MHz (DL) | 38.101-1, 5.2 | Rel-18 | pc_nrBand105_Supp | NR FDD FR1 Band n105 |
| 25 | NR Frequency band: 896 MHz-901 MHz (UL), 935 MHz-940 MHz (DL) | 38.101-1, 5.2 | Rel-18 | pc_nrBand106_Supp | NR FDD FR1 Band n106 |
| 26 | NR Frequency band: 703 MHz-733 MHz (UL), 1432 MHz-1517 MHz (DL) | 38.101-1, 5.2 | Rel-18 | pc_nrBand109_Supp | NR FDD FR1 Band n109 |

<Skipped unchanged part>

Table A.4.3.1-12: NR HD-FDD RedCap UE FR1 RF Baseline Implementation Capabilities

| Item | NR FDD FR1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | NR Frequency band: 1920-1980 MHz (UL), 2110-2170 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand1_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n1 |
| 2 | NR Frequency band: 1850-1910 MHz (UL), 1930-1990 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand2_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n2 |
| 3 | NR Frequency band: 1710-1785 MHz (UL), 1805-1880 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand3_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n3 |
| 4 | NR Frequency band: 824-849 MHz (UL), 869-894 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand5_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n5 |
| 5 | NR Frequency band: 2500-2570 MHz (UL), 2620-2690 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand7_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n7 |
| 6 | NR Frequency band: 880-915 MHz (UL), 925-960 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand8_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n8 |
| 7 to 11 | Reserved |  |  |  |  |
| 12 | NR Frequency band: 699-716 MHz (UL), 729-746 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand12_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n12 |
| 13 | NR Frequency band: 777-787 MHz (UL), 746-756 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand13_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n13 |
| 14 | NR Frequency band: 788-798 MHz (UL), 758-768 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand14_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n14 |
| 15 to 17 | Reserved |  |  |  |  |
| 18 | NR Frequency band: 815-830 MHz (UL), 860-875 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand18_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n18 |
| 19 | Reserved |  |  |  |  |
| 20 | NR Frequency band: 832-862 MHz (UL), 791-821 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand20_ HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n20 |
| 21 to 23 | Reserved |  |  |  |  |
| 24 | NR Frequency band: 1626.5-1660.5 MHz (UL), 1525-1559 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand24_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n24 |
| 25 | NR Frequency band: 1850-1915 MHz (UL), 1930- 1995 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand25_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n25 |
| 26 | NR Frequency band: 814-849 MHz (UL), 859-894 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand26_HD_FDD_ Supp | NR HD-FDD RedCap UE FR1 Band n26 |
| 27 | Reserved |  | Rel-17 |  |  |
| 28 | NR Frequency band: 703-748 MHz (UL), 758-803 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand28_ HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n28 |
| 29 | Reserved |  | Rel-17 |  |  |
| 30 | NR Frequency band: 2305-2315 MHz (UL), 2350-2360 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand30_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n30 |
| 31-64 | Reserved |  |  |  |  |
| 65 | NR Frequency band: 1920-2010 MHz (UL),2110-2200 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand65_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n65 |
| 66 | NR Frequency band: 1710-1780 MHz (UL), 2110-2200 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand66_ HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n66 |
| 67 to 69 | Reserved |  |  |  |  |
| 70 | NR Frequency band: 1695-1710 MHz (UL), 1995-2020 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand70_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n70 |
| 71 | NR Frequency band: 663-698 MHz (UL), 617-652 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand71_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n71 |
| 72 | Reserved |  |  |  |  |
| 73 | Reserved |  |  |  |  |
| 74 | NR Frequency band: 1427-1470 MHz (UL), 1475-1518 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand74_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n74 |
| 75 to 84 | Reserved |  |  |  |  |
| 85 | NR Frequency band: 698-716 MHz (UL), 728-746 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand85_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n85 |
| 86 to 90 | Reserved |  |  |  |  |
| 91 | NR Frequency band: 832-862 MHz (UL), 1427-1432 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand91_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n91 |
| 92 | NR Frequency band: 832-862 MHz (UL), 1432-1517 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand92_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n92 |
| 93 | NR Frequency band: 880-915 MHz (UL), 1427-1432 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand93_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n93 |
| 94 | NR Frequency band: 880-915 MHz (UL), 1432-1517 MHz (DL) | 38.101-1, 7.3I.2 | Rel-17 | pc_nrBand94_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n94 |
| 95 to 104 | Reserved |  |  |  |  |
| 105 | NR Frequency band: 663 MHz-703 MHz (UL), 612 MHz-652 MHz (DL) | 38.101-1, 7.3I.2 | Rel-18 | pc_nrBand105_HD_FDD_Supp | NR HD-FDD RedCap UE FR1 Band n105 |

<End of changes >