3GPP TSG-RAN5 Meeting #106	R5-250394

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0774 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of RRM Measurement Capability PICS for NR NTN Rel-18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM JAPAN LLC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NR NTN Rel-18 RRM PICS are missing from 38.508-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce following new PICS for NR NTN Rel-18 - pc_meas_ntn_quasi_earth_fixed_cell - pc_meas_ntn_earth_moving_cell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the work plan will remain unfulfilled. Test applicability cannot be defined for NR-NTN Rel-18 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Sections Omitted>

### A.4.3.6	Measurement Capabilities

Table A.4.3.6-1: UE Measurement Capabilities

| Item | UE Measurement Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support NR measurements and events A triggered reporting | 38.306, 4.2.9 | Rel-15 | pc_eventA_MeasAndReport | Yes |  |  |
| 2 | Support two independent measurement gap configurations for FR1 and FR2 | 38.306, 4.2.9 | Rel-15 | pc_independentGapConfig | No |  |  |
| 3 | Support NR intra-frequency and inter-frequency measurements and at least periodical reporting | 38.306, 4.2.9 | Rel-15 | pc_intraAndInterF_MeasAndReport | Yes |  |  |
| 4 | Support CSI-RSRP and CSI-RSRQ measurement as specified in TS38.215 [21], where CSI-RS resource is configured with an associated SS/PBCH | 38.306, 4.2.9 | Rel-15 | pc_csi_RSRP_AndRSRQ_MeasWithSSB | No |  |  |
| 5 | Support inter-RAT E-UTRA measurements and events B triggered reporting | 38.306, 4.2.9 | Rel-15 | pc_eventB_MeasAndReport | Yes |  |  |
| 6 | Support SS-SINR measurents | 38.306, 4.2.9 | Rel-15 | pc_ss_SINR_Meas | No |  |  |
| 7 | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is not configured. | 38.306, 4.2.9 | Rel-15 | pc_eutra_CGI_Reporting | Yes |  |  |
| 7a | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell using autonomous gap and reporting the acquired information to the network as specified in TS 38.331 [9] when MR-DC is not configured | 38.306,4.2.9 | Rel-16 | pc_eutra_AutonomousGaps_r16 | No |  |  |
| 8 | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when EN-DC is not configured. | 38.306, 4.2.9 | Rel-15 | pc_nr_CGI_Reporting | Yes |  |  |
| 8a | Support acquisition of relevant information from a neighbouring NR cell by reading the SI of the neighbouring cell using autonomous gap and reporting the acquired information to the network as specified in TS 38.331 [9] when MR-DC is not configured. | 38.306, 4.2.9 | Rel-16 | pc_nr_AutonomousGaps_r16 | No |  |  |
| 9 | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is configured. | 38.306, 4.2.9 | Rel-15 | pc_nr_CGI_Reporting_ENDC | Yes |  |  |
| 10 | Support shorter measurement gap length (i.e. gp2 and gp3) for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC. | 36.331, 6.3.6 | Rel-15 | pc_gp2_gp3_en_dc | No |  |  |
| 11 | Support NR supports gap pattern 4 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC | 36.331, 6.3.6 | Rel-15 | pc_gp4_en_dc | No |  |  |
| 12 | Support NR supports gap pattern 5 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC | 36.331, 6.3.6 | Rel-15 | pc_gp5_en_dc | No |  |  |
| 13 | Support NR supports gap pattern 6 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC | 36.331, 6.3.6 | Rel-15 | pc_gp6_en_dc | No |  |  |
| 14 | Support NR supports gap pattern 7 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC | 36.331, 6.3.6 | Rel-15 | pc_gp7_en_dc | No |  |  |
| 15 | Support NR supports gap pattern 8 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC | 36.331, 6.3.6 | Rel-15 | pc_gp8_en_dc | No |  |  |
| 16 | Support NR supports gap pattern 9 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC | 36.331, 6.3.6 | Rel-15 | pc_gp9_en_dc | No |  |  |
| 17 | Support NR supports gap pattern 10 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC | 36.331, 6.3.6 | Rel-15 | pc_gp10_en_dc | No |  |  |
| 18 | Support NR supports gap pattern 11 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC | 36.331, 6.3.6 | Rel-15 | pc_gp11_en_dc | No |  |  |
| 19 | Support measurement gap pattern 2 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp2_nr | No |  |  |
| 20 | Support measurement gap pattern 3 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp3_nr | No |  |  |
| 21 | Support measurement gap pattern 4 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp4_nr | No |  |  |
| 22 | Support measurement gap pattern 5 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp5_nr | No |  |  |
| 23 | Support measurement gap pattern 6 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp6_nr | No |  |  |
| 24 | Support measurement gap pattern 7 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp7_nr | No |  |  |
| 25 | Support measurement gap pattern 8 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp8_nr | No |  |  |
| 26 | Support measurement gap pattern 9 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp9_nr | No |  |  |
| 27 | Support measurement gap pattern 10 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp10_nr | No |  |  |
| 28 | Support measurement gap pattern 11 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp11_nr | No |  |  |
| 29 | Support measurement gap pattern 12 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp12_nr | No |  |  |
| 30 | Support measurement gap pattern 15 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp15_nr | No |  |  |
| 31 | Support measurement gap pattern 16 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp16_nr | No |  |  |
| 32 | Support measurement gap pattern 17 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp17_nr | No |  |  |
| 34 | Support measurement gap pattern 18 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp18_nr | No |  |  |
| 35 | Support measurement gap pattern 19 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp19_nr | No |  |  |
| 36 | Support measurement gap pattern 20 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp20_nr | No |  |  |
| 37 | Support measurement gap pattern 21 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp21_nr | No |  |  |
| 38 | Support measurement gap pattern 22 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp22_nr | No |  |  |
| 39 | Support measurement gap pattern 23 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp23_nr | No |  |  |
| 40 | Support CSI-RSRP and CSI-RSRQ measurement as specified in TS38.215 [21], where CSI-RS resource is configured without an associated SS/PBCH | 38.306, 4.2.9 | Rel-15 | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB | No |  |  |
| 41 | Support CSI-RS based Radio Link Monitoring for FR1 | 38.306, 4.2.9 | Rel-15 | pc_CSI_RS_RLM_FR1 | Yes |  | If the UE supports this feature, the UE needs to report maxNumberResource-CSI-RS-RLM in its capability report. If the UE doesn’t support CSI-RS based RLM, it will not include this IE in its capability report. |
| 41a | Support CSI-RS based Radio Link Monitoring for FR2 | 38.306, 4.2.9 | Rel-15 | pc_CSI_RS_RLM_FR2 | Yes |  | If the UE supports this feature, the UE needs to report maxNumberResource-CSI-RS-RLM in its capability report. If the UE doesn’t support CSI-RS based RLM, it will not include this IE in its capability report. |
| 42 | Support of E-UTRA RS-SINR measurements | 38.306, 4.2.10 | Rel-15 | pc_RS_SINR_MeasEUTRA | No |  |  |
| 43 | Support of SFTD measurements between a E-UTRA PCell and an NR PSCell in FDD | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasPSCell_MRDC_FDD | No |  | The SFTD measurement support should be indicated in MRDC capabilities for EN-DC. The support needs to be declared for FDD and TDD separately |
| 44 | Support of SFTD measurements between a E-UTRA PCell and an NR PSCell in TDD | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasPSCell_MRDC_TDD | No |  | The SFTD measurement support should be indicated in MRDC capabilities for EN-DC. The support needs to be declared for FDD and TDD separately |
| 45 | Support of relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE | 38.306, 5.6 | Rel-16 | pc_Relaxed_Measurement | No |  |  |
| 46 | Support of SFTD measurements between a E-UTRA PCell and an NR neighbour cell in FDD | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasNR_Cell_FDD | No |  | The support needs to be declared for FDD and TDD separately The SFTD measurement support can only be indicated in MRDC capabilities for EN-DC |
| 47 | Support of SFTD measurements between a E-UTRA PCell and an NR neighbour cell in TDD | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasNR_Cell_TDD | No |  | The support needs to be declared for FDD and TDD separately The SFTD measurement support can only be indicated in MRDC capabilities for EN-DC |
| 48 | Support of SFTD measurements between a NR PCell and an NR neighbour cell in FDD | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasNR_Neigh_FDD | No |  | The support needs to be declared for FDD and TDD separately |
| 49 | Support of SFTD measurements between a NR PCell and an NR neighbour cell in TDD | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasNR_Neigh_TDD | No |  | The support needs to be declared for FDD and TDD separately |
| 50 | Support of SFTD measurements between a NR PCell and an NR PSCell in FDD | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasPSCell_NRDC_FDD | No |  | The SFTD measurement support should be indicated in UE-NR-Capability |
| 51 | Support of SFTD measurements between a NR PCell and an NR PSCell in TDD | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasPSCell_NRDC_TDD | No |  | The SFTD measurement support should be indicated in UE-NR-Capability |
| 52 | Support of acquisition of CGI related information from a neighbouring intra-frequency or inter-frequency NPN CAG cell | 38.306, 4.2.9 | Rel-16 | pc_nr_CGI_Reporting_NPN_r16 | No |  |  |
| 53 | Supports periodic EUTRA measurement and reporting. | 38.306, 4.2.9 | Rel-15 | pc_periodicEUTRA_MeasAndReport | Yes |  |  |
| 54 | Support configuration of NR SSB measurements in RRC_IDLE/RRC_INACTIVE and reporting of the corresponding results upon network request as specified in TS 38.331 [9] | 38.306, 4.2.9 | Rel-16 | pc_idleInactiveNR_MeasReport | No |  |  |
| 55 | Support configuration of E-UTRA measurements in RRC_IDLE/RRC_INACTIVE and reporting of the corresponding results upon network request as specified in TS 38.331 [9] | 38.306, 4.2.9 | Rel-16 | pc_idleInactiveEUTRA_MeasReport | No |  |  |
| 56 | Support SRS-RSRP measurements between a NR Pcell and an interfering UE, upon network request as specified in 38.331 [9] | 38.306, 4.2.9 | Rel-16 | pc_nr_CLI_Reporting_r16 | No |  | If the UE supports this feature, the UE needs to report maxNumberCLI-SRS-RSRP-r16 and maxNumberPerSlotCLI-SRS-RSRP-r16 If the UE doesn’t support CLI SRS-RSRP measurement, it will not include this IE in its capability report. |
| 56A | Support SRS-RSRP measurements and periodical reporting and measurement event triggering based on SRS-RSRP | 38.306, 4.2.9 | Rel-16 | pc_cli_SRS_RSRP_Meas_r16 | No |  | If the UE supports this feature, the UE needs to report maxNumberCLI-SRS-RSRP-r16 and maxNumberPerSlotCLI-SRS-RSRP-r16 |
| 56B | Support CLI RSSI measurements and periodical reporting and measurement event triggering | 38.306, 4.2.9 | Rel-16 | pc_cli_RSSI_Meas_r16 | No |  | If the UE supports this feature, the UE needs to report maxNumberCLI-RSSI-r16 |
| 57 | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NE-DC is configured. | 38.306, 4.2.9 | Rel-15 | pc_eutra_CGI_Reporting_NEDC | No |  |  |
| 58 | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NR-DC is configured wherein MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if the DRX cycles are the same. | 38.306, 4.2.9 | Rel-15 | pc_eutra_CGI_Reporting_NRDC | No |  |  |
| 59 | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NE-DC is configured. | 38.306, 4.2.9 | Rel-15 | pc_nr_CGI_Reporting_NEDC | Yes |  |  |
| 60 | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NR-DC is configured wherein MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if the DRX cycles are the same. | 38.306, 4.2.9 | Rel-15 | pc_nr_CGI_Reporting_NRDC | Yes |  |  |
| 61 | Supports performing eNB-configured SSB-based RRM measurements for EN-DC configured NR FR1 carrier(s) in RRC_IDLE and reporting them when requested by the eNB while resuming from RRC_IDLE or in RRC_CONNECTED, as specified in TS 36.331 [5]. | 36.306, 4.3.6.41 | Rel-16 | pc_nrIdleInactiveNRFR1_MeasReport | No |  |  |
| 62 | Supports performing eNB-configured SSB-based RRM measurements for EN-DC configured NR FR2 carrier(s) in RRC_IDLE and reporting them when requested by the eNB while resuming from RRC_IDLE or in RRC_CONNECTED, as specified in TS 36.331 [5]. | 36.306, 4.3.6.42 | Rel-16 | pc_nrIdleInactiveNRFR2_MeasReport | No |  |  |
| 63 | Support more than 1 per-UE measurement gap configurations. | 38.306, 4.2.9 | Rel-17 | pc_concurrentPerUE_OnlyMeasGap_r17 | No |  |  |
| 64 | Support all concurrent gap combination configurations as specified in TS 38.133 [5] including support of more than 1 per-UE measurement gap configurations. | 38.306, 4.2.9 | Rel-17 | pc_concurrentPerUE_PerFRCombMeasGap_r17 | No |  |  |
| 65 | Support the configurations of E-UTRAN measurement objectives associated with more than 1 concurrent measurement gaps. | 38.306, 4.2.9 | Rel-17 | pc_concurrentMeasGapEUTRA_r17 | No |  |  |
| 66 | Support reporting of the NCSG and measurement gap requirement information for E-UTRA target bands in the UE response to a network configuration RRC message as specified in TS 38.331 [9]. | 38.306, 4.2.9 | Rel-17 | pc_eutra_NeedForGapNCSG_Reporting_r17 | No |  |  |
| 67 | Support two independent measurement gap configurations for FR1 and FR2 for PRS measurement. | 38.306, 4.2.9 | Rel-17 | pc_independentGapConfigPRS_r17 | No |  |  |
| 68 | Support NR-only NCSG patterns. | 38.306, 4.2.9 | Rel-17 | pc_ncsg_MeasGapNR_Patterns_r17 | No |  |  |
| 69 | Support NCSG patterns. | 38.306, 4.2.9 | Rel-17 | pc_ncsg_MeasGapPatterns_r17 | No |  |  |
| 70 | Support per-FR NCSG. | 38.306, 4.2.9 | Rel-17 | pc_ncsg_MeasGapPerFR_r17 | No |  |  |
| 71 | Support performing measurement with NCSG based on flag deriveSSB-IndexFromCell-inter and meeting the following requirements that the scheduling restriction in FR2 serving cell during NCSG ML is on SSB symbol level. | 38.306, 4.2.9 | Rel-17 | pc_ncsg_SymbolLevelScheduleRestrictionInter_r17 | No |  |  |
| 72 | Support reporting of the NCSG and measurement gap requirement information for SSB based measurement in the UE response to a network configuration RRC message. | 38.306, 4.2.9 | Rel-17 | pc_nr_NeedForGapNCSG_Reporting_r17 | No |  |  |
| 73 | Support the preconfigured measurement gap with UE-autonomous mechanism for activation and deactivation. | 38.306, 4.2.9 | Rel-17 | pc_preconfiguredUE_AutonomousMeasGap_r17 | No |  |  |
| 74 | Support the preconfigured measurement gap with network-controlled mechanism for activation and deactivation. | 38.306, 4.2.9 | Rel-17 | pc_preconfiguredNW_ControlledMeasGap_r17 | No |  |  |
| 75 | Support of SFTD measurements between the NR PCell and a configured E-UTRA PSCell | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasPSCell_NEDC | No |  |  |
| 76 | Support of location-based triggered measurement reporting (i.e., event D1) as specified in TS 38.331 [9]. | 38.306, 4.2.9 | Rel-17 | pc_eventD1_MeasReportTrigger_r17 | CY |  | If UE supports locationBasedCondHandover-r17 in any NTN band then UE needs to support this feature. |
| 77 | Support of Re-17 relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE | 38.306, 5.6 | Rel-17 | pc_Relaxed_Measurement_r17 | No |  |  |
| 78 | Support of Rel-17 BFD relaxation criteria | 38.306, 4.2.7.2 | Rel-17 | pc_bfd_Relaxation_r17 | No |  |  |
| 79 | Support of Rel-17 RLM relaxation criteria | 38.306, 4.2.7.2 | Rel-17 | pc_rlm_Relaxation_r17 | No |  |  |
| 80 | Indicates the support of simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA. If this field is included in ca-ParametersNR-ForDC, it indicates the UE supports simultaneous transmission and reception between any UL/DL band pair within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC. | 38.306, 4.2.7 | Rel-17 | pc_simultaneous_RxTx_InterBandCA | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |
| 81 | Indicates whether the UE can perform inter-frequency SSB based measurements without measurement gaps if the SSB is completely contained in the active BWP of the UE as specified in TS 38.133 [5]. If this parameter is indicated for FR1 and FR2 differently, each indication corresponds to the frequency range of cells to be measured. | 38.306, 4.2.9 | Rel-16 | pc_interFrequencyMeas_NoGap_r16 | No |  |  |
| 82 | Indicates whether the UE supports 2 parallel measurement gaps for NTN SSB based RRM measurements | 38.306, 4.2.9 | Rel-17 | pc_parallelMeasGap_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 83 | Indicates whether the UE supports Time-based measurement initiation for NTN while in RRC_IDLE/RRC_INACTIVE | 38.306, 5.6 | Rel-17 | pc_timeBasedMeasInit_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 84 | Indicates whether the UE supports Location-based measurement initiation for NTN while in RRC_IDLE/RRC_INACTIVE | 38.306, 5.6 | Rel-17 | pc_locationBasedMeasInit_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 85 | Indicates whether the UE supports enhanced RRM requirements for measurements in NTN bands while in RRC_IDLE/RRC_INACTIVE | 38.306, 5.6 | Rel-17 | pc_enhRRMreqMeas_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 86 | Void |  |  |  |  |  |  |
| 87 | Indicates whether the UE supports measurement of 1 LEO satellite in parallel within an SMTC for NTN (maxNumber-NGSO-SatellitesWithinOneSMTC-r17) | 38.306, 4.2.7.2 | Rel-17 | pc_maxNumNGSOsatPerSMTC_1_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 88 | Indicates whether the UE supports measurement of 2 LEO satellite in parallel within an SMTC for NTN (maxNumber-NGSO-SatellitesWithinOneSMTC-r17) | 38.306, 4.2.7.2 | Rel-17 | pc_maxNumNGSOsatPerSMTC_2_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 89 | Indicates whether the UE supports measurement of 3 LEO satellite in parallel within an SMTC for NTN (maxNumber-NGSO-SatellitesWithinOneSMTC-r17) | 38.306, 4.2.7.2 | Rel-17 | pc_maxNumNGSOsatPerSMTC_3_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 90 | Indicates whether the UE supports measurement of 4 LEO satellite in parallel within an SMTC for NTN (maxNumber-NGSO-SatellitesWithinOneSMTC-r17) | 38.306, 4.2.7.2 | Rel-17 | pc_maxNumNGSOsatPerSMTC_4_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 91 | Indicates whether the UE supports inter-satellite measurements for NTN (interSatMeas-r17) | 38.306, 4.2.9 | Rel-17 | pc_interSatMeas_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 92 | Support of measurement gap pattern 13 configured by NR RRC when in (NG)EN-DC | 38.306, 4.2.9 | Rel-15 | pc_gp13_en_dc | No |  |  |
| 93 | Support of measurement gap pattern 14 configured by NR RRC when in (NG)EN-DC | 38.306, 4.2.9 | Rel-15 | pc_gp14_en_dc | No |  |  |
| 94 | Support reduced TLTM_processing for cell switch from FR1 to FR1 | 38.306, 4.2.9 | Rel-18 | pc_ltm_FastUE_Processing_fr1_r18 | No |  |  |
| 95 | Support reduced TLTM_processing for cell switch from FR2 to FR2 | 38.306, 4.2.9 | Rel-18 | pc_ltm_FastUE_Processing_fr2_r18 | No |  |  |
| 96 | Support reduced TLTM_processing for cell switch from FR1/FR2 to FR2/FR1 | 38.306, 4.2.9 | Rel-18 | pc_ltm_FastUE_Processing_fr1_AndFR2_r18 | No |  |  |
| 97 | Support SSB based inter-frequency L1-RSRP measurements with measurement gaps for LTM | 38.306, 4.2.9 | Rel-18 | pc_ltm_InterFreqMeasGap_r18 | No |  | A UE supporting this feature shall also indicate support of interFreqL1-MeasConfig-r18 (pc_interFreqL1_MeasConfig_r18). |
| 98 | Support LTM for MCG with RACH with NR-DC configured | 38.306, 4.2.9 | Rel-18 | pc_ltm_MCG_NRDC_r18 | No |  | A UE indicating support for this feature shall also indicate support of ltm-MCG-IntraFreq-r18 (pc_ltm_MCG_IntraFreq_r18). |
| 99 | Support LTM for MCG with the release of NR-DC configuration as part of LTM execution when LTM cell switch command MAC CE is received | 38.306, 4.2.9 | Rel-18 | pc_ltm_MCG_NRDC_Release_r18 | No |  | A UE indicating support for this feature shall also indicate support of ltm-MCG-IntraFreq-r18 (pc_ltm_MCG_IntraFreq_r18). |
| 100 | Support inter-frequency MCG LTM on all the bands where the UE indicates support of ltm-MCG-IntraFreq-r18 or inter-frequency SCG LTM on all the bands where the UE indicates support of ltm-SCG-IntraFreq-r18 respectively | 38.306, 4.2.9 | Rel-18 | pc_ltm_InterFreq_r18 | No |  | A UE supporting this feature shall also indicate support of ltm-MCG-IntraFreq-r18 (pc_ltm_MCG_IntraFreq_r18) or ltm-SCG-IntraFreq-r18 (pc_ltm_SCG_IntraFreq_r18). |
| 101 | Support RACH-less LTM with configured grant for MCG LTM if the UE indicates support of ltm-MCG-IntraFreq-r18 or for SCG LTM if the UE indicates support of ltm-SCG-IntraFreq-r18 respectively | 38.306, 4.2.9 | Rel-18 | pc_ltm_RACH_LessCG_r18 | No |  | A UE indicating support for this feature shall also indicate support of either ltm-BeamIndicationJointTCI-r18 (pc_ltm_BeamIndicationJointTCI_r18) or ltm-BeamIndicationSeparateTCI-r18 (pc_ltm_BeamIndicationSeparateTCI_r18) for at least one band and either ta-IndicationCellSwitch-r18 (pc_ta_IndicationCellSwitch_r18) or ue-TA-Measurement-r18 (pc_ue_TA_Measurement_r18). |
| 102 | Support RACH-Less LTM with dynamic grant, for MCG LTM if the UE indicates support of ltm-MCG-IntraFreq-r18 or for SCG LTM if the UE indicates support of ltm-SCG-IntraFreq-r18 respectively | 38.306, 4.2.9 | Rel-18 | pc_ltm_RACH_LessDG_r18 | No |  | A UE indicating support for this feature shall also indicate support of either ltm-BeamIndicationJointTCI-r18 (pc_ltm_BeamIndicationJointTCI_r18) or ltm-BeamIndicationSeparateTCI-r18 (pc_ltm_BeamIndicationSeparateTCI_r18) for at least one band and TA indication in ta-IndicationCellSwitch-r18 (pc_ta_IndicationCellSwitch_r18) or ue-TA-Measurement-r18 (pc_ue_TA_Measurement_r18). |
| 103 | Support recovery procedure for MCG LTM execution when the selected cell in RRC re-establishment procedure is a LTM candidate | 38.306, 4.2.9 | Rel-18 | pc_ltm_Recovery_r18 | No |  | A UE indicating support for this feature shall also indicate support of ltm-MCG-IntraFreq-r18 (pc_ltm_MCG_IntraFreq_r18) for at least one band. |
| 104 | Support a reference configuration for LTM | 38.306, 4.2.9 | Rel-18 | pc_ltm_ReferenceConfig_r18 | No |  | A UE indicating support for this feature shall also indicate support of either ltm-MCG-IntraFreq-r18 (pc_ltm_MCG_IntraFreq_r18) or ltm-SCG-IntraFreq-r18 (pc_ltm_SCG_IntraFreq_r18) for at least one band. |
| 105 | Support reporting the measurement gap requirement information for NR target in the UE response to a network configuration RRC message. | 38.306, 4.2.9 | Rel-16 | pc_nr_NeedForGap_Reporting_r16 | No |  |  |
| 106 | Support reporting the interruption requirement information for SSB based measurement towards NR target without gap in the UE response to a network configuration RRC message | 38.306, 4.2.9 | Rel-18 | pc_nr_NeedForInterruptionReport_r18 | No |  | A UE supporting this feature shall also indicate support of nr-NeedForGap-Reporting-r16 (pc_nr_NeedForGap_Reporting_r16). |
| 107 | Support concurrent inter-RAT measurement on EUTRAN cell in non-DSS and PDCCH or PDSCH reception from the serving cell with a different numerology | 38.306, 4.2.9 | Rel-18 | pc_concurrentMeasCRS_InsideBWP_EUTRA_r18 | No |  | A UE supporting this feature shall also indicate support of eutra-NoGapMeasurementInsideBWP-r18 (pc_eutra_NoGapMeasurementInsideBWP_r18) or eutra-NoGapMeasurementOutsideBWP-r18 (pc_eutra_NoGapMeasurementOutsideBWP_r18). |
| 108 | Support multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG as specified in TS 38.133 | 38.306, 4.2.9 | Rel-18 | pc_concurrentMeasGapsNCSG_r18 | No |  | A UE supporting this feature shall also indicate support of nr-NeedForGapNCSG-Reporting-r17 (pc_nr_NeedForGapNCSG_Reporting_r17) and concurrentMeasGap-r17. |
| 109 | Support multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG as specified in TS 38.133 | 38.306, 4.2.9 | Rel-18 | pc_concurrentMeasGapsPreMG_r18 | No |  | A UE supporting this feature shall also indicate support of concurrentMeasGap-r17 and one of preconfiguredNW-ControlledMeasGap-r17 (pc_preconfiguredNW_ControlledMeasGap_r17) and preconfiguredUE-AutonomousMeasGap-r17 (pc_preconfiguredUE_AutonomousMeasGap_r17). |
| 110 | Support RRM requirements for handling dynamic collisions between a Pre-MG and another measurement gap or Pre-MG | 38.306, 4.2.9 | Rel-18 | pc_dynamicCollision_r18 | No |  | A UE supporting this feature shall also indicate support of concurrentMeasGapsPreMG-r18 (pc_concurrentMeasGapsPreMG_r18). |
| 111 | Support configuration of effective measurement window for inter-RAT EUTRAN measurements, including offset, duration and periodicity | 38.306, 4.2.9 | Rel-18 | pc_eutra_MeasEMW_r18 | No |  | A UE supporting this feature shall also indicate support of eutra-NoGapMeasurementOutsideBWP-r18 (pc_eutra_NoGapMeasurementOutsideBWP_r18) or eutra-NoGapMeasurementInsideBWP-r18 (pc_eutra_NoGapMeasurementInsideBWP_r18). |
| 112 | Support inter-RAT EUTRAN measurements without gap when CRS is completely contained within UE's active DL BWP | 38.306, 4.2.9 | Rel-18 | pc_eutra_NoGapMeasurementInsideBWP_r18 | No |  |  |
| 113 | Support inter-RAT EUTRAN measurements outside active DL BWP for nogap-noncsg | 38.306, 4.2.9 | Rel-18 | pc_eutra_NoGapMeasurementOutsideBWP_r18 | No |  | A UE supporting this feature shall also indicate support of eutra-NeedForGapNCSG-Reporting-r17 (pc_eutra_NeedForGapNCSG_Reporting_r17). |
| 114 | Indicates measurement gap pattern(s) optionally supported by the UE for NR SA and NR-DC when the frequencies to be measured within this measurement gap are all NR frequencies | 38.306, 4.2.9 | Rel-16 | pc_meas_gap_nr_only_r16 | No |  | The UE shall set the bits corresponding to the measurement gap pattern 2, 3 and 11 to 1 |
| xx1 | Support location based RRM measurements of neighbour cells in NTN quasi-Earth fixed cell in RRC_IDLE/RRC_INACTIVE | 38.306, 5.6 | Rel-18 | pc_meas_ntn_quasi_earth_fixed_cell | No |  | If a UE supports this feature, then it supports r18 NTN Enhanced |
| xx2 | Support location based RRM measurements of neighbour cells in NTN Earth-moving cell in RRC_IDLE/RRC_INACTIVE | 38.306, 5.6 | Rel-18 | pc_meas_ntn_earth_moving_cell | No |  | If a UE supports this feature, then it supports r18 NTN Enhanced |

<Unchanged Sections Omitted>