3GPP TSG-RAN5 Meeting #106	R5-251262

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4837 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.1.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM JAPAN LLC, Anristu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The TDD-UL-DL-ConfigCommon patterns defined for FR2 in Table 7.1.1.2.1.3.3-5 (to be active from step 19a1 onwards) conflict with the SchedulingRequestResourceConfig defined in Table 7.1.1.2.1.3.3-12 for SCS=120KHz. The 5th slot with such TDD-UL-DL-ConfigCommon is DL slot, therefore cannot be used for SR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | We propose to use SR slot offset=4 (instead of 5) in case of SCS=120kHz. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE cannot transmit SR after step 19a1 in FR2 version of this test thus will fail incorrectly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Final version of R5-250392 |  |  |  |  |  |  |  |  |

<Start of modified section>

##### 7.1.1.2.1	Correct Handling of DL MAC PDU / Assignment / HARQ process

7.1.1.2.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives downlink assignment on the PDCCH for the UE’s C-RNTI and receives data in the associated Slot and UE performs HARQ operation }

then { UE sends a HARQ feedback on the HARQ process }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { SS transmits downlink assignment on the PDCCH with a C-RNTI unknown by the UE and data is available in the associated Slot }

then { UE does not send any HARQ feedback on the HARQ process }

}

(3)

with { UE in  RRC_CONNECTED state }

ensure that {

when { the UE receives a MAC PDU addressed to its C-RNTI and decode fails in the associated Slot }

then { the UE transmits a NACK for the corresponding HARQ process }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { the UE receives a MAC PDU retransmission addressed to its C-RNTI, and results in successful decode in the associated Slot }

then { the UE transmits an ACK for the corresponding HARQ process and forward to higher layer }

}

(5)

with { UE in  RRC_CONNECTED state }

ensure that {

when { UE receives a MAC PDU containing multiple MAC sub PDUs each containing a MAC SDU that is larger than 256 bytes (16 bits L field used) with padding MAC sub PDU at the end }

then { UE successfully decodes the MAC PDU and forward to higher layer }

}

(6)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a MAC PDU containing multiple MAC sub PDUs each containing a MAC SDU that is smaller than 256 bytes (8 bits L field used) with padding MAC sub PDU at the end }

then { UE successfully decodes the MAC PDU and forward to higher layer }

}

(7)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a MAC PDU containing MAC sub PDU containing a MAC SDU and no padding MAC sub PDU }

then { UE successfully decodes the MAC PDU and forward to higher layer }

}

(8)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a MAC PDU containing MAC sub PDU containing a MAC SDU that is smaller than 256 bytes (8 bits L field used) plus MAC sub PDU containing a MAC SDU that is greater than 256 bytes (16 bits L field used)and no padding }

then { UE successfully decodes the MAC PDU and forwards the AMD PDUs to higher layer }

}

(9)

with { UE in RRC_CONNECTED state and configured with a specific TDD-UL-DL-ConfigCommon including configuration of pattern2 }

ensure that {

when { UE receives downlink assignment on the PDCCH associated with pattern2 for the UE’s C-RNTI and receives data in the associated Slot and UE performs HARQ operation }

then { UE sends a HARQ feedback on the HARQ process }

}

7.1.1.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.321, clauses 5.3.1, 5.3.2.1, 5.3.2.2 and 6.1.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.321, clause 5.3.1]

Downlink assignments received on the PDCCH both indicate that there is a transmission on a DL-SCH for a particular MAC entity and provide the relevant HARQ information.

When the MAC entity has a C-RNTI, Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion during which it monitors PDCCH and for each Serving Cell:

1>	if a downlink assignment for this PDCCH occasion and this Serving Cell has been received on the PDCCH for the MAC entity’s C-RNTI, or Temporary C-RNTI:

2>	if this is the first downlink assignment for this Temporary C-RNTI:

3>	consider the NDI to have been toggled.

2>	if the downlink assignment is for the MAC entity’s C-RNTI, and if the previous downlink assignment indicated to the HARQ entity of the same HARQ process was either a downlink assignment received for the MAC entity’s CS-RNTI or a configured downlink assignment:

3>	consider the NDI to have been toggled regardless of the value of the NDI.

2>	indicate the presence of a downlink assignment and deliver the associated HARQ information to the HARQ entity.

1>	else if a downlink assignment for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity’s CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	indicate the presence of a downlink assignment for this Serving Cell and deliver the associated HARQ information to the HARQ entity.

2>	if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate SPS deactivation:

4>	clear the configured downlink assignment for this Serving Cell (if any);

4>	if the timeAlignmentTimer associated with the PTAG is running:

5>	indicate a positive acknowledgement for the SPS deactivation to the physical layer.

3>	else if PDCCH content indicates SPS activation:

4>	store the downlink assignment for this Serving Cell and the associated HARQ information as configured downlink assignment;

4>	initialise or re-initialise the configured downlink assignment for this Serving Cell to start in the associated PDSCH duration and to recur according to rules in subclause 5.8.1;

4>	set the HARQ Process ID to the HARQ Process ID associated with this PDSCH duration;

4>	consider the NDI bit for the corresponding HARQ process to have been toggled;

4>	indicate the presence of a configured downlink assignment for this Serving Cell and deliver the stored HARQ information to the HARQ entity.

For each Serving Cell and each configured downlink assignment, if configured and activated, the MAC entity shall:

1>	if the PDSCH duration of the configured downlink assignment does not overlap with the PDSCH duration of a downlink assignment received on the PDCCH for this Serving Cell:

2>	instruct the physical layer to receive, in this PDSCH duration, transport block on the DL-SCH according to the configured downlink assignment and to deliver it to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with this PDSCH duration;

2>	consider the NDI bit to have been toggled;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity.

For configured downlink assignments, the HARQ Process ID associated with the slot where the DL transmission starts is derived from the following equation:

HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame × semiPersistSchedIntervalDL))] modulo nrofHARQ-Processes

where CURRENT_slot = [(SFN × numberOfSlotsPerFrame) + slot number in the frame] and numberOfSlotsPerFrame refers to the number of consecutive slots per frame as specified in TS 38.211 [8].

When the MAC entity needs to read BCCH, the MAC entity may, based on the scheduling information from RRC:

1>	if a downlink assignment for this PDCCH occasion has been received on the PDCCH for the SI-RNTI;

2>	indicate a downlink assignment and redundancy version for the dedicated broadcast HARQ process to the HARQ entity.

[TS 38.321, clause 5.3.2.2]

When a transmission takes place for the HARQ process, one or more (in case of downlink spatial multiplexing) TBs and the associated HARQ information are received from the HARQ entity.

For each received TB and associated HARQ information, the HARQ process shall:

1>	if the NDI, when provided, has been toggled compared to the value of the previous received transmission corresponding to this TB; or

1>	if the HARQ process is equal to the broadcast process, and this is the first received transmission for the TB according to the system information schedule indicated by RRC; or

1>	if this is the very first received transmission for this TB (i.e. there is no previous NDI for this TB):

2>	consider this transmission to be a new transmission.

1>	else:

2>	consider this transmission to be a retransmission.

The MAC entity then shall:

1>	if this is a new transmission:

2>	attempt to decode the received data.

1>	else if this is a retransmission:

2>	if the data for this TB has not yet been successfully decoded:

3>	instruct the physical layer to combine the received data with the data currently in the soft buffer for this TB and attempt to decode the combined data.

1>	if the data which the MAC entity attempted to decode was successfully decoded for this TB; or

1>	if the data for this TB was successfully decoded before:

2>	if the HARQ process is equal to the broadcast process:

3>	deliver the decoded MAC PDU to upper layers.

2>	else if this is the first successful decoding of the data for this TB:

3>	deliver the decoded MAC PDU to the disassembly and demultiplexing entity.

1>	else:

2>	instruct the physical layer to replace the data in the soft buffer for this TB with the data which the MAC entity attempted to decode;

1>	if the HARQ process is associated with a transmission indicated with a Temporary C-RNTI and the Contention Resolution is not yet successful (see subclause 5.1.5); or

1>	if the HARQ process is equal to the broadcast process; or

1>	if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is stopped or expired:

2>	not instruct the physical layer to generate acknowledgement(s) of the data in this TB.

1>	else:

2>	instruct the physical layer to generate acknowledgement(s) of the data in this TB.

The MAC entity shall ignore NDI received in all downlink assignments on PDCCH for its Temporary C-RNTI when determining if NDI on PDCCH for its C-RNTI has been toggled compared to the value in the previous transmission.

[TS 38.321, clause 6.1.2]

A MAC PDU consists of one or more MAC subPDUs. Each MAC subPDU consists of one of the following:

-	A MAC subheader only (including padding);

-	A MAC subheader and a MAC SDU;

-	A MAC subheader and a MAC CE;

-	A MAC subheader and padding.

The MAC SDUs are of variable sizes.

Each MAC subheader corresponds to either a MAC SDU, a MAC CE, or padding.

A MAC subheader except for fixed sized MAC CE and padding consists of the four header fields R/F/LCID/L. A MAC subheader for fixed sized MAC CE and padding consists of the two header fields R/LCID.

Figure 6.1.2-1: R/F/LCID/L MAC subheader with 8-bit L field

Figure 6.1.2-2: R/F/LCID/L MAC subheader with 16-bit L field

Figure 6.1.2-3: R/LCID MAC subheader

MAC CEs are placed together. DL MAC subPDU(s) with MAC CE(s) is placed before any MAC subPDU with MAC SDU and MAC subPDU with padding as depicted in Figure 6.1.2-4. UL MAC subPDU(s) with MAC CE(s) is placed after all the MAC subPDU(s) with MAC SDU and before the MAC subPDU with padding in the MAC PDU as depicted in Figure 6.1.2-5. The size of padding can be zero.

Figure 6.1.2-4: Example of a DL MAC PDU

Figure 6.1.2-5: Example of a UL MAC PDU

A maximum of one MAC PDU can be transmitted per TB per MAC entity.

7.1.1.2.1.3	Test description

7.1.1.2.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that set to return no data in uplink and parameters as in Table 7.1.1.2.1.3.1-1.

Table 7.1.1.2.1.3.1-1: MAC Parameters

| nrofHARQ-ProcessesForPDSCH | n16 |
| --- | --- |

7.1.1.2.1.3.2	Test procedure sequence

Table 7.1.1.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE | <-- | (PDCCH (C-RNTI)) | - | - |
| 2 | SS transmits in the indicated downlink assignment a MAC PDU including a RLC PDU with poll bit not set. | <-- | MAC PDU | - | - |
| 3 | Check: Does the UE transmit an HARQ ACK on PUCCH? | --> | HARQ ACK | 1 | P |
| 4 | SS transmits a downlink assignment to including a C-RNTI different from the assigned to the UE | <-- | (PDCCH (unknown C-RNTI)) | - | - |
| 5 | SS transmits in the indicated downlink assignment a RLC PDU in a MAC PDU including a RLC PDU with poll bit not set. | <-- | MAC PDU | - | - |
| 6 | Check: Does the UE send any HARQ ACK/NACK on PUCCH? | --> | HARQ ACK/NACK | 2 | F |
| - | EXCEPTION: Steps 7 to 10 are run repeated using test parameter values as given for each iteration in table 7.1.1.2.1.3.2.-2. | - | - | - | - |
| 7 | The SS indicates a new transmission on PDCCH and transmits a MAC PDU including a RLC PDU with poll bit not set, with content set so that UE could not successfully decode the data from its soft buffer. (Note 1) | <-- | MAC PDU | - | - |
| 8 | Check: Does the UE transmit a HARQ NACK? | --> | HARQ NACK | 3 | P |
| - | EXCEPTION: Step 9 shall be repeated till HARQ ACK is received at step 10 or until HARQ retransmission count = 4 is reached for MAC PDU at step 9 (Note 2). | - | - | - | - |
| 9 | The SS indicates a retransmission on PDCCH and transmits the same MAC PDU like step 7 (Note 1). | <-- | MAC PDU | - | - |
| - | EXCEPTION: Up to [3] HARQ NACK from the UE should be allowed at step 10 (Note 2). | - | - | - | - |
| 10 | Check: Does the UE send a HARQ ACK? | --> | HARQ ACK | 4 | P |
| 11 | The SS transmits a MAC PDU containing three MAC sub PDUs each containing a MAC SDU(RLC PDU) that is of 260 bytes (16 bits L field used) and a padding MAC sub PDU at the end. The third RLC PDU contained will have poll bit set. | <-- | MAC PDU | - | - |
| 12 | Check: Does the UE transmit a MAC PDU containing an RLC STATUS PDU acknowledging the reception of all the AMD PDUs in step 11? | --> | MAC PDU (RLC STATUS PDU ) | 5 | P |
| 13 | The SS transmits a MAC PDU containing three MAC sub PDUs each containing a MAC SDU(RLC PDU) that is of 128 bytes (8 bits L field used) and a padding MAC sub PDU at the end. The third RLC PDU contained will have poll bit set. | <-- | MAC PDU | - | - |
| 14 | Check: Does the UE transmit a MAC PDU containing an RLC STATUS PDU acknowledging the reception of all the AMD PDUs in step 13? | --> | MAC PDU (RLC STATUS PDU ) | 6 | P |
| 15 | The SS transmits a MAC PDU containing one MAC sub PDU containing a MAC SDU(RLC PDU) that is of [128] bytes (8 bits L field used) and no padding MAC sub PDU at the end. The RLC PDU contained will have poll bit set. | <-- | MAC PDU | - | - |
| 16 | Check: Does the UE transmit a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 15? | --> | MAC PDU (RLC STATUS PDU ) | 7 | P |
| 17 | The SS transmits a MAC PDU containing one MAC sub PDU containing a MAC SDU(RLC PDU) that is of [128] bytes (8 bits L field used), one MAC sub PDU containing a MAC SDU(RLC PDU) that is of [260] bytes (16 bits L field used) and no padding MAC sub PDU at the end. The second RLC PDU contained will have poll bit set. | <-- | MAC PDU | - | - |
| 18 | Check: Does the UE transmit a MAC PDU containing an RLC STATUS PDU acknowledging the reception of all the AMD PDUs in step 17? | --> | MAC PDU (RLC STATUS PDU ) | 8 | P |
| - | EXCEPTION : Steps 19a0 to 19a5 are executed for operation on NR TDD band only | - | - | - | - |
| 19a0 | The SS transmits an updated system information as specified in Table 7.1.1.3.1.3.3-14. (Note 5) | - | - | - | - |
| 19a1 | The SS transmits NR RRCReconfiguration message including TDD-UL-DL-ConfigCommon with pattern1 and pattern2 specified in Table 7.1.1.2.1.3.3-5 (Note 3) | <-- | (RRCReconfiguration) | - | - |
| 19a2 | The UE transmits a NR RRCReconfigurationComplete message. (Note 4) | --> | (RRCReconfigurationComplete) | - | - |
| 19a3 | SS transmits a downlink assignment addressed to the C-RNTI assigned to the UE indicating downlink reception in a symbol in a slot part of pattern2. | <-- | (PDCCH (C-RNTI)) | - | - |
| 19a4 | SS transmits in the indicated downlink assignment a MAC PDU including a RLC PDU with poll bit not set. | <-- | MAC PDU | - | - |
| 19a5 | Check: Does the UE transmit an HARQ ACK on PUCCH? | --> | HARQ ACK | 9 | P |
| Note 1: SS should transmit this PDU so as to ensure at least one NACK. Note 2: The value 4 for the maximum number of HARQ retransmissions has been chosen based on an assumption that, given the radio conditions used in this test case, a UE soft combiner implementation should have sufficient retransmissions to be able to successfully decode the data in its soft buffer. Note 3: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration 36.508 [7], Table 4.6.1-8 using condition EN-DC_EmbedNR_RRCRecon. Note 4: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 5: if pc_NG_RAN_NR only |  |  |  |  |  |

Table 7.1.1.2.1.3.2-2: Test Parameters

| Iteration | DL HARQ process (X) |
| --- | --- |
| K=1 to 16 | X=K-1 |

7.1.1.2.1.3.3	Specific message contents

Table 7.1.1.2.1.3.3-1: Void

Table 7.1.1.2.1.3.3-2: RRCReconfiguration (step 19a1, Table 7.1.1.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  | NR |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.2.1.3.3-3: CellGroupConfig (Table 7.1.1.2.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon | ServingCellConfigCommon |  |  |
| } |  |  |  |
| spCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.2.1.3.3-4: ServingCellConfigCommon (Tables 7.1.1.2.1.3.3-3 and 7.1.1.2.1.3.3-13)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| uplinkConfigCommon SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkCommon |  |  |
| } |  |  |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  |  |
| } |  |  |  |

Table 7.1.1.2.1.3.3-5: TDD-UL-DL-ConfigCommon (Table 7.1.1.2.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-192 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| referenceSubcarrierSpacing | SubcarrierSpacing |  |  |
| pattern1 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms5 |  | FR1 |
|  | ms0p625 |  | FR2 |
| nrofDownlinkSlots | 3 |  | FR1 |
|  | 2 |  | FR2 |
| nrofDownlinkSymbols | 6 |  | FR1 |
|  | 6 |  | FR2 |
| nrofUplinkSlots | 2 |  | FR1 |
|  | 2 |  | FR2 |
| nrofUplinkSymbols | 4 |  | FR1 |
|  | 2 |  | FR2 |
| dl-UL-TransmissionPeriodicity-v1530 | ms3 |  | FR1 |
| } |  |  |  |
| pattern2 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms2 |  | FR1 |
|  | ms0p625 |  | FR2 |
| nrofDownlinkSlots | 4 |  | FR1 |
|  | 3 |  | FR2 |
| nrofDownlinkSymbols | 0 |  | FR1 |
|  | 6 |  | FR2 |
| nrofUplinkSlots | 0 |  | FR1 |
|  | 1 |  | FR2 |
| nrofUplinkSymbols | 0 |  | FR1 |
|  | 2 |  | FR2 |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.2.1.3.3-6: BWP-UplinkCommon (Table 7.1.1.2.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup | RACH-ConfigCommon |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.2.1.3.3-7: RACH-ConfigCommon (Table 7.1.1.2.1.3.3-6)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon ::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| } |  |  |  |

Table 7.1.1.2.1.3.3-8: RACH-ConfigGeneric (Table 7.1.1.2.1.3.3-7)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-configurationIndex | 156 |  |  |
| } |  |  |  |

Table 7.1.1.2.1.3.3-9: ServingCellConfig (Table 7.1.1.2.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | BWP-UplinkDedicated |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.2.1.3.3-10: BWP-UplinkDedicated (Table 7.1.1.2.1.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup | PUCCH-Config |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.2.1.3.3-11: PUCCH-Config (Table 7.1.1.2.1.3.3-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-112 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-Config ::= SEQUENCE { |  |  |  |
| schedulingRequestResourceToAddModList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig { | 1 entry |  |  |
| SchedulingRequestResourceConfig[1] | SchedulingRequestResourceConfig | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.2.1.3.3-12: SchedulingRequestResourceConfig (Table 7.1.1.2.1.3.3-11)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-112 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl10 | 5 | With SCS = kHz15 results in repetition every 10 ms | SCS_15kHz |
| sl20 | 5 | With SCS = kHz30 results in repetition every 10 ms | SCS_30kHz |
| sl80 | 4 | With SCS = kHz120 results in repetition every 10 ms | SCS_120kHz |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.2.1.3.3-13: SIB1 (step 19a0, Table 7.1.1.2.1.3.2-1)

| Derivation path: TS 38.508-1 [4] table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommon | Same contents as in Table 7.1.1.2.1.3.3-4 |  |
| } |  |  |  |

<End of modified section>