3GPP TSG-RAN5 Meeting #106	R5-251268

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4824 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test case 8.1.3.1.24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_QoE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. The UM DRB for voice is not configured until step 10 of the generic procedure for IMS voice call establishment, specified in Tables 4.9.15.2.2-1 and 4.9.16.2.2-1 of TS 38.508-1. Steps 7 and 23 state that 38.508-1 Table 4.8.1-1CAA specifies the procedure, but this table simply contains the definition of RRCReconfiguration-SRB2-SRB4-DRB(n, m). 2. In step 10 in Tables 4.9.15.2-2-1 and 4.9.16.2-2-1 of TS 38.508-1, when the IMS voice DRB is established, the logical channel identity is set as per default with DRBid+3. But in this scenario SRB4 is configured, hence the logical channel id shall be set to DRBid+4. 3. According to TP2 application layer measurement is not configured. The default message content RRCReconfiguration-SRB2-SRB4-DRB(n, m) defined in in 38.508-1 Table 4.8.1-1CAA includes the condition QoE, therefore it can’t be used for TP2. 4. According to TS 38.331 clause 5.7.16.2 UE may initiate an application layer measurement reporting “when at least one measConfigAppLayer and SRB4 and/or SRB5 have been configured by the network.” The current TP2 definition requires to verify the behaviour of the UE under incomplete configuration on a reception of the application layer measurement report container. In the absence of the measConfigAppLayer contents it would be rather unreasonable to expect UE to generate a valid report container. Morever, there are no means in the SS to check whether such a container was received inside the UE or not. Same concern can be raised for TP3. It is proposed to replace conditioning “when { UE receives an application layer measurement report container from upper layers}” with when {speech call is ongoing} and to mention configured SRB4 in preconditions to ensure unambiguous testing terms. 5. In steps 15 and 31, the procedure from TS 38.508-1 shall be used instead of TS 34.229-5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Corrected text of steps 7 and 23. 2. Added specific message content for UM DRB establishemnt for IMS voice. 3. Added specific message content to establish SRB2, SRB4 and DRBs without QoE. 4. Updated wording for TP2 and TP3. 5. Updated steps 15 and 31 to use the test procedure TS 38.508-1 cl.4.9.18. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. Test does not implement real network behaviour. 2,3. Message contents will remain incorrect. 4. TP2 and TP3 definition will remain ambiguous. 5. Test case prose will remain inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.1.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 1 revision of R5-250326. |  |  |  |  |  |  |  |  |

##### 8.1.3.1.24	Measurement configuration control and reporting for voice / QoE

8.1.3.1.24.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives an RRCReconfiguration message containing srb-Identity-v1700 and servedRadioBearerSRB4-r17 }

then { UE configure SRB4 and sends an RRCReconfigurationComplete message }

}

(2)

with { UE in NR RRC_CONNECTED state configured with SRB4 and not configured with application layer measurement and reporting }

ensure that {

when { speech call is ongoing }

then { UE does not send a MeasurementReportAppLayer message }

}

(3)

with { UE in NR RRC_CONNECTED state configured with SRB4 and with application layer measurement and reporting and speech call is ongoing }

ensure that {

when { speech call is ongoing}

then { UE sends a MeasurementReportAppLayer message }

}

8.1.3.1.24.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clauses 4.2.2, 5.7.16.2 and TS 26.114 clause 16.3.

[TS 38.331 clause 4.2.2]

"Signalling Radio Bearers" (SRBs) are defined as Radio Bearers (RBs) that are used only for the transmission of RRC and NAS messages. More specifically, the following SRBs are defined:

…

-	SRB4 is for RRC messages which include application layer measurement report information, all using DCCH logical channel. SRB4 has a lower priority than SRB1 and can only be configured by the network after AS security activation.

[TS 38.331 clause 5.7.16.2]

A UE capable of application layer measurement reporting in RRC_CONNECTED may initiate the procedure when configured with application layer measurement and reporting, i.e. when at least one measConfigAppLayer and SRB4 and/or SRB5 have been configured by the network.

Upon initiating the procedure, the UE shall:

…

1>	for each measConfigAppLayerId received from upper layers:

2>	if the UE AS has received application layer measurement report container from upper layers which has not been transmitted; and

…

2>	set the measConfigAppLayerId in the MeasurementReportAppLayer message to the value of the measConfigAppLayerId received together with application layer measurement report information;

…

1>	for each encoded MeasurementReportAppLayer message generated above which contains either only encapsulated application layer measurement reports or both encapsulated application layer measurement reports and RAN visible application layer measurement reports:

2>	if reportingSRB is not configured for the measConfigAppLayerId:

…

3>	else:

4>	submit the MeasurementReportAppLayer message to lower layers for transmission via SRB4.

[TS 26.114 clause 16.3]

The QoE configuration shall only be evaluated by the client at the start of a QoE measurement and reporting session (“QoE session”) associated with a MTSI session. This includes evaluation of any filtering criteria such as by geographical area. Client evaluation of all measurement and reporting criterias for an ongoing QoE session shall be unaffected by any QoE configuration changes received during that session – i.e., any changes to the QoE configuration shall only affect QoE sessions started after these configuration changes have been received.

8.1.3.1.24.3	Test description

8.1.3.1.24.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2.

UE:

-	None.

Preamble:

-	The UE is in state RRC_IDLE using generic procedure parameter Connectivity (NR) according to TS 38.508-1 [4], Table 4.5.1-1.

8.1.3.1.24.3.2	Test procedure sequence

Table 8.1.3.1.24.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | Make the UE attempt an IMS voice call. | - | - | - | - |
| 2-6 | Steps 2-6 of test procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [4] are performed. | - | - | - | - |
| 7 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2, SRB4 and DRB(s). | <-- | NR RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT | - | - |
| 8 | Check: Does the UE transmit an RRCReconfigurationComplete message? | --> | NR RRC: RRCReconfigurationComplete | 1 | P |
| 9-13 | Steps 9-13 of test procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [4] are performed. | - | - | - | - |
| 14 | Check: Does the UE transmit a MeasurementReportAppLayer message during the next 40 seconds? | --> | NR RRC: MeasurementReportAppLayer | 2 | F |
| 15 | Test procedure for IMS MT call release in 5GS described in TS 38.508-1 [4] subclause 4.9.18 takes place. | - | - | - | - |
| 16 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 17-22 | Steps 1-6 of test procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 [4] are performed | - | - | - | - |
| 23 | The SS transmits an RRCReconfiguration message and a SERVICE ACCEPT message to establish SRB2, SRB4 and DRB(s). The RRCReconfiguration message is configured using RRCReconfiguration-SRB2-SRB4-DRB(n, m) where n and m are the number of DRB(s) configured during the preamble. | <-- | NR RRC: RRCReconfiguration 5GMM: SERVICE ACCEPT | - | - |
| 24 | The UE transmits a RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 25-29 | Steps 9-13 of test procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 [4] are performed | - | - | - | - |
| 30 | Check: Does the UE transmit a MeasurementReportAppLayer message? | --> | NR RRC: MeasurementReportAppLayer | 3 | P |
| 31 | Test procedure for IMS MT call release in 5GS described in TS 38.508-1 [4] subclause 4.9.18 takes place. | - | - | - | - |
| 32 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |

8.1.3.1.24.3.3	Specific message contents

Table 8.1.3.1.24.3.3-1: RRCReconfiguration (step 7, Table 8.1.3.1.24.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition NR |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig | RadioBearerConfig-SRB2-SRB4-DRB(n,m) | Note1 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup | CellGroupConfig-SRB2-SRB4-DRB(n, m) | OCTET STRING (CONTAINING CellGroupConfig) Note 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| Note 1: n and m are the number of DRB(s) configured in the preamble |  |  |  |  |

Table 8.1.3.1.24.3.3-2: RRCReconfiguration (steps 10 and 26, Table 8.1.3.1.24.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.8.1-1C |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element | Value/remark |  | Comment |  | Condition |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| masterCellGroup |  | CellGroupConfig-DRB |  | OCTET STRING (CONTAINING CellGroupConfig) |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |

Table 8.1.3.1.24.3.3-3: RadioBearerConfig (Table 8.1.3.1.24.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | Entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config SEQUENCE { | SDAP-Config |  |  |
| defaultDRB | false |  |  |
| } |  |  |  |
| } |  |  |  |
| drb-Identity | j | j is allocated according to internal TTCN mapping |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config with condition UM |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-4: CellGroupConfig-DRB (Table 8.1.3.1.24.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-19. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 1 entry |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfigWithSRB4 with conditions UM and DRBj | entry 1 |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.1.24.3.3-5: RRCReconfiguration (step 23, Table 8.1.3.1.24.3.2-1)

Derivation Path: TS 38.508-1 [4] Table 4.8.1-1CAA