3GPP TSG-RAN WG5 Meeting # 106	R5-251547

# Athens, Greece, February 17 – 21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3239 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on clause 6 for frequency error test requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Tejet, SRTC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | [Editorial Corrections] “The 10 frequency error Δf results must fulfil the test requirements …” should be “The each of 10 frequency error Δf results, derived in Annex E.3.1 must fulfil the test requirements …”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct “The 10 frequency error Δf results must fulfil the test requirements …” to “The each of 10 frequency error Δf results, derived in Annex E.3.1 must fulfil the test requirements …”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The frequency error test requirements will remain inaccurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.5, 6.4A.1.1.5, 6.4C.1.5, 6.4D.1.5, 6.4E.1.1.5, 6.4E.1.1D.5, 6.4F.1.5, 6.4G.1.5, 6.4H.1.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-251086r1: To change the wording to reference Annex E.3.1. R5-251086r2: To change the “Reason for change” and “Summary of change” in the coversheet accordingly. |  |  |  |  |  |  |  |  |

<< Start of changes >>

6.4.1.5	Test requirement

The each of 10 frequency error Δf results, derived in Annex E.3.1 must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz)

<< next change >>

6.4A.1.1.5	Test requirement

The each of 10 frequency error Δf results, derived in Annex E.3.1 must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + TT) for each test point

For inter-band CA, PPM refers to each CC UL frequency.

For intra-band contiguous CA, PPM refers to PCC UL frequency.

For intra-band non-contiguous CA, PPM refers to each CC UL frequency.

Table 6.4A.1.1.5-1: Test Tolerance for frequency error

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 15Hz | 15Hz |
| 40MHz < BW ≤ 100MHz | 15Hz | 15Hz |

<< next change >>

6.4C.1.5	Test requirement

The each of 10 frequency error Δf results, derived in Annex E.3.1 measured on the SUL carrier must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz)

<< next change >>

6.4D.1.5	Test requirement

The requirements apply to each layer.

The each of 10 frequency error Δf results, derived in Annex E.3.1 must fulfil the test requirement:

|Δf| ≤ (0.1PPM+15 Hz)

<< next change >>

6.4E.1.1.5	Test requirement

The each of 10 frequency error Δf results, derived in Annex E.3.1 must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz)

<< next change >>

6.4E.1.1D.5	Test requirement

The requirements apply to each transmit antenna connector.

The each of 10 frequency error Δf results, derived in Annex E.3.1 must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz)

<< next change >>

6.4F.1.5	Test requirement

The each of 10 frequency error Δf results, derived in Annex E.3.1 must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz)

<< next change >>

6.4G.1.5	Test requirement

For each transmit antenna connector, the each of 10 frequency error Δf results, derived in Annex E.3.1 must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + 15 Hz)

<< next change >>

6.4H.1.1.5	Test requirement

The each of 10 frequency error Δf results, derived in Annex E.3.1 must fulfil the test requirement:

|Δf| ≤ (0.1 PPM + TT) for each test point

For intra-band contiguous CA, PPM refers to PCC UL frequency.

Table 6.4H.1.1.5-1: Test Tolerance for frequency error

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 100MHz | 15Hz | 15Hz |

==============================================================

<< End of changes >>