3GPP TSG-RAN5 Meeting #106	R5-251737

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1135 | rev | 1 | Current version: | 18.5.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to TC 6.6.3 Beam correspondence for initial access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Italy |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_Ph3-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on previous agreement, it was decided to allow both approaches (with and without UE beamlock) for the FR2 UE beam correspondence for PRACH requirements. Based on the discussion paper R5-250548, the test procedure requires updates using the without UE beamlock approach. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the test procedure, message contents |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The FR2 PRACH UE beam correspondence test case will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated discussion paper R5-250548 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: updated the test procedure inline with R5-250548r2 R5-250549r1 R5-251737 |  |  |  |  |  |  |  |  |

### 6.6.3	Beam Correspondence in RRC_INACTIVE and initial access

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- .

- TP Analysis is pending

- MU/TT analysis is pending

6.6.3.1	Test purpose

The purpose of this test is to verify the UE RACH MSG1 performance and uplink spatial coverage of the UE in expected directions is acceptable.

Transmission of the wrong power increases interference or transmission errors in the uplink channel.

6.6.3.2	Test applicability

This test case applies to all types of NR UE release 18 and forward without need for [UE beamlock_idle] function.

This test case also applies to all types of NR UE release 18 and forward supporting [UE beamlock_idle] function.

6.6.3.3	Minimum conformance requirements

The minimum conformance requirements for beam correspondence in RRC_INACTIVE and initial access are same as specified in section 6.2.1.1.3.3.

For the beam correspondence requirement for UEs in initial access and in RRC_INACTIVE, the following applicability rules apply:

-	If a UE meets UE beam correspondence requirements in initial access, it is considered to have met the beam correspondence requirements in RRC_INACTIVE

Table 6.6.3.3-1: Conditions for beam correspondence in RRC_INACTIVE and initial access

| Angle of arrival | NR operating bands | Minimum SSB_RP Note 2 | SSB Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm / SCSSSB | dB |
|  |  | SCSSSB = 120 kHz |  |
| All angles Note 1 | n257 | -96.2 | ≥6 |
|  | n258 | -96.2 |  |
|  | n259 | -90.7 |  |
|  | n260 | -91.9 |  |
|  | n261 | -96.2 |  |
|  | n262 | -88.5 |  |
|  | n263 | -88.0 |  |
| NOTE 1: For UEs that support multiple FR2 bands, the Minimum SSB_RP values for all angles are increased by MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1. NOTE 2: Values specified at the radiated requirements reference point to give minimum SSB Ês/Iot, with no applied noise. |  |  |  |

The normative reference for this requirement is TS 38.101-2 [3] clause 6.6.3.

6.6.3.4	Test description

6.6.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in Table 6.6.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.6.3.4.1-1: Test Configuration Table

| Default Conditions |  |
| --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 | Normal |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 | Low range, Mid Range, High range |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 | 100 MHz |
| Test SCS as specified in Table 5.3.5-1 | 120 kHz |
| PRACH parameters |  |
| PRACH Configuration Index | 190 [format C2] |
| preambleReceivedTargetPower | -60 |
| preambleTransMax | n200 |
| ra-ResponseWindow | sl10 |
| SSB configuration |  |
| SSB index | 0 |
| SSB_RP | As per Table 6.6.3.3-1 depending on the band |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

4.	Propagation conditions are set according to Annex B.0.

        - Test procedure

Without UE_beamlock_idle

1.           Ensure UE is in RRC_IDLE

2.          Place UE in a grid point.

3.            TE switches to SSB transmitting from PolLink = theta

4.            Page the UE to trigger RACH

5.            UE starts sending PRACH. TE sends RAR containing Random access preamble identifier that do not match the transmitted random access preamble or no response

6.            TE measures simultaneously the power of both measurement polarization

7.        TE sends a valid RAR to the UE to ensure it goes to RRC_CONN. If UE fails to go to RRC_CONN, power cycle the UE.

8.     TE determines the EIRP for PolLink = theta by adding up the EIRP of the 2 measurement polarization

9.            TE uses a sliding window of X (where X is the number of RACH needed for total accumulated PRACH measurement period of 1ms X = 10 for format B4) and picks the set of consecutive RACH providing the highest average PRACH power. This is the EIRP PRACH power for PolLink = theta .

10.            Repeat steps 1-9 with TE switching to SSB transmitting from PolLink = phi. This is the EIRP PRACH power for PolLink = phi .

11.            TE calculates PRACH EIRP (PolMeas= Theta, PolLink = Phi)

12.         Continue with above steps for each grid point to complete the spherical scan.

With UE_beamlock_idle

6.6.3.4.3	Message contents

Table 6.6.3.4.3-1: RACH-ConfigGeneric for Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  | FR2 |
| msg1-FDM | one |  | FR2 |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -60 |  |  |
| preambleTransMax | N200 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl10 |  |  |
| } |  |  |  |

6.6.3.5	Test requirement

Table 6.2.1.1-3: UE spherical coverage for power class 1

| Operating band | Min EIRP at 85 %-tile CDF (dBm) |
| --- | --- |
| n257 | 30.0 |
| n258 | 30.0 |
| n260 | 28.0 |
| n261 | 30.0 |
| NOTE 1: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. NOTE 2: The requirements in this table are derived based on Table 6.2.1.1-3 of TS 38.101-2 |  |

Table 6.6.3.5-1: UE spherical coverage for power class 3

| Operating band | Min EIRP at 50 %-tile CDF (dBm) |
| --- | --- |
| n257 | 9.5 |
| n258 | 9.5 |
| n260 | 6 |
| n261 | 9.5 |
| NOTE 1: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. NOTE 2: The requirements in this table are derived based on Table 6.2.1.3-3 of TS 38.101-2 |  |