3GPP TSG-RAN5 Meeting #107	R5-251849

Malta, MT, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3865 | rev |  | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test tolerance and measurement uncertainty for less than 5 MHz test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | # NR_FR1_lessthan_5MHz_BW-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test tolerance and measurement uncertainty analysis for the test cases 6.5.1.13, 6.5.1.14, 6.5.1.16 are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test tolerance and measurement uncertainty analysis for the test cases 6.5.1.13, 6.5.1.14, 6.5.1.16 have been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases would be incomplete in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of change 1--------

# F.1	Measurement uncertainties and test tolerances for FR1 and FR2

## F.1.1	Acceptable uncertainty of test system (normative)

See TS 38.521-1 [17] annex F.1.

### F.1.1.1	Measurement of test environments

See TS 38.521-1 [17] Annex F1.1.

### F.1.1.2	Measurement of RRM requirements

-------- Unchanged Clauses Skipped --------

Table F.1.1.2-4: Maximum test system uncertainty for RRM requirements for SA FR1 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 6.1.1.2 NR SA FR1-FR1 cell re-selection | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| … |  |  |
| 6.5.1.1 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês / Noc ±0.3 dB Fading profile ±0.7 dB | Noc is the AWGN on cell 1 frequency Ês / Noc is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |
| 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.3 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.4 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.5 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.6 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.7 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.8 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.9 SA FR1 radio link monitoring out-of-sync Test for PSCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.13 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5MHz BW | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.14 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5MHz BW | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.16 NR SA FR1 Radio Link Monitoring In-sync Test for PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC | Noc1 ±1.5 dB Noc2 ±1.5 dB Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |
| 6.5.2.2 | Same as 4.5.2.1 | Same as 4.5.2.1 |
| … |  |  |
| 6.7.17.1.1 Inter-frequency L1-RSRP absolute accuracy requirements for neighbour cell in FR1 | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Noc1 is the AWGN on NR freq 1 Noc2 is the AWGN on NR freq 2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRBs are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |

-------- End of change 1--------

-------- Unchanged Clauses Skipped --------

## F.1.3	Test Tolerance and Derivation of Test Requirements (informative)

See TS 38.521-1 [17] Annex F.3.

### F.1.3.1	Measurement of test environments

See TS 38.521-1 [17] Annex F.3.1.

### F.1.3.2	Measurement of RRM requirements

-------- Unchanged Clauses Skipped --------

-------- Start of change 2--------

Table F.1.3.2-2: Derivation of test requirements for NR SA FR1 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |
| 6.1.1.2 NR SA FR1-FR1 cell re-selection | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -2dB 1.6dB 0.4dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB | During T1: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |
| … |  |  |  |
| 6.5.1.1 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | SNR during T1: 1dB T2: -7dB T3: -15dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |
| 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4.1.1 |
| 6.5.1.3 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.4 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode | Same as 6.5.1.2 | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 6.5.1.5 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.6 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 6.5.1.2 | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 6.5.1.7 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Same as 6.5.1.1 | Same as 6.5.1.1 | Same as 6.5.1.1 |
| 6.5.1.8 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | Same as 6.5.1.2 | Same as 6.5.1.2 | Same as 6.5.1.2 |
| 6.5.1.9 SA FR1 radio link monitoring out-of-sync Test for PSCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion | SNR during T1: 1dB T2: 1dB T3: -15dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 1.8dB T2: 1.8dB T3: -15.8dB |
| 6.5.1.13 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5MHz BW | SNR during T1: 4dB T2: -4dB T3: -12dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 4.8dB T2: -3.2dB T3: -12.8dB |
| 6.5.1.14 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5MHz BW | SNR during T1: 2dB T2: -6dB T3: -14dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 2.8dB T2: -5.2dB T3: -14.8dB |
| 6.5.1.16 NR SA FR1 Radio Link Monitoring In-sync Test for PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in DRX mode | SNR during T1: 4dB T2: -4dB T3: -12dB T4: 0dB T5: 5.5dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB | SNR during T1: 4.8dB T2: -3.2dB T3: -12.8dB T4: -0.8dB T5: 6.3dB |
| 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |
| … |  |  |  |
| 6.7.17.1.1 Inter-frequency L1-RSRP absolute accuracy requirements for neighbour cell in FR1 | Test configuration 1, 2 Test 1 : Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Normal condition : Reported Cell 2 SS-RSRP values ± 10dB Extreme condition : Reported Cell 2 SS-RSRP values ± 13dB Test 2 : Noc1: -109dBm/15kHz + ΔBG_offset Noc2: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13dB Ês2 / Noc2: -3dB Normal condition : Reported Cell 2 SS-RSRP values ± 6.5dB Extreme condition : Reported Cell 2 SS-RSRP values ± 11dB | Test configuration 1, 2 Test 1 : 0dB 0dB 0dB 0dB Via mapping Test 2 : 0dB 0dB 0dB 0.3dB Via mapping | Test configuration 1, 2 Test 1 : Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Normal condition: RSRP_62 to RSRP_82 Extreme condition: RSRP_59 to RSRP_85 Test 2: Noc1: -109dBm/15kHz + ΔBG_offset Noc2: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13dB Ês2 / Noc2: -2.7dB Normal condition: RSRP_30 to RSRP_43 RSRP_31 to RSRP_44 RSRP_31 to RSRP_44 RSRP_32 to RSRP_45 RSRP_32 to RSRP_45 RSRP_33 to RSRP_46 RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_37 to RSRP_50 Depending on band groups Extreme condition: RSRP_26 to RSRP_48 RSRP_26 to RSRP_48 RSRP_27 to RSRP_49 RSRP_27 to RSRP_49 RSRP_28 to RSRP_50 RSRP_28 to RSRP_50 RSRP_29 to RSRP_51 RSRP_29 to RSRP_51 RSRP_32 to RSRP_54 Depending on band groups |
|  | Test configuration 3 Test 1 : Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Normal condition : Reported Cell 2 SS-RSRP values ± 10dB Extreme condition : Reported Cell 2 SS-RSRP values ± 13dB Test 2 : Noc1: -109dBm/15kHz + ΔBG_offset Noc2: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13dB Ês2 / Noc2: -3dB Normal condition : Reported Cell 2 SS-RSRP values ± 6.5dB Extreme condition : Reported Cell 2 SS-RSRP values ± 11dB | Test configuration 3 Test 1 : -1.35dB -1.35dB 0dB 0dB Via mapping Test 2 : 0dB 0dB 0dB 0.3dB Via mapping | Test configuration 3 Test 1 : Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês1 / Noc1: +10dB Ês2 / Noc2: +10dB Normal condition: RSRP_63 to RSRP_84 Extreme condition: RSRP_60 to RSRP_87 Test 2: Noc1: -109dBm/15kHz + ΔBG_offset Noc2: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13dB Ês2 / Noc2: -2.7dB Normal condition: RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_34 to RSRP_47 RSRP_35 to RSRP_48 RSRP_35 to RSRP_48 RSRP_36 to RSRP_49 RSRP_36 to RSRP_49 RSRP_37 to RSRP_50 RSRP_40 to RSRP_53 Depending on band groups Extreme condition: RSRP_29 to RSRP_51 RSRP_29 to RSRP_51 RSRP_30 to RSRP_52 RSRP_30 to RSRP_52 RSRP_31 to RSRP_53 RSRP_31 to RSRP_53 RSRP_32 to RSRP_54 RSRP_32 to RSRP_54 RSRP_35 to RSRP_57 Depending on band groups |

-------- End of change 2--------