3GPP TSG-RAN5 Meeting #107	R5-251867

# Malta, MT, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3872 | rev | - | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Inter-frequency RRC Re-establishment in FR1 with unknown target cell without serving cell timing for ATG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton International INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Minimum requirement of Inter-frequency RRC Re-establishment in FR1 with unknown target cell without serving cell timing for ATG is defined in TS 38.133, so test case should be made in RAN5 TS 38.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Inter-frequency RRC Re-establishment in FR1 with unknown target cell without serving cell timing for ATG. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Cae 19.2.3.1.2 will be still incompleting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.2.3.1.2 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

19.2.3.1.2	Inter-frequency RRC Re-establishment in FR1 with unknown target cell without serving cell timing for ATG

Editor's Note: This test case is incomplete for the following configurations:

-	TT analysis is missing.

19.2.3.1.2.1	Test purpose

To verify the NR ATG UE’s ability to perform NR inter-frequency RRC re-establishment delay in FR1 with unknown target cell and without serving cell timing are within the specified limits for ATG.

19.2.3.1.2.2	Test applicability

This test applies to all types of NR ATG UE release 18 and forward.

19.2.3.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 19.2.3.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.19.2.3.1.2.

19.2.3.1.2.4	Test description

19.2.3.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

This test shall be tested using any of the test configurations in Table 19.2.3.1.1.4.1-1.

Table 19.2.3.1.2.4.1-1: Inter-frequency RRC re-establishment test configurations

| Config | Description |
| --- | --- |
| 19.2.3.1.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 19.2.3.1.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 19.2.3.1.2-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 19.2.3.1.2.4.1-2

Table 19.2.3.1.1.4.1-2: Initial conditions for NR ATG SA FR1 RRC re-establishment with known target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.2.3.1.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The test parameters are given in Tables 19.2.3.1.2.4.1-3.

2.	Message contents are defined in clause 19.2.3.1.2.4.3.

Table 19.2.3.1.2.4.1-3: General test parameters NR ATG SA FR1 RRC re-establishment

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell 1 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell 2 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell 2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2, 3 | 3 s | Synchronous cells |
| N310 |  | - | 1, 2, 3 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1, 2, 3 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1, 2, 3 | 0 | Radio link failure timer; |
| T311 |  | ms | 1, 2, 3 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | OFF |  |
| PRACH configuration |  |  | 1, 2, 3 | FR1 PRACH configuration 1 | Table A.3.8.2.1-1 |
| T1 |  | s | 1, 2, 3 | 5 |  |
| T2 |  | ms | 1, 2, 3 | 240 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133 ) The value applies for UEs that don’t support antennaArrayType-r18 and UEs that support antennaArrayType-18 |
| T3 |  | s | 1, 2, 3 | 5 | The value applies for UEs that don’t support antennaArrayType-r18 and UEs that support antennaArrayType-18 |

19.2.3.1.2.4.2	Test procedure

The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure. During T1, the UE shall be configured with the carrier frequency of cell 2 (with RF Channel Number #2) to ensure that the UE has the context of the carrier frequency of cell 2 by the end of T1.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message.

2.	Set the parameters according to T1 in Table 19.2.3.1.2.5-1. T1 starts.

3.	SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 19.2.3.1.2.5-1. T2 starts

6.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 19.2.3.1.2.5-1. T3 starts

7.	If the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2 within 3 s from the beginning of time period T3, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T3 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

9.	Set cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

10.	Repeat step 2-9 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

19.2.3.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 19.2.3.1.2.4.3-1: Common Exception messages inter-frequency RRC re-establishment

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |
| Specific message contents exceptions for Test Configuration 6.3.2.1.2-1 | Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.3.2.1.2-2 | Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.3.2.1.2-3 | Table H.3.1-3 with Condition INTER-FREQ MO, SSB.2 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 19.2.3.1.2.4.3-2: RLF-TimersAndConstants for inter-frequency RRC re-establishment

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms0 |  |  |
| t311 | ms5000 |  |  |
| } |  |  |  |

Table 19.2.3.1.2.4.3-3: MeasConfig for inter-frequency RRC re-establishment

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig::= SEQUENCE { |  |  |  |
| reportConfigToAddModList | Not present |  |  |
| measIdToAddModList | Not present |  |  |
| quantityConfig | Not present |  |  |
| } |  |  |  |

19.2.3.1.2.5	Test requirements

Table 19.2.3.1.2.5-1 defines the primary level settings for all tests.

Table 19.2.3.1.2.5-1: Cell specific test parameters for NR inter-frequency RRC Re-establishment test case in FR1 for ATG

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| RF Channel Number |  | 1, 2, 3 | 1 |  |  | 2 |  |  |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 | TRS.2.1 TDD |  |  | TRS.2.1 TDD |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| Active DL BWP confgiuration |  | 1, 2, 3 | DLBWP.1.1 | N/A | N/A | N/A | N/A | DLBWP.1.1 |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 | N/A | N/A | N/A | N/A | ULBWP.1.1 |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
|  | dB | 1 | 4 | -infinity | -infinity | -infinity | -infinity | 7 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 4 | -infinity | -infinity | -infinity | -infinity | 7 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 2 | -94 | -infinity | -infinity | -infinity | -infinity | -91 |
|  |  | 3 | -91 | -infinity | -infinity | -infinity | -infinity | -88 |
| Io | dBm/9.36 MHz | 1 | -64.59 | -70. 05 | -70. 05 | -70. 05 | -70. 05 | -62.26 |
|  | dBm/9.36 MHz | 2 | -64.59 | -70. 05 | -70. 05 | -70. 05 | -70.05 | -62.26 |
|  | dBm/38.16 MHz | 3 | -58.50 | -63.94 | -63.94 | -63.94 | -63.94 | -56.15 |
| Propagation Condition |  | 1, 2 | AWGN [220Hz] |  |  |  |  |  |
|  |  | 3 | AWGN [500Hz] |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to Cell 2 for sending the RRCReestablishmentRequest message to Cell 2.

The RRC re-establishment delay to an unknown NR inter frequency cell without serving cell timing shall be less than 3s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 2

Tidentify_intra_NR = 800 ms

Tidentify_inter_NR = 800 ms

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target inter-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 2945 ms, allow 3 s in the test case.

## << End of changes >>