3GPP TSG-RAN5 Meeting #107	R5-252028

# Malta, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4938 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new NR NTN enh Cell Selection Test case for CellBarredFixedVSAT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 6.7.1.8 needs to be added as per NTN Enh WP |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 6.7.1.8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of WP will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1.8(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR0573 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Sections Omitted>

##### 6.7.1.8	NR NTN Enhancements / Cell Selection / NTN Serving cell becomes non-suitable - CellBarredFixedVSAT

6.7.1.8.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state }

ensure that {

when { UE finds a suitable cell that provides access by NR NTN Enh Cell and cellBarredFixedVSAT in SIB1 is not present }

then { the UE considers the cell as barred and no camping on this cell can take place }

}

(2)

with { UE in NR RRC_IDLE state }

ensure that {

when { UE finds a suitable cell that provides access by NR NTN Enh Cell  and cellBarredFixedVSAT in SIB1 is “not barred” }

then { the UE considers the cell suitable and camps on it }

}

(3)

with { UE in NR RRC_IDLE state }

ensure that {

when { UE finds a suitable cell that provides access by NR NTN Enh Cell and cellBarredFixedVSAT in SIB1 changes to “barred” }

then { the UE considers the cell as barred and no camping on this cell can take place }

}

6.7.1.8.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.304 clauses 5.3.1 and TS 38.331 clauses 5.2.2.4.1. Unless otherwise stated these are Rel-18 requirements.

[TS 38.304, clause 5.3.1]

Cell status and cell reservations are indicated in the MIB or SIB1 message as specified in TS 38.331 [3] by means of the following fields:

. . . .

-	cellBarredFixedVSAT (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. This field is only applicable to VSAT UEs using NTN access.

-	cellBarredMobileVSAT (IE type: "barred" or "not barred")
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs. This field is only applicable to VSAT UEs using NTN access.

. . . .

When cellBarredFixedVSAT is not broadcast in this cell,

-	For NTN access, the fixed VSAT UE shall treat this cell as if cell status is "barred".

When cellBarredMobileVSAT is not broadcast in this cell,

-	For NTN access, the mobile VSAT UE shall treat this cell as if cell status is "barred".

. . . .

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls;

-	The UE shall select another cell according to the following rule:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MIB:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	else:

-	If the UE is a RedCap UE, the UE shall acquire SIB1 and, in the remainder of this procedure, consider 'intraFreqReselection in MIB' to be 'intraFreqReselectionRedCap in SIB1', if available; or,

-	If the UE is an eRedCap UE, the UE shall acquire SIB1 and, in the remainder of this procedure, consider 'intraFreqReselection in MIB' to be 'intraFreqReselection-eRedCap in SIB1', if available; or,

-	If the UE is a 2Rx XR UE, the UE shall acquire SIB1 and, in the remainder of this procedure, consider 'intraFreqReselection in MIB' to be 'intraFreqReselection2RxXR in SIB1', if available:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	If the cell status "barred" is indicated in MIB but the UE is unable to acquire the SIB1; or

-	If the UE is a RedCap UE and intraFreqReselectionRedCap in SIB1 is not available; or

-	If the UE is an eRedCap UE and intraFreqReselection-eRedCap in SIB1 is not available:

-	the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	the UE may select another cell on the same frequency if re-selection criteria are fulfilled.

-	If the UE is neither a RedCap UE nor an eRedCap UE; or

-	if the UE is a RedCap UE and intraFreqReselectionRedCap in SIB1 is available; or

-	if the UE is an eRedCap UE and intraFreqReselection-eRedCap in SIB1 is available:

-	If the field intraFreqReselection in MIB message is set to "allowed":

-	the UE may select another cell on the same frequency if re-selection criteria are fulfilled;

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

-	else:

-	the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the field intraFreqReselection in MIB message is set to "not allowed":

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

-	If the cell operates in licensed spectrum:

-	the UE shall not re-select to another cell on the same frequency as the barred cell and exclude such cell(s) as candidate(s) for cell selection/reselection for 300 seconds;

-	else:

-	the UE may select to another cell on the same frequency if the reselection criteria are fulfilled.

-	else:

-	If the cell operates in licensed spectrum, or if this cell belongs to a PLMN which is indicated as being equivalent to the registered PLMN or the selected PLMN of the UE, or if this cell belongs to an SNPN which is equal to or indicated as being equivalent to the registered SNPN or the selected SNPN of the UE:

-	the UE shall not re-select to another cell on the same frequency as the barred cell and exclude such cell(s) as candidate(s) for cell selection/reselection for 300 seconds;

-	else:

-	the UE may select to another cell on the same frequency if the reselection criteria are fulfilled.

-	the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

The cell selection of another cell may also include a change of RAT.

NOTE 2:	If barring of a cell is triggered by the condition of trackingAreaCode and trackingAreaList not being provided, as specified in TS 38.331 [3], the barring only applies to this PLMN and the UE can re-evaluate the barring condition again due to selection of another PLMN.

[TS 38.331, clause 5.2.2.4.1]

Upon receiving the SIB1 the UE shall:

1>	store the acquired SIB1;

1>	if the access is for NTN:

2>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running:

3>	if the cellBarredNTN in the acquired SIB1 is set to barred or the cellBarredNTN is not included in the acquired SIB1:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

3>	if the UE is a fixed VSAT UE and the cellBarredFixedVSAT in the acquired SIB1 is set to barred or the cellBarredFixedVSAT is not included in the acquired SIB1, or

3>	if the UE is a mobile VSAT UE and the cellBarredMobileVSAT in the acquired SIB1 is set to barred or the cellBarredMobileVSAT is not included in the acquired SIB1:

4>	consider the cell as barred in accordance with TS 38.304 [20];

4>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends;

6.7.1.8.3	Test description

6.7.1.8.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 according to TS 38.508-1 [4] Table 4.4.2-3.

-	System information combination NR-29 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [4].

6.7.1.8.3.2	Test procedure sequence

Table 6.7.1.8.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE | - | - | - | - |
| 2 | Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 3 | Power Off the UE | - | - | - | - |
| 4 | SS change NR Cell 1 SIB1 to include cellBarredFixedVSAT-r18 and set to "not barred" | - | - | - | - |
| 5 | Power on the UE | - | - | - | - |
| 6 | Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? | --> | NR RRC: RRCSetupRequest | 2 | P |
| 7-19 | Steps 3 to 15 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 1. | - | - | - | - |
| 20 | The SS transmits an RRCRelease message to move to RRC_IDLE state. | <-- | RRCRelease | - | - |
| 21 | SS change NR Cell 1 SIB1 to include cellBarredFixedVSAT-r18 and set to "barred" | - | - | - | - |
| 22 | The SS transmits a Short message on PDCCHusing P-RNTI indicating a systemInfoModification. | - | PDCCH (DCI 1_0): ShortMessage | - | - |
| 23 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 24 | The SS transmits a Paging message within a paging occasion | <-- | NR RRC: Paging | - | . |
| 25 | Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? | --> | NR RRC: RRCSetupRequest | 3 | F |

6.7.1.8.3.3	Specific message contents

Table 6.7.1.8.3.3-1: SIB1 for NR cell 1 (Preamble)

| Derivation Path: 38.508-1 [4] Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellBarredFixedVSAT-r18 | notBarred |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.1.8.3.3-2: SIB1 for NR cell 1 (Step 1 of Table 6.1.7.3.3.2-1)

| Derivation Path: 38.508-1 [4] Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellBarredFixedVSAT-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.1.8.3.3-3: SIB1 for NR Cell 1(Step 4 of Table 6.1.7.3.3.2-1)

| Derivation Path: 38.508-1 [4] Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellBarredFixedVSAT-r18 | notBarred |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.1.8.3.3-4: SIB1 for NR Cell 1 (Step 22 of Table 6.1.7.3.3.2-1)

| Derivation Path: 38.508-1 [4] Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellBarredFixedVSAT-r18 | barred |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<Unchanged Sections Omitted>