3GPP TSG-RAN5 Meeting #107	R5-252120

# Malta, Malta, 19th May 2025 - 23rd May 2025

Title:	Discussions on IoT NTN new RRM TCs

Source:	Mediatek Inc., Keysight Technologies UK Ltd, Thales

Agenda item:	5.3.28.6

Document for:	Discussion and endorsement

# 1.	Introduction

According to [1], below multi-cells RRM TCs are added in TS 36.521-3. However, only ephemeris of GEO is already captured in TS 36.508. That means those TCs are not completed yet.

Table 1. New RRM TC list of IoT_NTN_enh_plus_CT1-UEConTest

| TC ID | Sat type |
| --- | --- |
| 13.1.1.1_1 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage for GSO and NGSO | GSO/NGSO |
| 13.1.1.2_1 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation for GSO and NGSO | GSO/NGSO |
| 13.1.1.3_1 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX for GSO and NGSO | GSO/NGSO |
| 13.1.1.4 HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage | GSO/NGSO |
| 13.1.1.5 HD – FDD Intra frequency case for UE Category NB1 Standalone mode in enhanced coverage, location-based cell reselection for NGSO | GSO/NGSO |
| 13.1.1.6 HD – FDD Inter frequency case for UE Category NB1 Standalone mode in enhanced coverage, time-based cell reselection for NGSO | GSO/NGSO |
| 13.3.1.1_1 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage for GSO and NGSO | GSO/NGSO |
| 13.3.1.2_1 HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage for GSO and NGSO | GSO/NGSO |
| 13.3.1.3 HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage | GSO/NGSO |
| 13.5.1 HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access | GSO/NGSO |
| 13.5.2 HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access | GSO/NGSO |
| 13.5.3 HD-FDD Intra-frequency location-based neighbour cell measurement for UE category NB1 in standalone mode under normal coverage for Satellite Access | GSO/NGSO |

To promote the progress of IoT_NTN_enh_plus_CT1-UEConTest Work Item, this document proposes to define ephemeris in SIB31/SIB33 for those Multi-cells TCs.

# 2. 	Discussion

## 2.1	Satellite Orbit Definition

In TS 36.905, ephemeris of GSO, LEO600 and LEO1200 are already attached and used [2][3]. Similar work had been done for LTE_NBIOT_eMTC_NTN_req-UEConTest [4] under GEO condition. Now GSO and NGSO condition has to be handled.

Observation 1: The orbit of GSO/NGSO are already generated and captured in current RAN5 spec.

Proposal 1: Reuse existing orbits in TS 36.905.

- LEO 600: ReportFile-NGSO-LEO600km-MinElev0-Lat25.08439333-Long121.56076999-Step0.640sec

- LEO 1200: ReportFile-NGSO-LEO1200km-MinElev0-Lat25.08439333-Long121.56076999-Step0.640sec

- GSO: ReportFile2_GSO-Inc7deg-Lat55-Long121.56076999-Step2.56sec-v2 / ReportFile2_GSO-SATLong101.56076999-UELong121.56076999_Lat55-Inc7_v2 / ReportFile2_GSO-SATLong141.56076999-UELong121.56076999_Lat55-Inc7_v2

RAN5 yet have similar discussion in RAN5 #101 [6], the agreements are captured below.

For NR NTN [6] Proposal 3: For GSO satellites, use different satellites orbits for serving and neighbour cells and maximize the Doppler difference between them in those 3GPP TS 38.133 NR NTN RRM test cases requiring neighbour cells. Proposal 4: For NGSO satellites, use same satellite orbit for serving and neighbour cells and maximize the Doppler difference between them in those 3GPP TS 38.133 NR NTN RRM test cases requiring neighbour cells.

The physical effect of IoT NTN and NR NTN is the same. It’s reasonable to reuse same strategy of NR NTN.

Proposal 2: For GSO satellites, use different satellites orbits for serving and neighbour cells and maximize the Doppler difference between them in those 3GPP TS 36.133 IoT NTN RRM test cases requiring neighbour cells. For NGSO satellites, use same satellite orbit for serving and neighbour cells and maximize the Doppler difference between them in those 3GPP TS 36.133 IoT NTN RRM test cases requiring neighbour cells.

## 2.2	Doppler and Delay Effect

New TCs can be separated to 3 TC groups.

- Group 1: Reselection

- Group 2: Re-establishment

- Group 3: Neighbour cell measurement

The essential and universe requirement of above TCs is detecting a new cell and measuring on the newly detected cell. As SIB33 contains neighbour cell ephemeris will be broadcasted to the UE, from UE side, there is no big difference to detect or measuring on the neighbour cell. Same orbit selection strategy can be used to those TC groups.

Additionally, there is no need to differentiate intra and inter detecting or measuring as the UE pre-compensating behaviour is the same.

Proposal 3: Use same strategy when defining the orbit for all TC groups.

## 2.3	Orbit Selecting Strategy for GSO and NGSO

Based on previous consensus in RAN4 and RAN5, following strategy should be complied,

- If UE supports both NGSO and GSO, the GSO test Config can be skipped if the UE passes NGSO test Config [5].

- 1200km Orbit can suffice the requirements.

For both IoT NTN and NR NTN [7] Proposal 2: When testing RRM NTN test cases for UEs which support only NGSO or both GSO and NGSO satellites, ephemeris values shall correspond to a 1200 km satellite altitude.

If this is the case, then the test configuration can be summarized as below,

Table 2. Test configuration for different UE capability

| UE capability | Test Configuration |
| --- | --- |
| Support GSO only | GSO orbit |
| Support NGSO only or support both GSO and NGSO | LEO1200 orbit |

According to TS 36.133 Clause 13, only 1 NBR cell is needed for IoT NTN. That’s the difference between IoT NTN and NR NTN.

Observation 2: Only 1 NBR cell is needed for IoT NTN RRM TCs.

The maximum doppler(ΔD) and delay(ΔT) differential of NR NTN is listed in table 3. For IoT NTN, 1 NBR cell ephemeris need to be selected to maximize the doppler differential between SRV and NBR cell.

Table 3. Ephemeris of NR NTN SRV and 2 NBR cells

| Sat type | Ephemeris in 38.508-1 Clause 7 | ΔD(SRV-NBR1) | ΔD(SRV-NBR2) |
| --- | --- | --- | --- |
| GSO |  | 0.27 ppm | 0.16 ppm |
| LEO1200 |  | 35.2 ppm | 17.5 ppm |

Obviously, NBR 1 can be chosen to maximize the doppler differential.

Proposal 4: Use the ephemeris of serving cell and neighbour cell 1 of NR NTN for those IoT NTN newly added RRM TCs in TS 36.521-3.

# 3. 	Conclusion

This document proposes a way forward to define the ephemeris of Neighbour cells of IoT NTN.

The following observations and proposals that pertains IoT NTN new RRM TCs are made:

Observation 1: The orbit of GSO/NGSO are already generated and captured in current RAN5 spec.

Proposal 1: Reuse existing orbits in TS 36.905.

- LEO 600: ReportFile-NGSO-LEO600km-MinElev0-Lat25.08439333-Long121.56076999-Step0.640sec

- LEO 1200: ReportFile-NGSO-LEO1200km-MinElev0-Lat25.08439333-Long121.56076999-Step0.640sec

- GSO: ReportFile2_GSO-Inc7deg-Lat55-Long121.56076999-Step2.56sec-v2 / ReportFile2_GSO-SATLong101.56076999-UELong121.56076999_Lat55-Inc7_v2 / ReportFile2_GSO-SATLong141.56076999-UELong121.56076999_Lat55-Inc7_v2

Proposal 2: For GSO satellites, use different satellites orbits for serving and neighbour cells and maximize the Doppler difference between them in those 3GPP TS 36.133 IoT NTN RRM test cases requiring neighbour cells. For NGSO satellites, use same satellite orbit for serving and neighbour cells and maximize the Doppler difference between them in those 3GPP TS 36.133 IoT NTN RRM test cases requiring neighbour cells.

Proposal 3: Use same strategy when defining the orbit for all TC groups.

Observation 2: Only 1 NBR cell is needed for IoT NTN RRM TCs.

Proposal 4: Use the ephemeris of serving cell and neighbour cell 1 of NR NTN for those IoT NTN newly added RRM TCs in TS 36.521-3.

# 4. 	References

- R5-250107, “WP Rel-18 IoT NTN enh after RAN5#106” , CMCC

- R5-237213, “GSO satellite Ephemeris file generation methodology for NTN NR UE testing”, THALES	, RAN5#101.

- R5-237216, “NGSO satellite Ephemeris file generation methodology for NTN NR UE testing”, THALES	, RAN5#101.

- R5-241943, “IoT NTN RRM Ephemeris: definition for neighbour cells and updates for serving cells”, Keysight Technologies UK Ltd, Thales, RAN5#102.

- TS 36.521-3 i80, Clause 13

- R5-242001, “Ephemeris definition for NR NTN RRM neighbour cells ”, Keysight Technologies UK Ltd, Thales, RAN5#101.

- R5-237884, “Selection of ephemeris information for NTN RRM test cases”, Keysight Technologies UK Ltd, MediaTek, RAN5#101