3GPP TSG-RAN5 Meeting #107	R5-252167

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3305 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new TC Error Vector Magnitude including symbols with transient period for ATG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TC 6.4J.2.1a Error Vector Magnitude including symbols with transient period for ATG missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | TC 6.4J.2.1a Error Vector Magnitude including symbols with transient period for ATG introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_ATG-UEConTest work item cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4J.2.1a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

#### 6.4J.2.1a	Error Vector Magnitude including symbols with transient period for ATG

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- MU and TT are FFS

- Test procedure and even minimum requirements are FFS in case of devices with more than one antenna/TAB connector.

6.4J.2.1a.1	Test Purpose

Same test purpose as in 6.4.2.1a.1

6.4J.2.1a.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR standalone ATG.

Support of 256QAM is optional.only applicable if indicating support

6.4J.2.1a.3	Minimum conformance requirements

The requirements for transmit modulation quality defined in clause 6.4.2.1a.3 shall apply for ATG UE except for the requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4J.2

6.4J.2.1a.4	Test description

Same test description as in clause 6.4.2.1a.4

6.4J.2.1a.5	Test requirement

The EVMbefore and EVMafter derived in Table 6.4J.2.1a.3-1 shall not exceed the values in Table 6.4J.2.1a.5-1. Both rising and falling edges of transient occurring at the respective slot boundaries are considered to have the same EVM requirements.

Table 6.4J.2.1a.5-1: Test requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| 64 QAM | % | 10 + TT |
| 256 QAM | % | 8 + TT |
| Note 1: TT is defined in Table 6.4J.2.1a.5-2. |  |  |

Table 6.4J.2.1a.5-2: Test Tolerance

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| 64 QAM | % | FFS |
| 256 QAM | % | FFS |

<< END OF CHANGES >>

Release 17

3GPP TS 38.521-1 V17.6.0 (2022-09)

Release 17

3GPP TS 38.521-1 V17.6.0 (2022-09)