3GPP TSG-RAN5 Meeting #107	R5-252185

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4959 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new ATG TC 6.8.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Tejet, SRTC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The TC 6.8.1.1 of ATG idle mode operations / Cell selection / GNSS position needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TC 6.8.1.1 has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work plan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.1.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0580 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.8		Air To Ground

6.8.1	ATG idle mode operations

6.8.1.1	ATG idle mode operations / Cell selection / GNSS position

6.8.1.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state and UE finds a cell which provides ATG access }

ensure that {

when { UE has no valid GNSS position }

then { UE shall not establish RRC connection to perform a Registration procedure }

}

(2)

with { UE in NR RRC_IDLE state and UE finds a cell which provides ATG access }

ensure that {

when { UE has a valid GNSS position and the cell broadcasts SIB22 }

then { UE establishes RRC connection and successfully performs registration procedure}

}

6.8.1.1.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in:TS 38.300, clauses 16.19.1 and 16.19.2.2, TS 38.331 clauses  5.2.2.1, 5.2.2.4.1, 5.2.2.4.2 and 5.2.2.5. Unless otherwise stated these are Rel-18 requirements.

[TS 38.300 clause 16.19.1]

In Air-to-ground (ATG) network, an NG-RAN consisting of ground-based gNBs, which provide cell towers that send signals up to an aircraft's antenna(s) of onboard ATG terminal, with typical vertical altitude of around 10,000m and take-off/landing altitudes down to 3000m.

CA and DC are not supported for ATG in this release of the specification.

[TS 38.300 clause 16.19.2.2]

For the serving cell, the network broadcasts coarse gNB location information. The UE shall have valid GNSS position as well as gNB location before connecting to an ATG cell. To achieve synchronisation, before and during connection to an ATG cell, the UE shall compute the RTT between UE and the gNB based on the GNSS position and the gNB location parameters, and autonomously pre-compensate the TTA for the RTT between the UE and the gNB.

In connected mode, the UE shall be able to continuously update the Timing Advance.

The UE may be configured to report Timing Advance during Random Access procedures or in connected mode. For an RRC_CONNECTED UE, event-triggered reporting of the Timing Advance is supported.

[TS 38.331 clause 5.2.2.1]

Figure 5.2.2.1-1: System information acquisition

The UE applies the SI acquisition procedure to acquire the AS, NAS- and positioning assistance data information. The procedure applies to UEs in RRC_IDLE, in RRC_INACTIVE and in RRC_CONNECTED.

The UE in RRC_IDLE and RRC_INACTIVE shall ensure having a valid version of (at least) the MIB, SIB1 through SIB4, SIB5 (if the UE supports E-UTRA), SIB11 (if the UE is configured for idle/inactive measurements), SIB12 (if UE is capable of NR sidelink communication/discovery and is configured by upper layers to receive or transmit NR sidelink communication/discovery), and SIB13, SIB14 (if UE is capable of V2X sidelink communication and is configured by upper layers to receive or transmit V2X sidelink communication), SIB15 (if UE is configured by upper layers to report disaster roaming related information), SIB16 (if the UE is capable of slice-based cell reselection and the UE receives NSAG information for cell reselection from upper layer), SIB17 or SIB17bis (if the UE is using TRS resources for power saving in RRC_IDLE and RRC_INACTIVE), SIB19 (if UE is accessing NR via NTN access), SIB22 (for ATG access), and SIB23 (if UE is capable of NR sidelink positioning and is configured by upper layers to receive or transmit SL-PRS).

The UE capable of MBS broadcast which is receiving or interested to receive MBS broadcast service(s) via a broadcast MRB shall ensure having a valid version of SIB20, regardless of the RRC state the UE is in.

The UE shall ensure having a valid version of the posSIB requested by upper layers.

[TS 38.331 clause 5.2.2.4.1]

Upon receiving the MIB the UE shall:

1>	store the acquired MIB;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE, or if the UE is in RRC_CONNECTED while T311 is running:

...

2>	else:

3>	apply the received systemFrameNumber, pdcch-ConfigSIB1, subCarrierSpacingCommon, ssb-SubcarrierOffset and dmrs-TypeA-Position.

NOTE 1:	A UE capable of NTN access should acquire SIB1 to determine whether the cell is an NTN cell.

NOTE 2:	A UE capable of ATG access should acquire SIB1 to determine whether the cell is an ATG cell.

NOTE 3:	A UE indicating any of the values in nes-CellDTX-DRX should acquire SIB1 to determine the cell barring status when the cellBarred in MIB is set to barred.

[TS 38.331 clause 5.2.2.5]

The UE shall:

1>	if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running:

2>	if the UE is unable to acquire the MIB or SIB1:

3>	consider the cell as barred in accordance with TS 38.304 [20];

3>	perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20].

NOTE 1:	The SIB19 is essential for NTN access. If UE is unable to acquire the SIB19 for NTN access, the action is up to UE implementation (e.g., cell re-selection to other cells).

NOTE 2: The SIB22 is essential for ATG access. If UE is unable to acquire the SIB22 for ATG access, the action is up to UE implementation (e.g., cell re-selection to other cells).

6.8.1.1.3	Test description

6.8.1.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 as defined in TS 38.508-1 [4] Table 4.4.2-2 is used.

UE:

- No UE location is pre-configured.

Preamble:

-	The UE is in Switched OFF (State 0N-B) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

6.8.1.1.3.2	Test procedure sequence

Table 6.8.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE. | - | - | - | - |
| 2 | Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 seconds? | - | - | 1 | F |
| 3 | Power off the UE. | - | - | - | - |
| 4 | Power on the UE and pre-configure the UE location. (Note 1) | - | - | - | - |
| 5 | Check: Does the UE send an RRCSetupRequest on NR Cell 1? | --> | RRCSetupRequest | 2 | P |
| 6-11 | Steps 3 to 8 of the procedure described in TS 38.508-1 [4] Table 4.5.4.2-3 are performed on NR Cell 1. | - | - | - | - |
| 12 | The SS transmits an RRCRelease message. | <-- | RRCRelease | - | - |
| Note 1: The pre-configuration could be done by use of test function UPDATE UE LOCATION INFORMATION defined in TS 38.509 [6], and the detailed UE location is any allowed value for ATG. |  |  |  |  |  |

6.8.1.1.3.3	Specific message contents

-	None.