3GPP TSG-RAN5 Meeting #107	R5-252213

# St Julian’s, Malta, 19th - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1594 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of annex A.2.19 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Due to the changes of R5-251276 conditions A1 and A2 are not used anywhere in A.2.19 anymore. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A1, A2 set to void |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.19 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | no TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## A.2.19	MO INFO for eCall over IMS / PSAP callback over IMS

| Header/param |  | Cond |  | Value/remark |  | Rel |  | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request-Line |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| Method |  |  |  | INFO |  |  |  | RFC 6086 [139] |  |
| Request-URI |  |  |  | same URI as SS sent in its Contact header before when it sent 200 OK for INVITE |  |  |  |  |  |
| SIP-Version |  |  |  | SIP/2.0 |  |  |  |  |  |
| Via |  |  |  |  |  |  |  | RFC 3261 [15] RFC 3581 [96] |  |
| sent-protocol |  |  |  | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |  |  |  |  |  |
| sent-by |  |  |  | IP address or FQDN and protected server port of the UE |  |  |  |  |  |
| via-branch |  |  |  | value starting with ‘z9hG4bK’ |  |  |  |  |  |
| Route |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| route-param |  |  |  | MO Call (eCall): URIs of the Record-Route header field of the dialog creating response in reverse order (200 OK for eCall, 183 Session Progress for test eCall) MT Call (PSAP callback): URIs of the Record-Route header field as specified for the INVITE in A.2.9 |  |  |  |  |  |
| From |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | SIP URI of the UE URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |  | TS 24.229 [10] |  |
| tag |  |  |  | local tag of the dialog ID |  |  |  |  |  |
| To |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | SIP URI of the SS. URI must be the same as used for the endpoint in the earlier requests within the dialog. |  |  |  |  |  |
| tag |  |  |  | remote tag of the dialog ID |  |  |  |  |  |
| Call-ID |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| callid |  |  |  | same as value received in INVITE message |  |  |  |  |  |
| CSeq |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | value of CSeq sent by the UE within its previous request in the same dialog but increased by one |  |  |  |  |  |
| method |  |  |  | INFO |  |  |  |  |  |
| Call-Info |  |  |  |  |  |  |  |  |  |
| info |  |  |  |  |  |  |  |  |  |
| absoluteURI |  |  |  | any cid URL |  | Rel-14 |  | RFC 8147 [149] |  |
| info-param |  | A3 |  | purpose=EmergencyCallData.eCall.MSD |  | Rel-14 |  | RFC 8147 [149] |  |
|  |  | A4 |  | purpose=EmergencyCallData.Control |  | Rel-14 |  | RFC 8147 [149] |  |
| Info-Package |  |  |  | EmergencyCallData.eCall.MSD |  | Rel-14 |  | RFC 8147 [149] |  |
| Content-Type |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| media-type |  |  |  | multipart/mixed;boundary=any value |  |  |  | TS 24.229 [10] |  |
| Content-Length |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | length of message-body |  |  |  |  |  |
| Content-Disposition |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| disp-type |  |  |  | Info-Package |  | Rel-14 |  | RFC 8147 [149] |  |
| Message-body |  |  |  |  |  |  |  |  |  |
|  |  | A3 |  | --boundary value (as provided in Content-Type) Content-Type: application/EmergencyCallData.eCall.MSD Content-ID: converted cid URL from Call-Info header. Note that this is done by removing the cid: prefix. Content-Disposition: by-reference MSD in ASN.1 PER encoding --boundary value (as provided in Content-Type) |  | Rel-14 |  | RFC 8147 [149] |  |
|  |  | A4 |  | --boundary value (as provided in Content-Type) Content-Type: application/EmergencyCallData.Control+xml Content-ID: converted cid URL from Call-Info header. Note that this is done by removing the cid: prefix.Content-ID: same cid as in Call-Info header Content-Disposition: by-reference <?xml version=”1.0” encoding=”UTF-8”?> <EmergencyCallData.Control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack ref=cid of the body part of corresponding INFO request from SS> <actionResult action=”send-data” success=”false” reason=any value/> </ack> </EmergencyCallData.Control> --boundary value (as provided in Content-Type) |  | Rel-14 |  | RFC 8147 [149] |  |

| Condition | Explanation |
| --- | --- |
| A1 | Void |
| A2 | Void |
| A3 | UE able to provide an updated MSD |
| A4 | UE not able to provide an updated MSD |