3GPP TSG-RAN5 Meeting #107	R5-252224

# St Julian’s, Malta, 19th - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5443 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction of test cases 9.2.3.3.5a and 9.2.3.4.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test cases 9.2.3.3.5a and 9.2.3.4.1.3 refer to 34.229-1 C.32 (MO call release) instead of 34.229-1 C.2 (Initial registration). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction of tables 9.2.3.3.5a.3.2-1 and 9.2.3.4.1.3.2-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.3.5a, 9.2.3.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | no TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

##### 9.2.3.3.5a	Periodic Location Update

9.2.3.3.5a.1	Test Purpose (TP)

(1)

with { UE in MM IDLE substate NORMAL SERVICE with periodic location update timer T3212 running}

ensure that {

when {  UE camps on a E-UTRAN cell and successfully performs a Tracking Area Update and T3212 expires}

then { the UE does not perform a periodic location update whilst in E-UTRAN coverage and does not change RAT because T3212 has expired}

}

9.2.3.3.5a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 23.401 clauses 4.3.5.2; and TS 24.008 clauses 4.7.1.4.1 and 4.7.5.1.1; TS 24.301 clause 5.1.5.Unless otherwise stated these are Rel-8 requirements.

[TS 23.401, clause 4.3.5.2 "Reachability Management for UE in ECM-IDLE state"]

…

If the UE is EPS attached only and either camps on an E UTRAN cell or is in ECM CONNECTED state when the UE's periodic LAU timer expires, the UE shall perform a Location Area Update procedure in NMO II/III or combined RA/LA update in NMO I when it next returns to GERAN/UTRAN coverage.

…

Expiry of the periodic TAU timer, or, the periodic RAU timer, or, the periodic LAU timer shall not cause the UE to change RAT.

…

E-UTRAN RRC state transitions shall have no impact on the periodic RAU timer or periodic LAU timer except that handover from GERAN/UTRAN to E-UTRAN shall cause the periodic RAU timer to be started from its initial value.

[TS 24.008, 4.2.2.1 "Service State, NORMAL SERVICE"]

When in state MM IDLE and service state NORMAL SERVICE, the mobile station shall:

-	perform normal location updating when a new location area is entered;

-	perform location updating procedure at expiry of timer T3211 or T3213;

-	perform periodic updating at expiration of timer T3212;

-	perform IMSI detach;

-	support requests from the CM layer;

-	respond to paging; and

-	for an eCall only mobile station (as determined by information configured in USIM), perform the eCall inactivity procedure at expiry of timer T3242 or timer T3243.

[TS 24.008, 4.4.1 "Location updating procedure"]

…

Upon successful location updating the mobile station sets the update status to UPDATED in the SIM/USIM, and stores the Location Area Identification received in the LOCATION UPDATING ACCEPT message in the SIM/USIM. The attempt counter shall be reset.

[TS 24.008, 4.4.2 "Periodic updating"]

…

The procedure is controlled by the timer T3212 in the mobile station. If the timer is not already started, the timer is started each time the mobile station enters the MM IDLE substate NORMAL SERVICE or ATTEMPTING TO UPDATE. When the MS leaves the MM Idle State the timer T3212 shall continue running until explicitly stopped.

…

When the timer T3212 expires, the location updating procedure is started and the timer shall be set to its initial value for the next start. If the mobile station is in other state than MM Idle when the timer expires the location updating procedure is delayed until the MM Idle State is entered.

…

If the mobile station is in service state NO CELL AVAILABLE, LIMITED SERVICE, PLMN SEARCH or PLMN SEARCH-NORMAL SERVICE when the timer expires the location updating procedure is delayed until this service state is left.

[TS 24.301, clause 5.1.5]

UEs that operate in CS/PS mode 1 or CS/PS mode 2 of operation should not use any MM timers related to MM specific procedures (e.g. T3210, T3211, T3212, T3213) while camped on E-UTRAN, unless the re-activation of these timers is explicitly described. If the MM timers are already running, the UE should not react on the expiration of the timers.

9.2.3.3.5a.3	Test description

9.2.3.3.5a.3.1	Pre-test conditions

System Simulator:

-	cell A is configured as Non-suitable cell;

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, cell 5 (LAI-1, NMO II) is configured as Serving cell. The value of T3212 (in SIB1) is 0110 (6 minutes);

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, Cell 24 (LAI-1, NMO II) is configured as Serving cell and does not support Dual Transfer Mode (DTM_SUPPORT is configured as default (see TS 51.010-1 clause 40.2.1.1 [23]. The value of T3212 (Octet 4 of the Control Channel Description IE) is 0110 (6 minutes);

-	System information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	if pc_UTRA AND px_RATComb_Tested = EUTRA_UTRA, the UE is previously registered on UTRAN cell 5 using default message contents according to TS 36.508 [18].

-	if pc_GERAN AND px_RATComb_Tested = EUTRA_GERAN, the UE is previously registered on GERAN cell 24 using default message contents according to TS 36.508 [18].

-	the UE is configured to initiate combined EPS/IMSI attach.

Preamble:

-	The UE is in state Switched Off (state 1).

9.2.3.3.5a.3.2	Test procedure sequence

Table 9.2.3.3.5a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on and camps on Cell 24 or Cell 5. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 5 to 7 the step specified in Table 9.2.3.3.5a.3.2-3 should take place. | - | - | - | - |
| 2 | Void | - | - | - | - |
| - | EXCEPTION: The messages in the next two steps are sent only on Cell 24 | - | - | - | - |
| 2Aa1 | The UE transmits a Classmark Change message | --> | CLASSMARK CHANGE |  |  |
| - | EXCEPTION: The next step describes behaviour that depends on UE capability. | - | - | - | - |
| 2Aa2 | IF pc_UTRA THEN the UE transmits a Utran Classmark Change message. | --> | UTRAN CLASSMARK CHANGE. |  |  |
| 3-4 | Void | - | - | - | - |
| 5 | The UE transmits an ATTACH REQUEST message on Cell 24 or Cell 5 indicating "GPRS attach”. | --> | ATTACH REQUEST | - | - |
| - | EXCEPTION: The messages in the next two steps are only on Cell 24 |  |  |  |  |
| 5Aa1 | The SS transmits an Authentication and Ciphering Request message on Cell 24 | <-- | Authentication and Ciphering Request | - | - |
| 5Aa2 | The UE transmits an Authentication and Ciphering Response message on Cell 24 | --> | Authentication and Ciphering Response | - | - |
| 6 | The SS transmits an ATTACH ACCEPT message indicating "GPRS only attached”. The UE sets the update status to UPDATED on the USIM. The SS allocates a P-TMSI. | <-- | ATTACH ACCEPT | - | - |
| 7 | The UE transmits an ATTACH COMPLETE to acknowledge the P-TMSI allocation. | --> | ATTACH COMPLETE |  |  |
| 7A | The UE may attempt the generic procedures for IMS signalling defined in 36.508 4.5A.3A (for cell 5) or 4.5A.3B (for cell 24) |  |  |  |  |
| 8 | The UE requests the activation of a default PDP context | --> | ACTIVATE PDP CONTEXT REQUEST |  |  |
| 9 | The SS accepts the request for default PDP context request and return an ACTIVATE PDP CONTEXT ACCEPT | <-- | ACTIVATE PDP CONTEXT ACCEPT |  |  |
| - | EXCEPTION: Step 9Aa1 describes behaviour that depends on the RAT combination; the "lower case letter" identifies a step sequence that takes place if a UTRA is applied. | - | - | - | - |
| 9Aa1 | IF px_RATComb_Tested = EUTRA_UTRA THEN the SS releases the RRC connection. | - | - | - | - |
| 10 | The signal strength of Cell A is raised to that of the Serving Cell and Cell 24 or Cell 5 is lowered to that of a Suitable Neighbour Cell as defined in table 6.2.2.1-1 of TS 36.508 [18]. Note: Cell 24 or Cell 5 is still suitable but the UE shall select Cell A | - | - | - | - |
| 11 | The UE camps on E-UTRAN cell A and transmits a TRACKING AREA UPDATE REQUEST message as defined in table 6.4.2.7A.1-1 of TS 36.508 [18]. | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 12 | The SS transmits an AUTHENTICATION REQUEST message to initiate the EPS authentication and AKA procedure. | <-- | AUTHENTICATION REQUEST | - | - |
| 13 | The UE transmits an AUTHENTICATION RESPONSE message and establishes mutual authentication. | --> | AUTHENTICATION RESPONSE | - | - |
| 14 | The SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security. | <-- | SECURITY MODE COMMAND | - | - |
| 15 | The UE transmits a NAS SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | SECURITY MODE COMPLETE | - | - |
| 16 | SS responds with TRACKING AREA UPDATE ACCEPT message including a valid TAI list containing the TAI of Cell A; with PLMN ID of Cell A included in the GUTI. The UE sets the TIN = GUTI. Note: This Accept message includes EPS update result = combined TA/LA updated. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 17 | The UE transmits a TRACKING AREA UPDATE COMPLETE message | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 18 | The SS releases the RRC Connection | - | - | - | - |
| - | Exception: Steps 18A to 18Aa4/18Ab1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 18A | If pc_IMS = TRUE SS starts 10s timer | - | - | - | - |
| 18Aa1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | -> | SERVICE REQUEST | - | - |
| 18Aa2 | The SS establishes SRB2 and DRB | - | - | - | - |
| 18Aa3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - | - |
| 18Aa4 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.2 steps 4-11. | - | - | - | - |
| 18Aa5 | The SS releases the RRC connection | - | - | - | - |
| 18Ab1 | The 10s timer expires | - | - | - | - |
| 19-28 | Void | - | - | - | - |
| 29 | Check: Does the UE transmit a LOCATION UPDATING REQUEST (on Cell 24 or Cell 5) and changes RAT. Check this until 7 minutes after step 3 in Table 9.2.3.3.5a.3.2-3 (when in GERAN) or 7 (when in UTRAN) | --> | LOCATION UPDATING REQUEST | 1 | F |
| 30-36 | Void | - | - | - | - |

Table: 9.2.3.3.5a.3.2-2: Void

Table 9.2.3.3.5a.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a LOCATION UPDATING REQUEST with location updating type set to "IMSI attach” on Cell 24 or Cell 5. | --> | LOCATION UPDATING REQUEST | - | - |
| 2 | The SS transmits a LOCATION UPDATING ACCEPT, and the UE stores the received LAI and sets the update status to UPDATED on the USIM. The SS allocates a TMSI. | <-- | LOCATION UPDATING ACCEPT | - | - |
| 3 | The UE transmits a TMSI REALLOCATION COMPLETE message to acknowledge the TMSI allocation. | --> | TMSI REALLOCATION COMPLETE | - | - |

9.2.3.3.5a.3.3	Specific message contents

Table 9.2.3.3.5a.3.3-1: Void

Table 9.2.3.3.5a.3.3-2:  Void

<End of modification>

<Start of modification>

##### 9.2.3.4.1	TAU/RAU procedure for inter-system cell reselection between A/Gb and S1 modes

9.2.3.4.1.1	Test Purpose (TP)

(1)

with { UE attached to GERAN with a PDP context active, and, E-UTRAN NAS and Security parameters including a valid GUTI stored on the USIM  }

ensure that {

when { UE performs a cell reselection to E-UTRAN and performs a Tracking Area Update }

then { the UE encodes the RRC parameters in the RRC Connection Establishment messages correctly (i.e. in the RRCConnectionRequest message, the ue-Identity is set to “randomValue” and the establishmentcause is set to MO-signalling; and, in the RRCConnectionSetupComplete message the selectedPLMN-identity, mmegi and mmec indicate the value of the registered MME  }

}

(2)

with { UE attached to GERAN with a PDP context active, and, E-UTRAN NAS and EPS Security parameters including a valid GUTI stored on the USIM, and ISR not activated  }

ensure that {

when { UE performs a cell reselection to E-UTRAN and performs a Tracking Area Update }

then { the UE encodes the parameters in the TRACKING AREA UPDATE REQUEST correctly i.e.:
           - the NAS key set identifierASME IE is set to the value stored in the USIM/allocated in
                  the last TRACKING AREA UPDATE ACCEPT message;
           - the Old GUTI IE is derived from the mapped P-TMSI and RAI;
           - the Additional GUTI IE is set to the GUTI stored in the USIM/allocated in the last
                  TRACKING AREA UPDATE ACCEPT message;
           - the GPRS ciphering key sequence number IE is set to the value allocated in the
                  AUTHENTICATION AND CIPHERING REQUEST message; and
           - the NONCEue IE is included;
           - the DRX parameter IE is not included. }

}

(3)

with { UE powered on in GERAN }

ensure that {

when { UE makes its first Tracking Area Update to E-UTRAN}

then { UE sends the UE radio capability information update needed IE }

}

(4)

with { UE registered in E-UTRAN, and, GERAN NAS and Security parameters including a valid P-TMSI available in the UE and ISR not activated }

ensure that {

when { UE performs a cell reselection to GERAN and performs a Routeing Area Update }

then { the UE derives the TLLI parameter in the RLC/MAC header from the GUTI allocated in the TRACKING AREA UPDATE ACCEPT message }

}

(5)

with { UE registered in E-UTRAN, and, GERAN NAS and GERAN Security parameters including a valid P-TMSI available in the UE, and ISR not activated }

ensure that {

when { UE performs a cell reselection to GERAN and performs a Routeing Area Update }

then { the UE encodes the parameters in the ROUTING AREA UPDATE REQUEST correctly i.e.:
           - [the GPRS ciphering key sequence number IE is set to the value mapped from
                  KSI-ASME;;]            - the Old routing area identification IE, and the Old P-TMSI signature IE are
                  mapped from the GUTI allocated in the TRACKING AREA UPDATE ACCEPT message;
           - the Additional mobile identity IE contains the P-TMSI allocated in the last received
                  ATTACH ACCEPT/ROUTING AREA UPDATE ACCEPT message; 
           - the Additional old routing area identification IE contains the RAI allocated in the
                  last received ATTACH ACCEPT/ROUTING AREA UPDATE ACCEPT message; and
           - [the DRX parameter IE is not included]. }

}

(6)

with { UE registered }

ensure that {

when { UE performs Tracking Area Update to E-UTRAN which is not for the "first TAU following GERAN/UTRAN Attach" or for a "UE radio capability update"}

then { UE does not send the UE radio capability information update needed IE }

}

(7)

with { UE attached to GERAN with a PDP context active, and, E-UTRAN NAS and Security parameters including a valid GUTI stored on the USIM  }

ensure that {

when { UE performs a cell reselection to E-UTRAN and performs a Tracking Area Update }

then { the UE encodes the RRC parameters in the RRC Connection Establishment messages correctly (i.e. in the RRCConnectionRequest message, the ue-Identity is set to S-TMSI or random value for pre-Rel-12 UE,or set to random value from Rel-12 and onwards, and the establishmentcause is set to MO-signalling; and, in the RRCConnectionSetupComplete message the registeredMME is not included when s-TMSI is included, or include the registeredMME and the mapped GUMMEI otherwise }

}

9.2.3.4.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 23.003 clauses 2.8.2.1 and 2.8.2.2; TS 23.401 clauses 4.3.5.6, 5.3.3.1 and 5.3.3.3; TS 24.008 clauses 4.7.1.4.1 and 4.7.5.1.1; and TS 24.301 clauses 5.3.1.1 and 5.5.3.2.2.

[TS 23.003, clause 2.8.2.1 “Mapping from GUTI to RAI, P-TMSI and P-TMSI signature”]

The mapping of the GUTI shall be done to the combination of RAI of GERAN / UTRAN and the P-TMSI:

E-UTRAN <MCC> maps to GERAN/UTRAN <MCC>

E-UTRAN <MNC> maps to GERAN/UTRAN <MNC>

E-UTRAN <MME Group ID> maps to GERAN/UTRAN <LAC>

E-UTRAN <MME Code> maps to GERAN/UTRAN <RAC> and is also copied into the 8 most significant bits of the NRI field within the P-TMSI;

E-UTRAN <M-TMSI>			maps as follows:

-	6 bits of the E-UTRAN <M-TMSI> starting at bit 29 and down to bit 24 are mapped into bit 29 and down to bit 24 of the GERAN/UTRAN <P-TMSI>;

-	16 bits of the E-UTRAN <M-TMSI> starting at bit 15 and down to bit 0 are mapped into bit 15 and down to bit 0 of the GERAN/UTRAN <P-TMSI>;

-	and the remaining 8 bits of the E-UTRAN <M-TMSI> are mapped into the 8 MBS bits of the <P-TMSI signature> field.

For UTRAN, the 10-bit long NRI bits are masked out from the P-TMSI and also supplied to the RAN node as IDNNS (Intra Domain NAS Node Selector). However, the RAN configured NRI length should not exceed 8 bits.

[TS 23.003, clause 2.8.2.2 “Mapping from RAI and P-TMSI to GUTI”]

The mapping of P-TMSI (TLLI) and RAI in GERAN/UTRAN to GUTI in E-UTRAN shall be performed as follows:

GERAN/UTRAN <MCC> maps to E-UTRAN <MCC>

GERAN/UTRAN <MNC> maps to E-UTRAN <MNC>

GERAN/UTRAN <LAC> maps to E-UTRAN <MME Group ID>

GERAN/UTRAN <RAC> maps into bit 23 and down to bit 16 of the M-TMSI

The 8 most significant bits of GERAN/UTRAN <NRI> map to the MME code.

GERAN/UTRAN <P-TMSI> maps as follows:

-	6 bits of the GERAN/UTRAN <P-TMSI> starting at bit 29 and down to bit 24 are mapped into bit 29 and down to bit 24 of the E-UTRAN <M-TMSI>;

-	16 bits of the GERAN/UTRAN <P-TMSI> starting at bit 15 and down to bit 0 are mapped into bit 15 and down to bit 0 of the E-UTRAN <M-TMSI>.

…

[TS 23.401, clause 4.3.5.6, “Idle mode signalling reduction function”]

The TIN can take one of the three values, "P-TMSI", "GUTI" or "RAT-related TMSI". The UE shall set the TIN when receiving an Attach Accept, a TAU Accept or RAU Accept message according to the rules in table 4.3.5.6-1.

Table 4.3.5.6-1: Setting of the TIN

| Message received by UE | Current TIN value stored by UE | TIN value to be set by the UE when receiving message |
| --- | --- | --- |
| Attach Accept via E-UTRAN (never indicates ISR activation) | Any value | GUTI |
| Attach Accept via GERAN/UTRAN (never indicates ISR activation) | Any value | P-TMSI |
| TAU Accept not indicating ISR | Any value | GUTI |
| TAU Accept indicating ISR | GUTI P-TMSI or RAT-related TMSI | GUTI RAT-related TMSI |
| RAU Accept not indicating ISR | Any value | P-TMSI |
| RAU Accept indicating ISR | P-TMSI GUTI or RAT-related TMSI | P-TMSI RAT-related TMSI |

[TS 23.401, clause 5.3.3.1, step 2, “Tracking Area Update procedure with Serving GW change”]

….

If the UE's TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and related RAI then these two elements are indicated as the old GUTI.

…

[TS 23.401, clause 5.3.3.3, step 2, “Routeing Area Update with MME interaction and without S-GW change”]

…

If the UE's internal TIN indicates "GUTI" and the UE holds a valid GUTI then the UE indicates the GUTI as the old P-TMSI and old RAI

…

[TS 24.008, clause 4.7.1.4.1, “Radio resource sublayer address handling (A/Gb mode only)”]

For an MS supporting S1 mode, the following five cases can be distinguished:

a)	the TIN indicates "P-TMSI" or "RAT-related TMSI" and the MS holds a valid P-TMSI and a RAI;

b)	the TIN indicates "GUTI" and the MS holds a valid GUTI;

c)	the TIN is deleted and the UE holds a valid P-TMSI and RAI;

d)	the TIN is deleted and the UE holds a valid GUTI, but no valid P-TMSI and RAI; or

e)	none of the previous cases is fulfilled.

In case a) the MS shall derive a foreign TLLI from the P-TMSI and proceed as specified for case i) above.

In case b), the MS shall derive a P-TMSI from the GUTI and then a foreign TLLI from this P-TMSI and proceed as specified for case i) above.

…

[TS 24.008, clause 4.7.5.1.1, “Normal and periodic routing area updating procedure initiation”]

To initiate the normal routing area updating procedure, the MS sends the message ROUTING AREA UPDATE REQUEST to the network, starts timer T3330 and changes to state GMM-ROUTING-AREA-UPDATING-INITIATED.

If the MS supports S1 mode, the MS shall handle the P-TMSI IE as follows:

-	If the TIN indicates "GUTI" and the MS holds a valid GUTI, the MS shall map the GUTI into a P-TMSI, P-TMSI signature and RAI as specified in 3GPP TS 23.003 [4]. The MS shall include the mapped RAI in the Old routing area identification IE and the mapped P-TMSI signature in the P-TMSI signature IE. When the routing area updating procedure is initiated in Iu mode, the MS shall also include the mapped P-TMSI in the P-TMSI IE. Additionally, in Iu mode and A/Gb mode, if the MS holds a valid P-TMSI and RAI, the MS shall indicate the P-TMSI in the Additional mobile identity IE and the RAI in the Additional old routing area identification IE.

-	If the TIN indicates "P-TMSI" or "RAT-related TMSI" and the MS holds a valid P-TMSI and RAI, the MS shall indicate the RAI in the Old routing area identification IE. When the routing area updating procedure is initiated in Iu mode, the MS shall also include the P-TMSI in the P-TMSI IE.

If the routing area updating procedure is not initiated by the MS due to an S1 mode to Iu mode or S1 mode to A/Gb mode intersystem change, the message ROUTING AREA UPDATE REQUEST shall contain the P-TMSI signature when received within a previous ATTACH ACCEPT or ROUTING AREA UPDATE ACCEPT message.

If the routing area updating procedure is initiated by the MS due to an S1 mode to Iu mode or S1 mode to A/Gb mode inter-system change in idle mode and the MS does not have a valid current UMTS security context, the message ROUTING AREA UPDATE REQUEST shall include a P-TMSI signature filled with a NAS token as specified in 3GPP TS 33.401 [119]. If the MS has a valid current UMTS security context, the MS shall indicate it in the GPRS ciphering key sequence number IE.

NOTE:	If the TIN indicates "GUTI", 8 bits of the NAS token will be filled with bits from the M-TMSI (see 3GPP TS 23.003 [4]).

If the routing area updating procedure is initiated by the MS due to the S1 mode to Iu mode or S1 mode to A/Gb mode inter-system change in connected mode or in idle mode if the MS does not have a valid current security context, the MS shall derive CK' and IK' from the KASME and the NAS downlink COUNT value corresponding to the NAS token derived as specified in 3GPP TS 33.401 [119]. The MS shall indicate the eKSI value in the CKSN field of the GPRS ciphering key sequence number IE in the ROUTING AREA UPDATE REQUEST message. Then, the MS shall reset the START value and store the mapped UMTS security context replacing the current UMTS security context.

…

In order to indicate the new DRX parameter while in GERAN or UTRAN coverage, the MS shall send the ROUTING AREA UPDATE REQUEST message containing the DRX parameter in the DRX parameter IE to the network, with the exception of the case if the MS had indicated its UE specific DRX parameter (3GPP TS 24.301 [120]) to the network while in E-UTRAN coverage. In this case, when the MS enters GERAN or UTRAN coverage and initiates a routing area updating procedure, the MS shall not include the DRX parameter in the DRX parameter IE in the ROUTING AREA UPDATE REQUEST message.

[TS 24.008, clause 4.7.5.1.3, “Normal and periodic routing area updating procedure accepted by the network”]

If the MS has indicated in the ROUTING AREA UPDATE REQUEST message that it supports PS inter-RAT HO from GERAN to E-UTRAN, the network may include in the ROUTING AREA UPDATE ACCEPT message a request to provide the E-UTRAN inter RAT information container.

[TS 24.301, clause 5.3.1.1 “Establishment of the NAS signalling connection”]

For the routing of the initial NAS message to the appropriate MME, the UE NAS provides the lower layers with either the S-TMSI or the registered globally unique MME identifier (GUMMEI) that consists of the PLMN ID, the MME group ID, and the MME code (see 3GPP TS 23.003 [2]).

-	When the UE is registered in the tracking area of the current cell during the NAS signalling connection establishment, the UE NAS shall provide the lower layers with the S-TMSI, but shall not provide the registered MME identifier to the lower layers. Exceptionally, when the UE in EMM-IDLE mode initiates a tracking area updating procedure for load balancing purposes, the UE NAS shall provide the lower layers with neither S-TMSI nor registered MME identifier.

-	When the UE is not registered in the tracking area of the current cell during the NAS signalling connection establishment, the UE NAS does not provide the lower layers with the S-TMSI. If the UE has a valid registered MME identifier from a previous registration, the UE NAS shall provide the lower layers with the registered MME identifier.

[TS 24.301, clause 5.5.3.2.2 “Normal and periodic tracking area updating procedure initiation”]

…

In order to indicate its UE specific DRX parameter while in E-UTRAN coverage, the UE shall send the TRACKING AREA UPDATE REQUEST message containing the UE specific DRX parameter in the DRX parameter IE to the network, with the exception of the case if the UE had indicated its DRX parameter (3GPP TS 24.008 [13]) to the network while in GERAN or UTRAN coverage. In this case, when the UE enters E-UTRAN coverage and initiates a tracking area updating procedure, the UE shall not include the UE specific DRX parameter in the DRX parameter IE in the TRACKING AREA UPDATE REQUEST message.

…

When initiating a tracking area updating procedure as a result of an Iu mode to S1 mode or A/Gb mode to S1 mode inter-system change, the UE shall handle the GUTI as follows:

-	if the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE shall map the P-TMSI and RAI into the old GUTI IE. Additionally, if the UE holds a valid GUTI, the UE shall indicate the GUTI in the Additional GUTI IE.

NOTE:	Mapping the P-TMSI and RAI to the GUTI is specified in Annex H of 3GPP TS 23.401 [10].

-	if the TIN indicates "GUTI" or "RAT-related TMSI" and the UE holds a valid GUTI, the UE shall indicate the GUTI in the Old GUTI IE.

…

When the tracking area updating procedure is initiated to perform an inter-system change from A/Gb mode or Iu mode to S1 mode, the UE shall include the KSIASME in the TRACKING AREA UPDATE REQUEST message if the UE has a cached EPS security context. Otherwise, the UE shall set the KSIASME to the value "no key is available".

When the tracking area updating procedure is initiated in EMM-IDLE mode to perform an inter-system change from A/Gb mode or Iu mode to S1 mode, the UE shall include the KSISGSN in the TRACKING AREA UPDATE REQUEST message. If the UE does not have a cached EPS security context, the UE shall include the NonceUE IE in the TRACKING AREA UPDATE REQUEST message. The TRACKING AREA UPDATE REQUEST message shall be integrity protected with the cached EPS security context if the UE has one. If the UE does not have a cached EPS security context, the TRACKING AREA UPDATE REQUEST message shall not be integrity protected.

…

When the tracking area updating procedure is initiated in EMM-IDLE mode to perform an inter-system change from A/Gb mode or Iu mode to S1 mode, the UE shall include the KSISGSN in the TRACKING AREA UPDATE REQUEST message. If the UE does not have a cached EPS security context, the UE shall include the NonceUE IE in the TRACKING AREA UPDATE REQUEST message. The TRACKING AREA UPDATE REQUEST message shall be integrity protected with the cached EPS security context if the UE has one. If the UE does not have a cached EPS security context, the TRACKING AREA UPDATE REQUEST message shall not be integrity protected.

…

If the UE initiates the first tracking area updating procedure following an attach in A/Gb mode or Iu mode, the UE shall include a UE radio capability information update needed IE in the TRACKING AREA UPDATE REQUEST message.

…

[TS 24.301, clause 4.4.2 “Handling of EPS security contexts”]

…

The key set identifier eKSI is assigned by the MME either during the authentication procedure or, for the mapped security context, during the handover procedure. The eKSI consists of a value and a type of security context parameter indicating whether an EPS security context is a native EPS security context or a mapped EPS security context. When the EPS security context is a native EPS security context, the eKSI has the value of KSIASME, and when the current EPS security context is a mapped EPS security context, the eKSI has the value of KSISGSN.

…

[TS 24.301, clause 5.3.1.1 “Establishment of the NAS signalling connection”]

b)	if the TIN indicates "P-TMSI" and the UE holds a valid P-TMSI and RAI, the UE NAS shall provide the lower layers with the MME identifier part of the mapped GUTI, which is generated from the P-TMSI and RAI.

9.2.3.4.1.3	Test description

9.2.3.4.1.3.1	Pre-test conditions

System Simulator:

NOTE: 	while this test describes the uses of 3 cells, it is intended that this test only requires 2 cells to be active at any one instant.

-	cell A (E-UTRAN), cell B (E-UTRAN), cell 24 and cell 26 (GERAN)

-	with the exception of the MCC and MNC, cell 24 and cell 26 take the default parameters of the GERAN cells as defined in clause 6.3.1 of TS 36.508 [18], cells 24 and 26 are configured with the same MCC and MNC as cell A. Cells 24 and 26 are in the same routing area;

-	as defined in clause 40.1 of TS 51.010 [23], the GERAN cells are configured to use Network Mode of Operation I;

-	the power level of Cell 24 is the Serving Cell level;

-	the power level of Cell A is set to the Non-suitable level;

-	the power level of Cell B is set to the Non-suitable" Off" level;

-	the power level of Cell 26 is set to the Non-suitable Off.

-	System information combination 5 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	the UE was previously registered on E-UTRAN cell B, and when on E-UTRAN, the UE was last authenticated and registered on a cell (no name assigned) frequency 2 belonging to a PLMN with (MCC, MNC) = (65, 987) and allocated a GUTI with (MMEGI, MMEC) = (0xfedc, 0xba) and an arbitrary M-TMSI value.

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.3.4.1.3.2	Test procedure sequence

Table 9.2.3.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message on Cell 24 | --> | ATTACH REQUEST | - | - |
| 3 | The SS transmits an AUTHENTICATION REQUEST message. | <-- | AUTHENTICATION REQUEST | - | - |
| 4 | The UE transmits an AUTHENTICATION RESPONSE message. | --> | AUTHENTICATION RESPONSE | - | - |
| 5 | The SS transmits a NAS SECURITY MODE COMMAND message | <-- | SECURITY MODE COMMAND | - | - |
| 6 | The UE transmits a NAS SECURITY MODE COMPLETE message ciphered with the new EPS security context identified by the KSIASME received in the SECURITY MODE COMMAND message in step 5 | --> | SECURITY MODE COMPLETE | - | - |
| 8 | SS responds with ATTACH ACCEPT message including valid TMSI, P-TMSI (TIN set to P-TMSI) and RAI. Editor’s Note: the default message contents in 51.010 clause 40.2.4.3 allocate a Negotiated READY timer value of 32 seconds. | <-- | ATTACH ACCEPT | - | - |
| 9 | The UE transmits an ATTACH COMPLETE message. | - | ATTACH COMPLETE | - | - |
| 9AA | Exception: if pc_IMS = TRUE then the generic procedure specified in TS 36.508 4.5A.3B is performed. NOTE: The SS rejects any PDP context request to prevent IMS signalling on the GERAN cell. | - | - | - | - |
| 9A | The activation of a PDP context is triggered by MMI or AT command. | - | - | - | - |
| 9B | The UE transmits an ACTIVATE PDP CONTEXT REQUEST message | --> | ACTIVATE PDP CONTEXT REQUEST | - | - |
| 9C | The SS responds with an ACTIVATE PDP CONTEXT ACCEPT message | <-- | ACTIVATE PDP CONTEXT ACCEPT | - | - |
| 10 | Void | - | - | - | - |
| 11 | Void | - | - | - | - |
| 12 | Void | - | - | - | - |
| 13 | The signal strength of Cell A is raised to that of the Serving Cell and that of Cell 24 is lowered to that of a Suitable Neighbour Cell. Note: Cell 24 is still suitable but the UE is expected to select Cell A. | - | - | - | - |
| 14 | Void | - | - | - | - |
| 15 | Check: Does the UE send an RRCConnectionRequest with the ue-Identity set to “randomValue” and the establishmentcause set to MO-signalling on Cell A? Editor’s Note: Cell reselection to E-UTRAN might not occur until the GPRS READY timer has expired. Hence this step might occur up to 40 seconds after step 13. | --> | RRCConnectionRequest | 1 | P |
| 16 | The SS responds with RRCConnectionSetup | <-- | RRCConnectionSetup |  |  |
| 17 | Check: Does the UE send an RRCConnectionSetupComplete with the mmegi and mmec set to the values derived from the mapped RAI and P-TMSI | --> | RRCConnectionSetupComplete(RRC parameters, TRACKING AREA UPDATE REQUEST) | 1 | P |
|  | Check: Are the contents of the TRACKING AREA UPDATE REQUEST with the correct parameters? |  |  | 2, 3 |  |
| 18 | The SS sends TRACKING AREA UPDATE ACCEPT Note: the default message contents cause the allocation of a new GUTI and new TAI list | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 19 | The UE sends TRACKING AREA UPDATE COMPLETE | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 20 | The SS releases the RRC connection | - | - | - | - |
| - | Exception: Steps 20A to 20Aa4/20Ab1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 20A | If pc_IMS = TRUE SS starts 10s timer | - | - | - | - |
| 20Aa1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message | -> | SERVICE REQUEST | - | - |
| 20Aa2 | The SS establishes SRB2 and DRB | - | - | - | - |
| 20Aa3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 4.5A.16.3. | - | - | - | - |
| 20Aa4 | The UE performs IMS registration using the generic procedure defined in 34.229-1 [35] Annex C.2 steps 4-11. | - | - | - | - |
| 20Aa5 | The SS releases the RRC connection | - | - | - | - |
| 20Ab1 | The 10s timer expires | - | - | - | - |
| 21 | Cell 24 is switched off | - | - | - | - |
| 22 | The signal strength of Cell 26 is raised to that of the Serving Cell and that of Cell A is lowered to that of a Non suitable Neighbour Cell. Note: Cell A is still suitable but the UE is expected to select Cell 26. | - | - | - | - |
| 23 | Check: Does the UE send on Cell 26 an RLC/MAC header with the TLLI derived from the P-TMSI that is derived from the GUTI allocated in step 18? | - | - | 4 | P |
| 24 | Check: Does the UE send on Cell 26 a ROUTING AREA UPDATE REQUEST with the correct parameters? | --> | ROUTING AREA UPDATE REQUEST | 5 | P |
| 24A | The SS transmits an AUTHENTICATION AND CIPHERING REQUEST message. | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 24B | The UE transmits an AUTHENTICATION AND CIPHERING RESPONSE message. | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 25 | The SS sends ROUTING AREA UPDATE ACCEPT (without the allocated P-TMSI IE). | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| - | EXCEPTION: Step 26 describes behaviour that depends on the UE capability. | - | - | - | - |
| 26 | IF pc_GERAN_2_E_UTRAN_PSHO, then the UE sends ROUTING AREA UPDATE COMPLETE message | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| 27 | The signal strength of Cell A is raised to that of the Serving Cell and that of Cell 26 is lowered to that of a Suitable Neighbour Cell. Note: Cell 26 is still suitable but the UE is expected to select Cell A. | - | - | - | - |
| 27A | Check: Does the UE send an RRCConnectionRequest with the ue-Identity set for Rel-8 to Rel-11 UE to s-TMSI or randomValue, and, for Rel-12 and onwards UE set to randomValue? | --> | RRCConnectionRequest | 7 | P |
| 27B | The SS responds with RRCConnectionSetup | <-- | RRCConnectionSetup |  |  |
| 28 | Check: Does the UE send on Cell A an RRCConnectionSetupComplete without the registeredMME when s-TMSI is included in step 27A, otherwise with the registeredMME and the mapped GUMMEI? | --> | RRCConnectionSetupComplete(RRC parameters, TRACKING AREA UPDATE REQUEST) | 7 | P |
| - | Check: Does the UE send a TRACKING AREA UPDATE REQUEST with the correct parameters? |  |  | 2, 6 |  |
| 29 | The SS sends TRACKING AREA UPDATE ACCEPT Note: the default message contents cause the allocation of a new GUTI. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 30 | The UE sends TRACKING AREA UPDATE COMPLETE | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

9.2.3.4.1.3.3	Specific message contents

Table 9.2.3.4.1.3.3-1: Message RRCConnectionRequest (step 15, Table 9.2.3.4.1.3.2-1)

| Derivation path: 36.508 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| InitialUE-Identity CHOICE { |  |  |  |
| random-Value | Any allowed value |  |  |
| } |  |  |  |
| establishmentCause | Mo-Signalling |  |  |

Table 9.2.3.4.1.3.3-2: Message RRCConnectionSetupComplete (step 17, Table 9.2.3.4.1.3.2-1)

| Derivation path: 36.508 table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |
| selectedPLMN-Identity |  | Indicates the PLMN of cell A. |  |  |
| registeredMME SEQUENCE { |  |  |  |  |
| plmn-Identity |  | Not Checked |  |  |
| Mmegi |  | LAC sent to the UE in step 8 |  |  |
| Mmec |  | Bit 23 to bit 16 of P-TMSI sent to the UE in step 8 |  |  |
| } |  |  |  |  |
| nas-DedicatedInformation |  | Not checked at RRC layer |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.2.3.4.1.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 17, Table 9.2.3.4.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Sent in SECURITY PROTECTED NAS MESSAGE with valid integrity check |  |  |  |
| EPS update type |  |  |  |
| EPS update type Value | '000'B | TA updating | TA_only |
|  | '001'B or ‘010’B | Combined TA/LA updating or combined TA/LA updating with IMSI attach | No_LAU_CombinedRAU_GERAN_UTRA |
|  | ‘010’B | Combined TA/LA updating with IMSI attach | LAU_CombinedRAU_GERAN_UTRA |
| NAS key set identifierASME |  |  |  |
| NAS key set identifier | The valid NAS key set identifier KSIASME of the UE | As stored on the USIM in EFEPSNSC |  |
| TSC | '0'B | native security context |  |
| Old GUTI | Mapped from the P-TMSI and RAI allocated in step 8 |  |  |
| Additional GUTI | GUTI1 | Set to the value stored in the USIM in EFEPSLOCI |  |
| DRX parameter | Not present |  |  |
| UE radio capability information update needed | ‘1’B | UE radio capability information update needed |  |
| GPRS ciphering key sequence number | Value mapped from the AUTHENTICATION AND CIPHERING REQUEST message |  |  |
| Old Ptmsi Signature | Any Value |  |  |
| Nonce | Any Value |  |  |

Table 9.2.3.4.1.3.3-3A: Message TRACKING AREA UPDATE REQUEST Conditions (Table 9.2.3.4.1.3.3-3)

| Condition | Explanation |
| --- | --- |
| TA_only | See the definition below 36.508 table 4.7.2-24. |
| No_LAU_CombinedRAU_GERAN_UTRA | UE has not performed location area update procedure or a combined routing area update procedure in A/Gb or Iu mode |
| LAU_CombinedRAU_GERAN_UTRA | UE has performed location area update procedure or a combined routing area update procedure in A/Gb or Iu mode |

Table 9.2.3.4.1.3.3-4: Message ROUTING AREA UPDATE REQUEST (step 24, Table 9.2.3.4.1.3.2-1)

| Derivation path: 24.008 table 9.4.14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| - Protocol discriminator | GMM |  |  |
| Skip indicator | ‘0000’ |  |  |
| Routing area update request message identity | ‘0000 1000’ B |  |  |
| Update type | Any allowed value |  |  |
| GPRS ciphering key sequence number | set to the value to KSI-ASME | With ISR inactive and TIN indicates GUTI, then eKSI value is set to KSI-ASME |  |
| Old routing area identification | Mapped from the GUTI received in step 18. |  |  |
| MS Radio Access capability | Any allowed value |  |  |
| Old P-TMSI signature | Mapped from the GUTI received in step 18. |  |  |
| Requested READY timer value | If present, any allowed value. |  |  |
| DRX parameter | Not present | Shall be absent |  |
| TMSI status | If present, any allowed value. |  |  |
| P-TMSI | Not present |  |  |
| MS network capability | Any allowed value. |  |  |
| PDP context status | If present, any allowed value. |  |  |
| PS LCS Capability | If present, any allowed value. |  |  |
| MBMS context status | If present, any allowed value. |  |  |
| UE network capability | Any allowed value. |  |  |
| Additional mobile identity | Set to the P-TMSI allocated in step 8 |  |  |
| Additional old routing area identification | Set to the RAI allocated in step 8 |  |  |
| Mobile station classmark 2 | If present, any allowed value. |  |  |
| Mobile station classmark 3 | If present, any allowed value. |  |  |
| Supported Codecs | If present, any allowed value. |  |  |

Table 9.2.3.4.1.3.3-5: Message ROUTING AREA UPDATE ACCEPT (step 25, Table 9.2.3.4.1.3.2-1)

| Derivation path: 24.008 table 9.4.15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| allocated P-TMSI | Absent |  |  |
| PDP context status | Same value as received in ROUTING AREA UPDATE REQUEST in step 24 |  |  |

Table 9.2.3.4.1.3.3-6: Void

Table 9.2.3.4.1.3.3-6A: Message RRCConnectionRequest (step 27A, Table 9.2.3.4.1.3.2-1)

| Derivation path: 36.508 table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| ue-Identity CHOICE { |  |  |  |
| s-TMSI | Received from NAS layer | Rel-8 to Rel-11 inclusive |  |
| randomValue | Any allowed value | Rel-8 and onwards |  |
| } |  |  |  |
| establishmentCause | Mo-Signalling |  |  |

Table 9.2.3.4.1.3.3-7: Message RRCConnectionSetupComplete (step 28, Table 9.2.3.4.1.3.2-1)

| Derivation path: 36.508 table 4.6.1-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |
| Rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| C1 CHOICE { |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |
| selectedPLMN-Identity |  | Indicates the PLMN of cell 24. |  |  |
| registeredMME |  | Not present | Shall be absent if the upper layers provided S-TMSI in RRCConnectionRequest message in step 27A for pre-Rel-12 UE |  |
| registeredMME |  | generated from the P-TMSI and RAI | Present only when the InitialUE-Identity in step 27A is set to randomValue |  |
| nas-DedicatedInformation |  | Not checked at RRC layer |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| gummei-Type-r10 |  | mapped | Present when the registeredMME is included, |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.2.3.4.1.3.3-8: Message TRACKING AREA UPDATE REQUEST (step 28, Table 9.2.3.4.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Sent in SECURITY PROTECTED NAS MESSAGE with valid integrity check |  |  |  |
| EPS update type | 000 | TA updating | TA_only |
|  | 010 | combined TA/LA updating with IMSI attach | combined_TA_LA |
| EPS bearer context status | (octet 3 = '00100000'B and octet 4 = '00000000'B) or (octet 3 = '00100000'B and octet 4 = '00010000'B) |  |  |
| NAS key set identifierASME |  |  |  |
| NAS key set identifier | The valid NAS key set identifier KSIASME of the UE | As stored on the USIM in EFEPSNSC in the pre-test conditions |  |
| TSC | '0'B | native security context |  |
| Old GUTI | Mapped from the P-TMSI allocated in Step 8 and RAI allocated in |  |  |
| Additional GUTI | GUTI1 | Set to the value allocated in step 18 |  |
| DRX parameter | Not present |  |  |
| UE radio capability information update needed | Not present |  |  |
| Old Ptmsi Signature | Any Value |  |  |
| Nonce | Any Value |  |  |

<End of modification>