3GPP TSG-RAN5 Meeting #107	R5-252229

# St Julian’s, Malta, 19th - 23rd May 2025

Agenda Item:	6.6.6.6

Source:	MCC TF160

Title:	Issues with non-UE detectable emergency call test cases

Document for:	Endorsement

# 1	Introduction

According to the definition (19.3.1.1) 34.229-1 test case 19.3.1 shall test that when the UE receives a response containing a P-Asserted-Identity header field with an emergency number then the UE shall send an UPDATE with proper location information and Contact header field. But according to the test case specification a UE not sending an UPDATE at all may pass the test case too.

In addition, the test case selection expression does not mandate that the UE supports reporting of location information and the requirements for the Contact header field of the UPDATE are the same as for the INVITE
 For a UE not supporting location information test case 19.3.1 does not deviate from a normal MO call. The same applies for 34.229-5 test case 10.4.

# 2	Discussion

### 2.1	Call establishment scenarios with/without UPDATE

At call establishment with preconditions the UE may use the PRACK to send the 2nd SDP offer when it has gotten the resources already (dedicated EPS bearer in case of LTE). In this case at normal call establishment the UE does not send an UPDATE. In the test case implementation this depends on at which point in time the IPCAN gets triggered to add the dedicated EPS bearer or QoS flow.

For 4G the trigger happens after the 183 Session Progress is sent to the UE:

| UE --> SS | INVITE (1st SDP offer) |
| --- | --- |
| UE <-- SS | 100 Trying |
| UE --> SS | 183 Session (1st SDP answer) |
| Trigger to activate dedicated EPS bearer |  |
| UE --> SS | PRACK (with 2nd SDP offer depending on whether the UE has recognised the dedicated EPS bearer as activated already) |
| UE <-- SS | 200 OK for PRACK |
| UE --> SS | (UPDATE) |

According to 24.229 clause 5.1.6.10 in case of a non-UE detectable emergency call the UE shall send an UPDATE even when the PRACK contains the 2nd SDP offer - what is not considered by test case 19.3.1. Nevertheless, whether or not the UE sends the 2nd SDP offer in the PRACK depends on the timing.

When the PRACK contains the 2nd SDP offer then

Issue 2.1-1:	a UE sending an UPDATE according to 24.229 clause 5.1.6.10 would fail test case 19.3.1.

Issue 2.1-2:	a UE sending no UPDATE at all would pass test case 19.3.1 (even when it has geographical location information available).

5G test case 10.4 specifies the PDU session modification to happen after the 200 OK for PRACK:

| UE --> SS | INVITE (1st SDP offer) |
| --- | --- |
| UE <-- SS | 100 Trying |
| UE --> SS | 183 Session (1st SDP answer) |
| UE --> SS | PRACK (without 2nd SDP offer) |
| UE <-- SS | 200 OK for PRACK |
| Trigger for PDU session modification |  |
| UE --> SS | UPDATE (with 2nd SDP offer) |

In case of preconditions, the UE is forced to send an UPDATE and there are no issues like 2.1-1 and 2.1-2.

Proposal 2.1-1:	4G test case 19.3.1 to be aligned to 5G test case 10.4 forcing the UPDATE.

In the scenario without preconditions in case of a non-UE detectable emergency call according to 24.229 clause 5.1.6.10 the UE still needs to send an UPDATE (when it has geographical location information available).

Proposal 2.1-2:	RAN5 to check whether the case of non-UE detectable emergency call with no preconditions needs test coverage.

### 2.2	Content of the UPDATE

Test cases 19.3.1 and 10.4 are applicable even when a UE does not support reporting of location information. In this case the UPDATE's message body only contains the 2nd SDP offer (as for a normal call).

In addition, the requirements for the Contact header field in the UDATE of the test cases are the same as for the INVITE in 34.229-1 A.2.1 (Basically the requirements are that the UE shall apply GRUU when it supports it).

When the UE does not provide location information and especially when the UE uses the same Contact in the UPDATE as in the INVITE then the UE behaviour in case of non-UE detectable emergency call is the same as at normal call establishment:

NOTE 1:	Only if - for what reason ever - the UE changes in the UPDATE its contact so that it does not fulfil the requirements as specified in 34.229-1 A.2.1 anymore, then test cases 19.31 and 10.4 will fail - in contrast to normal call establishment where the UPDATE's contact is not checked at all.

NOTE 2:	Due to the implementation, even when a UE supports location information by setting pc_IMS_Geolocation to false the UE will not be updated with location information, i.e. a non-conformant UE may pass the test case by setting pc_IMS_Geolocation to false.

Issue 2.2-1:	Unless the UE provides location information, the test cases do not add significant test coverage.

Proposal 2.2-1:	RAN5 to discuss whether test case applicability of 19.3.1 and 10.4 shall require support of location information (pc_IMS_Geolocation = true).

### 2.3	Proposals

Proposal 2.1-1:	4G test case 19.3.1 to be aligned to 5G test case 10.4 forcing the UPDATE.

Proposal 2.1-2:	RAN5 to check whether the case of non-UE detectable emergency call with no preconditions needs test coverage.

Proposal 2.2-1:	RAN5 to discuss whether test case applicability of 19.3.1 and 10.4 shall require support of location information (pc_IMS_Geolocation = true).