3GPP TSG-RAN5 Meeting #107	R5-252268

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5445 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IoT NTN enhancement test case 22.4.32 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Sweden AB |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Whether the UE had acquired the SIB information was undeterminable so step 2 was not needed. The UE was in an unstable state after step 29. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Step 23a3, step 23b2 and step 30 were added. Steps 2, step 24 and step 28-29 were set to void. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error will exist in this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.32 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

22.4.32	NB-IoT / NTN / GNSS validity duration reporting / UE-autonomous GNSS measurement

22.4.32.1	Test Purpose (TP)

(1)

with { UE has acquired SystemInformationBlockType2-NB with gnss-PositionFixDurationReporting present }

ensure that {

when { UE receives RRCConnectionSetup-NB }

then { UE transmits RRCConnectionSetupcomplete-NB with gnss-PositionFixDuration and gnss-validityDuration included }

}

(2)

with { UE has started T390 in RRC_CONNECTED state and gnss-AutonomousEnabled and ul-TransmissionExtensionEnabled are configured }

ensure that {

when { T390 expires and GNSS position is out-of-date }

then { UE transmits a GNSS Validity Duration Report MAC CE upon a new GNSS position becomes valid }

}

(3)

with { UE has started T390 in RRC_CONNECTED state and ul-TransmissionExtensionEnabled is configured and gnss-AutonomousEnabled is not configured }

ensure that {

when { T390 expires and GNSS position is out-of-date }

then { UE moves to RRC_IDLE state }

}

22.4.32.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331 clause 5.3.3.4, 5.3.3.21, 5.3.3.24 and 5.5.9, TS 38.321 clauses 5.4.10. Unless otherwise stated these are Rel-18 requirements.

[TS 36.331, clause 5.3.3.4]

1>	set the content of RRCConnectionSetupComplete message as follows:

……

2>	if the UE is connected to NTN:

3>	include gnss-validityDuration in accordance with the remaining time of the GNSS validity duration;

3>	if UE supports GNSS position fix in RRC_CONNECTED and gnss-PositionFixDurationReporting is present in SystemInformationBlockType2(-NB):

4>	include gnss-PositionFixDuration in accordance with the time duration required for the UE to acquire a GNSS position;

[TS 36.331, clause 5.3.3.21]

Upon indication that the GNSS position has become out-of-date while in RRC_CONNECTED, the UE shall:

1>	if the UE does not support performing GNSS position fix in RRC_CONNECTED and ul-TransmissionExtensionEnabled is not configured:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

1>	else if ul-TransmissionExtensionEnabled is configured:

2>	if timeAlignmentTimer is configured to be infinity:

3>	start timer T390 with the timer value set to ul-TransmissionExtensionValue;

2>	else:

3>	start timer T390 with the timer value set to the remaining time of timeAlignmentTimer;

1>	else if ul-TransmissionExtensionEnabled is not configured and no indication of network triggered GNSS measurement is received from lower layers:

2>	if gnss-AutonomousEnabled is configured:

3>	perform GNSS measurement using autonomous gaps as specified in clause 5.5.9;

2>	else:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

[TS 36.331, clause 5.3.3.24]

The UE shall:

1>	if timer T390 expires and no indication of network triggered GNSS measurement has been received from lower layers:

2>	if gnss-AutonomousEnabled is configured:

3>	perform GNSS measurement using autonomous gaps as specified in clause 5.5.9;

2>	else:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

[TS 36.331, clause 5.5.9]

For BL UEs or UEs in CE or NB-IoT UEs that are connected to NTN, GNSS measurement can be triggered aperiodically by the GNSS Measurement Command MAC CE (see TS 36.321 [6]), or triggered by the UE autonomously if enabled by the network, or triggered by the UE using available idle periods.

The UE shall:

1>	if an indication to perform GNSS measurement is received from lower layers:

2>	perform GNSS measurement using the measurement gap with a gap length indicated by lower layers, as specified in TS 36.213 [23];

2>	stop timer T390, if running;

1>	if gnss-AutonomousEnabled is configured:

2>	if the gap length is indicated by lower layers:

3>	set the autonomous gap length to the gap length indicated by lower layers;

2>	else:

3>	set the autonomous gap length to the latest reported time duration required for the UE to acquire a GNSS position;

2>	perform GNSS measurement using the autonomous gap starting from T390 expiry if ul-TransmissionExtensionEnabled is configured, otherwise starting from GNSS validity duration expiry;

NOTE:	UE can autonomously start GNSS measurements during available idle periods in RRC_CONNECTED to keep GNSS valid and stop T390 upon indication that a new GNSS position becomes valid. The exact time of starting GNSS measurements during available idle periods is left to UE implementation.

1>	upon starting GNSS measurement:

2>	stop timer T318, if running;

1>	upon indication that a new GNSS position becomes valid:

2>	instruct lower layers to report the remaining GNSS validity duration (see TS 36.321 [6]);

1>	upon indication that GNSS measurement has failed:

2>	if GNSS position is out-of-date; and

2>	if ul-TransmissionExtensionEnabled is not configured or T390 is not running:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

[TS 36.321, clause 5.4.10]

For an NB-IoT UE, a BL UE or a UE in enhanced coverage in a non-terrestrial network, an indication may be sent by upper layers to report the remaining GNSS validity duration.

If the MAC entity receives an indication from upper layers to report the remaining GNSS validity duration:

-	stop the timeAlignmentTimer associated with the pTAG, if running.

-	initiate a Random Access procedure (see clause 5.1).

NOTE:	When in RRC_CONNECTED, if the UE autonomously starts and completes GNSS acquisition using available idle periods, the UE reports the remaining GNSS validity duration but it is up to UE implementation whether to stop timeAlignmentTimer and initiate a Random Access procedure.

If the GNSS validity duration reporting procedure has been triggered and not cancelled:

-	if the MAC entity has UL resources allocated for new transmission for this TTI, and;

-	if the allocated UL resources can accommodate the GNSS Validity Duration Report MAC control element plus its subheader, as a result of logical channel prioritization:

-	instruct the Multiplexing and Assembly procedure to generate the GNSS Validity Duration Report MAC control element as defined in clause 6.1.3.23.

All triggered GNSS validity duration reports shall be cancelled when a GNSS Validity Duration Report MAC control element is included in a MAC PDU for transmission.

22.4.32.3	Test description

22.4.32.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 is configured according to Table 8.1.4.2-1 and Table 8.1.4.2-2 in TS 36.508 [18].

-	System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used.

UE:

-	The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble:

-	The UE is in State 1-NB switched off according to TS 36.508 [18].

22.4.32.3.2	Test procedure sequence

Table 22.4.32.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on UE. | - | - | - | - |
| 2 | Void | - | - | - | - |
| 3 | UE transmits an RRCConnectionRequest-NB message. | --> | RRCConnectionRequest-NB | - | - |
| 4 | SS transmits an RRCConnectionSetup-NB message. | <-- | RRCConnectionSetup-NB | - | - |
| 5 | Check: Does the UE transmit an RRCConnectionSetupComplete-NB message with gnss-PositionFixDuration and gnss-validityDuration included? | --> | RRCConnectionSetupComplete-NB | 1 | P |
| 6-20 | The attach procedure is completed by executing steps 5-14b1 of the UE registration procedure in TS 36.508 clause 8.1.5.2.3. | - | - | - | - |
| 21 | Wait 30 sec till T390 expire upon GNSS position is out of date. | - | - | - | - |
| 22 | The SS transmits Update UE location information message. | <-- | UPDATE UE LOCATION INFORMATION | - | - |
| - | EXCEPTION: Steps 23a1 to 23b2 describe behaviour depending on UE implementation; the "lower case letter" identifies a step sequence that take place if the UE performs a specific action. | - | - | - | - |
| 23a1 | The UE transmits a preamble on NPRACH upon GNSS position becomes valid. | --> | NPRACH Preamble | - | - |
| 23a2 | The SS transmits Random Access Response including UL grant suitable for transmitting GNSS Validity Duration Report MAC CE. | <-- | Random Access Response | - | - |
| 23a3 | Check: Does UE transmit a MAC PDU to report the remaining GNSS measurement validity duration? | --> | MAC PDU (GNSS Validity Duration Report MAC CE) | 2 | P |
| 23b1 | Void | - | - | - | - |
| 23b2 | Check: Does UE transmit a MAC PDU to report the remaining GNSS measurement validity duration? | --> | MAC PDU (GNSS Validity Duration Report MAC CE) | 2 | P |
| 24 | Void | - | - | - | - |
| 25 | The SS transmits RRCConnectionReconfiguration-NB with gnss-AutonomousEnabled not configured. | <-- | RRCConnectionReconfiguration-NB | - | - |
| 26 | UE transmits an RRCConnectionReconfigurationComplete-NB message. | --> | RRCConnectionReconfigurationComplete-NB | - | - |
| 27 | Wait 30 sec till T390 expire upon GNSS position is out of date. | - | - | - | - |
| 28-29 | Void | - | - |  |  |
| 30 | Check: Does the test result of generic test procedure in TS 36.508 subclause 8.1.5A.2 indicate that the UE responds to the paging message? | - | - | 3 | P |

22.4.32.3.3	Specific message contents

Table 22.4.32.3.3-1: SystemInformationBlockType2-NB (step 2, Table 22.4.32.3.2-1)

| Derivation Path: 36.508 Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| gnss-PositionFixDurationReporting-r18 | true |  |  |
| } |  |  |  |

Table 22.4.32.3.3-2: RRCConnectionSetup-NB (step 4, Table 22.4.32.3.2-1)

| Derivation path: 36.508 table 8.1.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetup-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetup-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13 SEQUENCE { |  |  |  |
| mac-MainConfig-r13 CHOICE { |  |  |  |
| explicitValue-r13 SEQUENCE { |  |  |  |
| timeAlignmentTimerDedicated-r13 | infinity |  |  |
| } |  |  |  |
| } |  |  |  |
| gnss-AutonomousEnabled-r18 | true |  |  |
| ul-TransmissionExtensionEnabled-r18 | true |  |  |
| ul-TransmissionExtensionValue-r18 | sf5120 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.4.32.3.3-3: RRCConnectionSetupComplete-NB (step 5, Table 22.4.32.3.2-1)

| Derivation Path: 36.508 table 8.1.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r13 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| gnss-ValidityDuration-r17 | Any allowed value |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| gnss-PositionFixDuration-r18 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.4.32.3.3-4:  RRCConnectionReconfiguration-NB (step 25, Table 22.4.32.3.2-1)

| Derivation Path: 36.508 table 8.1.6.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13 SEQUENCE { |  |  |  |
| mac-MainConfig-r13 CHOICE { |  |  |  |
| explicitValue-r13 SEQUENCE { |  |  |  |
| timeAlignmentTimerDedicated-r13 | infinity |  |  |
| } |  |  |  |
| } |  |  |  |
| gnss-AutonomousEnabled-r18 | Not present |  |  |
| ul-TransmissionExtensionEnabled-r18 | true |  |  |
| ul-TransmissionExtensionValue-r18 | sf5120 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.4.32.3.3-5: UPDATE UE LOCATION INFORMATION (step 22, Table 22.4.32.3.2-1)

| Derivation Path: TS 36.509 [25] Table 6.12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ellipsoidPointWithAltitude | '23AC F356 7176 0000'H | UE Location: - Latitude: 25.08439433 - Longitude: 121.56076999 - Altitude: 0 |  |
| horizontalVelocity | '00 0000'H | Speed: 0 km/s |  |
| gnss-TOD-msec | '00 0000'H |  |  |

[End of change]