3GPP TSG-RAN WG5 Meeting #107	R5-252273

St Julian’s, Malta, 19th-23nd May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0191 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of sidelink positioning test condition to 37.571-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Support of sidelink positioning in NR systems is one of the objectibve of the work item Expanded and improved NR positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added TS 38.355 as reference file and added SLPP related abbreviations - Added sidelink positioning methods sub-tests - Added default SLPP REQUEST CAPABILITIES message contents |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The release 18 positioning work item will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3, 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS) ".

[3]	3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[4]	3GPP TS 36.355: "LTE Positioning Protocol (LPP)".

[5]	3GPP TS 24.171: "Control Plane Location Services (LCS) procedures in the Evolved Packet System (EPS)".

[6]	3GPP TS 24.030: "Location Services (LCS); Supplementary service operations; Stage 3".

[7]	3GPP TS 24.080: "Mobile radio interface layer 3 supplementary services specification; Formats and coding".

[8]	3GPP TS 36.508: "Common test environments for User Equipment (UE)".

[9]	3GPP TS 37.571-1: "User Equipment (UE) conformance specification for UE positioning; 
Part 1: Conformance test specification".

[10]	3GPP TS 37.571-3: "User Equipment (UE) conformance specification for UE positioning; 
Part 3: Implementation Conformance Statement (ICS)".

[11]	3GPP TS 37.571-4: "User Equipment (UE) conformance specification for UE positioning; 
Part 4: Test suites".

[12]	3GPP TS 37.571-5: "User Equipment (UE) conformance specification for UE positioning; 
Part 5: Test scenarios and assistance data".

[13]	3GPP TS 36.509: "Special conformance testing functions for User Equipment (UE)".

[14]	3GPP TS 34.123-1: "User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[15]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); 
Stage 3".

[16]	3GPP TS 34.108: "Common Test Environments for User Equipment (UE) Conformance Testing".

[17]	3GPP TS 25.331: "RRC Protocol Specification".

[18]	3GPP TS 34.109: "Terminal logical test interface; Special conformance testing functions".

[19]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols;
Stage 3".

[20]	3GPP TS 33.102: "3G security; Security architecture".

[21]	ICD-GPS-200: "Navstar GPS Space Segment/Navigation User Interface".

[22]	3GPP TS 23.171: "Location Services (LCS); Functional description; Stage 2 (UMTS)".

[23]	GSM TS 03.71: "Location Services (LCS); Functional description; Stage 2".

[24]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[25]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[26]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[27]	3GPP TS 22.101: "Service aspects; Service principles".

[28]	OMA-TS-LPPe-V2_0: "LPP Extensions Specification", Open Mobile Alliance.

[29]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance", May 2015.

[30]	3GPP TS 38.508-1: "User Equipment (UE) conformance specification; Part 1: Common test environment".

[31]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[32]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[33]	3GPP TS 38.331: "NR Radio Resource Control (RRC) protocol specification".

[34]	3GPP TS 38.509: "Special conformance testing functions for User Equipment (UE)".

[35]	3GPP TS 38.355: "NR; Sidelink Positioning Protocol (SLPP); Protocol specification".

[36]	3GPP TS 38.355: "NR; Sidelink Positioning Protocol (SLPP); Protocol specification".

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 23.271 [2],

TS 36.305 [3], TS 36.355 [4] , TS 38.305 [31] , TS 37.355 [32], TS 38.355 [36] apply.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BDS	BeiDou Navigation Satellite System

DL	Downlink

DL-AoD	Downlink Angle-of-Departure

DL-PRS	Downlink Positioning Reference Signal

DL-TDOA	Downlink Time Difference Of Arrival

LCS	Location Services

LPP	LTE Positioning Protocol

MBS	Metropolitan Beacon System

MO-LR	Mobile Originated Location Request

Multi-RTT	Multi-Round Trip Time

MT-LR	Mobile Terminated Location Request

NAS	Non-Access-Stratum

NI-LR	Network Induced Location Request

NR	New Radio

NR E-CID	NR Enhanced Cell ID (positioning method)

OTA	Over The Air

RSCP	Reference Signal Carrier Phase

RSCPD	Reference Signal Carrier Phase Difference

RSRP	Reference Signal Received Power

RSRPP	Reference Signal Received Path Power

SL-AoA	Sidelink Angle-of-Arrival

SL-PRS	Sidelink Positioning Reference Signal

SL-PRS-RSRP	Sidelink PRS Reference Signal Received Power

SL-PRS-RSRPP	Sidelink PRS Reference Signal Received Path Power

SL-RSTD	Sidelink Reference Signal Time Difference

SL-RTOA	Sidelink Relative Time of Arrival

SL-RTT	Sidelink Round Trip Time

SL-TDOA	Sidelink Time Difference Of Arrival

SL-TOA	Sidelink Time Of Arrival

SLPP	Sidelink Positioning Protocol

TBS	Terrestrial Beacon System

TEG	Timing Error Group

UL	Uplink

WLAN	Wireless Local Area Network

UL-SRS	Uplink Sounding Reference Signal

< Unchanged sections omitted >

8	Default Conditions for NR

8.1	LCS Sub-Test Cases

Some test cases defined in clause 9 may include several sub-test cases dependent on the positioning method(s) supported by the UE. Each sub-test case is identified by a sub-test case number as defined in Table 8.1-1. The applicable sub-tests for each test case are specified in the test procedure sequence clause of each test case. If no sub-tests are defined for a specific test case it means that this particular test case is not dependent on a specific positioning method and is applicable in all cases, independently of the positioning method(s) supported by the UE.

Table 8.1-1: Sub-Test Case Numbers for NR

| Sub-Test Case Number | Supported Positioning Methods |
| --- | --- |
| 1 | Void |
| 2 | Void |
| 3 | Void |
| 4 | Void |
| 5 | UE supporting OTDOA (LTE) |
| 6 | UE supporting ECID (LTE) |
| 7 | UE supporting GNSS(1) and OTDOA (LTE) |
| 8 | Void |
| 9 | Void |
| 10 | Void |
| 11 | UE supporting WLAN (Rel-13 only) |
| 12 | UE supporting MBS(2) (Rel-13 only) |
| 13 | UE supporting Bluetooth |
| 14 | UE supporting Sensor (Rel-13 only) |
| 15 | UE supporting GNSS(1) |
| 16 | UE supporting MBS(2) (Rel-14 onwards) |
| 17 | UE supporting WLAN (Rel-14 onwards) |
| 18 | UE supporting Sensor (Rel-14 onwards) |
| 19 | UE supporting Multi-RTT (Rel-16 onwards) |
| 20 | UE supporting DL-AoD (Rel-16 onwards) |
| 21 | UE supporting DL-TDOA (Rel-16 onwards) |
| 22 | UE supporting NR E-CID (Rel-16 onwards) |
| 23 | UE supporting MBS(2) (Rel-16 onwards) |
| 24 | UE supporting Sensor (Rel-16 onwards) |
| 25 | UE supporting GNSS(1) (Rel-15 onwards) |
| 26 | UE supporting Multi-RTT (Rel-17 onwards) |
| 27 | UE supporting DL-AoD (Rel-17 onwards) |
| 28 | UE supporting DL-TDOA (Rel-17 onwards) |
| 29 | UE supporting UL-TDOA (Rel-17 onwards) |
| 30 | UE supporting Multi-RTT (Rel-18 onwards) |
| 31 | UE supporting DL-AoD (Rel-18 onwards) |
| 32 | UE supporting DL-TDOA (Rel-18 onwards) |
| 33 | UE supporting SL-RTT (Rel-18 onwards) |
| 34 | UE supporting SL-AoA (Rel-18 onwards) |
| 35 | UE supporting SL-TDOA (Rel-18 onwards) |
| 36 | UE supporting SL-TOA (Rel-18 onwards) |
| NOTE 1: The GNSS combination of BDS, Galileo, GLONASS, GPS supported by the UE NOTE 2: Metropolitan Beacon System (MBS) is a specific type of Terrestrial Beacon System (TBS) [29] |  |

< Unchanged sections omitted >

8.2.13	Simulated UL-TDOA environment

For UL-TDOA signalling test cases NR Cell 1 is used, as defined in 3GPP TS 38.508-1 [30].

The simulated general NR environment as specified in 8.2.8 also applies.

UL-TDOA test is only applicable for Test Configuration B (Table 8.1A-1). The NR Cell 1 frequency to be used for testing and other default conditions is as specified for signalling test cases in 3GPP TS 38.508-1 [30].

The simulated general NR environment as specified in 8.2.8 also applies.

NR E-CID tests are only applicable for Test Configuration B (Table 8.1A-1). The NR Cell 1 frequency to be used for testing and other default conditions is as specified for signalling test cases in 3GPP TS 38.508-1 [30].

8.2.14	Simulated SL-RTT environment

FFS

8.2.15	Simulated SL-AoA environment

FFS

8.2.16	Simulated SL-TDOA environment

FFS

8.2.17	Simulated SL-TOA environment

FFS

< Unchanged sections omitted >

8.5	Default SLPP message and information elements contents

This clause contains the default values of SLPP messages and information elements used, unless indicated otherwise in specific clauses of this specification.

-	SLPP REQUEST CAPABILITIES

Table 8.5-1: RequestCapabilities

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Not present |  |  |
|  | '00 00 01 00'H | Application Layer ID in initiating UE side | UE-only mode |
| transactionID | (0..255) |  |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | Not present |  |  |
| sessionID | Not present |  |  |
|  | Present. Any value acceptable. | OCTET STRING (SIZE (6)) | UE-only mode |
| acknowledgement | Not present |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestCapabilities SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| requestCapabilities SEQUENCE { |  |  |  |
| commonIEsRequestCapabilities | Present | OCTET STRING containing CommonIEsRequestCapabilities |  |
| commonSL-PRS-MethodsIEsRequestCapabilities | Present | OCTET STRING containing CommonSL-PRS-MethodsIEsRequestCapabilities |  |
| sl-AoA-RequestCapabilities | Present | OCTET STRING containing SL-AoA-RequestCapabilities |  |
| sl-RTT-RequestCapabilities | Present | OCTET STRING containing SL-RTT-RequestCapabilities |  |
| sl-TDOA-RequestCapabilities | Present | OCTET STRING containing SL-TDOA-RequestCapabilities |  |
| sl-TOA-RequestCapabilities | Present | OCTET STRING containing SL-TOA-RequestCapabilities |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UE-only mode | The SLPP signalling procedure is between sidelink UEs |