3GPP TSG-RAN5 Meeting #107	R5-252373

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5447 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT NTN testcase 22.4.30 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Step 24C to 32b test the behaviour when T318 expires, the handling of starting the timers on the SS side and UE side cannot be aligned and this leads to test case failures. At the expiry of T317, T318 is started. At Step 24A, UE starts T317 at reception of SystemInformationBlockType31, The SS timer is only started at 24B when the ESM DATA TRANSPORT is received. If we try to align the start of T317, taking into account the Koffset and SystemInformationBlockType31-NB periodicity, it cannot be guaranteed that SS and UE timing will be in Sync (as it cannot be guaranteed that the UE first SystemInformationBlockType31-NB transmission). At Step 26, if UE T318 has expired before SS T318 expiry UE monitors SIB31, and since we are not broadcasting SIB31 schedule in SIB1 UEs shall bar the cell for NTN. At Step 26, if SS T318 has expired before UE T318 expiry, we schedule SIB31, before T318 has expired at UE side, UE aquires SIB31 and UE loops back the data. Since the start of timer T317 depends on when UE has acquired SIB31, we cannot guarantee the SS and UE timers can be aligned, therefore it is proposed to remove this test purpose |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test case titled updated and TP3 is removed. Steps 24C to 32b1 is removed TP1 is tested as part of steps 23 to 24B, therefore added TP 1 at Step 24A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.30 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

### 22.4.30	NB-IoT / NTN / Ephemeris information update / T317 Expiry

#### 22.4.30.1	Test Purpose (TP)

(1)

with { UE has acquired SystemInformationBlockType31-NB and starts timer T317 with the duration ul-SyncValidityDuration }

ensure that {

when { T317 expires in RRC_CONNECTED state }

then { UE informs lower layers that the UL synchronization is lost and start T318 }

}

(2)

with { UE has started T318 in RRC_CONNECTED state }

ensure that {

when { SystemInformationBlockType31-NB is acquired successfully }

then { UE stops T318 and informs lower layers that the UL synchronization is restored }

}

#### 22.4.30.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331 clauses 5.2.1.3, 5.2.2.4, 5.2.2.39, 5.3.18, 5.3.11.33 and 5.3.7.2; TS 36.321 clause 5.2a. Unless otherwise stated these are Rel-17 requirements.

[TS 36.331, clause 5.2.1.3]

In RRC_CONNECTED, BL UEs or UEs in CE or NB-IoT UEs are not required to acquire system information except when T311 is running, or upon handover where the UE is only required to acquire the MasterInformationBlock in the target PCell, or for UEs in CE to receive ETWS/CMAS information, or upon expiry of T317 where the UE is only required to acquire the SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT). In RRC_IDLE, E-UTRAN may notify BL UEs or UEs in CE or NB-IoT UEs about SI update, and except for NB-IoT, ETWS and CMAS notification, EAB modification and UAC modification, using Direct Indication information, as specified in 6.6 (or 6.7.5 in NB-IoT) and TS 36.212 [22].

NOTE 2:	Upon system information change essential for BL UEs, UEs in CE, or NB-IoT UEs in RRC_CONNECTED, E-UTRAN may initiate connection release.

NOTE 3:	When acquiring SIB31(-NB) in RRC_CONNECTED, UE may assume that the scheduling is unchanged.

[TS 36.331, clause 5.2.2.4]

The UE shall:

…

1>	if the UE is NTN capable:

2>	if schedulingInfoList indicates that SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) is present:

3>	immediately before establishing, resuming or re-establishing an RRC connection; or

3>	if in RRC_CONNECTED and T317 is not running:

4>	acquire SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT);

[TS 36.331, clause 5.2.2.39]

Upon receiving SystemInformationBlockType31 (SystemInformationBlockType31-NB), the UE shall:

1>	start or restart timer T317 with the duration ul-SyncValidityDuration from the subframe indicated by epochTime.

[TS 36.331, clause 5.3.18]

The UE shall:

1>	if in RRC_CONNECTED:

2>	inform lower layers that the UL synchronisation is lost;

2>	start timer T318;

2>	acquire SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) as specified in 5.2.2;

2>	upon successful acquisition of SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT):

3>	stop timer T318;

3>	inform lower layers that the UL synchronisation is restored;

NOTE:	SystemInformationBlockType31 (SystemInformationBlockType31-NB in NB-IoT) may be broadcast on a different narrowband or different NB-IoT carrier than the one configured to the UE.

[TS 36.331, clause 5.3.7.2]

The UE shall only initiate the procedure either when AS security has been activated or for a NB-IoT UE supporting RRC connection re-establishment for the Control Plane CIoT EPS/5GS optimisation. The UE initiates the procedure when one of the following conditions is met:

[TS 36.321, clause 5.2a]

If upper layer informs that the UL synchronization is lost according to the clause 5.3.18 of TS 36.331 [8], the MAC entity shall:

-	flush all HARQ buffers;

-	not perform any uplink transmission.

If upper layer informs that the UL synchronization is restored for the SpCell according to the clause 5.3.18 of TS36.331 [8], uplink transmissions can be performed.

#### 22.4.30.3	Test description

##### 22.4.30.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 is configured according to Table 8.1.4.2-1 and Table 8.1.4.2-2 in TS 36.508 [18].

-	System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used.

UE:

-	The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble:

-	The UE is in State 1-NB switched off according to TS 36.508 [18].

##### 22.4.30.3.2	Test procedure sequence

Table 22.4.30.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on UE. | - | - | - | - |
| 2 | UE acquires SystemInformationBlockType31-NB and start T317 = 60 seconds. | - | - | - | - |
| 3-18b1 | Steps 1 to 16b1 of the Attach, connected mode and test mode activated procedure described in TS 36.508 [18] subclause 8.1.5.2A.3 with TEST LOOP MODE MODE G (loopback of the User data container content of any received downlink ESM DATA TRANSPORT) are performed on Ncell 1. Note: After receiving the message in step 4, SS start a timer T1 with value = T317. | - | - | - | - |
| 18A-18B | Steps 1 to 2 of the UE Test Loopback Activated procedure described in TS 36.508 [18] subclause 8.1.5.2B.3 are performed on Ncell 1. | - | - | - | - |
| 19 | SS stops broadcasting SystemInformationBlockType31-NB. | - | - | - | - |
| 19 A | SS stops periodic grant allocation | - | - | - | - |
| 20 | The SS transmits one IP packet embedded in an ESM DATA TRANSPORT and DLInformationTransfer-NB. | <-- | NAS: ESM DATA TRANSPORT | - | - |
| 21-22 | Void | - | - | - | - |
| 23 | Wait until expiry of T1. Note: UE starts timer T318. SS start a timer T2 with value = T318. | - | - | - | - |
| 24 | The SS starts the UL periodic grant transmissionallowing the UE to return the IP packet as received in step 20. | - | - | - | - |
| 24A | SS resumes broadcasting SystemInformationBlockType31-NB. Note: Timer T318 shall be stopped and T317 shall be started in UE side. | - | - | - | - |
| 24B | Check: Does the UE transmit one IP packet embedded in an ESM DATA TRANSPORT and ULInformationTransfer-NB corresponding to grant in step 24? | --> | NAS: ESM DATA TRANSPORT | 1, 2 | P |
| 24C-32 | Void | - | - | - | - |

##### 22.4.30.3.3	Specific message contents

Table 22.4.30.3.3-1: Void

Table 22.4.30.3.3-2: SystemInformationBlockType31-NB (steps 2, and 24A, Table 22.4.30.3.2-1)

| Derivation Path: TS 36.508 [18], Table 8.1.4.3.3-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ul-SyncValidityDuration-r17 | s60 | Value of T317 |  |
| } |  |  |  |
| } |  |  |  |

Table 22.4.30.3.3-3: CLOSE UE TEST LOOP (step 18A, Table 22.4.30.3.2-1)

| Derivation path: 36.508 Table 8.1.5.2B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode | '00000110'B | UE test loop mode G setup | TL_MODE_G |
| Operation mode and repetitions |  |  |  |
| M0 | 0 |  | return_via_EMM_SMC |
| R6..R0 | '0000001'B | 1 The received DL message in uplink shall be looped back 1 time (once) |  |
| Uplink data delay | '00000000'B | T_delay_modeGH timer = 0 sec |  |