3GPP TSG-RAN5 Meeting #107		R5-252409

Malta, Malta, 19th – 23rd May 2025

Agenda item:	5.3.3.5

Source:   Verizon

Title:       Discussions on 38.521-3 Inter-band NRDC cases between FR1 and FR2

Document for:	DISCUSSION AND ENDORSEMENT

# 1	Introduction

Currently in 38.521-3, PRD21 completed NRDC between FR1 and FR2 band combos are not listed in corresponding band configuration tables in Chapter 5. In addition, for transmitter and receiver characteristics testing, there is no separate test configurations and test procedures for cases of NRDC between FR1 and FR2, except 6.2B.5 (Configured output power for NR-DC). There is a need to close the lope of NRDC cases between FR1 and FR2 in 38.521-3, so that it can be referred from the certification organizations properly.

# 2	Discussion

### 2.1	Inter-band NRDC cases between FR1 and FR2 in 38.521-3

Currently in 38.521-3, the PRD21 completed NRDC band combos are not listed in Chapter 5 (Operating bands and channel arrangement). In addition, for Chapter6 and Chapter7 transmitter and receiver characteristics, there is no separate test configurations and test procedures for cases of NRDC between FR1 and FR2, except 6.2B.5 (Configured output power for NR-DC), while the ENDC part is fully specified. There is a need to close the lope of NRDC cases between FR1 and FR2 in 38.521-3, so that it can be referred properly from the certification organizations for certification purposes.

Observation 1: In section 5.2 (Operating bands for CA), Table 5.2A.1-1: Band combinations for inter-band NR CA between FR1 and FR2, is empty with the following editor’s note, while PRD21 has a list of completed NRDC between FR1 FR2 band combos which are operated in the current network

Observation 2: In section 5.5A.1 (Configuration for CA), Table 5.5A.1-1: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands), is empty with the following editor’s note, while PRD21 has a list of completed NRCA between FR1 FR2 band combos which are operated in the current network

Observation 3: In section 5.5B (configuration for DC), the subsection 5.5B.1 refers to section 5.5A.1 of NR CA configurations between FR1 FR2, which has an empty table (see Observation 2)

Observation 4: In section 5.5B (configuration for DC), the subsection 5.5B.7 refers to section 5.5A.1 of NR CA configurations between FR1 FR2, which has an empty table (see Observation 2)

Observation 5: In section 5.5B (configuration for DC) subsection 5.5B.7, Table 5.5B.7-1: Inter-band NR-DC configurations between FR1 and FR2 (two bands), is empty with the following editor’s note, while PRD21 has a list of completed NRDC between FR1 FR2 band combos which are operated in the current network

Observation 6: For Chapter6 and Chapter7 transmitter and receiver characteristics, there is no separate test configurations and test procedures for cases of NRDC between FR1 and FR2, except 6.2B.5 (Configured output power for NR-DC). This issue is clarified by RAN4 and RAN5 CRs ([1] and [2]) through modifying the general sections of Chapter 6 and Chapter 7 with the following statement:

Proposal 1: Complete Table 5.2A.1-1: Band combinations for inter-band NR CA between FR1 and FR2, with operator completed band combos

Proposal 2: Complete Table 5.5A.1-1: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands), with operator completed band combos

Proposal 3: Complete Table 5.5B.7-1: Inter-band NR-DC configurations between FR1 and FR2 (two bands), with operator completed band combos

Proposal 4: Include the following Note to general sections of 6.1 and 7.1 to further clarify the NRDC between FR1 FR2 test requirements:

Note: For NRDC between FR1 FR2, no test cases are specified. The related tests are performed for FR1 and FR2 separately and independently. Follow the TS 38.521-1 to test the corresponding procedures for FR1 part and TS 38.521-2 to test the corresponding procedures for FR2 part.

# 3	Summary

Observation 1: In section 5.2 (Operating bands for CA), Table 5.2A.1-1: Band combinations for inter-band NR CA between FR1 and FR2, is empty with the following editor’s note, while PRD21 has a list of completed NRDC between FR1 FR2 band combos which are operated in the current network

Observation 2: In section 5.5A.1 (Configuration for CA), Table 5.5A.1-1: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands), is empty with the following editor’s note, while PRD21 has a list of completed NRCA between FR1 FR2 band combos which are operated in the current network

Observation 3: In section 5.5B (configuration for DC), the subsection 5.5B.1 refers to section 5.5A.1 of NR CA configurations between FR1 FR2, which has an empty table (see Observation 2)

Observation 4: In section 5.5B (configuration for DC), the subsection 5.5B.7 refers to section 5.5A.1 of NR CA configurations between FR1 FR2, which has an empty table (see Observation 2)

Observation 5: In section 5.5B (configuration for DC) subsection 5.5B.7, Table 5.5B.7-1: Inter-band NR-DC configurations between FR1 and FR2 (two bands), is empty with the following editor’s note, while PRD21 has a list of completed NRDC between FR1 FR2 band combos which are operated in the current network

Proposal 1: Complete Table 5.2A.1-1: Band combinations for inter-band NR CA between FR1 and FR2, with operator completed band combos

Proposal 2: Complete Table 5.5A.1-1: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands), with operator completed band combos

Proposal 3: Complete Table 5.5B.7-1: Inter-band NR-DC configurations between FR1 and FR2 (two bands), with operator completed band combos

Proposal 4: Include the following Note to general sections of 6.1 and 7.1 to further clarify the NRDC between FR1 FR2 test requirements:

Note: For NRDC between FR1 FR2, no test cases are specified. The related tests are performed for FR1 and FR2 separately and independently. Follow the TS 38.521-1 to test the corresponding procedures for FR1 part and TS 38.521-2 to test the corresponding procedures for FR2 part.

CR for proposal 1, 2, 3: R5-252415

CRs for proposal 4: R5-252418, R5-252419

# 4	References

- R4-2322000 - [NR_newRAT-Core] Clarifications for FR2 testing with NR-DC and NR-CA; 3GPP TSG-RAN WG4 #109; Chicago, United States, 13th Nov 2023 - 17th Nov 2023

- R5-237963 - Clarifications for FR2 testing with NR-DC and NR-CA; 3GPP TSG-RAN5 Meeting #101; Chicago, United States, 13th Nov 2023 - 17th Nov 2023

TS 38.101-3: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios (Release 18)

TS 38.521-3: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios (Release 18)