3GPP TSG-RAN5 Meeting #107	R5-252455

# St Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3476 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test procedure for IMS MT call release in 5GC - release IMS data channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_IMS_dataCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For the IMS data channel (IMS DC) test cases, IMS data channel may be added simutaniously with Voice/Video Call so when SS reelase the IMS Call, the resource for IMS DC also need to release together. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Release the Qos flow and DRB for IMS DC when the MT Call release. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Resource for IMS DC may maintain after call release. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.9.18	Test procedure for IMS MT call release in 5GC

4.9.18.1	Scope

The purpose of this procedure is to make the network release an ongoing IMS call.

4.9.18.2	Procedure description

4.9.18.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The UE is in state 3N-A on the NR Cell with an active IMS call.

4.9.18.2.2	Procedure sequence

Table 4.9.18.2.2-1: IMS MT call release in 5GC

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1-2 | Generic test procedure for MT release of speech call, steps 1-2, as defined in annex A.8 of TS 34.229-5 [47] are performed. | - | - | - | - |
| 3 | The SS transmits an RRCReconfiguration message and an PDU SESSION MODIFICATION COMMAND | <-- | NR RRC: RRCReconfiguration 5GMM: DL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| - | EXCEPTION: Steps 4 and 5 can occur in any order. | - | - | - | - |
| 4 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 5 | Check: Does the UE transmit a ULInformationTransfer message, an UL NAS TRANSPORT message and an PDU SESSION MODIFICATION COMPLETE message? | --> | NR RRC: ULInformationTransfer 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION MODIFICATION COMPLETE | - | P |

4.9.18.2.3	Specific message contents

Table 4.9.18.2.3-1: RRCReconfiguration (step 3, Table 4.9.18.2.2-1)

| Derivation Path: Table 4.6.1-13. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig SEQUENCE { |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { |  | 1 entry |  | IMS_VOICE |
| DRB-Identity[1] |  | DRB-Identity linked to the IMS speech bearer | entry 1 |  |
| } |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { |  | 2 entries |  | IMS_VIDEO |
| DRB-Identity[1] |  | DRB-Identity linked to the IMS video bearer | entry 1 |  |
| DRB-Identity[2] |  | DRB-Identity linked to the IMS video bearer | entry 2 |  |
| } |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { |  | 2 entries |  | IMS_ VOICE AND IMS_BDC |
| DRB-Identity[1] |  | DRB-Identity linked to the IMS speech bearer | entry 1 |  |
| DRB-Identity[2] |  | DRB-Identity linked to the IMS data channel bearer | entry 2 |  |
| } |  |  |  |  |
| drb-ToReleaseList SEQUENCE (SIZE (1..maxDRB)) OF DRB-Identity { |  | 3 entries |  | IMS_VIDEO AND IMS_BDC |
| DRB-Identity[1] |  | DRB-Identity linked to the IMS speech bearer | entry 1 |  |
| DRB-Identity[2] |  | DRB-Identity linked to the IMS video bearer | entry 2 |  |
| DRB-Identity[3] |  | DRB-Identity linked to the IMS data channel bearer | entry 3 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup SEQUENCE { |  |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1..maxLC-ID)) OF LogicalChannelIdentity { |  | 1 entry | entry 1 | IMS_VOICE |
| LogicalChannelIdentity[1] |  | Same value as drb-Identity[1] above |  |  |
| } |  |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1..maxLC-ID)) OF LogicalChannelIdentity { |  | 2 entries |  | IMS_VIDEO |
| LogicalChannelIdentity[1] |  | Same value as drb-Identity[1] above | entry 1 |  |
| LogicalChannelIdentity[2] |  | Same value as drb-Identity[2] above | entry 2 |  |
| } |  |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1..maxLC-ID)) OF LogicalChannelIdentity { |  | 2 entries |  | IMS_ VOICE AND IMS_BDC |
| LogicalChannelIdentity[1] |  | Same value as drb-Identity[1] above | entry 1 |  |
| LogicalChannelIdentity[2] |  | Same value as drb-Identity[2] above | entry 2 |  |
| } |  |  |  |  |
| rlc-BearerToReleaseList SEQUENCE (SIZE (1..maxLC-ID)) OF LogicalChannelIdentity { |  | 3 entries |  | IMS_VIDEO AND IMS_BDC |
| LogicalChannelIdentity[1] |  | Same value as drb-Identity[1] above | entry 1 |  |
| LogicalChannelIdentity[2] |  | Same value as drb-Identity[2] above | entry 2 |  |
| LogicalChannelIdentity[3] |  | Same value as drb-Identity[3] above | entry 3 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| IMS_VOICE | If this QoS rule is used to setup an IMS voice session |
| IMS_VIDEO | If this QoS rule is used to setup an IMS video session |
| IMS_BDC | If this QoS rule is used to setup an IMS bootstrap data channel session |

Table 4.9.18.2.3-2: PDU SESSION MODIFICATION COMMAND (step 3, Table 4.9.18.2.2-1)

| Derivation Path: Table 4.7.2-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PDU session ID |  | Same value as sent in PDU SESSION ESTABLISHMENT REQUEST message. |  |  |
| Authorized QoS rules |  |  |  |  |
| QoS rule[1] |  |  |  |  |
| QoS rule identifier |  | ‘00000011’B | QoS rule 3 | IMS_VOICE, IMS_VIDEO |
| Rule operation code |  | ‘010’B | Delete existing QoS rule |  |
| QoS rule[2] |  |  |  | IMS_VIDEO |
| QoS rule identifier |  | ‘00000100’B | QoS rule 4 |  |
| Rule operation code |  | ‘010’B | Delete existing QoS rule |  |
| QoS rule[3] |  |  |  | IMS_BDC |
| QoS rule identifier |  | ‘00000101’B | QoS rule 5 |  |
| Rule operation code |  | ‘010’B | Delete existing QoS rule |  |
| Authorized QoS flow descriptions |  |  |  |  |
| QoS flow descriptions[1] |  |  |  |  |
| QFI |  | ‘00000111’B | QFI 7 | IMS_VOICE, IMS_VIDEO |
| Operation code |  | ‘010’B | Delete existing QoS flow |  |
| QoS flow descriptions[2] |  |  |  | IMS_VIDEO |
| QFI |  | ’00001000’B | QFI 8 |  |
| Operation code |  | ‘010’B | Delete existing QoS flow |  |
| QoS flow descriptions[3] |  |  |  | IMS_BDC |
| QFI |  | ’00001001’B | QFI 9 |  |
| Operation code |  | ‘010’B | Delete existing QoS flow |  |

| Condition | Explanation |
| --- | --- |
| IMS_VOICE | If this QoS rule is used to setup an IMS voice session |
| IMS_VIDEO | If this QoS rule is used to setup an IMS video session |
| IMS_BDC | If this QoS rule is used to setup an IMS bootstrap data channel session |

<End of modified section 1>