3GPP TSG-RAN5 Meeting #107	R5-252459

# St Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1605 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new LTE IMS data channel test case 12.33 - MO Voice call with precondition and BDC success |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_IMS_dataCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This new IMS data channel (IMS DC) test case is added to test: UE send SDP offer with IMS DC and NW reply SDP answer with IMS DC. The UE can establish voice call with precondition + IMS DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new LTE IMS data channel test case 12.33 IMS Data Channel / DCMTSI MO Voice call / Bootstrap Data Channel success / with preconditions at both originating UE and terminating UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NG_RTC_IMS_dataCH-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.33 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0369 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

12.33	IMS Data Channel / DCMTSI MO Voice call / Bootstrap Data Channel success / with preconditions at both originating UE and terminating UE

12.33.1	Definition

Test to verify that the UE correctly performs IMS mobile originated voice call setup with preconditions and IMS Bootstrap data channel setup. This process is described in 3GPP TS 23.228 [3] ,3GPP TS 24.186 [159] and TS 26.114 [66].

12.33.2	Conformance requirement

Same as 12.32.2

12.33.3	Test purpose

1)	To verify that UE setups IMS Bootstrap data channel successfully when initiating MO Voice call or after the establishment of MO Voice call; and

2)	To verify that UE includes media feature tag +sip.app-subtype="webrtc-datachannel" in (re-)INVITE message.

3)	To verify that UE includes correct SDP offer for IMS data channel media negotiation in (re-)INVITE message.

12.33.4	Method of test

Initial conditions

UE contains either SIM application (GIBA), ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security). UE is configured to use the precondition mechanism. UE is configured to use IMS data channel.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-9)	UE executes the procedures described in TS 36.508 [94] table 4.5A.6.3-1 steps 1 to 14. If INVITE in step2 does not include SDP offer containing media lines for IMS data channel, Step 14a1-14a4 are excuted to establish IMS data channel after voice call establishment. If INVITE in step2 include SDP offer containing media lines for both voice and IMS data channel, SS accept IMS data channel by replying the SDP answer without media-level attributes “a = dcmap”, and the qos flow for IMS bootstrap data channel (specified in TS 36.508[94] table 4.5A.32.3-1 ) is established after 200OK for INVITE.

14a1)	UE send re-INVITE with an SDP offer containing media lines for both voice and IMS data channel.

14a2)	 SS responds with a 100 Trying provisional response.

14a3)	SS responds re-INVITE with 200 OK, and SS reject IMS data channel by repling the SDP answer without media-level attributes “a = dcmap”, then SS establishs qos flow for IMS bootstrap data channel (specified in TS 36.508[94] table 4.5A.32-1.

14a4)	UE acknowledge the receipt of 200 OK for re-INVITE.

15-16)	Test procedure for MT release of IMS call defined in annex C.33 steps 1 to 4 is executed.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-13 |  |  | Steps 1-13 defined in annex C.21 | MTSI MO speech call. Referred from 36.508 [94] table 4.5A.6.3-1 for a UE with E-UTRA support. If INVITE in step2 include SDP offer containing media lines for both voice and IMS data channel, the qos flow for IMS bootstrap data channel is established after 200OK for INVITE message, and the test procedure is referred from 36.508 [94] table 4.5A.32.3-1.. |
| - | - |  | EXCEPTION: Step 14a1 to 14a4 executed if INVITE in step2 does not include SDP offer containing media lines for IMS data channel. |  |
| 14a1 |  |  | INVITE | UE send re-INVITE with an SDP offer containing media lines for both voice and IMS data channel. |
| 14a2 |  |  | 100 Trying | SS responds with a 100 Trying provisional response. |
| 14a3 |  |  | 200 OK | SS responds re-INVITE with 200 OK. |
| 14a4 |  |  | ACK | UE acknowledge the receipt of 200 OK for re-INVITE. |
| 15-16 |  |  | Steps 1-2 defined in annex C.33 | MT release of IMS call. Referred from C.33. |

Specific Message Contents

INVITE (Step 2)

Use the default message “INVITE” in annex C.21 with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Step 2 of C.21 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Message-body |  | SDP body is same as Step 2 of C.21 or same as Step 2 of C.21 except adding at least one Media description for IMS bootstrap data channel as follow: Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel c=IN (addrtype) (connection-address for UE) [Note 1] Attributes for media for IMS data channel: a=max-message-size:(max-message-size-value) [Note 2] a=sctp-port:(sctp-port-value) [Note 3] a=fingerprint:(hash-func) (fingerprint) [Note 4] a=setup:actpass a=tls-id:(tls-id-value) a=dcmap: (dcmap-stream-id) subprotocol="http” [Note 5,6] Note 1: At least one "c=" field shall be present. Note 2: Optional attribute. If the SDP ”max-message-size” attribute is not present the default value is 64K. Note 3: SCTP port range is between 0 and 65535(both included).Leading zeroes must not be used. Note 4: SHA-1,SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note 5: Multiple data channels may be mapped to a single data channel SDP media description, each with a corresponding "a=dcmap" SDP attribute and stream IDs that are unique within that media description. Note 6: dcmap-stream-id range is between 0 and 999(both included). |

183 Session Progress (Step 4)

Use the default message “183 Session Progress” in annex C.21 with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Step 4 of C.21 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Message-body | A1 A2 | SDP body is same as Step 4 of C.21 except adding one Media description as follow: Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel [Note 1] b=AS:500 Attributes for media for IMS data channel: a=max-message-size:64000 a=sctp-port:(sctp-port-value) [Note 2] a=setup:passive a=fingerprint: (hash-func) (fingerprint) [Note 3] a=tls-id: (tls-id-value) [Note 4] a=dcmap: (dcmap-stream-id) subprotocol="http" [Note 5] SDP body is same as TS 34.229-5, Step 3 of A.4.1a Note 1: Transport port is the port number of the SS. Note 2: The sctp-port-value is the SCTP port of the SS. Note 3: SHA-1,SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note 4: The tls-id-value is random number generated by SS. Note 5: The dcmap-stream-id is the same as the dcmap-stream-id in the INVITE message. |

| Condition | Explanation |
| --- | --- |
| A1 | INVITE includes SDP offer containing media lines both for voice and IMS data channel in Step2. |
| A2 | INVITE does not include SDP offer containing media lines for IMS data channel in Step2. |

UPDATE (Step 7)

Use the default message “UPDATE” in annex C.21 with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Message-body | A1 A2 | SDP body is same as Step 7 of C.21 except adding one Media description as follow: Media description for IMS data channel: Same as it in the INVITE message Attributes for media for IMS data channel: Same as it in the INVITE message with following exception: a = setup: active or actpass SDP answer is same as Step 7 of C.21 |

| Condition | Explanation |
| --- | --- |
| A1 | INVITE includes SDP offer containing media lines both for voice and IMS data channel in Step2. |
| A2 | INVITE does not include SDP offer containing media lines for IMS data channel in Step2. |

200OK for UPDATE (Step 8)

Use the default message “200OK for UPDATE” in annex C.21 with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Step 8 of C.21 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Message-body | A1 A2 | SDP body is same as Step 8 of C.21 except adding one Media description as follow: Media description for IMS data channel: Same as it in the 183 Session Progress message. Attributes for media for IMS data channel: Same as it in the 183 Session Progress message. SDP answer is same as Step 8 of C.21 |

| Condition | Explanation |
| --- | --- |
| A1 | INVITE includes SDP offer containing media lines both for voice and IMS data channel in Step2. |
| A2 | INVITE does not include SDP offer containing media lines for IMS data channel in Step2. |

200OK for INVITE(Step 12)

Use the default message “200OK for INVITE” in annex C.21 with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Step 12 of C.21 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |

Re-INVITE (Step 14a1)

Use the default message “INVITE for MO Call” in annex A.2.1 with condition A5

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Annex A.2.1 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Supported option-tag |  | precondition |
| Message-body |  | SDP body is same as Step 7 of C.21 (UPDATE) with following exception: Attributes for preconditions of audio media: a=curr:qos remote sendrecv Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel c=IN (addrtype) (connection-address for UE) [Note 1] Attributes for media for IMS data channel: a=max-message-size:(max-message-size-value) [Note 2] a=sctp-port:(sctp-port-value) [Note 3] a=fingerprint:(hash-func) (fingerprint) [Note 4] a=setup:actpass a=tls-id:(tls-id-value) a=dcmap: (dcmap-stream-id) subprotocol="http” [Note 5,6] Note 1: At least one "c=" field shall be present. Note 2: Optional attribute. If the SDP ”max-message-size” attribute is not present the default value is 64K. Note 3: SCTP port range is between 0 and 65535(both included).Leading zeroes must not be used. Note 4: SHA-1,SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note 5: Multiple data channels may be mapped to a single data channel SDP media description, each with a corresponding "a=dcmap" SDP attribute and stream IDs that are unique within that media description. Note 6: dcmap-stream-id range is between 0 and 999(both included). |

100 Trying (Step 14a2)

Use the default message “100 Trying” in annexA.2.2 with condition A1.

200OK (Step 14a3)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annexA.3.1 with condition A23.

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Annex A.3.1 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Supported option-tag |  | precondition |
| Content-Type media-type |  | application/sdp |
| Content-Length value |  | length of message-body |
| Message-body |  | SDP body is same as Step 8 of C.21(200OK for UPDATE) with follow exception: Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel [Note 1] b=AS:500 Attributes for media for IMS data channel: a=max-message-size:64000 a=sctp-port:(sctp-port-value) [Note 2] a=setup:passive a=fingerprint: (hash-func) (fingerprint) [Note 3] a=tls-id: (tls-id-value) [Note 4] a=dcmap: (dcmap-stream-id) subprotocol="http" [Note 5] Note 1: Transport port is the port number of the SS. Note 2: The sctp-port-value is the SCTP port of the SS. Note 3: SHA-1,SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note 4: The tls-id-value is random number generated by SS. Note 5: The dcmap-stream-id is the same as the dcmap-stream-id in the INVITE message. |

ACK (Step 14a4)

Use the default message “ACK” in annexA.2.7 with condition A5.

<End of modified section 1>