3GPP TSG-RAN5 Meeting #107	R5-252462

# St Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0665 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new NR IMS data channel test case 7.37 - MO Voice call without precondition and BDC failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_IMS_dataCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This new IMS data channel (IMS DC) test case is added to test: UE send SDP offer with IMS DC and NW reply SDP answer using port 0 in m line for IMS DC. The UE can establish voice call without precondition even if IMS DC establishment failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new NR IMS data channel test case 7.37 IMS Data Channel / DCMTSI MO Voice call / Bootstrap Data Channel failure / without preconditions at both originating UE and terminating UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NG_RTC_IMS_dataCH-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.37 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0368 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 34.229-1: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[3]	3GPP TS 34.229-2: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification".

[4]	3GPP TS 34.229-3: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 3: Abstract Test Suites (ATS)".

[5]	3GPP TS 34.229-4: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 4: Enabler for IP multimedia applications testing".

[6]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[7]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[8]	ETSI ES 201 873-1: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 1: TTCN-3 Core Language".

[9]	ETSI ES 201 873-2: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 2: TTCN-3 Tabular Presentation Format (TFT)".

[10]	ETSI TR 201 873-3: "Methods for Testing and Specification (MTS); The Testing and Test Control Notation version 3; Part 3: TTCN-3 Graphical Presentation Format (GFT)".

[11]	ETSI ETS 300 406: "Methods for testing and Specification (MTS); Protocol and profile conformance testing specifications; Standardization methodology".

[12]	ISO/IEC 9646-1: "Information technology - Open systems interconnection - Conformance testing methodology and framework - Part 1: General concepts".

[13]	ISO/IEC 9646-7: "Information technology - Open systems interconnection - Conformance testing methodology and framework - Part 7: Implementation Conformance Statements".

[14]	3GPP TS 24.341: "Support of SMS over IP networks; Stage 3".

[15]	IETF RFC 3310: "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA)".

[16]	3GPP TS 33.203: "3G security;Access security for IP-based services".

[17]	IETF RFC 3329: "Security Mechanism Agreement for the Session Initiation Protocol (SIP)".

[18]	IETF RFC 3680: "A Session Initiation Protocol (SIP) Event Package for Registrations".

[19]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[20]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[21]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[22]	3GPP TS 27.007: "AT command set for User Equipment (UE)".

[23]	IETF RFC 2617: "HTTP Authentication: Basic and Digest Access Authentication".

[24]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[25]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[26]	3GPP TS 24.237: "IP Multimedia (IM) Core Network (CN) subsystem IP Multimedia Subsystem (IMS) Service Continuity".

[27]	3GPP TS 23.003: "Numbering, addressing and identification".

[28]	IETF RFC 6665: “SIP-Specific Event Notification”.

[29]	IETF RFC 3312: “Integration of Resource Management and SIP”.

[30]	IETF RFC 3262: “Reliability of Provisional Responses in the Session Initiation Protocol (SIP)”.

[31]	GSMA PRD NG.114: "IMS Profile for Voice, Video and Messaging over 5GS".

[32]	3GPP TS 24.610: "Communication HOLD (HOLD) using IP Multimedia (IM) Core Network (CN) subsystem".

[33]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[34]	3GPP TS 24.606: "Message Waiting Indication (MWI) using IP Multimedia (IM) Core Network (CN) subsystem".

[35]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem".

[36]	3GPP TS 24.629: "Explicit Communication Transfer (ECT) using IP Multimedia (IM) Core Network (CN) subsystem".

[37]	IETF RFC 4028: "Session Timers in the Session Initiation Protocol (SIP)".

[38]	IETF RFC 4566: "SDP: Session Description Protocol".

[39]	IETF RFC 7462: "URNs for the Alert-Info Header Field of the Session Initiation Protocol (SIP) ".

[40]	IETF RFC 3891: "The Session Initiation Protocol (SIP) "Replaces" Header".

[41]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[42]	IETF RFC 3326: "The Reason Header Field for the Session Initiation Protocol (SIP)".

[43]	3GPP TS 24.109: "Bootstrapping interface (Ub) and network application function interface (Ua); Protocol details".

[44]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture".

[45]	IETF RFC 4825: "The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)".

[46]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

[47]	IETF RFC 3310: "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA)".

[48]	3GPP TS 38.509: "5GS; Special conformance testing functions for User Equipment (UE)".

[49]	IETF RFC 6228: "Session Initiation Protocol (SIP) Response Code for Indication of Terminated Dialog".

[50]	IETF RFC 3323: "A Privacy Mechanism for the Session Initiation Protocol (SIP)".

[51]	IETF RFC 3325: "Private Extensions to the Session Initiation Protocol (SIP) for Asserted Identity within Trusted Networks".

[52]	3GPP TS 38.508-2: "5GS; User Equipment (UE) conformance specification; Part 2: Common
Implementation Conformance Statement (ICS) proforma".

[53]	3GPP TS 36.523-3: "LTE; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 3: Test suites".

[aa]	3GPP TS 24.186: "IMS Data Channel applications; Protocol specification".

[bb]	IETF RFC 8841: "Session Description Protocol (SDP) Offer/Answer Procedures for Stream Control Transmission Protocol (SCTP) over Datagram Transport Layer Security (DTLS) Transport".

[cc]	IETF RFC 8122: "Connection-Oriented Media Transport over the Transport Layer Security (TLS) Protocol in the Session Description Protocol (SDP)".

[dd]	IETF RFC 8842: " Session Description Protocol (SDP) Offer/Answer Considerations for Datagram Transport Layer Security (DTLS) and Transport Layer Security (TLS)".

[ee]	IETF RFC 8864: "Negotiation Data Channels Using the Session Description Protocol (SDP)".

[ff]	IETF RFC 4145: "TCP-Based Media Transport in the Session Description Protocol (SDP)".

<End of modified section 1>

<Start of modified section 2>

7.37	IMS Data Channel / DCMTSI MO Voice call / Bootstrap Data Channel failure / without preconditions at both originating UE and terminating UE

7.37.1	Test Purpose (TP)

(1)

with { UE supporting IMS data channel and is allowed to use IMS data channel and UE determined its home network supports the IMS data channel capability }

ensure that {

when { UE is made to establish IMS Voice Call }

then { UE shall include SDP media-level attributes "a=dcmap" in data channel media negotiation in initial INVITE request or a re-INVITE request }

}

(2)

with { UE having send (re-)INVITE without preconditions, and (re-)INVITE include SDP media-level attributes "a=dcmap" in data channel media negotiation }

ensure that {

when { receiving SDP answer without media-level attributes "a=dcmap" }

then { UE proceeds the voice call without preconditions}

}

(3)

with { UE supporting IMS data channel and is allowed to use IMS data channel and UE determined its home network supports the IMS data channel capability }

ensure that {

when { UE is made to establish IMS Voice Call }

then { UE shall include the media feature tag for supported streaming media type in the Contact header field to the remote UE and use +sip.app-subtype="webrtc-datachannel" in the initial INVITE request }

}

(4)

with { UE supporting IMS data channel and is allowed to use IMS data channel and UE determined its home network supports the IMS data channel capability }

ensure that {

when { UE determins to establish the IMS data channels after the establishment of the Voice Call }

then { UE shall include the media feature tag for supported streaming media type in the Contact header field to the remote UE and use +sip.app-subtype="webrtc-datachannel" in the re-INVITE request }

}

7.37.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-18 requirements.

[TS 24.186 clause 9.2.3.1]

Upon generating an initial INVITE request or a re-INVITE request, the UE supporting IMS data channel and is allowed to use IMS data channel and if the UE determined its home network supports the IMS data channel capability, the UE shall include the media feature tag defined in IETF RFC 5688 [5] for supported streaming media type in the Contact header field to the remote UE and use +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4], regardless of IMS data channel media description being part of the SDP or not. The UE may include in the initial INVITE request an Accept-Contact header field containing the "sip.app-subtype" media feature tag defined in IETF RFC 5688 [5] with a value of "webrtc-datachannel" as specified in 3GPP TS 24.173 [10].

[TS 24.186 clause 9.3.2.1.2]

If the UE initiates an MMTel session with IMS data channels, the UE:

1)	shall generate an initial INVITE request in accordance with 3GPP TS 24.229 [9] and 3GPP TS 24.173 [10];

2)	shall include the media feature tag defined in IETF RFC 5688 [5] for supported streaming media type with +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4] in the Contact header field;

3)	may include an Accept-Contact header field containing the "sip.app-subtype" media feature tag defined in IETF RFC 5688 [5] with a value of "webrtc-datachannel" as specified in 3GPP TS 26.114 [4]; and

4)	if the configuration described in clause 9.3.2.1.1, allows the establishment of bootstrap data channels simultaneously with the setup of the MMTel session, shall include an SDP offer containing the media descriptions for the MMTel media according 3GPP TS 24.173 [10] and data channel media descriptions for both the local and remote bootstrap data channels in accordance with 3GPP TS 26.114 [4].

[TS 24.186 clause 9.3.2.1.3.1]

If the UE wants to establish a bootstrap data channel, the UE shall take into account the data channel configuration as specified in clause 9.3.2.1.1, and if the UE determines that the configuration allows the establishment of IMS data channels after the establishment of the MMTel session, the UE shall:

1)	generate a re-INVITE request in accordance with 3GPP TS 24.229 [9] and 3GPP TS 24.173 [10];

2)	include the media feature tag defined in IETF RFC 5688 [5] for supported streaming media type with +sip.app-subtype="webrtc-datachannel" as specified in 3GPP TS 26.114 [4] in the Contact header field; and

3)	include an updated SDP offer that contains data channel media descriptions for both the local and remote bootstrap data channels according to 3GPP TS 26.114 [4].

[TS 26.114 clause 6.2.10.1]

A bootstrap data channel shall be configured as ordered, reliable, with normal SCTP multiplexing priority. The sub-protocol for a bootstrap data channel shall be HTTP (not encapsulating HTTP in TCP), represented by the following, example SDP "a=dcmap" line, which therefore shall be present in each data channel media description in an SDP offer from a DCMTSI client in terminal:

a=dcmap:0 subprotocol="http"

[TS 26.114 clause 6.2.10.3]

An answering DCMTSI client in terminal that desires to reject the entire SCTP association for all offered data channels shall set the port to 0 (zero) on the corresponding "m=application" line in SDP, as described in [172]. An SCTP association that initially, or as a result of session modification, has no open data channels ("a=dcmap" lines) should be rejected or closed by modifying the session, setting port number to 0 (zero).

[TS 26.114 clause 6.2.10.4]

An offering DCMTSI client in terminal receiving an SDP answer where the data channel SCTP association is accepted (port is not 0) may use any offered stream ID that has a corresponding "a=dcmap" line in the SDP answer, as described by section 6.5 in [172]. Data channels with "a=dcmap" lines in the SDP offer that are not included in the SDP answer must be considered as rejected and shall not be used, as described by section 6.5 in [172]. The "req-app-id" parameter of the "a=3gpp-req-app" attribute is used to identify the application for which the application data channels are added/updated.

7.37.3	Test description

7.37.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

-	The UE is configured to not use preconditions.

-	The UE is configured to use IMS data channel.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS.

7.37.3.2	Test procedure sequence

Table 7.37.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE attempt an IMS voice call. | - | - | - | - |
| 2-7 | Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| - | EXCEPTION: In parallel with Step 8, parallel behaviour defined in table 7.37.3.2-2 takes place | - | - | - | - |
| 8 | Check: Does UE sends INVITE with the first SDP offer containing media lines for both voice and IMS data channel, or containing media lines only for voice? (Note 1) (Step 1 of Annex A.4.2a happens) | --> | INVITE | 1,3 | P |
| 9 | SS sends a 100 Trying provisional response. (Step 2 of Annex A.4.2a happens) | <-- | 100 Trying | - | - |
| 10 | SS sends an SDP answer in 183 Session Progress. (Step 3 of Annex A.4.2a happens) | <-- | 183 Session Progress | - | - |
| 11 | UE acknowledges reception of 183 Session Progress. (Step 4 of Annex A.4.2a happens) | --> | PRACK | - | - |
| 12 | SS responds to PRACK. (Step 5 of Annex A.4.2a happens) | <-- | 200 OK | - | - |
| 13-15 | Steps 10-12 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 16 | SS sends 180 Ringing reliably. (Step 6 of Annex A.4.2a happens) | <-- | 180 Ringing | - | - |
| 17 | UE acknowledges reception of 180 Ringing. (Step 6A of Annex A.4.2a happens) | --> | PRACK | - | - |
| 18 | SS responds to PRACK. (Step 6B of Annex A.4.2a happens) | <-- | 200 OK | - | - |
| 19 | SS responds to INVITE. (Step 7 of Annex A.4.2a happens) | <-- | 200 OK | - | - |
| 20 | UE acknowledges. (Step 8 of Annex A.4.2a happens) | --> | ACK | 2 | P |
| - | EXCEPTION: Step 21a1 to 21a4 describe behaviour that depends on events happening prior to their execution; the "lower case letter" identifies a step sequence that take place if a specific prior event takes place. | - | - | - | - |
| 21a1 | IF INVITE in step8 does not include SDP offer containing media lines for IMS data channel, THEN Check: Does the UE send re-INVITE with an SDP offer containing media lines for both voice and IMS data channel? (Note 1) | --> | INVITE | 1,4 | P |
| 21a2 | SS responds with a 100 Trying provisional response | <-- | 100 Trying | - | - |
| 21a3 | SS responds re-INVITE with 200 OK. | <-- | 200 OK | - | - |
| 21a4 | Check: Does the UE acknowledge the receipt of 200 OK for re-INVITE? | --> | ACK | 2 | P |
| 22-23 | SS releases the call. (Annex A.8) | - | - | - | - |
| Note 1: At least one "a=dcmap" field shall be present. |  |  |  |  |  |

Table 7.37.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |

7.37.3.3	Specific message contents

Table 7.37.3.3-1: INVITE (step 8, table 7.37.3.2-1)

| Derivation Path: TS 34.229-5, Step 1 of A.4.2a, with following exceptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-5, Step 1 of A.4.2a except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [aa] |
| Message-body |  | SDP body is same as TS 34.229-5, Step 1 of A.4.2a or same as TS 34.229-5, Step 1 of A.4.2a except adding at least one Media description for IMS bootstrap data channel as follow: Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel c=IN (addrtype) (connection-address for UE) [Note 1] Attributes for media for IMS data channel: a=max-message-size:(max-message-size-value) [Note 2] a=sctp-port:(sctp-port-value) [Note 3] a=fingerprint:(hash-func) (fingerprint) [Note 4] a=setup:actpass a=tls-id:(tls-id-value) a=dcmap: (dcmap-stream-id) subprotocol="http” [Note 5,6] Note 1: At least one "c=" field shall be present. Note 2: Optional attribute. If the SDP ”max-message-size” attribute is not present the default value is 64K. Note 3: SCTP port range is between 0 and 65535(both included).Leading zeroes must not be used. Note 4: SHA-1,SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note 5: Multiple data channels may be mapped to a single data channel SDP media description, each with a corresponding "a=dcmap" SDP attribute and stream IDs that are unique within that media description. Note 6: dcmap-stream-id range is between 0 and 999(both included). |  | TS 26.114 [33] RFC 8841 [bb] RFC 8841 [bb] RFC 8122 [cc] RFC 4145 [ff] RFC 8842 [dd] RFC 8864 [ee] |

Table 7.37.3.3-2: 183 Session Progress (step 10, table 7.37.3.2-1)

| Derivation Path: TS 34.229-5, Step 3 of A.4.2a, with following exceptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-5, Step 3 of A.4.2a except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [aa] |
| Message-body | A1 A2 | SDP body is same as TS 34.229-5, Step 3 of A.4.2a except adding one Media description as follow: Media description for IMS data channel: m=application 0 UDP/DTLS/SCTP webrtc-datachannel Attributes for media for IMS data channel: Not present SDP answer is same as TS 34.229-5, Step 3 of A.4.2a |  | TS 26.114 [33] |

| Condition | Explanation |
| --- | --- |
| A1 | INVITE includes SDP offer containing media lines both for voice and IMS data channel in Step8. |
| A2 | INVITE includes SDP offer containing media lines only for voice in Step8. |

Table 7.37.3.3-3: 200 OK for INVITE (step 19, table 7.38.3.2-1)

| Derivation Path: TS 34.229-5, Step 7 of A.4.2a, with following exceptions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-5, Step 7 of A.4.2a except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [aa] |

Table 7.37.3.3-4: re-INVITE (step 21a1, table 7.37.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.2.1, condition A5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-1 [2], Annex A.2.1 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [aa] |
| Message-body |  | SDP body is same as TS 34.229-5, Step 1 of A.4.2a (INVITE) with following exception: Session description: "o=" line identical to previous SDP sent by UE except that sess-version is incremented by one Media description for audio: Same as in previous SDP sent by the SS in the 183 Session Progress at TS 34.229-5, Step 3 of A.4.2a. Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel c=IN (addrtype) (connection-address for UE) [Note 1] Attributes for media for IMS data channel: a=max-message-size:(max-message-size-value) [Note 2] a=sctp-port:(sctp-port-value) [Note 3] a=fingerprint:(hash-func) (fingerprint) [Note 4] a=setup:actpass a=tls-id:(tls-id-value) a=dcmap: (dcmap-stream-id) subprotocol="http” [Note 5,6] Note 1: At least one "c=" field shall be present. Note 2: Optional attribute. If the SDP ”max-message-size” attribute is not present the default value is 64K. Note 3: SCTP port range is between 0 and 65535(both included).Leading zeroes must not be used. Note 4: SHA-1,SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note 5: Multiple data channels may be mapped to a single data channel SDP media description, each with a corresponding "a=dcmap" SDP attribute and stream IDs that are unique within that media description. Note 6: dcmap-stream-id range is between 0 and 999(both included). |  | TS 26.114 [33] RFC 8841 [bb] RFC 8841 [bb] RFC 8122 [cc] RFC 4145 [ff] RFC 8842 [dd] RFC 8864 [ee] |

Table 7.37.3.3-5: 100 Trying (step 21a2, table 7.37.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.2, with condition A1.

Table 7.37.3.3-6: 200 OK for re-INVITE (step 21a3, table 7.37.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.3.1, with condition A23 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact feature-param |  | Feature-param is same as TS 34.229-1 [2], Annex A.3.1 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |  | TS 24.186 [aa] |
| Content-Type media-type |  | application/sdp |  |  |
| Content-Length value |  | length of message-body |  |  |
| Message-body |  | SDP body is same as TS 34.229-5, Step 3 of A.4.2a with follow exception: Session description: "o=" line identical to previous SDP sent by SS except that sess-version is incremented by one Media description for IMS data channel: m=application 0 UDP/DTLS/SCTP webrtc-datachannel Attributes for media for IMS data channel: Not present |  | TS 26.114 [33] |

Table 7.37.3.3-7: ACK for re-INVITE (step 21a4, table 7.23.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.2.7, with condition A5

<End of modified section 2>