3GPP TSG-RAN5 Meeting #107	R5-252578

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5448 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Inter RAT cell reselection test case 6.2.3.3a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Steps 3 and 4 are duplicated in main behavior Table 6.2.3.3a.3.2-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed duplicate steps 3 and 4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test specification remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.3a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 6.2.3.3a	Inter-RAT Cell reselection / From UTRA_Idle to E-UTRA RRC_IDLE (QqualminEUTRA, SqualServingCell < Threshserving,low2, SqualnonServingCell,x > Threshx, low2 and SqualnonServingCell,x > Threshx, high2)

6.2.3.3a.1	Test Purpose (TP)

(1)

with { UE in UTRA_Idle state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the lower priority inter-RAT E-UTRA cell}

then { UE reselects the cell which belongs to the lower priority inter-RAT E-UTRA cell }

}

(2)

with { UE in UTRA_Idle state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority inter-RAT E-UTRA cell}

then { UE reselects the cell which belongs to the higher priority inter-RAT E-UTRA cell }

}

6.2.3.3a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, clause 5.2.6.1.2a and 5.2.6.1.4a.

[TS 25.304, clause 5.2.6.1.2a]

If the UE has received absolute priority information for inter-RAT layers, the UE shall follow these rules:

-	The UE shall perform measurements of inter-RAT layers with a priority higher than the priority of the current serving cell.

NOTE:	The rate of these measurements may vary depending on whether Srxlev and Squal of the serving cell are above or below Sprioritysearch1 and Sprioritysearch2. This is specified in [10].

-	For inter-RAT layers with a priority lower than the priority of the current serving cell:

-	If SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2 the UE may choose not to perform measurements of inter-RAT layers of lower priority.

-	If SrxlevServingCell <= Sprioritysearch1 or SqualServingCell <= Sprioritysearch2 the UE shall perform measurements of inter-RAT layers of lower priority.

[TS 25.304, clause 5.2.6.1.4a]

Absolute priorities of different absolute priority layers may be provided to the UE. The following cell reselection criteria are used for inter-frequency cells if absolute priority inform69ation for inter-frequency is available to the UE, and inter-RAT cells if absolute priority information for inter-RAT is available to the UE.

...

The following definitions apply for the layers for which Threshx,high2 or Threshx,low2 are not provided:

-	Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high during a time interval Treselection;

…

-	Criterion 3: (SrxlevServingCell < Threshserving,low or SqualServingCell <= 0) and the SrxlevnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low during a time interval Treselection;

The following definitions apply for the layers for which both Threshx,high2 and Threshx,low2 are provided:

-	Criterion 4: the SqualnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high2 during a time interval Treselection;

-	Criterion 5: (SqualServingCell < Threshserving,low2 or SrxlevServingCell <= 0) and the SqualnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low2 during a time interval Treselection;

Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 or 4 is fulfilled.

...

Cell reselection to a cell on a lower absolute priority layer than the camped frequency shall be performed if criterion 3 or 5 is fulfilled.

If more than one cell meets the above criteria, the UE shall reselect the cell with the highest SrxlevnonServingCell,x among the cells meeting the criteria on the highest absolute priority layer.

The UE shall not perform cell reselection to cells for which the cell selection criterion S is not fulfilled.

The UE shall not perform cell reselection until more than 1 second has elapsed since the UE camped on the current serving cell.

...

Cell reselection from UTRAN to E-UTRAN, for which Squal (RSRQ) based cell reselection parameters are broadcast in system information, shall be performed based on the Squal criteria (criterion 4 or 5) if the UE supports Squal based cell reselection from E-UTRAN to UTRAN. Otherwise, cell reselection to E-UTRAN shall be performed based on Srxlev criteria.

6.2.3.3a.3	Test description

6.2.3.3a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 16.508 [18] on Cell 5 (serving cell).

6.2.3.3a.3.2	Test procedure sequence

Tables 6.2.3.3a.3.2-1 & Table 6.2.3.3a.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 6.2.3.3a.3.2-3.

Table 6.2.3.3a.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | Squal nonServingCell, Cell1 > Thresh Cell1,low2 (4 dB) |
|  | RSRQ | dB | -3 |  |
|  | Qqualmin | dB | -20 |  |
|  | Noc | dBm/15kHz | Off |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -115 |  |
|  | RSRQ | dB | -3 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |
|  | RSRQ | dB | -3 | S nonServingCell, Cell1 > Thresh Cell1,high2 |

Table 6.2.3.3a.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T1 | CPICH_Ec | dBm/3.84 MHz | -80 | SqualServingCell < ThreshServing,low2 (25 dB) |
|  | P-CCPCH | dBm/1.28 MHz | -82 |  |
|  | CPICH_Ec/No | dB | -2.89 |  |
|  | Qrxlevmin | dBm | -99 |  |
|  | Qqualmin | dB | -20 |  |
|  | Ioc | dBm/3.84MHz | off |  |
| T2 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | CPICH_Ec/No | dB | -2.89 |  |
| T3 | CPICH_Ec | dBm/3.84 MHz | -80 |  |
|  | P-CCPCH | dBm/1.28 MHz | -82 |  |
|  | CPICH_Ec/No | dB | -2.89 |  |

Table 6.2.3.3a.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 5 levels according to the row "T1" in table 6.2.3.3a.3.2-1 and table 6.2.3.3a.3.2-2. | - | - | - | - |
| - | EXCEPTION : Step 2a1 to 2b1 describe behaviour that depends on whether a UE that has a PDP context activated on UTRAN |  |  |  |  |
| 2 a1 | If PDP context is not active: Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 1 | - |
| 2b1 | If PDP context is active: Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 is performed and the UE is camped on E-UTRAN Cell 1? |  |  |  |  |
| 3 | Void | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 5 levels according to the row "T2" in table 6.2.3.3a.3.2-1 and table 6.2.3.3a.3.2-2. | - | - | - | - |
| 5 | The UE selects UTRAN cell 5 and performs the generic test procedure in TS 36.508 subclause 6.4.2.8. | - | - | - | - |
| 6 | The SS changes the E-UTRA cell priority broadcast in system information. | - | - | - | - |
| 7 | Notify UE change of System Information. | <-- | PAGING TYPE 1 | - | - |
| 8 | The SS changes Cell 1 and Cell 5 levels according to the row "T3" in table 6.2.3.3a.3.2-1 and table 6.2.3.3a.3.2-2. | - | - | - | - |
| 9 | Check: Does the test result of generic test procedure in TS 36.508 Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 2 | - |

6.2.3.3a.3.3	Specific message contents

Table 6.2.3.3a.3.3-0: Conditions for specific message contents
in Table 6.2.3.3a.3.3-1 and Table 6.2.3.3a.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.3a.3.3-1: System Information Block type 19 for Cell 5 (preamble, Table 6.2.3.3a.3.2-3)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark | Comment |  |  | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |  |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |  |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |  |  |  |  |
| priority |  |  | 4 |  |  |  |  |
| s-PrioritySearch1 |  |  | 12 (24 dB) |  |  |  |  |
| s-PrioritySearch2 |  |  | 0 (0 dB) |  |  |  |  |
| threshServingLow |  |  | 2 (4 dB) |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| eutra-FrequencyAndPriorityInfoList ::= SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { |  |  |  |  |  |  |  |
| earfcn [n] |  |  | Same downlink EARFCN as used for Cell 1 |  |  |  |  |
| measurementBandwidth [n] |  |  | Not present |  |  |  |  |
| priority [n] |  |  | 3 |  |  |  |  |
| threshXhigh [n] |  |  | 15 (30 dB) |  |  |  |  |
| } |  |  |  |  |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |  |  |  |  |
| utra-PriorityInfoList-v920ext SEQUENCE { |  |  |  |  |  |  |  |
| threshServingLow2 | 25 (25dB) |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |  |  |  |  |
| threshXhigh2[n] | 30 (30dB) |  |  |  |  |  |  |
| threshXlow2[n] | 4 (4dB) |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  | Band > 64 |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |  |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |  |  |  |  |
| earfcn [n] |  |  | Same downlink EARFCN as used for Cell 1 |  |  |  |  |
| measurementBandwidth [n] |  |  | Not present |  |  |  |  |
| priority [n] |  |  | 3 |  |  |  |  |
| threshXhigh [n] |  |  | 15 (30 dB) |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| Note: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 0 to 7. |  |  |  |  |  |  |  |

Table 6.2.3.3a.3.3-2: System Information Block type 19 for Cell 5 (step 6, Table 6.2.3.3a.3.2-3)

| Derivation Path: 36.508 table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList ::= SEQUENCE { |  |  |  |
| utra-ServingCell ::= SEQUENCE { |  |  |  |
| priority | 4 |  |  |
| s-PrioritySearch1 | 2 (4 dB) |  |  |
| s-PrioritySearch2 | 0 (0 dB) |  |  |
| threshServingLow | 2 (4 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList ::= SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [n] | Not present |  |  |
| priority [n] | 5 |  |  |
| threshXhigh [n] | 15 (30 dB) |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |
| utra-PriorityInfoList-v920ext SEQUENCE { |  |  |  |
| threshServingLow2 | 8 (8dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| threshXhigh2[n] | 4 (4dB) |  |  |
| threshXlow2[n] | 4 (4dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn [n] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth [n] | Not present |  |  |
| measurementBandwidth [n] | Not present |  |  |
| priority [n] | 5 |  |  |
| threshXhigh [n] | 15 (30 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 0 to 7. |  |  |  |

Table 6.2.3.3a.3.3-3: PAGING TYPE 1 (step 7, Table 6.2.3.3a.3.2-3)

| Derivation path: 34.108 clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PagingType1 ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| bcch-ModificationInfo ::= SEQUENCE { |  |  |  |
| mib-ValueTag | Set to (Current MIB value tag + 1) |  |  |
| bcch-ModificationTime | Not present |  |  |
| } |  |  |  |
| laterNonCriticalExtensions SEQUENCE {} | Not present |  |  |
| } |  |  |  |

Table 6.2.3.3a.3.3-4:SystemInformationBlockType3 for Cell 1(preamble, Table 6.2.3.3a.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 3 |  |  |
| } |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| threshServingLowQ-r9 | 0 dB |  |  |
| } |  |  |  |

Table 6.2.3.3a.3.3-5:SystemInformationBlockType3 for Cell 1(step 6, Table 6.2.3.3a.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 5 |  |  |
| } |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| threshServingLowQ-r9 | 0 dB |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.3a.3.3-6:SystemInformationBlockType6 for Cell 1(preamble, Table 6.2.3.3a.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| CarrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 4 |  |  |
| q-RxLevMin[n] | -36 (-71 dBm) |  |  |
| q-QualMin[n] | -20 (-20 dB) |  |  |
| threshX-Q-r9[n] SEQUENCE { |  |  |  |
| threshX-HighQ-r9 | 31 (31dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| CarrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[n] | 4 |  |  |
| q-RxLevMin[n] | -36 (-71 dBm) |  |  |
| q-QualMin[n] | -20 (-20 dB) |  |  |
| threshX-Q-r9[n] SEQUENCE { |  |  |  |
| threshX-HighQ-r9 | 31 (31dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

<End of modified section>