3GPP TSG-RAN5 Meeting #107	R5-252609

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5014 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to testcases 7.1.1.14.2.1 and 7.1.1.14.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Testcase is unfinished. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of specific messages and test sequence in test case 7.1.1.14.2.1 and 7.1.1.14.2.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase remains unfinished. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.14.2.1 and 7.1.1.14.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------- Start of first change -----------------------------------------------

##### 7.1.1.14.2	NES spatial domain adaptations

###### 7.1.1.14.2.1		NES spatial domain Type 1 adaptation / Periodic CSI reporting

7.1.1.14.2.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state configured for Periodic CSI-RS measurement reporting }

ensure that {

when { UE receives RRCReconfiguration message to report periodical CSI measurements with CSI-ReportSubconfig along with minimum 2 CSI-ReportSubconfigID }

then { UE applies CSI-ReportConfig and sends periodical CSI measurement reports for all CSI-ReportSubconfig present in CSI-ReportConfig }

}

7.1.1.14.2.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.2.1.4.2 and 5.2.4. Unless otherwise stated these are Rel-18 requirements.

[TS 38.214, clause 5.2.1.4.2]

…

If the UE is configured with a CSI-ReportConfig that contains a list of sub-configurations, provided by csi-ReportSubConfigToAddModList:

-	The UE expects to be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'. If the UE indicates a capability for supporting mixed codebook combination in a slot with mixCodeBookSpatialAdaptation, each sub-configuration which is configured with portSubsetIndictor can be configured with the higher layer parameter codebookType set to 'typeI-SinglePanel' or 'typeI-MultiPanel'.

-	Each sub-configuration can be configured with an antenna port subset using the higher layer bitmap parameter portSubsetIndicator which contains the bit sequence p0,p1,...,pPm-1, where p0 is the MSB and pPm-1 is the LSB, bit pi corresponds to antenna port 3000+i, and Pm is the number of ports nrofPorts configured for the CSI-RS resources(s) within a NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement that corresponds to the CSI-ReportConfig. A bit value 0 in portSubsetIndicator indicates that the corresponding antenna port is disabled for the sub-configuration, whereas bit value 1 indicates that the antenna port is enabled and belongs to the antenna port subset for the sub-configuration. For the derivation of PMI, antenna ports corresponding to all bits with value of 1 in portSubsetIndicator are mapped to consecutive antenna ports starting at CSI-RS antenna port 3000 in increasing order of the bit position in portSubsetIndicator.

-	If a sub-configuration is configured with an antenna port subset, then the sub-configuration can be configured with a ri-Restriction, codebookMode and, if the number of antenna ports of the subset greater than 2, with n1-n2 if the higher layer parameter codebookType is set to 'typeI-SinglePanel' or with ng-n1-n2 if the higher layer parameter codebookType is set to 'typeI-MultiPanel', and, if the corresponding number of antenna ports of the subset is 2, with twoTX-CodebookSubsetRestriction, where the parameters ri-Restriction, codebookMode, n1-n2, ng-n1-n2, twoTX-CodebookSubsetRestriction are as described in Clauses 5.2.2.2.1 and 5.2.2.2.2. If a sub-configuration is configured with an antenna port subset, and if higher layer parameter reportQuantity is set to 'cri-RI-i1-CQI', and if the higher layer parameter codebookType is set to 'typeI-SinglePanel', then the sub-configuration is configured with higher layer parameter typeI-SinglePanel-codebookSubsetRestriction-i2, where typeI-SinglePanel-codebookSubsetRestriction-i2 is as described in Clause 5.2.2.2.1.

-	If a sub-configuration is configured with an antenna port subset, and if the CSI-ReportConfig that contains a mix of sub-configuration(s) each corresponding to 'typeI-SinglePanel' and some other sub-configuration(s) each corresponding to 'typeI-MultiPanel', then the sub-configuration(s) which is configured with portSubsetIndictor is configured with the higher layer parameter codebookMode.

-	A sub-configuration can be configured with a power offset provided by powerOffset.

-	A sub-configuration can be configured with a list of NZP CSI-RS resources, provided by nzp-CSI-RS-ResourceList, which indicates one or more NZP CSI-RS resources, within a NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig. If there is no sub-configuration configured with a power offset provided by powerOffset, the list of NZP CSI-RS resources has no intersection with a list of NZP CSI-RS resources configured for any other sub-configuration(s) within the CSI-ReportConfig, otherwise, the list of NZP CSI-RS resources is identical to or has no intersection with a list of NZP CSI-RS resources configured for any other sub-configuration(s) within the CSI-ReportConfig.

-	If a sub-configuration is configured with a list of NZP CSI-RS resources with more than one resource, the UE shall derive the CSI parameters other than CRI conditioned on the reported CRI, where the CRI k (k ≥ 0) for the sub-configuration corresponds to the configured (k+1)-th entry of associated NZP-CSI-RS-Resource in the list of NZP CSI-RS resources.

-	If a sub-configurations is not configured with nzp-CSI-RS-ResourceList then the sub-configuration shall be associated with all the NZP CSI-RS resources within a NZP-CSI-RS-ResourceSet contained in the CSI-ResourceConfig for channel measurement which corresponds to the CSI-ReportConfig.

-	the UE reports CSI(s) for one or more sub-configurations according to Clauses 5.2.1.5.1, 5.2.1.5.2, 5.2.3 and 5.2.4, and according to the higher layer parameter reportQuantity configured for that CSI-ReportConfig.

-	The UE does not expect the higher layer parameter reportQuantity to be set to 'cri-RSRP', 'cri-SINR', 'cri-SINR- Index', 'cri-RSRP-Index', 'none', 'ssb-Index-RSRP', 'ssb-Index-SINR', 'ssb-Index-RSRP- Index', 'ssb-Index-SINR- Index', or 'tdcp'.

…

[TS 38.214, clause 5.2.4]

A UE is semi-statically configured by higher layers to perform periodic CSI Reporting on the PUCCH. A UE can be configured by higher layers for multiple periodic CSI Reports corresponding to multiple higher layer configured CSI Reporting Settings, where the associated CSI Resource Settings are higher layer configured. For a Reporting Setting for which the CSI-ReportConfig contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, CSI reporting is provided for all the sub-configurations in each corresponding reporting instance. Periodic CSI reporting on PUCCH formats 2, 3, 4 supports Type I CSI with wideband granularity.

7.1.1.14.2.1.3	Test description

7.1.1.14.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

- None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

7.1.1.14.2.1.3.2	Test procedure sequence

Table 7.1.1.14.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR Cell 1 including CSI-MeasConfig to setup measurement and periodic reporting with CSI-ReportSubconfig. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | Check: Does the UE transmit CSI report on PUCCH for all CSI-ReportSubconfig configured at first periodicity? | --> | CSI Report on PUCCH | 1 | P |
| 4 | Check: Does the UE transmit CSI report on PUCCH for all CSI-ReportSubconfig configured at second periodicity? | --> | CSI Report on PUCCH | 1 | P |
| 5 | The SS transmits an RRCReconfiguration including CSI-MeasConfig to remove measurements | <-- | NR RRC: RRCReconfiguration | - | - |
| 6 | The UE transmits an RRCReconfigurationComplete message to confirm the remove. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 7 | Check: Does the UE transmit CSI report on PUCCH for all CSI-ReportSubconfig configured within two periods? | --> | CSI Report on PUCCH | 1 | F |

7.1.1.14.2.1.3.3	Specific message contents

Table 7.1.1.14.2.1.3.3-1: RRCReconfiguration (Step 1 and Step 5, Table 7.1.1.14.2.1.3.2-1)

Derivation Path: TS 38.508-1 [4] table 4.6.1-13 with CellGroupConfig with condition NR_MEAS

Table 7.1.1.14.2.1.3.3-2: ServingCellConfig (Table 7.1.1.14.2.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], clause 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig |  |  |
| } |  |  |  |

Table 7.1.1.14.2.1.3.3-3: CSI-MeasConfig (Table 7.1.1.14.2.1.3.3-2)

Derivation Path: TS 38.508-1 [4], clause 4.6.3-38

Table 7.1.1.14.2.1.3.3-4: CSI-ReportConfig (Step 1, Table 7.1.1.14.2.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], clause 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| csi-ReportSubConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfig-r18 { | 2 entries |  |  |
| CSI-ReportSubConfig-r18[1] SEQUENCE{ |  | Entry 1 |  |
| reportSubConfigId-r18 | 0 |  |  |
| reportSubConfigParams-r18 CHOICE { |  |  |  |
| a1-parameters SEQUENCE { |  |  |  |
| codebookSubConfig-r18 | CodebookConfig |  |  |
| portSubsetIndicator-r18 CHOICE { |  |  |  |
| p32 | FFFFFFFF’H |  |  |
| } |  |  |  |
| non-PMI-PortIndication-r18 | Not present |  |  |
| } |  |  |  |
| a2-parameters | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| CSI-ReportSubConfig-r18[2] SEQUENCE{ |  | Entry 2 |  |
| reportSubConfigId-r18 | 1 |  |  |
| reportSubConfigParams-r18 CHOICE { |  |  |  |
| a1-parameters SEQUENCE { |  |  |  |
| codebookSubConfig-r18 | CodebookConfig |  |  |
| portSubsetIndicator-r18 CHOICE { | TBD |  |  |
| p16 | FFFF’H |  |  |
| } |  |  |  |
| non-PMI-PortIndication-r18 | Not present |  |  |
| } |  |  |  |
| a2-parameters | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.14.2.1.3.3-5: CSI-ReportConfig (Step 5, Table 7.1.1.14.2.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], clause 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| csi-ReportSubConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfigId-r18 { | 1 entry |  |  |
| CSI-ReportSubConfigId-r18[1] | 0 |  |  |
| CSI-ReportSubConfigId-r18[2] | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

----------------------------------------------- End of first change -----------------------------------------------

----------------------------------------------- Start of second change -----------------------------------------------

###### 7.1.1.14.2.3	NES Power domain adaptation / Semi-Periodic CSI reporting

7.1.1.14.2.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state configured for Semi-Periodic CSI-RS measurement reporting }

ensure that {

when { UE receives RRCReconfiguration message to report Semi-periodical CSI measurements with CSI-ReportSubconfig with multiple CSI-ReportSubconfigID }

then { UE applies CSI-ReportConfig and sends Semi-periodical CSI measurement reports for all CSI-ReportSubconfig present in CSI-ReportConfig }

}

7.1.1.14.2.3.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.2.1.5.2, 5.2.4 and 6.1.3.16. Unless otherwise stated these are Rel-18 requirements.

[TS 38.214, clause 5.2.1.5.2]

…

For semi-persistent reporting on PUCCH, the PUCCH resource used for transmitting the CSI report are configured by reportConfigType. Semi-persistent reporting on PUCCH is activated by an activation command as described in clause 6.1.3.16 of [10, TS 38.321], which selects one of the semi-persistent Reporting Settings for use by the UE on the PUCCH. For a selected reporting setting for which the CSI-ReportConfig contains a list of sub-configurations provided by the higher layer parameter csi-ReportSubConfigToAddModList, the activation command can also select one or more sub-configurations to use by the UE as described in clause 6.1.3.16 of [10, TS 38.321]. When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated semi-persistent Reporting Setting should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH

…

[TS 38.214, clause 5.2.4]

…

A UE shall perform semi-persistent CSI reporting on the PUCCH applied starting from the first slot that is after slot n+3Nslotsubframe,µ when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command described in clause 6.1.3.16 of [10, TS 38.321] where  is the SCS configuration for the PUCCH. The activation command will contain one or more Reporting Settings, with or without containing one or more sub-configurations for each Reporting Setting for which the CSI-ReportConfig contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, where the associated CSI Resource Settings are configured. Semi-persistent CSI reporting on the PUCCH supports Type I CSI. Semi-persistent CSI reporting on the PUCCH format 2 supports Type I CSI with wideband frequency granularity. Semi-persistent CSI reporting on PUCCH formats 3 or 4 supports Type I CSI with wideband and sub-band frequency granularities and Type II CSI Part 1

…

[TS 38.321, clause 6.1.3.80]

The enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE is identified by a MAC subheader with eLCID as specified in Table 6.2.1-1b. It has a variable size and consists of the following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field indicates a UL BWP for which the MAC CE applies as the codepoint of the DCI bandwidth part indicator field as specified in TS 38.212 [9]. The length of the BWP ID field is 2 bits;

-	Si: This field indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within csi-ReportConfigToAddModList, as specified in TS 38.331 [5]. S0 refers to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the lowest CSI-ReportConfigId within the list with type set to semiPersistentOnPUCCH, S1 to the report configuration which includes PUCCH resources for SP CSI reporting in the indicated BWP and has the second lowest CSI-ReportConfigId and so on. If the number of report configurations within the list with type set to semiPersistentOnPUCCH in the indicated BWP is less than i + 1, MAC entity shall ignore the Si field. The Si field is set to 1 to indicate that the corresponding Semi-Persistent CSI report configuration shall be activated. The Si field is set to 0 to indicate that the corresponding Semi-Persistent CSI report configuration i shall be deactivated;

-	Ni,x: This field indicates the activation/deactivation status of the Semi-Persistent CSI report sub-configuration x within csi-ReportSubConfigToAddModList, as specified in TS 38.331 [5]. If Si set to 1, the octet corresponding to Ni,0 to Ni,7 is present. If Si set to 0, the octet corresponding to Ni,0 to Ni,7 is not present. N0,0 refers to the report sub-configuration which has the lowest csi-ReportSubConfigID within the list, N0,1 to the report sub-configuration which has the second lowest csi-ReportSubConfigID and so on. If the number of report sub-configurations within the list in the indicated CSI-ReportConfig is less than x + 1, the MAC entity shall ignore the Ni,x field. The Ni,x field is set to 1 to indicate that the corresponding Semi-Persistent CSI report sub-configuration x shall be activated. The Ni,x field is set to 0 to indicate that the corresponding Semi-Persistent CSI report sub-configuration x shall be deactivated;

-	R: Reserved bit, set to 0.

Figure 6.1.3.80-1: Enhanced SP CSI reporting on PUCCH Activation/Deactivation MAC CE

7.1.1.14.2.3.3	Test description

7.1.1.14.2.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

UE:

-	None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

7.1.1.14.2.3.3.2	Test procedure sequence

Table 7.1.1.14.2.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR Cell 1 including CSI-MeasConfig to setup measurement and semiPersistentOnPUCCH in CSI-ReportSubconfig with different powerOffset-r18 | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS transmits a SP CSI reporting on PUCCH Activation MAC-CE | <-- | MAC PDU (S0=1 and N0,0=1, N0,1=1) | - | - |
| 4 | Check: Does the UE transmit CSI report on PUCCH for all CSI-ReportSubconfig configured in first periodicity? | --> | CSI report on PUCCH | 1 | P |
| 5 | Check: Does the UE transmit CSI report on PUCCH for all CSI-ReportSubconfig configured in second periodicity? | --> | CSI report on PUCCH | 1 | P |
| 6 | The SS transmits a SP CSI reporting on PUCCH Deactivation MAC-CE | <-- | MAC PDU | - | - |
| 7 | Check: Does the UE transmit CSI report on PUCCH for all CSI-ReportSubconfig configured within two periods? | --> | CSI report on PUCCH | 1 | F |
| 8 | The SS transmits an RRCReconfiguration including CSI-measConfig to remove measurements | <-- | NR RRC: RRCReconfiguration | - | - |
| 9 | The UE transmits an RRCReconfigurationComplete message to confirm the remove | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 10 | Check: Does the UE transmit CSI report on PUCCH for all CSI-ReportSubconfig configured within two periods? | --> | CSI report on PUCCH | 1 | F |

7.1.1.14.2.3.3.3	Specific message contents

Table 7.1.1.14.2.3.3.3-1: RRCReconfiguration (Step 1 and Step 8, Table 7.1.1.14.2.3.3.2-1)

Derivation Path: TS 38.508-1 [4] table 4.6.1-13 with condition NR_MEAS

Table 7.1.1.14.2.3.3.3-2: ServingCellConfig (Table 7.1.1.14.2.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], clause 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig |  |  |
| } |  |  |  |

Table 7.1.1.14.2.3.3.3-3: CSI-MeasConfig (Table 7.1.1.14.2.3.3.3-2)

Derivation Path: TS 38.508-1 [4], clause 4.6.3-38

Table 7.1.1.14.2.3.3.3-4: CSI-ReportConfig (Step 1, Table 7.1.1.14.2.3.3.3-3)

| Derivation Path: TS 38.508-1 [4], clause 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| csi-ReportSubConfigToAddModList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfig-r18 { | 2 entries |  |  |
| CSI-ReportSubConfig-r18[1] SEQUENCE{ |  | Entry 1 |  |
| reportSubConfigId-r18 | 0 |  |  |
| reportSubConfigParams-r18 CHOICE { |  |  |  |
| a1-parameters SEQUENCE { |  |  |  |
| codebookSubConfig-r18 | CodebookConfig |  |  |
| portSubsetIndicator-r18 CHOICE { |  |  |  |
| p32 | FFFFFFFF’H |  |  |
| } |  |  |  |
| non-PMI-PortIndication-r18 | Not present |  |  |
| } |  |  |  |
| a2-parameters | Not present |  |  |
| } |  |  |  |
| powerOffset-r18 | 3 |  |  |
| } |  |  |  |
| CSI-ReportSubConfig-r18[2] SEQUENCE{ |  | Entry 2 |  |
| reportSubConfigId-r18 | 1 |  |  |
| reportSubConfigParams-r18 CHOICE { |  |  |  |
| a1-parameters SEQUENCE { |  |  |  |
| codebookSubConfig-r18 | CodebookConfig |  |  |
| portSubsetIndicator-r18 CHOICE { |  |  |  |
| p16 | FFFF’H |  |  |
| } |  |  |  |
| non-PMI-PortIndication-r18 | Not present |  |  |
| } |  |  |  |
| a2-parameters | Not present |  |  |
| } |  |  |  |
| powerOffset-r18 | 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.14.2.3.3.3-5: CSI-ReportConfig (Step 8, Table 7.1.1.14.2.3.3.3-3)

| Derivation Path: TS 38.508-1 [4], clause 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| csi-ReportSubConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofCSI-ReportSubconfigPerCSI-ReportConfig-r18)) OF CSI-ReportSubConfigId-r18 { | 1 entry |  |  |
| CSI-ReportSubConfigId-r18[1] | 0 |  |  |
| CSI-ReportSubConfigId-r18[2] | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

----------------------------------------------- End of second change -----------------------------------------------