3GPP TSG-RAN5 Meeting #107	R5-252636

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5459 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Idle mode TC 6.2.3.3a and 6.2.3.4a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The specific RRC message needs to be updated based on TS 36.331 and TS 36.508. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specific message contents were updated. Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error will exist in this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.3a and 6.2.3.4a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

6.2.3.3a	Inter-RAT Cell reselection / From UTRA_Idle to E-UTRA RRC_IDLE (QqualminEUTRA, SqualServingCell < Threshserving,low2, SqualnonServingCell,x > Threshx, low2 and SqualnonServingCell,x > Threshx, high2)

6.2.3.3a.1	Test Purpose (TP)

(1)

with { UE in UTRA_Idle state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the lower priority inter-RAT E-UTRA cell }

then { UE reselects the cell which belongs to the lower priority inter-RAT E-UTRA cell }

}

(2)

with { UE in UTRA_Idle state }

ensure that {

when { UE detects the cell re-selection criteria are met for the cell which belongs to the higher priority inter-RAT E-UTRA cell }

then { UE reselects the cell which belongs to the higher priority inter-RAT E-UTRA cell }

}

6.2.3.3a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, clause 5.2.6.1.2a and 5.2.6.1.4a.

[TS 25.304, clause 5.2.6.1.2a]

If the UE has received absolute priority information for inter-RAT layers, the UE shall follow these rules:

-	The UE shall perform measurements of inter-RAT layers with a priority higher than the priority of the current serving cell.

NOTE:	The rate of these measurements may vary depending on whether Srxlev and Squal of the serving cell are above or below Sprioritysearch1 and Sprioritysearch2. This is specified in [10].

-	For inter-RAT layers with a priority lower than the priority of the current serving cell:

-	If SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2 the UE may choose not to perform measurements of inter-RAT layers of lower priority.

-	If SrxlevServingCell <= Sprioritysearch1 or SqualServingCell <= Sprioritysearch2 the UE shall perform measurements of inter-RAT layers of lower priority.

[TS 25.304, clause 5.2.6.1.4a]

Absolute priorities of different absolute priority layers may be provided to the UE. The following cell reselection criteria are used for inter-frequency cells if absolute priority inform69ation for inter-frequency is available to the UE, and inter-RAT cells if absolute priority information for inter-RAT is available to the UE.

...

The following definitions apply for the layers for which Threshx,high2 or Threshx,low2 are not provided:

-	Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high during a time interval Treselection;

…

-	Criterion 3: (SrxlevServingCell < Threshserving,low or SqualServingCell <= 0) and the SrxlevnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low during a time interval Treselection;

The following definitions apply for the layers for which both Threshx,high2 and Threshx,low2 are provided:

-	Criterion 4: the SqualnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high2 during a time interval Treselection;

-	Criterion 5: (SqualServingCell < Threshserving,low2 or SrxlevServingCell <= 0) and the SqualnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low2 during a time interval Treselection;

Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 or 4 is fulfilled.

...

Cell reselection to a cell on a lower absolute priority layer than the camped frequency shall be performed if criterion 3 or 5 is fulfilled.

If more than one cell meets the above criteria, the UE shall reselect the cell with the highest SrxlevnonServingCell,x among the cells meeting the criteria on the highest absolute priority layer.

The UE shall not perform cell reselection to cells for which the cell selection criterion S is not fulfilled.

The UE shall not perform cell reselection until more than 1 second has elapsed since the UE camped on the current serving cell.

...

Cell reselection from UTRAN to E-UTRAN, for which Squal (RSRQ) based cell reselection parameters are broadcast in system information, shall be performed based on the Squal criteria (criterion 4 or 5) if the UE supports Squal based cell reselection from E-UTRAN to UTRAN. Otherwise, cell reselection to E-UTRAN shall be performed based on Srxlev criteria.

6.2.3.3a.3	Test description

6.2.3.3a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 36.508 [18] on Cell 5 (serving cell).

6.2.3.3a.3.2	Test procedure sequence

Tables 6.2.3.3a.3.2-1 & Table 6.2.3.3a.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 6.2.3.3a.3.2-3.

Table 6.2.3.3a.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | Squal nonServingCell, Cell1 > Thresh Cell1,low2 (4 dB) |
|  | RSRQ | dB | -3 |  |
|  | Qqualmin | dB | -20 |  |
|  | Noc | dBm/15kHz | Off |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -115 |  |
|  | RSRQ | dB | -3 |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | -85 |  |
|  | RSRQ | dB | -3 | S nonServingCell, Cell1 > Thresh Cell1,high2 |

Table 6.2.3.3a.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T1 | CPICH_Ec | dBm/3.84 MHz | -80 | SqualServingCell < ThreshServing,low2 (25 dB) |
|  | P-CCPCH | dBm/1.28 MHz | -82 |  |
|  | CPICH_Ec/No | dB | -2.89 |  |
|  | Qrxlevmin | dBm | -99 |  |
|  | Qqualmin | dB | -20 |  |
|  | Ioc | dBm/3.84MHz | off |  |
| T2 | CPICH_Ec | dBm/3.84 MHz | -60 |  |
|  | P-CCPCH | dBm/1.28 MHz | -62 |  |
|  | CPICH_Ec/No | dB | -2.89 |  |
| T3 | CPICH_Ec | dBm/3.84 MHz | -80 |  |
|  | P-CCPCH | dBm/1.28 MHz | -82 |  |
|  | CPICH_Ec/No | dB | -2.89 |  |

Table 6.2.3.3a.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 5 levels according to the row "T1" in table 6.2.3.3a.3.2-1 and table 6.2.3.3a.3.2-2. | - | - | - | - |
| - | EXCEPTION: Step 2a1 to 2b1 describe behaviour that depends on whether a UE that has a PDP context activated on UTRAN. | - | - | - | - |
| 2a1 | If PDP context is not active: Check: Does the test result of generic test procedure in TS 36.508 [18] Table 6.4.2.7A-2 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 1 | - |
| 2b1 | If PDP context is active: Check: Does the test result of generic test procedure in TS 36.508 [18] Table 6.4.2.7A-1 is performed and the UE is camped on E-UTRAN Cell 1? | - | - | - | - |
| 3 | Void | - | - | - | - |
| 4 | The SS changes Cell 1 and Cell 5 levels according to the row "T2" in table 6.2.3.3a.3.2-1 and table 6.2.3.3a.3.2-2. | - | - | - | - |
| 5 | The UE selects UTRAN cell 5 and performs the generic test procedure in TS 36.508 [18] subclause 6.4.2.8. | - | - | - | - |
| 6 | The SS changes the E-UTRA cell priority broadcast in system information. | - | - | - | - |
| 7 | Notify UE change of System Information. | <-- | PAGING TYPE 1 | - | - |
| 8 | The SS changes Cell 1 and Cell 5 levels according to the row "T3" in table 6.2.3.3a.3.2-1 and table 6.2.3.3a.3.2-2. | - | - | - | - |
| 9 | Check: Does the test result of generic test procedure in TS 36.508 [18] Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1? | - | - | 2 | - |

6.2.3.3a.3.3	Specific message contents

Table 6.2.3.3a.3.3-0: Conditions for specific message contents
in Table 6.2.3.3a.3.3-1 and Table 6.2.3.3a.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.3a.3.3-1: System Information Block type 19 for Cell 5 (preamble, Table 6.2.3.3a.3.2-3)

| Derivation Path: TS 36.508 [18] table 4.4.4.1-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  |  | Value/remark | Comment |  |  | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |  |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |  |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |  |  |  |  |
| priority |  |  | 4 |  |  |  |  |
| s-PrioritySearch1 |  |  | 12 (24 dB) |  |  |  |  |
| s-PrioritySearch2 |  |  | 0 (0 dB) |  |  |  |  |
| threshServingLow |  |  | 2 (4 dB) |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { |  |  |  |  |  |  |  |
| earfcn[1] |  |  | Same downlink EARFCN as used for Cell 1 |  |  |  |  |
| measurementBandwidth[1] |  |  | Not present |  |  |  |  |
| priority[1] |  |  | 3 |  |  |  |  |
| threshXhigh[1] |  |  | 15 (30 dB) |  |  |  |  |
| } |  |  |  |  |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |  |  |  |  |
| utra-PriorityInfoList-v920ext SEQUENCE { |  |  |  |  |  |  |  |
| threshServingLow2 | 25 (25dB) |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |  |  |  |  |
| threshXhigh2[1] | 30 (30dB) |  |  |  |  |  |  |
| threshXlow2[1] | 4 (4dB) |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  | Band > 64 |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |  |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |  |  |  |  |
| earfcn[1] |  |  | Same downlink EARFCN as used for Cell 1 |  |  |  |  |
| measurementBandwidth[1] |  |  | Not present |  |  |  |  |
| priority[1] |  |  | 3 |  |  |  |  |
| threshXhigh[1] |  |  | 15 (30 dB) |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| Note: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 0 to 7. |  |  |  |  |  |  |  |

Table 6.2.3.3a.3.3-2: System Information Block type 19 for Cell 5 (step 6, Table 6.2.3.3a.3.2-3)

| Derivation Path: TS 36.508 [18] table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 4 |  |  |
| s-PrioritySearch1 | 2 (4 dB) |  |  |
| s-PrioritySearch2 | 0 (0 dB) |  |  |
| threshServingLow | 2 (4 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { |  |  |  |
| earfcn[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth[1] | Not present |  |  |
| priority[1] | 5 |  |  |
| threshXhigh[1] | 15 (30 dB) |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |
| utra-PriorityInfoList-v920ext SEQUENCE { |  |  |  |
| threshServingLow2 | 8 (8dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| threshXhigh2[1] | 4 (4dB) |  |  |
| threshXlow2[1] | 4 (4dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth[1] | Not present |  |  |
| measurementBandwidth[1] | Not present |  |  |
| priority[1] | 5 |  |  |
| threshXhigh[1] | 15 (30 dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 0 to 7. |  |  |  |

Table 6.2.3.3a.3.3-3: PAGING TYPE 1 (step 7, Table 6.2.3.3a.3.2-3)

| Derivation path: TS 34.108 [5] clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PagingType1 ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| bcch-ModificationInfo ::= SEQUENCE { |  |  |  |
| mib-ValueTag | Set to (Current MIB value tag + 1) |  |  |
| bcch-ModificationTime | Not present |  |  |
| } |  |  |  |
| laterNonCriticalExtensions | Not present |  |  |
| } |  |  |  |

Table 6.2.3.3a.3.3-4: SystemInformationBlockType3 for Cell 1 (preamble, Table 6.2.3.3a.3.2-3)

| Derivation path: TS 36.508 [18] table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 3 |  |  |
| } |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| threshServingLowQ-r9 | 0 dB |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.3a.3.3-5: SystemInformationBlockType3 for Cell 1 (step 6, Table 6.2.3.3a.3.2-3)

| Derivation path: TS 36.508 [18] table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 5 |  |  |
| } |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| threshServingLowQ-r9 | 0 dB |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.3a.3.3-6: SystemInformationBlockType6 for Cell 1 (preamble, Table 6.2.3.3a.3.2-3)

| Derivation path: TS 36.508 [18] table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[1] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[1] | 4 |  |  |
| q-RxLevMin[1] | -36 (-71 dBm) |  |  |
| q-QualMin[1] | -20 (-20 dB) |  |  |
| threshX-Q-r9[1] SEQUENCE { |  |  |  |
| threshX-HighQ-r9 | 31 (31dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[1] | Same downlink UARFCN as used for cell 5 |  |  |
| cellReselectionPriority[1] | 4 |  |  |
| q-RxLevMin[1] | -36 (-71 dBm) |  |  |
| q-QualMin[1] | -20 (-20 dB) |  |  |
| threshX-Q-r9[1] SEQUENCE { |  |  |  |
| threshX-HighQ-r9 | 31 (31dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

[Skip unchanged text]

6.2.3.4a	Inter-RAT Cell reselection / From UTRA_CELL_PCH state to E-UTRA RRC_IDLE based on RSRQ+RSRP evaluation

6.2.3.4a.1	Test Purpose (TP)

(1)

with { UE in UTRA_CELL_PCH state }

ensure that {

when { UE detects the cell re-selection criteria ( RSRQ => Squal >0 , RSRP => Srxlev >0 )are met for the cell which belongs to the higher priority inter-RAT E-UTRA cell }

then { UE reselects the cell which belongs to the higher priority inter-RAT E-UTRA cell }

}

6.2.3.4a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 25.304, clause 5.2.6.1.2a and 5.2.6.1.4a; TS 25.331, clause 8.3.9.2, 8.3.9.3, Annex E.

[TS 25.304, clause 5.2.6.1.2a]

The measurement rules below apply in Idle, URA_PCH, CELL_PCH states.

...

If the UE has received absolute priority information for inter-RAT layers, the UE shall follow these rules:

-	The UE shall perform measurements of inter-RAT layers with a priority higher than the priority of the current serving cell.

NOTE:	The rate of these measurements may vary depending on whether Srxlev and Squal of the serving cell are above or below Sprioritysearch1 and Sprioritysearch2. This is specified in [10].

-	For inter-RAT layers with a priority lower than the priority of the current serving cell:

-	If SrxlevServingCell > Sprioritysearch1 and SqualServingCell > Sprioritysearch2 the UE may choose not to perform measurements of inter-RAT layers of lower priority.

-	If SrxlevServingCell <= Sprioritysearch1 or SqualServingCell <= Sprioritysearch2 the UE shall perform measurements of inter-RAT layers of lower priority.

[TS 25.304, clause 5.2.6.1.4a]

Absolute priorities of different absolute priority layers may be provided to the UE. The following cell reselection criteria are used for inter-frequency cells if absolute priority information for inter-frequency is available to the UE, and inter-RAT cells if absolute priority information for inter-RAT is available to the UE.

...

The following definitions apply:

-	Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high during a time interval Treselection;

…

Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 is fulfilled.

...

[TS 25.304, clause 5.2.6.1.4a]

Absolute priorities of different absolute priority layers may be provided to the UE. The following cell reselection criteria are used for inter-frequency cells if absolute priority information for inter-frequency is available to the UE, and inter-RAT cells if absolute priority information for inter-RAT is available to the UE.

...

The following definitions apply for the layers for which Threshx,high2 or Threshx,low2 are not provided:

-	Criterion 1: the SrxlevnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high during a time interval Treselection;

…

-	Criterion 3: (SrxlevServingCell < Threshserving,low or SqualServingCell <= 0) and the SrxlevnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low during a time interval Treselection;

The following definitions apply for the layers for which both Threshx,high2 and Threshx,low2 are provided:

-	Criterion 4: the SqualnonServingCell,x of a cell on an evaluated higher absolute priority layer is greater than Threshx,high2 during a time interval Treselection;

-	Criterion 5: (SqualServingCell < Threshserving,low2 or SrxlevServingCell <= 0) and the SqualnonServingCell,x of a cell on an evaluated lower absolute priority layer is greater than Threshx,low2 during a time interval Treselection;

Cell reselection to a cell on a higher absolute priority layer than the camped frequency shall be performed if criterion 1 or 4 is fulfilled.

...

Cell reselection to a cell on a lower absolute priority layer than the camped frequency shall be performed if criterion 3 or 5 is fulfilled.

The UE shall not perform cell reselection until more than 1 second has elapsed since the UE camped on the current serving cell.

…

If more than one cell meets the above criteria, the UE shall reselect the cell with the highest SrxlevnonServingCell,x among the cells meeting the criteria on the highest absolute priority layer.

The UE shall not perform cell reselection to cells for which the cell selection criterion S is not fulfilled.

The UE shall not perform cell reselection until more than 1 second has elapsed since the UE camped on the current serving cell.

For UE in RRC connected mode states CELL_PCH or URA_PCH the interval Treselections,PCH applies, if provided in SIB4 (see [4]), while for UE in RRC connected mode state CELL_FACH the interval Treselections,FACH applies, if provided in SIB4 (see [4]).

In all the above criteria the values of Treselections, Treselections,PCH or Treselections,FACH apply for Treselection and are scaled according to the UE mobility state and target RAT, as specified in 5.2.6.1.1a.

...

Cell reselection from UTRAN to E-UTRAN, for which Squal (RSRQ) based cell reselection parameters are broadcast in system information, shall be performed based on the Squal criteria (criterion 4 or 5) if the UE supports Squal based cell reselection from E-UTRAN to UTRAN. Otherwise, cell reselection to E-UTRAN shall be performed based on Srxlev criteria.

[TS 25.331, 8.3.9.2]

This procedure is applicable in states CELL_FACH, CELL_PCH or URA_PCH. Inter-RAT cell reselection to E-UTRAN in CELL_FACH state is not supported in this version of the specification.

When the UE based on received system information makes a cell reselection to a radio access technology other than UTRAN, e.g. GSM/GPRS or E-UTRA, according to the criteria specified in [4], the UE shall:

...

[TS 25.331, 8.3.9.3]

When the UE has succeeded in reselecting a cell in the target radio access technology, the UE shall:

1>	release all UTRAN specific resources.

UTRAN should:

1>	release all UE dedicated resources upon indication that the UE has completed a connection establishment to the other radio access technology.

[TS 25.331, Annex E]

This annex contains the definitions of the bits in EUTRA Feature Group Indicators.

In this release of the specification the UE shall include the optional EUTRA Feature Group Indicators IE in the UE multi-mode/multi RAT capability IE of the RRC message. For a specific indicator, if all functionalities for a feature group listed in Table E.1 have been implemented and tested, the UE shall set the indicator as "true" (as one), else (i.e. if any one of the functionalities in a feature group listed in Table E.1 have not been implemented or tested), the UE shall set the indicator as zero.

The UE shall set all indicators, which do not have a definition in the table E.1, as "false" (as zero).

If the optional EUTRA Feature Group Indicators IE is not included by a UE supporting a future release of the specification, the network may assume the UE supports all features in the feature groups listed in table E.1 and deployed in the network.

Table E.1: Definitions of feature group indicators

| Index of indicator (bit number) | Definition (description of the supported functionality, if indicator set to ‘true’) | Notes |
| --- | --- | --- |
| 1 (leftmost bit) | - UTRA CELL_PCH to EUTRA RRC_IDLE cell reselection - UTRA URA_PCH to EUTRA RRC_IDLE cell reselection |  |
| 2 | EUTRAN measurements and reporting in connected mode |  |
| 3 | Undefined |  |
| 4 | Undefined |  |

Clarification for mobility to EUTRAN

For mobility to E-UTRAN, it is assumed that we have 6 main "functions":

A.	Support of measurements and cell reselection procedure in idle mode

B.	Support of measurements and cell reselection procedure in CELL/URA_PCH

C.	Support of RRC release with redirection procedure

D.	Support of RRC reject with redirection procedure

E.	Support of EUTRAN measurements and reporting in connected mode

F.	Support of handover procedure in connected mode

Of the above, all UEs that indicate support of E-UTRA in UE capability signalling "Support of E-UTRA FDD" or "Support of E-UTRA TDD" support A), C) and D) EUTRA bands.

For B) above, UEs indicate support by the Group 1 indicator bit (if Group 1 is set to "true", it is supported for all EUTRA bands the UE supports).

For E) above, UEs indicate support by the Group 2 indicator bit (if Group 2 is set to "true", it is supported for all EUTRA bands the UE supports). The compressed mode capability for supported E-UTRA frequency bands ("Need for compressed mode") in "Measurement capability extension" is ignored by the network when Group 2 is set to "false".

For F) above, UEs indicate support by the separate UE capability signalling "Support of Inter-RAT PS Handover to E-UTRA FDD/TDD" defined in TS 25.306 (if this bit is set to "true", PS handover is supported for all EUTRA bands the UE supports). This bit can only be set to "true" if the UE has set the Group 2 indicator bit to "true".

6.2.3.4a.3	Test description

6.2.3.4a.3.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 5.

-	Cell 1 suitable neighbour E-UTRA cell

-	Cell 5 UTRA serving cell

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode according to section 4.5A.3A of TS 36.508 [18] on Cell 5 (serving cell).

-	The cell power level and parameter for Cell 5 is as follow:

| Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- |
| CPICH_Ec | dBm/3.84 MHz | -60 |  |
| P-CCPCH | dBm/1.28 MHz | -62 |  |
| Qrxlevmin (FDD) | dBm | -79 | Default value |
| Qrxlevmin(TDD) | dBm | -81 | Default value |

6.2.3.4a.3.2	Test procedure sequence

Tables 6.2.3.4a.3.2-1 & Table 6.2.3.4a.3.2-2 illustrate the downlink power levels and other changing parameters to be applied for the cells applied at the points indicated in the Main behaviour description in Table 6.2.3.4a.3.2-3.

Table 6.2.3.4a.3.2-1: Time instances of cell power level and parameter changes for E-UTRA cell

|  | Parameter | Unit | Cell 1 | Remark |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -75 | Steps 1-3, SqualnonServingCell,1 < Threshx,high2(26 dB) Steps 4-7, SqualnonServingCell,1 >Threshx,high2(4 dB) |
|  | RSRQ | dB | -3 |  |
|  | Qrxlevmin | dBm | -84 |  |
|  | Qqualmin | dB | -20 |  |
|  | Noc | dBm/15kHz | off |  |
| Note: The downlink signal level uncertainty is specified in TS 36.508 [18] section 4.3.4.1. |  |  |  |  |

Table 6.2.3.4a.3.2-2: Time instances of cell power level and parameter changes for UTRA cell

|  | Parameter | Unit | Cell 5 | Remark |
| --- | --- | --- | --- | --- |
| T1 | CPICH_Ec | dBm/3.84 MHz | -70 |  |
|  | P-CCPCH | dBm/1.28 MHz | -72 |  |
|  | CPICH_Ec/Io | dB | -2.89 |  |
|  | QqualminFDD | dB | -20 |  |
|  | Ioc | dBm/3.84MHz | off |  |

Table 6.2.3.4a.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 1 and Cell 5 levels according to the row "T1" in table 6.2.3.4a.3.2-1 and 6.2.3.4a.3.2-2. | - | - | - | - |
| 2 | The UE is brought to state CELL_PCH (state 6-12) on Cell 5 according to TS 34.108 [5] with modified SIB-19 info so that Cell 1 Squal cell 1 < threshXhigh2 according to table 6.2.3.4a.3.2-1. | - | - | - | - |
| 3 | Check: Does the test result of generic test procedure in TS 36.508 [18] Table 6.4.2.7A-2 indicate that the UE is NOT camped on E-UTRAN Cell 1? (Note 2) | - | - | - | - |
| 4 | The SS changes the SIB-19 info so that for Cell 1 Squalcell1> threshXhigh2 according to table 6.2.3.4a.3.2-2. | - | - | - | - |
| 5 | Notify UE change of System Information. | <- | PAGING TYPE 1 | - | - |
| 6 | Wait for 6 s for UE to receive system information. | - | - | - | - |
| 7 | Check: Does the test result of generic test procedure in TS 36.508 [18] Table 6.4.2.7A-1 indicate that the UE is camped on E-UTRAN Cell 1? (Note 2) | - | - | 1 | - |
| - | Exception: Steps 8 to 8a5/8b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 8 | If pc_IMS = TRUE SS starts 10s timer. | - | - | - | - |
| 8a1 | The UE establishes an RRC connection and transmits a SERVICE REQUEST message. | - | - | - | - |
| 8a2 | The SS establishes SRB2 and DRB. | - | - | - | - |
| 8a3 | If MULTI-PDN = TRUE the UE executes the generic procedure for establishment of additional PDN connectivity defined in TS 36.508 [18] 4.5A.16.3. | - | - | - | - |
| 8a4 | The UE performs IMS registration using the generic procedure defined in TS 34.229-1 [35] Annex C.2 steps 4-11. | - | - | - | - |
| 8a5 | The SS releases the RRC connection. | - | - | - | - |
| 8b1 | The 10s timer expires. | - | - | - | - |
| Note 1: Void Note 2: UE shall search higher priority layer (i.e. E-UTRA carrier frequency of Cell 1) at least every (60 * Nlayers) seconds, where Nlayers = 1 as specified in TS 36.133 [34] clause 4.2.2. |  |  |  |  |  |

6.2.3.4a.3.3	Specific message contents

Table 6.2.3.4a.3.3-0: Conditions for specific message contents
in Table 6.2.3.4a.3.3-1 and Table 6.2.3.4a.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.4a.3.3-1: System Information Block type 19 for Cell 5 (at preamble, Table 6.2.3.4a.3.2-3)

| Derivation Path: TS 36.508 [18] table 4.4.4.1-1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment |  |  | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |  |  |  |
| priority |  | 3 |  |  |  |  |
| s-PrioritySearch1 |  | 2 (4 dB) |  |  |  |  |
| s-PrioritySearch2 |  | 0 (0 dB) |  |  |  |  |
| threshServingLow |  | 2 (4 dB) |  |  |  |  |
| } |  |  |  |  |  |  |
| utran-FDD-FrequencyList |  | Not present |  |  |  | UTRA-FDD |
| utran-TDD-FrequencyList |  | Not present |  |  |  | UTRA-TDD |
| } |  |  |  |  |  |  |
| gsm-PriorityInfoList |  | Not present |  |  |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { |  |  |  |  |  |  |
| earfcn[1] |  | Same downlink EARFCN as used for Cell 1 |  |  |  |  |
| measurementBandwidth[1] |  | Not present |  |  |  |  |
| priority[1] |  | 5 |  |  |  |  |
| qRxlevMinEUTRA[1] |  | -84 (-168 dBm) |  |  |  |  |
| qQualminEUTRA[1] |  | -20 |  |  |  |  |
| eutra-exclude-ListedCellList[1] |  | Not present |  |  |  |  |
| eutraDetection[1] |  | TRUE |  |  |  |  |
| } |  |  |  |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |  |  |  |  |  |
| utra-PriorityInfoList-v920ext SEQUENCE { |  |  |  |  |  |  |  |  |
| threshServingLow2 |  | 8 (8dB) |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |  |  |  |  |  |
| qQualMinEUTRA[1] |  | -20 (-20dB) |  |  |  |  |  |  |
| threshXhigh2[1] |  | 26 (26dB) |  |  |  |  |  |  |
| threshXlow2[1] |  | 4 (4dB) |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  |  |  |  | Band > 64 |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |  |  |  |
| earfcn[1] |  | Same downlink EARFCN as used for Cell 1 |  |  |  |  |
| measurementBandwidth[1] |  | Not present |  |  |  |  |
| priority[1] |  | 5 |  |  |  |  |
| qRxlevMinEUTRA[1] |  | -84 (-168 dBm) |  |  |  |  |
| qQualminEUTRA[1] |  | -20 |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| Note: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 0 to 7. |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.2.3.4a.3.3-2: System Information Block type 19 for Cell 5 (at step 4, Table 6.2.3.4a.3.2-3)

| Derivation Path: TS 36.508 [18] table 4.4.4.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SysInfoType19 ::= SEQUENCE { |  |  |  |
| utra-PriorityInfoList SEQUENCE { |  |  |  |
| utra-ServingCell SEQUENCE { |  |  |  |
| priority | 3 |  |  |
| s-PrioritySearch1 | 2 (4 dB) |  |  |
| s-PrioritySearch2 | 0 (0 dB) |  |  |
| threshServingLow | 2 (4 dB) |  |  |
| } |  |  |  |
| utran-FDD-FrequencyList | Not present |  | UTRA-FDD |
| utran-TDD-FrequencyList | Not present |  | UTRA-TDD |
| } |  |  |  |
| gsm-PriorityInfoList | Not present |  |  |
| eutra-FrequencyAndPriorityInfoList SEQUENCE (SIZE (1..maxNumEUTRAFreqs) OF SEQUENCE { |  |  |  |
| earfcn[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth[1] | Not present |  |  |
| priority[1] | 5 |  |  |
| qRxlevMinEUTRA[1] | -84 (164 dBm) |  |  |
| threshXhigh[1] | 15 (30 dB) |  |  |
| qQualminEUTRA[1] | -20 |  |  |
| eutra-exclude-ListedCellList[1] | Not present |  |  |
| eutraDetection[1] | TRUE |  |  |
| } |  |  |  |
| v920NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-v920ext SEQUENCE { |  |  |  |
| utra-PriorityInfoList-v920ext SEQUENCE { |  |  |  |
| threshServingLow2 | 8 (8dB) |  |  |
| } |  |  |  |
| eutra-FrequencyAndPriorityInfoList-v920ext (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| qQualMinEUTRA[1] | -20 (-20dB) |  |  |
| threshXhigh2[1] | 4 (4dB) |  |  |
| threshXlow2[1] | 4 (4dB) |  |  |
| } |  |  |  |
| } |  |  |  |
| va80NonCriticalExtensions SEQUENCE { |  |  | Band > 64 |
| vb30NonCriticalExtensions SEQUENCE { |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { |  |  |  |
| sysInfoType19-vb50ext SEQUENCE { |  |  |  |
| eutra-FrequencyAndPriorityInfoExtensionList SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF SEQUENCE { |  |  |  |
| earfcn[1] | Same downlink EARFCN as used for Cell 1 |  |  |
| measurementBandwidth[1] | Not present |  |  |
| priority[1] | 5 |  |  |
| qRxlevMinEUTRA[1] | -84 (164 dBm) |  |  |
| threshXhigh[1] | 15 (30 dB) |  |  |
| qQualminEUTRA[1] | -20 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note: GERAN cell priority is not allocated, hence it will be considered as the less than all allowed values of 0 to 7. |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 6.2.3.4a.3.3-3: PAGING TYPE 1 for Cell 5 (step 5, Table 6.2.2.4a.3.2-3)

| Derivation path: TS 34.108 [5] clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PagingType1 ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| bcch-ModificationInfo SEQUENCE { |  |  |  |
| mib-ValueTag | Set to (Current MIB value tag + 1) |  |  |
| bcch-ModificationTime | Not present |  |  |
| } |  |  |  |
| laterNonCriticalExtensions | Not present |  |  |
| } |  |  |  |

Table 6.2.3.4a.3.3-4: SystemInformationBlockType3 for Cell 1 (preamble and all steps, Table 6.2.3.4a.3.2-3)

| Derivation path: TS 36.508 [18] table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 5 |  |  |
| } |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| s-IntraSearch-v920 SEQUENCE {} | Not present |  |  |
| s-NonIntraSearch-v920 SEQUENCE {} | Not present |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| threshServingLowQ-r9 | 0 dB |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.4a.3.3-5: SystemInformationBlockType6 for Cell 1 (preamble and all steps, Table 6.2.3.4a.3.2-3)

| Derivation path: TS 36.508 [18] table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[1] | Same as Cell 5 |  |  |
| cellReselectionPriority[1] | 3 |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[1] | Same as Cell 5 |  |  |
| cellReselectionPriority[1] | 3 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

[End of change]