3GPP TSG-RAN5 Meeting #107	R5-252645

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1612 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of EPS eCall test case 21.30 for CEN alignments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New eCall test case 21.30 to be added for CEN requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 21.30 [eCall capable / Automatic initiation / Emergency registration / 200 OK with ACK / PSAP Callback / SIP INFO request for MSD Update / Success]. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eCallCEN-UEConTest WI will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.30 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 34.229-2 CR 0371 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

## 21.30	eCall capable / Automatic initiation / Emergency registration / 200 OK with ACK / PSAP Callback / SIP INFO request for MSD Update / Success

### 21.30.1	Definition

Test to verify that the ecall capable UE sends correctly composed SIP INVITE and SIP INFO for MSD transfer during automatically initiated eCall over IMS and PSAP callback respectively. This process is described in 3GPP TS 24.229 [10].

### 21.30.2	Conformance requirement

[Rel-19, TS 24.229, clause 5.1.6B]:

This clause specifies a feature which applies to UEs subject to national regulations in CEN EN 17184:2024 [300]. The procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD shall be supported by a UE during an incoming IMS call when the following conditions are fulfilled:

1)	the UE previously established or attempted to establish an emergency call either over IMS or in the CS domain of the automatically initiated eCall type of emergency service or of the manually initiated eCall type of emergency service;

2)	the incoming IMS call occurred within an implementation dependent time period of at least one hour;

3)	the INVITE for the incoming IMS call contains a Priority header field with header field value "psap-callback";

4)	the INVITE for the incoming IMS call contains an Accept header field that includes the value “application/EmergencyCallData.Control+xml”; and

5)	the INVITE for the incoming IMS call contains a Recv-Info header field with header field value EmergencyCallData.eCall.MSD.

NOTE 1:	When there are multiple attempts to establish the emergency call, the emergency call in condition 2) refers to the last attempt.

When conditions 1) to 5) are fulfilled, the UE shall include in the 200 OK:

a)	a Recv-Info header field with header field value EmergencyCallData.eCall.MSD; and

b)	an Accept header field indicating the UE is willing to accept an "application/EmergencyCallData.Control+xml" MIME type as defined in RFC 8147 [244].

The UE shall also support the procedures specified in subclause 5.1.6.11.3 for transfer of an updated MSD.

When conditions 1) and 2) and, as an implementation option, condition 3) are fulfilled, the UE shall support request and transfer of an updated MSD using audio media stream as described in 3GPP TS 26.267 [9C].

NOTE 2:	A PSAP can request transfer of an updated MSD using audio media stream if the procedures specified in subclause 5.1.6.11.3 cannot be supported by the PSAP, the serving PLMN, the HPLMN or by some other intermediate network.

Reference(s)

3GPP TS 24.229 [10], clause 5.1.6B.

### 21.30.3	Test purpose

1) 	To verify that an ecall capable UE checks the header fields in incoming IMS call for PSAP callback and responds with 200 OK including correctly composed Recv-Info and Accept header fields.

2) 	To verify that the UE in ecall and normal service sends updated MSD in a correctly composed SIP INFO during PSAP callback after automatic eCall over IMS is successfully released.

### 21.30.4	Method of test

Initial conditions

The UE contains either ISIM and USIM applications or only USIM application on UICC with eCall subscription. The UE has discovered P-CSCF and registered to IMS services.

The SS is configured with the IMSI within the USIM application, the home domain name, public and private user identities together with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) that is configured on the UICC card equipped into the UE. SS is listening to SIP default port 5060 for both UDP and TCP protocols. SS is able to perform AKAv1-MD5 authentication algorithm for that IMPI, according to 3GPP TS 33.203 [14] clause 6.1 and RFC 3310 [17].

Test procedure applicable for a UE subjected to national regulations in CEN EN 17184:2024 for IMS eCall over EPS (TS 34.229-2 [5] A.4.5/67)

1)	Automatic eCall is initiated at UE.

2-5) Emergency registration according to C.20 is executed.

6-8) Automatic eCall is established successfully according to C.47.

9) SS sends BYE to the UE.

10) SS expects and receives 200 OK.

11)	 SS sends SIP INVITE with correctly composed Accept, Priority and Recv-Info header.

12) SS expects call setup with correctly composed 200 OK with Accept and Recv-Info headers and sends ACK.

13)	 SS sends INFO request to transfer MSD.

14)	 SS expects and receives 200 OK.

15)	 SS expects and receives INFO request from the UE with MSD contents.

16) SS responds with 200 OK for INFO request from UE.

17) SS sends BYE to the UE.

18) SS expects and receives 200 OK for BYE from the UE.

Expected sequence:

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 | - |  | UE is triggered to initiate an automatic eCall |  |
| 2-5 | - |  | Step 1-4 defined in Annex C.20 | UE establishes the emergency PDN and performs IMS emergency registration. Referred from 36.508 [94] table 4.5A.26.3-1 for a UE with E-UTRA support. |
| 6-8 | - |  | Steps 1-3 as defined in Annex C.47 | eCall setup. |
| 9-12 | - |  | Steps 1-4 as defined in Annex C.33 | SS releases the eCall. |
| 13-25 | - |  | Steps 1-13 as defined in Annex C.11 | Steps 1 to 23A in generic procedure 36.508 [94] table 4.5A.7.3-1. |
| 26-29 | - |  | Steps 4-7 as defined in Annex C.47 | UE responds to incoming SIP INFO with 200 OK and sends updated MSD. |
| 30-33 | - |  | Steps 1-4 as defined in Annex C.33 | SS releases the call. |

Specific Message Contents:

INVITE (Step 6, step 1 of C.47)

Use INVITE as specified for step 1 in annex C.47 with condition A21.

200 OK for INVITE (Step 7, step 2 of C.47)

Use 200 OK as specified for step 2 in annex C.47 with conditions A12 and A13.

INVITE (Step 13, step 1 of C.11)

Use INVITE as specified for step 1 in annex C.11 with condition A12.

200 OK for INVITE (Step 24, step 12 of C.11)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in annex A.3.1 with condition A24.

BYE (Step 9 and Step 30, step 1 of C.33)

Use the default message “BYE” in annex A.2.8 with condition A9.

### 21.30.5	Test requirements

The UE shall send requests and responses as described in clause 21.30.4.

<End of modified section>