3GPP TSG-RAN5 Meeting #107	R5-252667

Malta, Malta, May 19th – 23rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3367 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding testing for NS_47 power class 1.5 in Additional spurious emissions for UL MIMO test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16,LTE_NR_B41_Bn41_PC29dBm-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The power class 1.5 in Additional spurious emissions for UL MIMO test case is absent in 6.5D.3.3 and need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding test Configuration table for NS_47 power class 1.5 in 6.5D.3.3.4.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The band n41 test cases for NS_47 power class 1.5 is absent in Additional spurious emissions for MIMO test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5D.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.5D.3.3	Additional spurious emissions for UL MIMO

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- There are NS-XXs other than NS_04, NS_21 and NS_47 that are FFS:

6.5D.3.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5D.3.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 that support 2-layer codebook based UL MIMO.

6.5D.3.3.3	Minimum conformance requirements

The additional spurious emission requirements specified in 6.5.3.3.3 apply to the sum of the emissions from both UE antenna connectors.

The normative reference for this requirement is TS 38.101-1 [2] clauses 6.5D.3 and 6.5.3.3

6.5D.3.3.4	Test description

6.5D.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, and are shown in Table 6.5D.3.3.4.1-1 through Table 6.5D.3.3.4.1-6. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5D.3.3.4.1-1: Test Configuration Table (network signalling value "NS_04")

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | (See Freq column) |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest |  |
| Additional spurious emissions test parameters for NS_04 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq |  |  | N/A for A-MPR testing | Modulation |  | RB allocation (NOTE 1) |
|  |  |  |  |  | (NOTE 2) |  |  |
| 1 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 2 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 3 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 4 | Low |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 5 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 6 | High |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 7 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 8 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 9 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 10 | Low |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 11 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 12 | High |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 13 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 14 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 15 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 16 | Low |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 17 | High |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 18 | High |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 19 | Low |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 20 | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 21 | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 22 | Low |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| 23 | High |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 24 | High |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |  |  |  |  |

Table 6.5D.3.3.4.1-2: Test Configuration table for NS_47 power class 3 (contiguous allocation)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-19 and 6.2.3.4.1-20 |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 30 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest |  |  |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 43 | Default | 30 | Default |  | CP-OFDM | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 44 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 45 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 46 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 47 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 48 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 49 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 50 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |
| 51 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 52 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |  |
| 53 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |  |
| 54 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 55 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 56 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 57 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |  |
| 58 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 59 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Right (A3) |  |  |  |
| 60 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |  |
| 61 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 62 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 63 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 64 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |  |
| 65 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 66 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 67 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 68 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 69 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 70 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n41. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n41, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n41. NOTE 5: Void |  |  |  |  |  |  |  |  |  |  |

Table 6.5D.3.3.4.1-3: Test Configuration table for NS_47 power class 3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-19 and 6.2.3.4.1-20 |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 30 MHz |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation |  | RB allocation (Note 1) |
| 1 | Default | 30 | Default |  | CP-OFDM | QPSK | Outer_Full (A2) |
| 2 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |
| 3 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |
| 4 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.2.2.4.1-4. NOTE 2: Void. |  |  |  |  |  |  |  |

Table 6.5D.3.3.4.1-4: Test Configuration table for NS_47 power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-19 and 6.2.3.4.1-20 |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 30 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest |  |  |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 36 | Default | 30 | Default |  | CP-OFDM | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 37 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 38 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 39 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 40 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 41 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 42 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 43 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |
| 44 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 45 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A3) |  |  |  |
| 46 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |  |
| 47 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 48 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 49 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 50 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |  |
| 51 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 52 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Right (A3) |  |  |  |
| 53 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |  |
| 54 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 55 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 56 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 57 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |  |
| 58 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 59 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 60 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 61 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 62 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 63 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Void |  |  |  |  |  |  |  |  |  |  |

Table 6.5D.3.3.4.1-5: Test Configuration table for NS_47 power class 1.5

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  |  | As specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 30 MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest |  |  |
| A-MPR test parameters for NS_47 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 30 | Default |  | CP-OFDM | QPSK | Edge_1RB_Left (A1) |  |  |  |
| 2 | Default | 30 | Default |  |  | QPSK | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 3 | Default | 30 | Deafult |  |  | QPSK | Edge 1RB_Right (A3) |  |  |  |
| 4 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 5 | Default | 30 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 6 | Default | 30 | Default |  |  | QPSK | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 7 | Default | 30 | Default |  |  | QPSK | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 8 | Default | 30 | Default |  |  | QPSK | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 9 | Default | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A1) |  |  |  |
| 10 | Default | 30 | Default |  |  | 16 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 11 | Default | 30 | Default |  |  | 16 QAM | Edge 1RB_Right (A3) |  |  |  |
| 12 | Default | 30 | Default |  |  | 16 QAM | Outer_Full (A2) |  |  |  |
| 13 | Default | 30 | Default |  |  | 16 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 14 | Default | 30 | Default |  |  | 16 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 15 | Default | 30 | Default |  |  | 16 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 16 | Default | 30 | Default |  |  | 16 QAM | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 17 | Default | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A1) |  |  |  |
| 18 | Default | 30 | Default |  |  | 64 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 19 | Default | 30 | Default |  |  | 64 QAM | Edge 1RB_Right (A3) |  |  |  |
| 20 | Default | 30 | Default |  |  | 64 QAM | Outer_Full (A2) |  |  |  |
| 21 | Default | 30 | Default |  |  | 64 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 22 | Default | 30 | Default |  |  | 64 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 23 | Default | 30 | Default |  |  | 64 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 24 | Default | 30 | Default |  |  | 64 QAM | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| 25 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A1) |  |  |  |
| 26 | Default | 30 | Default |  |  | 256 QAM | 1@29 (A2) |  | 1@15 (A2) | 1@8 (A2) |
| 27 | Default | 30 | Default |  |  | 256 QAM | Edge 1RB_Right (A3) |  |  |  |
| 28 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 29 | Default | 30 | Default |  |  | 256 QAM | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 30 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A4) |  | 40@0 (A4) | 20@0 (A4) |
| 31 | Default | 30 | Default |  |  | 256 QAM | 54@0 (A2) |  | 27@0 (A2) | 12@0 (A2) |
| 32 | Default | 30 | Default |  |  | 256 QAM | 80@35 (A5) |  | 40@18 (A5) | 20@9 (A5) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |  |

Table 6.5D.3.3.4.1-6: Test Configuration table for NS_21

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |
| A-MPR test parameters for NS_21 |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |
|  |  |  |  | N/A for A-MPR test cases |  |  |  |  |
| 26 | Low | Default | Default |  | CP-s OFDM | QPSK | Edge_1RB_Left |  |
| 27 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |  |
| 28 | Default | Default | Default |  |  | QPSK | Outer_Full |  |
| 29 | Default | 10 MHz | Default |  |  | QPSK | 4@0 | 2@0 |
| 30 | Default | 10 MHz | Default |  |  | QPSK | 4@48 | 2@22 |
| 31 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |  |
| 32 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |  |
| 33 | Default | Default | Default |  |  | 16 QAM | Outer_Full |  |
| 34 | Default | 10 MHz | Default |  |  | 16 QAM | 4@0 | 2@0 |
| 35 | Default | 10 MHz | Default |  |  | 16 QAM | 4@48 | 2@22 |
| 36 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |  |
| 37 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |  |
| 38 | Default | Default | Default |  |  | 64 QAM | Outer_Full |  |
| 39 | Default | 10 MHz | Default |  |  | 64 QAM | 4@0 | 2@0 |
| 40 | Default | 10 MHz | Default |  |  | 64 QAM | 4@48 | 2@22 |
| 41 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |  |
| 42 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |  |
| 43 | Default | Default | Default |  |  | 256 QAM | Outer_Full |  |
| 44 | Default | 10 MHz | Default |  |  | 256 QAM | 4@0 | 2@0 |
| 45 | Default | 10 MHz | Default |  |  | 256 QAM | 4@48 | 2@22 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

<Unchanged Skipped>

<End of Changes>