3GPP TSG-RAN5 Meeting #107	R5-252670

# St Julian’s, Malta, May 19th – 23rd 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on testing AMPR for UL MIMO

Document for:	Endorsement

Introduction

It has been verified again in the A-MPR for UL MIMO test case in clause 6.2D.3 38.521-1. A large number of test points need to be tested with different frequency bands or network signalling value, resulting in redundant test points and great time costing.

This document is proposed to remove the ULFPTx feature from the AMPR for UL MIMO testing in RAN5.

Discussion

The test procedure for verifying whether the UE can reach the maximum transmit power when the ULFPTx feature is enabled in clause 6.2D.1, 6.2D.2, 6.2D.4 38.521-1[1]. If the test requirements are met, the feature has been verified as shown in the following contents:

[Quote from 38.521-1 clause 6.2D.1, 6.2D.2, 6.2D.4]

6.2D.1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 FWA UEs, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

6.2D.2.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

6.2D.4.2	Test applicability

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

This test case applies to all types of NR Power Class 1.5, Power Class 2 and Power Class 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

[Unquote]

Observation 1:	For the uplink full power transmission (ULFPTx) feature, whether the UE could set the transmit power to the target power within allowed tolerance has been verified in multiple test points in clause 6.2D.1, 6.2D.2 and 6.2D.4 38.521-1.

A-MPR is used to verify the additional maximum power reduction capability of the UE when it receives different NS indications from the network, as described in clause 6.2D.3 38.521-1[1]. It includes two aspects: 1) The UE could interpret the NS indications correctly and set the target power properly; 2) The UE could transmit according to the target power. Whether the UE could correctly interpret the NS indications is verified in several test cases such as 6.2.3, 6.2D.3 for UL-MIMO, and whether the UE could transmit the output power properly is also verified by 6.2D.1, 6.2D.2, 6.2D.4 as mentioned in Observation 1. As a result, the verification for ULFPTx in A-MPR MIMO test cases could be covered by other test cases and doesn’t need to be repeated.

Observation 2:	The verification for ULFPTx in A-MPR MIMO test cases could be covered by other test cases and doesn’t need to be repeated.

In 6.2D.3 38.521-1[1], UE additional maximum output power reduction for UL MIMO need to test many band and NS values as described below.

[Quote from 38.521-1 clause 6.2D.3]

6.2D.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated a unique with network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2D.1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2D.3.2	Test applicability

The requirements of this test apply in test case 6.5D.2.3 Additional Spectrum Emission mask for UL MIMO for network signalling value NS_03, NS_03U, NS_04, NS_06, NS_07 to all types of NR UE release 15 and forward that support UL MIMO.

The requirements of this test apply in test case 6.5D.2.3 Additional Spectrum Emission mask for UL MIMO for network signalling value NS_03, NS_03U, NS_04 to all types of NR UE release 16 and forward that support UL full power transmission (ULFPTx).

The requirements of this test apply in test case 6.5D.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, and NS_100 to all types of NR Power Class 1.5, Power Class 2 and 3 UE release 15 and forward that support 2-layer codebook based UL MIMO.

The requirements of this test apply in test case 6.5D.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U to all types of NR Power Class 2 and 3 UE release 16 and forward that support UL full power transmission (ULFPTx).

The requirements of this test apply in test case 6.5D.3.3 Additional Spurious Emissions for network signalling value NS_04, NS_07, NS_12, NS_13, NS_14_NS_15, NS_47, NS_21, NS_43, NS_43U, NS_56, NS_50 to all types of NR UE release 15 and forward that support UL-MIMO.

The requirements of this test apply in test case 6.5D.3_1.3 Additional Spurious Emissions for network signalling value NS_04 to all types of NR UE release 16 and forward that support UL full power transmission (ULFPTx).

NOTE:	Test execution is not necessary if 6.5D.2.3, 6.5D.2.4.2 and 6.5D.3.3 are executed

[Unquote]

Observation 3:	The A-MPR test workload is large due to the different bands and NS network signaling need to be tested.

Proposal 1: 	To avoid redundant testing, the test execution of the ULFPTx feature in A-MPR test case for UL MIMO can be removed.

Conclusion

Based on the above discussion, following observations can be obtained:

Observation 1:	The uplink full power transmission (ULFPTx) feature for UL MIMO UEs supporting has been verified in UE maximum out power for UL MIMO test case in clause 6.2D.1 38.521-1.

Observation 2:	The verification for ULFPTx in A-MPR MIMO test cases could be covered by other test cases and doesn’t need to be repeated.

Observation 3:	The A-MPR test workload is large due to the different bands and NS network signaling need to be tested.

To avoid redundant testing, RAN5 can consider the following proposal:

Proposal 1: 	To avoid redundant testing, the test execution of the ULFPTx feature in A-MPR test case for UL MIMO can be removed.

- 4 References

- TS 38.521-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone