3GPP TSG-RAN5 Meeting #107	R5-252679

# Malta, Malta, 19th May 2025 - 23rd May 2025

# Source:	Qualcomm Incorporated

# Title:	Impact of unsecure IRAT redirection on legacy LTE testcases

# Document for:	Discussion

## 1.	Introduction

3GPP CT1 introduced the network policy IE including redir-policy bit in ATTACH ACCEPT and TRACKING AREA UPDATE ACCEPT messages from Rel-14 onwards with [1]. 3GPP RAN2 modified the UE behaviour at reception of RRCConnectionRelease in [2] such that UE, based on indication from NAS, discards redirection to GERAN unless AS security is activated.

In Rel-18, the redir-policy bit is made mandatory to include in ATTACH ACCEPT and TRACKING AREA UPDATE ACCEPT messages, and it was extended to also allow prohibition of insecure redirection to UTRAN with [3] and [4].

Below test cases were added in [5] based on discussion paper [6] that checks whether the UE discards redirection from E-UTRAN to UTRAN/GERAN unless AS security is activated.

- 8.1.3.6b		RRC connection release / Redirection from E-UTRAN to UTRAN / redir-policy bit

- 8.1.3.8a		RRC connection release / Redirection from E-UTRAN to GERAN / redir-policy bit

In these two test cases, SS includes Network policy IE in Attach Accept message and redir-policy bit is set to 1 (Unsecured redirection to GERAN or UTRAN not allowed). Impact of this feature on existing LTE test cases involving IRAT redirection was not assessed.

## 2.	Discussion

In Rel-18, on top of [3], 3GPP CT1 CR [7] further clarified that the UE shall set the network policy to "Unsecured redirection to GERAN or UTRAN not allowed" when it is not included in the ATTACH ACCEPT message.

TS 24.301 [4] Clause 5.5.1.2.4

If the redir-policy bit is set to "Unsecured redirection to GERAN or UTRAN not allowed" in the Network policy IE of the ATTACH ACCEPT message or the Network policy IE is not included in the ATTACH ACCEPT message, the UE shall set the network policy on unsecured redirection to GERAN or UTRAN for the current PLMN to "Unsecured redirection to GERAN or UTRAN not allowed" and indicate to the lower layers that unsecured redirection to a GERAN or UTRAN cell is not allowed.

Hence, when Network policy IE is not included in Attach Accept message, UE shall consider Unsecured redirection to GERAN or UTRAN as not allowed and discard the redirection procedure.

In existing LTE test cases of [5] from prior releases (Rel-8, 9), Network policy IE is not included in Attach Accept message. On further analysis, six such test cases in clause 13.1 (Multi-layer Procedures - Call setup) of [5] have been identified where redirection from E-UTRAN to UTRAN/GERAN is performed in an unsecured manner.

Impacted test cases:

- 13.1.2 - Call setup from E-UTRAN RRC_IDLE / CS fallback to UTRAN with redirection / MO call

- 13.1.2a - Call setup from E-UTRAN RRC_IDLE / CS fallback to UTRAN with redirection including System Information / MO call

- 13.1.7 - Call setup from E-UTRA RRC_IDLE / CS fallback to GSM with redirection / MT call

- 13.1.15 - Call setup from E-UTRAN RRC_IDLE / CS fallback to UTRAN with redirection / MT call / UTRAN cell is barred

- 13.1.19 - Emergency call setup from E-UTRAN RRC_IDLE / IMS VoPS supported / EMC BS not supported / CS fallback to UTRAN or GERAN with redirection

- 13.1.20 - Emergency call setup from E-UTRAN RRC_IDLE / IMS VoPS not supported / EMC BS supported / CS fallback to UTRAN or GERAN with redirection

A Rel-18 conformant UE may unfairly fail these test cases as unsecure redirection procedure is discarded by the UE.

## 3.  Proposal/Way Forward

These impacted test cases do not pertain to unsecure redirection. Therefore, to verify CSFB call setup functionality, proposals are as follows:

Proposal – 1: 	All the impacted test cases to be made applicable only till Rel-17. This would result in CSFB coverage miss for a Rel-18 and later UEs.

Proposal – 2: 	Add AS security mode procedure before redirection procedure to all the impacted test cases.

Proposal – 3.1: 	Update the Attach Accept message of all impacted test cases to include network policy IE and set value of redir-policy bit to 0 ("Unsecured redirection to GERAN or UTRAN allowed").

Proposal – 3.2: 	Set redir-policy bit of network policy IE in default message content of Attach Accept of [8] to 0 ("Unsecured redirection to GERAN or UTRAN allowed") based on the UE release (Rel-18 or later).

## 4.	References

[1]	C1-181204	Network policy indication on LTE call redirection to GERAN (Rel-14)

[2]	R2-1714284	Reject of unprotected redirect to GERAN

[3]	C1-236303	Network support and clarification of the redir-policy bit

[4]	R2-2312806	Network support and clarification of redirection to 3G

[5]	TS 36.523-1	LTE; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification

[6] 	R5-241178	Proposal for new test case related to network policy indication on LTE call redirection to GERAN and UTRAN

[7]	C1-243059	UE behaviour if Network policy IE is not included

[8]	TS 36.508	LTE; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing