3GPP TSG-RAN5 Meeting #107	R5-252732

# Malta, Malta, May.19th – 23rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5025 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of V2X SIG test case 12.1.9.2.2 - AM RLC polling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add V2X SIG test case 12.1.9.2.2 PC5-only operation / RLC / acknowledged mode / polling and retransmission |  |  |  |  |  |  |  |  |
| Summary of change: |  | - V2X SIG test case 12.1.9.2.2 PC5-only operation / RLC / acknowledged mode / polling and retransmission is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.9.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

12.1.9.2.2	PC5-only operation / RLC / acknowledged mode / polling and retransmission

12.1.9.2.2.1	Test Purpose (TP)

(1)

with { UE has established unicast link with peer UE and has established an unicast SL DRB with RLC-AM and sl-SN-FieldLengthAM = size12 }

ensure that {

when { A RLC SDU arrives and PDU_WITHOUT_POLL >= pollPDU after sending the AM PDU corresponding to the RLC SDU. }

then { UE sends AM PDU corresponding to the RLC SDU with polling }

}

(2)

with { UE has established unicast link with peer UE and has established an unicast SL DRB with RLC-AM and sl-SN-FieldLengthAM = size12 }

ensure that {

when { A RLC SDU arrives and BYTE_WITHOUT_POLL >= pollByte after sending the AM PDU corresponding to the RLC SDU }

then { UE sends AM PDU corresponding to the RLC SDU with polling }

}

(3)

with { UE has established unicast link with peer UE and has established an unicast SL DRB with RLC-AM and sl-SN-FieldLengthAM = size12 }

ensure that {

when { UE receives a STATUS PDU sent from peer UE }

then { UE retransmits all SDU/SDU segments indicated as NACK in the STATUS PDU }

}

12.1.9.2.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, subclause 5.3.2, 5.3.3.2 and 5.3.3.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.322, subclause 5.3.2]

The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following:

-	STATUS PDU from its peer AM RLC entity.

When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall:

-	if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer:

-	consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission.

…

When retransmitting an RLC SDU or an RLC SDU segment, the transmitting side of an AM RLC entity shall:

…

-	form a new AMD PDU which will fit within the total size of AMD PDU(s) indicated by lower layer at the particular transmission opportunity;

-	submit the new AMD PDU to lower layer.

When forming a new AMD PDU, the transmitting side of an AM RLC entity shall:

-	only map the original RLC SDU or RLC SDU segment to the Data field of the new AMD PDU;

-	modify the header of the new AMD PDU in accordance with the description in clause 6.2.2.4;

-	set the P field according to clause 5.3.3.

[TS 38.323, subclause 5.3.3.2]

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall:

-	increment PDU_WITHOUT_POLL by one;

-	increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU;

-	if PDU_WITHOUT_POLL >= pollPDU; or

-	if BYTE_WITHOUT_POLL >= pollByte:

-	include a poll in the AMD PDU as described below.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or

-	if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

-	include a poll in the AMD PDU as described below.

NOTE:	Empty RLC buffer (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation.

To include a poll in an AMD PDU, the transmitting side of an AM RLC entity shall:

-	set the P field of the AMD PDU to "1";

-	set PDU_WITHOUT_POLL to 0;

-	set BYTE_WITHOUT_POLL to 0.

Upon submission of an AMD PDU including a poll to lower layer, the transmitting side of an AM RLC entity shall:

-	set POLL_SN to the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer;

-	if t-PollRetransmit is not running:

-	start t-PollRetransmit.

-	else:

-	restart t-PollRetransmit.

[TS 38.323, subclause 5.3.3.3]

Upon reception of a STATUS report from the receiving RLC AM entity the transmitting side of an AM RLC entity shall:

-	if the STATUS report comprises a positive or negative acknowledgement for the RLC SDU with sequence number equal to POLL_SN:

-	if t-PollRetransmit is running:

-	stop and reset t-PollRetransmit.

12.1.9.2.2.3	Test description

12.1.9.2.2.3.1	Pre-test conditions

System Simulator:

-	NR-SS-UE

-	NR-SS-UE1 is as defined in TS 38.508-1 [4], configured for and operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	NR-SS-UE1 uses GNSS as the synchronization reference source.

-	GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- UE is authorised to perform NR sidelink communication.

-	The UE is equipped with a USIM containing default values as per TS 38.508-1 [4] clause 4.8.3.3.3 except for those listed in Table 12.1.9.2.2.3.1-1.

- The UE uses GNSS as the synchronization reference source.

Table 12.1.9.2.2.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°119 (V2X) supported |  |
| EFVST |  | As per TS 38.508-1 [4] clause 4.8.3.3.3 |  |
| EFV2XP_PC5 |  | As per TS 38.508-1[4] clause 4.8.3.3.3 SL-PreconfigurationNR included in V2X data policy over PC5 is defined in Table 12.1.9.2.2.3.3-1 |  |

Preamble:

-	The UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On) using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22.

12.1.9.2.2.3.2	Test procedure sequence

Table 12.1.9.2.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The NR-SS-UE sends a RRCReconfigurationSidelink message to reconfigure pollPDU =p8 and pollByte = infinity for SL DRB#1 | <-- | PC5 RRC: RRCReconfigurationSidelink |  |  |
| 2 | The UE sends a RRCReconfigurationCompleteSidelink message | --> | PC5 RRC: RRCReconfigurationCompleteSidelink |  |  |
| 3 | Trigger UE to close UE test loop mode E. (TRANSMIT). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). NOTE: The UE sends multiple packets continuously. | - | - | - | - |
| 4 | Check: Does the UE keep transmiting AMD PDU, with the poll bit set in the last PDU of every 4 PDUs? | --> | AMD PDUs | 1 | P |
| 5 | The NR-SS-UE transmits a Status Report with ACK_SN= k, NACK_SN=k-4, NACK_SN=k-5 (constructed by NACK_SN Range), NACK_SN = k-1, where k is the SN of the RLC PDU with polling sent in step 4 | <-- | Status Report | - | - |
| 6 | Check: Does the UE transmit AMD PDUs with the following SN? AMD PDU, SN=k-5, AMD PDU, SN=k-4 AMD PDU, SN=k-1 | --> | AMD PDU (SN=k-5), AMD PDU (SN=k-4) AMD PDU (SN=k-1) | 3 | P |
| 5 | Trigger UE to open UE test loop mode E. NOTE: Opening of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). | - | - | - | - |
| 6 | The NR-SS-UE sends a RRCReconfigurationSidelink message to reconfigure pollPDU =infinity and pollByte = kB2 for SL DRB#1 | <-- | PC5 RRC: RRCReconfigurationSidelink |  |  |
| 7 | The UE sends a RRCReconfigurationCompleteSidelink message | --> | PC5 RRC: RRCReconfigurationCompleteSidelink |  |  |
| 8 | Trigger UE to close UE test loop mode E. (TRANSMIT). NOTE: Closing of UE test loop mode E may be performed by MMI or AT command (+CCUTLE). NOTE: The UE sends multiple packets continuously. | - | - | - | - |
| 9 | Check: Does the UE keep transmiting AMD PDU, with the poll bit set in the last PDU of every 6 PDUs? (Note 2) | --> | AMD PDUs | 2 | P |
| Note 1: Selected SL grant of 357 bytes is chosen to allow the UE to send only one complete RLC SDU of size 347 bytes. The RLC PDU size is calculated as follows: RLC SDU size = 300 (SDAP SDU size given in 38.509 Table 5.3.4.3.1-1) +40 (Ipv6 header size) + 1 (SDAP header size) + 4 (PDCP header size) = 345 Bytes. RLC PDU size = 345 (RLC SDU size) + 2 (AM RLC header w/ 12bit SN and no SO)= 347 Bytes. TBS size = 357 Bytes based on IMCS = 22, sl-LengthSymbols = 10, sl-RB-Number = 10, sl-TimeResourcePSCCH = 2 and sl-PSSCH-DMRS-TimePatternList = {2,4} . Selected sidelink grant size - RLC SDU size = 357 (TBS size) – 4 (SL-SCH subheader size) – 3 (subPDU subheader size) – 347 (RLC PDU size) = 3 Bytes < 12 Bytes. Then the UE will include only an complete RLC SDU in the MAC PDU and include no RLC SDU segment according to the rules specified in TS 38.321 cl 5.22.1.4.1.3. Note 2: 345 Bytes x 6 PDUs = 2070 > 2 KB, with 1 kB = 1000 Bytes (TS 38.331 [12], clause 3.2). |  |  |  |  |  |

12.1.9.2.2.3.3	Specific message contents

Table 12.1.9.2.2.3.3-1: SL-PreconfigurationNR (Pre-configuration, UE under test)

| Derivation Path: TS 38.508-1 [4], Table 4.10.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-PreconfigurationNR-r16 ::= SEQUENCE { |  |  |  |  |
| sidelinkPreconfigNR-r16 SEQUENCE { |  |  |  |  |
| sl-PreconfigFreqInfoList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfigCommon-r16 { |  | 1 entry |  |  |
| SL-FreqConfigCommon-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-BWP-List-r16 SEQUENCE (SIZE (1..maxNrofSL-BWPs-r16)) OF SL-BWP-ConfigCommon-r16 { |  | 1 entry |  |  |
| SL-BWP-ConfigCommon-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-BWP-Generic-r16 SEQUENCE { |  |  |  |  |
| sl-LengthSymbols-r16 |  | sym10 |  |  |
| } |  |  |  |  |
| sl-BWP-PoolConfigCommon-r16 |  | SL-BWP-PoolConfigCommon | Table 12.1.9.2.2.3.3-2 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-RLC-BearerPreConfigList-r16 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16 { |  | 1 entry |  |  |
| SL-RLC-BearerConfig-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-MAC-LogicalChannelConfig-r16 SEQUENCE { |  |  |  |  |
| sl-HARQ-FeedbackEnabled-r16 |  | disabled |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| sl-UE-SelectedPreConfig-r16 SEQUENCE { |  |  |  |  |
| sl-PSSCH-TxConfigList-r16 SEQUENCE (SIZE (1..maxPSSCH-TxConfig-r16)) OF SL-PSSCH-TxConfig-r16 { |  | 1 entry |  |  |
| SL-PSSCH-TxConfig-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-ParametersBelowThres-r16 SEQUENCE { |  |  |  |  |
| sl-MinMCS-PSSCH-r16 |  | 22 | To ensure that only IMCS = 22 is used (R = 0.6504, Qm = 6) |  |
| sl-MaxMCS-PSSCH-r16 |  | 22 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.1.9.2.2.3.3-2: SL-BWP-PoolConfigCommon (Table 12.1.9.2.2.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-4 with RXPOOL and SELECTED |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-BWP-PoolConfigCommon-r16 ::= SEQUENCE { |  |  |  |  |
| sl-RxPool-r16 SEQUENCE (SIZE (1..maxNrofRXPool-r16)) OF SL-ResourcePool-r16 { |  | 1 entry |  |  |
| SL-ResourcePool-r16[1] |  | SL-ResourcePool | entry 1 Table 12.1.9.2.2.3.3-3 |  |
| } |  |  |  |  |
| sl-TxPoolSelectedNormal-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 { |  | 1 entry |  |  |
| SL-ResourcePoolConfig-r16[1] SEQUENCE { |  |  | entry 1 |  |
| sl-ResourcePool-r16 |  | SL-ResourcePool | Table 12.1.9.2.2.3.3-3 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 12.1.9.2.2.3.3-3: SL-ResourcePool (Table 12.1.9.2.2.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.6-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ResourcePool-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PSFCH-Config-r16 |  | Not present | PSFCH is not configured in resource pool |  |
| sl-NumSubchannel-r16 |  | 1 | To ensure there is only 1 subchannel in resource pool |  |
| sl-RB-Number-r16 |  | 10 |  |  |
| sl-TimeResource-r16 |  | 1111111111 |  |  |
| } |  |  |  |  |

Table 12.1.9.2.2.3.3-4: RRCReconfigurationSidelink (Step 1 and 6, Table 12.1.9.2.2.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1A-3 with condition RX and SL_DRB |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfigurationSidelink ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfigurationSidelink-r16 SEQUENCE { |  |  |  |  |
| slrb-ConfigToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-Config-r16 { |  | 1 entry |  |  |
| sl-RLC-ConfigPC5-r16 CHOICE { |  |  |  |  |
| sl-AM-RLC-r16 SEQUENCE { |  |  |  |  |
| sl-T-PollRetransmit-r16 |  | ms4000 |  |  |
| sl-PollPDU-r16 |  | p8 |  | Step 1 |
|  |  | infinity |  | Step 6 |
| sl-PollByte-r16 |  | infinity |  | Step 1 |
|  |  | kB2 |  | Step 6 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

<End of Change 1>