3GPP TSG-RAN5 Meeting #107	R5-252734

# Malta, Malta, May.19th – 23rd 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3507 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of default configurations of DCI formats for dynamic waveform switching SIG test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add default configuration of DCI format 0_1/0_2 for dynamic waveform switching test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - default configuration of DCI format 0_1/0_2 for dynamic waveform switching test cases are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.1.1.2, 4.3.6.1.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

4.3.6.1.1.2	Physical layer parameters for DCI format 0_1

DCI format 0_1 is used for the scheduling of PUSCH in one cell.

Default physical layer parameters for DCI format 0_1 are specified in table 4.3.6.1.1.2-1.

Table 4.3.6.1.1.2-1: Physical layer parameters for DCI format 0_1

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Carrier indicator | Not present | - |  |
| UL/SUL indicator | Not present (0 bit for UEs not configured with SUL in the cell) | - |  |
|  | 1 | 1 | SUL |
| Identifier for DCI formats | Indicating an UL DCI format | “0” |  |
| Bandwidth part indicator | Not present (indicating active BWP, not present in case of only one BWP-Id as per Table 4.6.3-8) | - |  |
| Frequency domain resource assignment | Dependent on test parameters | - |  |
| Time domain resource assignment | Indicating the first entry of PUSCH-TimeDomainResourceAllocationList to be used | “0000” |  |
| Frequency hopping flag | Not present | - |  |
| Modulation and coding scheme | Dependent on test parameters | - |  |
| New data indicator | Set for every data transmission / retransmission according to the rules specified in TS 38.321 [20] | - |  |
| Redundancy version | Dependent on test parameters | - |  |
| Transform precoder indicator | 1 bit. A value of "1" indicates that transform precoder is disabled, i.e. CP-OFDM waveform is used. | “1” | DWS |
|  | Not present | - |  |
| HARQ process number | Dependent on test parameters | - |  |
| 1st downlink assignment index | Dependent on test parameters |  |  |
| 2nd downlink assignment index | Not present (0 bit if one HARQ-ACK sub-codebook) | - |  |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) as per Table 7.1.1-1 in TS 38.213 [22] | “01” |  |
| SRS resource indicator | Not present | - |  |
| Precoding information and number of layers | Not present (0 bits for 1 antenna port and if the higher layer parameter txConfig = Codebook as per clause 7.3.1.1.2 TS 38.212 [27]) | - |  |
|  | 2 | “10” | 2TX_UL_MIMO or ULFPTx_Mode1 |
|  | Dependent on UE reported full power PMI | 1 bit as per Table 7.3.1.1.2-5 in TS 38.212 | ULFPTx_Mode2 |
|  | 0 or 1 | 1 bit | ULFPTx_ModeFull |
| Antenna ports | Port 0 (NOTE 2) | “000” |  |
|  |  | “00” | TRANSFORM _PRECODER _ENABLED |
| SRS request | No aperiodic SRS resource set triggered as per Table 7.3.1.1.2-24 in TS 38.212 (no SUL configured) | “00” |  |
| CSI request | Not present | - |  |
| CBG transmission information | Not present | - |  |
| PTRS-DMRS association | DMRS port 0 | “00” | PTRS_UL_CONFIG |
|  | Not present | - |  |
| beta_offset indicator | Not present (0 bit if the higher layer parameter dynamic in uci-on-PUSCH is not configured) | - |  |
| DMRS sequence initialization | = 0 (ScramblingID0 is not present as per Table 4.6.3-50) | “0” |  |
|  | Not present | - | TRANSFORM _PRECODER _ENABLED |
| UL-SCH indicator | Dependent on test parameters 1 bit. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. | - |  |
| ChannelAccess-CPext-CAPC | Not present (0 bit) | - |  |
|  | 2 as per Table 7.3.1.1.1-4A in TS 38.212 | "10" | SharedSpectrum AND pc_ul_Semi_StaticChAccess_r16 |
| Open-loop power control parameter set indication | Not present | - |  |
| Priority indicator | Not present | - |  |
| Invalid symbol pattern indicator | Not present | - |  |
| Minimum applicable scheduling offset indicator | Not present | - |  |
| SCell dormancy indication | Not present | - |  |
| Sidelink assignment index | Dependent on test parameters 2 bits |  | SL_Mode1 |
| NOTE 1: codebookSubset = nonCoherent, 2 layers,TPMI = 0 as specified in TS 38.212 [27] Table 7.3.1.1.2-4 NOTE 2: Bitsize depends on transform precoder being enabled/disabled (PUSCH_Config, Table 4.6.3-118) and on dmrs-Type and maxLength (DMRS-UplinkConfig, Table 4.6.3-51); 3 bits (transform precoder disabled) or 2 bits (transform precoder enabled) for DMRS type 1 and len1 |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| PTRS_UL_CONFIG | When PTRS Uplink is configured |
| TRANSFORM_PRECODER_ENABLED | Transform precoding is enabled (PUSCH_Config, Table 4.6.3-118) |
| SUL | On the SUL carrier when supplementary carrier is configured |
| ULFPTx_Mode1 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-1 |
| ULFPTx_Mode2 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-2 |
| ULFPTx_ModeFull | UL-MIMO test cases with UEs supporting UL full power transmission Mode-full power |
| SL_Mode1 | For SL mode 1 transmission test cases. |
| SharedSpectrum | Operation with shared spectrum channel access |
| DWS | When dynamic waveform switching is enabled for DCI format 0_1 |

4.3.6.1.1.3	Physical layer parameters for DCI format 0_2

DCI format 0_2 is used for the scheduling of PUSCH in one cell.

Default physical layer parameters for DCI format 0_2 are specified in table 4.3.6.1.1.3-1.

Table 4.3.6.1.1.3-1: Physical layer parameters for DCI format 0_2

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Identifier for DCI formats | Indicating an UL DCI format | “0” |  |
| Carrier indicator | Not present | - |  |
| UL/SUL indicator | Not present (0 bit for UEs not configured with SUL in the cell) | - |  |
| Bandwidth part indicator | Not present (indicating active BWP, not present in case of only one BWP-Id as per Table 4.6.3-8) | - |  |
| Frequency domain resource assignment | Dependent on test parameters | - |  |
| Time domain resource assignment | Indicating the first entry of PUSCH-TimeDomainResourceAllocationList to be used | “0000” |  |
| Frequency hopping flag | Not present (0 bit if the higher layer parameter frequencyHoppingDCI-0-2 is not configured) | - |  |
| Modulation and coding scheme | Dependent on test parameters | - |  |
| New data indicator | Set for every data transmission / retransmission according to the rules specified in TS 38.321 [20] | - |  |
| Redundancy version | Dependent on test parameters | - |  |
| HARQ process number | Dependent on test parameters | - |  |
| Transform precoder indicator | 1 bit. A value of "1" indicates that transform precoder is disabled, i.e. CP-OFDM waveform is used. | “1” | DWS |
|  | Not present | - |  |
| Downlink assignment index | Not present (0 bit if the higher layer parameter downlinkAssignmentIndexDCI-0-2 is not configured) |  |  |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) as per Table 7.1.1-1 in TS 38.213 [22] | “01” |  |
| SRS resource indicator | Not present | - |  |
| Precoding information and number of layers | Not present (0 bits for 1 antenna port and if the higher layer parameter txConfig = Codebook as per clause 7.3.1.1.2 TS 38.212 [27]) | - |  |
| Antenna ports | Not present (0 bit if higher layer parameter antennaPortsFieldPresenceDCI-0-2 is not configured) | - |  |
| SRS request | Not present (0 bit if the higher layer parameter srs-RequestDCI-0-2 is not configured) | - |  |
| CSI request | Not present | - |  |
| PTRS-DMRS association | DMRS port 0 | “00” | PTRS_UL_CONFIG |
|  | Not present | - |  |
| beta_offset indicator | Not present (0 bit if the higher layer parameter betaOffsets = semiStatic) | - |  |
| DMRS sequence initialization | Not present (0 bit if the higher layer parameter dmrs-SequenceInitializationDCI-0-2 is not configured or if transform precoder is enabled) | - |  |
| UL-SCH indicator | Dependent on test parameters 1 bit. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. | - |  |
| Open-loop power control parameter set indication | Not present (0 bit if the higher layer parameter p0-PUSCH-SetList is not configured) | - |  |
| Priority indicator | Not present (0 bit if higher layer parameter priorityIndicatorDCI-0-2 is not configured) | - |  |
| Invalid symbol pattern indicator | Not present (0 bit if higher layer parameter invalidSymbolPatternIndicatorDCI-0-2 is not configured) | - |  |
| NOTE 1: codebookSubset = nonCoherent, 2 layers,TPMI = 0 as specified in TS 38.212 [27] Table 7.3.1.1.2-4 NOTE 2: Bitsize depends on transform precoder being enabled/disabled (PUSCH_Config, Table 4.6.3-118) and on dmrs-Type and maxLength (DMRS-UplinkConfig, Table 4.6.3-51); 3 bits (transform precoder disabled) or 2 bits (transform precoder enabled) for DMRS type 1 and len1 |  |  |  |

| Condition | Explanation |
| --- | --- |
| PTRS_UL_CONFIG | When PTRS Uplink is configured |
| TRANSFORM_PRECODER_ENABLED | Transform precoding is enabled (PUSCH_Config, Table 4.6.3-118) |
| DWS | When dynamic waveform switching is enabled for DCI format 0_2 |

<End of Change 1>