3GPP TSG-RAN5 Meeting #107	R5-252776

# Malta, May 19 - 23 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3950 | rev | - | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR SA NES triggering conditional handover test cases including test tolerance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Netw_Energy_NR-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NR SA FR2 intra-frequency NES triggering conditional handover test cases were incomplete |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test case 7.3.3.3 and 7.3.3.4 is corrected, the following has been added, modified: Test Tolerance has been included in test case 7.3.3.3. Editors’ notes removed. Due to NR FR1 - FR2 OTA testability issue, RRM TT was not included to test case 7.3.3.4. +TT was added to the test parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test cases will remain incomplete and WP cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.3, 7.3.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/// Start of changes

#### 7.3.3.3	NR SA FR2 intra-frequency NES triggering conditional handover

7.3.3.3.1	Test purpose

This test is to verify the requirement for the NR FR2 intra-frequency NES triggering conditional handover requirements specified in 38.133 [6] clause 6.1.4.4.

7.3.3.3.2	Test applicability

This test applies to all types of NR UE from Release 18 onwards and supporting NES triggering conditional handover.

7.3.3.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.3.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.3.3.

7.3.3.3.4	Test description

7.3.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 7.3.3.3.4.1-1.

Table 7.3.3.3.4.1-1: test configurations

| Config | Description |
| --- | --- |
| 7.3.3.3-1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 7.3.3.3.4.1-2

Table 7.3.3.3.4.1-2: Initial conditions for intra-frequency NES triggering conditional handover

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 7.3.3.3.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR2 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 7.3.3.3.4.1-3 below.

Table 7.3.3.3.4.1-3: General test parameters for NES triggering Intra-frequency conditional handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset for condition |  | dBm | -2 Note 1 | Trigger HO to cell which may be measured as -1dB relative to cell 1. Actual SS-RSRP is 5dB stronger. |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |
| Note 1: Including test tolerance given in Annex F.1.3.2 |  |  |  |  |

7.3.3.3.4.2	Test procedure

At the start of time duration T1, the UE may not have any timing information of Cell 2. The UE is configured with a NES-based condition implying handover to cell 2 firstly, during T1, at a time earlier than TRRC before the beginning of T2 and then configured with DCI 2-9 with NES-mode indication during T2. No interruption shall be observed in time period T1,

From start of T2, the Cell 2 becomes detectable and NES-based event is fulfilled. During T2, the UE performs measurement on Cell 2 and evaluates the execution condition, and starts handover procedure when execution condition is satisfied. The UE shall send PRACH to Cell 2 less than Tmeasure + TCHO_execution + Tinterrupt ms from the start of T2. The Interruption length Tinterrupt shall also be verified in T2.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRCReconfiguration message. Cell 1 is the active cell. Set Cell 2 physical cell identity to the initial physical cell identity.

2.	Set the parameters according to T1 in Table 7.3.3.3.5-1. T1 starts.

3.	The SS starts continuously scheduling the UE to perform DL reception in every DL slot on Cell 1 and monitoring corresponding ACK/NACK feedback sent by the UE.

4.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

5.	The SS shall transmit an RRCReconfiguration message with conditionalReconfiguration on Cell 1 to configure NES-based CHO execution condition for the UE.

6.	The UE shall transmit an RRCReconfigurationComplete message.

7.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.3.3.3.5-1. T2 starts.

8.	The SS shall transmit DCI 2-9 command of ‘1’ value for indicating NES-specific CHO execution condition.

9.	If the UE transmits the PRACH preambles to Cell 2 less than 1672 ms for PC1 or 1152 ms for PC3 from the beginning of time period T2, and the interruption during T2 is no longer than 62ms, then the number of successful test iterations is increased by one. Otherwise, the number of failure test iterations is increased by one.

10.	After T2 expires, the SS sends an RRCReconfiguration with reconfigurationWithSync to cause UE handover back to Cell 1.

11.	If UE is not in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1, switch off and on the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1.

12.	Repeat steps 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.3.3.3.4.3	Message contents

Same as in clause 6.3.3.6.4.3.

7.3.3.3.5	Test requirements

Table 7.3.3.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.3.3.3.5-1: Cell specific test parameters for intra-frequency NES triggering conditional handover

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | 1 |  | 1 |  |
| AoA setup |  |  |  | Setup 1 as defined in A.3.15 |  |  |  |
| Assumption for UE beamsNote 6 |  |  |  | Rough |  |  |  |
| Duplex mode |  |  |  | TDD |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  |  |  |
| BWP BW |  |  | MHz | 100: NRB,c = 66 |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |
| CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |
| SSB Configuration |  |  |  | SSB.1 FR2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |
| TCI configuration |  |  |  | CSI-RS.Config.0 |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -106.2 |  | -106.2 |  |
| Note2 | Config 1 |  | dBm/SCS | -97.2 |  | -97.2 |  |
| EsIotBB Note 8 |  |  | dB | 4.69 | -5.44 | -Infinity | 3.73 |
|  |  |  | dB | 6 | 6 | -Infinity | 11 |
| IoNote3 | Config 1 |  | dBm/ BW | -61.21 | -55.73 | -61.21 | -55.73 |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 7: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated REFSENS requirement in TS 38.101-2 [19] clause 7.3.2, and an allowance of 1dB for UE multi-band relaxation factor ΔMBS specified in TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

TRRC + TEvent_DU occurs during T1 as the NES triggering handover condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + Tinterrupt + TCHO_execution =1600+62+10=1672 ms (power class 1) or 1080+62+10 =1152 (power classes 2,3 and 4) from the start of T2 and the interruption during T2 shall not exceed Tinterrupt=Tprocessing + TIU + T∆ + Tmargin =40+20+2 = 62ms excluding any transmissions which do not occur due to scheduling restrictions.

The rate of correct handovers observed during repeated tests shall be at least 90% with the confidence level of 95%.

#### 7.3.3.4	NR SA FR2-FR1 inter-frequency NES triggering conditional handover

Editor’s Note: This test case is incomplete in following aspects:

-	TT analysis is missing.

- 	NR FR1 - FR2 OTA testability is still FFS.

7.3.3.4.1	Test purpose

To verify the requirement for the NR FR2 inter-frequency NES triggering conditional handover requirements specified in 38.133 [6] clause 6.1.4.4.

7.3.3.4.2	Test applicability

This test applies to all types of NR UE from Release 18 onwards and supporting NES triggering conditional handover.

7.3.3.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.3.3.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.3.4.

7.3.3.4.4	Test description

7.3.3.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 7.3.3.4.4.1-1.

Table 7.3.3.4.4.1-1: test configurations

| Config | Description |
| --- | --- |
| 7.3.3.4-1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.3.3.4.4.1-2

Table 7.3.3.4.4.1-2: Initial conditions for inter-frequency NES triggering conditional handover

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 7.3.3.4.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR2 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 7.3.3.4.4.1-3 below, with A3-Offset modified by Test Tolerance.

Table 7.3.3.4.4.1-3: General test parameters for inter-frequency NES triggering conditional handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset in condition |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 3 |  |

7.3.3.4.4.2	Test procedure

At the start of time duration T1, the UE may not have any timing information of Cell 2. The UE is configured with a NES-based condition implying handover to cell 2 firstly, during T1, at a time earlier than TRRC before the beginning of T2 and then configured with DCI 2-9 with NES-mode indication 950 ms from the start of T2. UE is expected to decode the DCI command 2-9 later than the time when the NES condition is considered met. No interruption shall be observed in time period T1,

From start of T2, the Cell 2 becomes detectable and NES-based event is fulfilled. During T2, the UE performs measurement on Cell 2 and evaluates the execution condition and starts handover procedure when the DCI command is received. The UE shall send PRACH to Cell 2 less than Tmeasure + TCHO_execution + Tinterrupt ms from the start of T2. The Interruption length Tinterrupt shall also be verified in T2.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRCReconfiguration message. Cell 1 is the active cell. Set Cell 2 physical cell identity to the initial physical cell identity.

2.	Set the parameters according to T1 in Table 7.3.3.4.5-1. T1 starts.

3.	The SS starts continuously scheduling the UE to perform DL reception in every DL slot on Cell 1 and monitoring corresponding ACK/NACK feedback sent by the UE.

4.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

5.	The SS shall transmit an RRCReconfiguration message with conditionalReconfiguration on Cell 1 to configure NES-based CHO execution condition for the UE.

6.	The UE shall transmit an RRCReconfigurationComplete message.

7.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.3.3.4.5-1. T2 starts.

8.	The SS shall transmit DCI 2-9 command of ‘1’ value for indicating NES-specific CHO execution condition 950 ms from the start of T2.

9.	If the UE transmits the PRACH preambles to Cell 2 less than 1022 ms from the beginning of time period T2, and the interruption during T2 is no longer than 62ms, then the number of successful test iterations is increased by one. Otherwise, the number of failure test iterations is increased by one.

10.	After T2 expires, the SS sends an RRCReconfiguration with reconfigurationWithSync to cause UE handover back to Cell 1.

11.	If UE is not in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1, switch off and on the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1.

12.	Repeat steps 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.3.3.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.3.3.4.4.3-1: RRCReconfiguration (Step 4)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition NR_MEAS and CHO |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| measConfig |  | MeasConfig | Table 7.3.3.4.4.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| conditionalReconfiguration-r16 |  | ConditionalReconfiguration | Table 7.3.3.4.4.3-5 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.3.3.4.4.3-2: MeasConfig (Table 7.3.3.4.4.3-1)

| Derivation Path: Table H.3.1-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | 1 | entry 1 |  |
| measObject[1] CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with Condition INTRA-FREQ MO |  |  |
| } |  |  |  |
| measObjectId[2] | 2 |  |  |
| measObject[2] CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 7.3.3.4.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |

| Information Element | Value/remark | Comment | Condition |
| --- | --- | --- | --- |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 7.3.3.4.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR | Table 7.3.3.4.4.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig specified in Table H.3.1-5 |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR | Table 7.3.3.4.4.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  |  |  |
| measId | 2 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig specified in Table H.3.1-5 |  |  |
| } |  |  |  |

Table 7.3.3.4.4.3-3: MeasObjectNR (Table 7.3.3.4.4.3-2)

| Derivation Path: TS 38.508-1[14], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR ::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR of the SSB associated to inter-frequency neighbour cell |  |  |
| referenceSignalConfig SEQUENCE { |  |  |  |
| ssb-ConfigMobility SEQUENCE { |  |  |  |
| ssb-ToMeasure | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| } |  |  |  |

Table 7.3.3.4.4.3-4: ReportConfigNR (Table 7.3.3.4.4.3-2)

| Derivation Path: TS 38.508-1[14], Table 4.6.3-142 with condition CHO |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| condTriggerConfig SEQUENCE { |  |  |  |
| condEventId CHOICE { |  |  |  |
| condEventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 0 | actuall value = -2*0 = 0dB |  |
| } |  |  |  |
| hysteresis | 0 | actuall value = 0*0.5 = 0dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| nesEvent-r18 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.3.4.4.3-5: ConditionalReconfiguration (Table 7.3.3.4.4.3-1)

| Derivation Path: TS 38.508-1[14], Table 4.6.3-25D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ConditionalReconfiguration-r16::= SEQUENCE { |  |  |  |
| condReconfigToAddModList-r16 SEQUENCE (SIZE (1.. maxNrofCondCells-r16)) OF CondReconfigToAddMod-r16 { | 1 entry |  |  |
| CondReconfigToAddMod-r16 [1] SEQUENCE { |  | entry 1 |  |
| condReconfigId-r16 | 1 |  |  |
| condExecutionCond-r16 SEQUENCE (SIZE (1..2)) OF MeasId { | 1 entry |  |  |
| MeasId[1] | 1 | The MeasId configured in Table 7.3.3.4.4.3-2 |  |
| } |  |  |  |
| condRRCReconfig-r16 | OCTET STRING (CONTAINING RRCReconfiguration Specified in Table 4.8.1-1A with condition RBConfig_NoKeyChange) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

7.3.3.4.5	Test requirements

Table 7.3.3.4.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.3.3.4.5-1: Cell specific test parameters for NR FR2-FR1 Inter frequency conditional handover test case

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  | 1 |  |  | 2 |  |
| AoA setup |  |  | Setup 1 as defined in A.3.15 |  | NA |  |  |
| Assumption for UE beamsNote 6 |  |  | Rough |  | NA |  |  |
| Duplex mode |  |  | TDD |  | FDD |  |  |
| TDD configuration |  |  | TDDConf.3.1 |  | NA |  |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |  |  |  |
| BWP BW |  | MHz | 100: NRB,c = 66 |  |  |  |  |
| DRx Cycle |  | ms | Not Configured |  | Not Configured |  |  |
| Gap pattern ID |  |  | gp0 |  | NA |  |  |
| PDSCH Reference measurement channel |  |  | SR3.1 TDD |  | SR.1.1 FDD |  |  |
| CORESET Reference Channel |  |  | CR3.1 TDD |  | CR.1.1 FDD |  |  |
| OCNG Patterns |  |  | OCNG pattern 1 |  | OCNG pattern 1 |  |  |
| SMTC Configuration |  |  | SMTC pattern 1 |  | SMTC pattern 1 |  |  |
| SSB Configuration |  |  | SSB.1 FR2 |  | SSB.1 FR1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 kHz |  | 15 kHz |  |  |
| PUCCH/PUSCH subcarrier spacing |  | kHz | 120 kHz |  | 15 kHz |  |  |
| PRACH configuration |  |  | FR2 PRACH configuration 1 |  | FR1 PRACH configuration 1 |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  | NA |  |  |
| TCI configuration |  |  | CSI-RS.Config.0 |  | NA |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Propagation condition |  | - | AWGN |  | N/A Link only, see clause A.3.7A |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |  |  |

Table 7.3.3.4.5-2: OTA related test parameters for NR FR2-FR1 Inter frequency conditional handover test case

| Parameter |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| Angle of arrival configuration |  |  | According to clause A.3.15.1 |  |  |  | N/A |  |  |  |
| Assumption for UE beams Note 7 |  |  | Rough |  |  |  | N/A |  |  |  |
| Note 1 | Config 1,2,3 | dBm/15kHz | -104.7+TT |  |  |  | Link only, see clause A.3.7A |  |  |  |
| Note 1 | Config 1,2,3 | dBm/SCS | -95.7+TT |  |  |  |  |  |  |  |
|  | Config 1,2,3 | dB | 7+TT |  |  |  |  |  |  |  |
|  | Config 1,2,3 | dB | 7+TT |  |  |  |  |  |  |  |
| SSB_RPNote 2, Note 4 | Config 1,2,3 | dBm/SCS | -88.7+TT |  |  |  |  |  |  |  |
| IoNote 2, Note 4 | Config 1,2,3 | dBm/95.04 MHz | -58.92+TT |  |  |  |  |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: Void Note 6: Void Note 7: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |  |  |

TRRC + TEvent_DU occurs during T1 as the handover condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + Tinterrupt + TCHO_execution = 1022ms from the start of T2 and the interruption during T2 shall not exceed Tinterrupt=Tprocessing + TIU + T∆ + Tmargin = 62 ms excluding any transmissions which do not occur due to measurement gaps.

# F.1	Measurement uncertainties and test tolerances for FR1 and FR2

## F.1.1	Acceptable uncertainty of test system (normative)

See TS 38.521-1 [17] annex F.1.

### F.1.1.1	Measurement of test environments

See TS 38.521-1 [17] Annex F1.1.

### F.1.1.2	Measurement of RRM requirements

/// unchanged clauses omitted

Table F.1.1.2-6: Maximum test system uncertainty for RRM requirements for SA FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.1.1.1 NR SA FR2 cell re-selection | Noc, averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es1/Noc averaged over BWConfig ±0.3 dB Es1/Noc averaged over Measurement PRB ±0.3 dB Es2/Noc averaged over BWConfig ±0.3 dB Es2/Noc averaged over Measurement PRB ±0.3 dB | Noc averaged over BWConfig is the AWGN absolute power of RF channel 1 averaged over BWConfig Noc averaged over Measurement PRB is the AWGN absolute power of RF channel 1 averaged over Measurement PRB Es1/Noc averaged over BWConfig is Ratio of cell 1 signal / AWGN averaged over BWConfig Es1/Noc averaged over Measurement PRB is Ratio of cell 1 signal / AWGN averaged over Measurement PRB Es2/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |
| /// unchanged rows omitted |  |  |
| 7.3.3.1 NR SA FR2 conditional handover | Noc averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es1/Noc averaged over BWConfig ±0.3 dB Es1/Noc averaged over Measurement PRB ±0.3 dB Es2/Noc ±0.3 dB Noc, averaged over BWConfig, ±5.65 dB Es2/Noc averaged over Measurement PRB ±0.3 dB | Noc averaged over BWConfig,is the AWGN absolute power of RF channel 1 averaged over BWConfig, Noc averaged over Measurement PRB,is the AWGN absolute power of RF channel 1 averaged over Measurement PRB, Es1/Noc averaged over BWConfig is Ratio of cell 1 signal / AWGN averaged over BWConfig Es1/Noc averaged over Measurement PRB is Ratio of cell 1 signal / AWGN averaged over Measurement PRB Es2/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |
| 7.3.3.3 | Same as 7.3.3.1 | Same as 7.3.3.1 |
| 7.4.1.1 SA FR2 UE transmit timing accuracy | Noc ±5.65 dB Ês1 / Noc ±0.3 dB ±48Tc Uplink signal transmit timing relative to downlink ±40Tc Relative during UE timing adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |

/// unchanged clauses omitted

## F.1.3	Test Tolerance and Derivation of Test Requirements (informative)

See TS 38.521-1 [17] Annex F.3.

### F.1.3.1	Measurement of test environments

See TS 38.521-1 [17] Annex F.3.1.

### F.1.3.2	Measurement of RRM requirements

/// unchanged clauses omitted

Table F.1.3.2-4: Derivation of test requirements for NR SA FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 7.1.1.1 NR SA FR2 cell re-selection | UE PC1 |  |  |
|  | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -3dB Es2/Noc = 1.5dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 1.5dB Es2/Noc = -3dB In signalling: SintraSearchP = 50 Qrxlevmin = -138dBm/SCS for config. 1 Qrxlevmin = -135dBm/SCS for config. 2 | During T1: 0dB 0dB N/A During T2: 0dB 0.5dB 1.0dB During T3: 0dB 1.0dB 0.5dB In signalling: 12 37 dB 37 dB | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -2.50dB Es2/Noc = 2.5dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 2.5dB Es2/Noc = -2.50dB In signalling: SintraSearchP = 62 Qrxlevmin = -101dBm/SCS for config. 1 Qrxlevmin = -98dBm/SCS for config. 2 |
|  | UE PC3 |  |  |
|  | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -3dB Es2/Noc = 1.5dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 1.5dB Es2/Noc = -3dB In signalling: Qrxlevmin = -138dBm/SCS for config. 1 Qrxlevmin = -135dBm/SCS for config. 2 | During T1: 0dB 0dB N/A During T2: 0dB 0.1dB 0.55dB During T3: 0dB 0.55dB 0.1dB In signalling: 18 dB 18 dB | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -2.90dB Es2/Noc = 2.05dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 2.05dB Es2/Noc = -2.90dB In signalling: Qrxlevmin = -120dBm/SCS for config. 1 Qrxlevmin = -117dBm/SCS for config. 2 |
| /// unchanged rows omitted |  |  |  |
| 7.3.3.1 NR SA FR2 conditional handover | During T1: Noc -104.7dBm/15kHz Es1/Noc 6 dB Es2/Noc -infinity During T2: Noc -104.7dBm/15kHz Es1/Noc 6 dB Es2/Noc 11dB In signaling: A3-offset = -1dB | During T1 -1.5dB 0dB 0dB During T2 -1.5dB 0dB 0dB In signaling: -1dB | During T1: Noc -106.2 dBm/15kHz Es1/Noc 6 dB Es2/Noc -infinity During T2: Noc -106.2 dBm/15kHz Es1/Noc 6 dB Es2/Noc 11dB In signaling: A3-offset = -2dB |
| 7.3.3.3 | Same as 7.3.3.1 | Same as 7.3.3.1 | Same as 7.3.3.1 |
| 7.4.1.1 SA FR2 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB |

/// End of changes