3GPP TSG-RAN5 Meeting #107 	R5-252808

# St Julian's, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3964 | rev | - | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test Tolerance update for Time based and Location based Measurement Initiation inter frequency NR cell reselection for Satellite Access test 14.1.7 and 14.1.8. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Innovation Center Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Test tolerance update for test case 14.1.7 and 14.1.8 needs to be updated |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test tolerance is added for test cases 14.1.7 and 14.1.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Test case implementation will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.7,14.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.903 CR 1022 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on RAN4 CR R4-25xxxx |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

## 14.1	RRC_IDLE state mobility

### 14.1.0	Minimum conformance requirements

<< Skipped sections >>

### 14.1.7	NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access

Editor’s Note:

-	Message contents are FFS

-	Call setup and test procedure needs to be updated

-	Several test parameters and configuration are still in brackets

-	Annex E need to be updated

14.1.7.1	Test purpose

This test is to verify the requirement for the inter frequency NR cell reselection requirements for satellite access specified in clause 14.1.0.2.

14.1.7.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access and time-based measurement initiation.

14.1.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.14.1.7.

14.1.7.4	Test description

The test scenario comprises of 1 NR carrier and 2 cells as given in tables 14.1.7.4.1-1, 14.1.7.4.1-3 and 14.1.7.5-1. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2. t-Service broadcasted in SIB19 of Cell 1 is set to the time point that is 36s after start of T2.

14.1.7.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.1.7.4.1-1.

Table 14.1.7.4.1-1: Supported test configurations for NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access

| Configuration | Description |
| --- | --- |
| 14.1.7-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.1.7-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 14.1.7.4.1-2

Table 14.1.7.4.1-2: Initial conditions for NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.1.7.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |

1.	The general test parameter settings are set up according to Table 14.1.7.4.1-3.

2.	Message contents are defined in clause 14.1.7.4.3.

3.	Cell 1 and Cell 2 are NR cells with the power level set according to clauses C.1.2 and C.1.3 for this test. Both Cell 1 and Cell 2 are satellite access cells.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.1.7.4.1-3: General test parameters for NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2 | Cell2 |  |
|  | Neighbour cells |  | 1, 2 | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1, 2 | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 102 | The detailed configuration is specified in TS 38.211 [7] clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2 | 40 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

14.1.7.4.2	Test procedure

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility.

For NGSO configurations, the number of LEO satellites that a UE can measure in parallel within an SMTC is a capability the UE reports via maxNumber-NGSO-SatellitesWithinOneSMTC-r17. This value will determine the requirement to be used in the test, as well as the duration of T2.

1.	Ensure the UE is in [state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is an active cell and Cell 2 is powered off.

2.	Set the parameters according to T1 in Table 14.1.7.5-1. Propagation conditions are set according to Annex C clause C.2.2. T-Service broadcasted in SIB19 of Cell 1 is set to the time point that is 36s after start of T2, according to Table 14.1.7.4.3-FFS (this timestamp indicates the UE when Cell 1 will stop providing service). T1 starts.

3.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for this iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.1.7.5-1. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

6.	If the UE responds on the newly detectable cell, Cell 2 during time duration T2 within [36] seconds for GSO configurations or for NGSO configurations for UEs supporting measurement of only one LEO satellite in parallel within an SMTC, or within FFS seconds for NGSO configurations for UEs supporting measurement of two or more LEO satellites in parallel within an SMTC, from the beginning of time period T2, then count a success for the test.

7.	If the UE has re-selected to Cell 2 within T2, after the re-selection or when T2 expires, the SS shall send an RRCRelease message to the UE and then proceed with step 1 for the next iteration. Otherwise proceed with step 7a.

7a.	The SS shall switch off and on the UE, and skip to step 1 for the next iteration.

8.	Repeat step 2-7 until a test verdict has been achieved.

14.1.7.4.3	Message contents

FFS

14.1.7.5	Test requirements

Table 14.1.7.5-1 defines the primary level settings including test tolerances for NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access.

Table 14.1.7.5-1: Cell specific parameters for NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 |  |  | T2 |
| Satellite information |  | SSC.1 for Config 1 SSC.2 for Config 2 |  | NSC.1 for Config 1 NSC.2 for Config 2 |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  | SR.1.1 FDD |  |
| RMSI CORESET configuration |  | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET configuration |  | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
| OCNG Pattern |  | OP.1 |  | OP.1 |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| SSB configuration |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration |  | #1: SMTC.1 for Cell 1 and Cell 2 |  | #1: SMTC.1 for Cell 1 and Cell 2 |  |
| RLM-RS |  | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | -130 |  | -130 |  |
| Pcompensation | dB | 0 |  | 0 |  |
| Qhysts | dB | 0 |  | 0 |  |
| Qoffsets, n | dB | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  | SS-RSRP |  |
|  | dB | 16 | 13 | -infinity |  |  | 16 |
| Note2 | dBm/SCS | -98 |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |
|  | dB | 16 | 13 | -infinity |  |  | 16 |
| SS-RSRP Note3 | dBm/SCS | -82 | -85 | -infinity |  |  | -82 |
| Io | dBm/9.36 MHz | -53.94 | -56.84 | -70.05 | -53.94 |  |  |
| Treselection | s | 0 | 0 | 0 |  |  | 0 |
| SnonIntrasearchP | dB | 50 |  | 50 |  |
| Threshx, highP | dB | 48 |  | 48 |  |
| Threshserving, lowP | dB | 54 |  | 44 |  |
| Threshx, lowP | dB | 50 |  | 40 |  |
| Propagation Condition |  | AWGN |  |  |  |

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a lower priority cell shall be less than [36] s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR,

Where:

Tevaluate, NR_ inter	See Table 4.2C.2.4-1 in 38.133 [6] clause 4.2C.2.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 80ms period.

[In this test, SMTC.1 (period 20ms, duration 1ms, offset 0ms) is configured on both cells. Since only one SMTC is configured there is no chance for overlap, therefore according to clause 14.1.0.2, Kmulti_SMTC = 1 for GEO, Kmulti_SMTC = 1 for LEO for UEs supporting measurement of only one LEO satellite in parallel within an SMTC (NLEO,simul=1), and Kmulti_SMTC = 2 for LEO for UEs supporting measurement of two or more LEO satellites in parallel within an SMTC (NLEO,simul=2,3,4).]

### 14.1.8	NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access

Editor’s Note:

-	Message contents are FFS

-	Call setup and test procedure needs to be updated

-	Several test parameters and configuration are still in brackets

-	Annex E need to be updated

14.1.8.1	Test purpose

This test is to verify the requirement for the inter frequency NR cell reselection requirements for satellite access specified in clause 14.1.0.2.

14.1.8.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access and location-based measurement initiation.

14.1.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.14.1.8.

14.1.8.4	Test description

The test scenario comprises of 1 NR carrier and 2 cells as given in tables 14.1.8.4.1-1, 14.1.8.4.1-3 and 14.1.8.5-1. The test consists of two successive time periods, with time duration of T1and T2, respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

At 4s after the start of T2, the UE location is changed such that the distance to the reference location broadcasted in SIB19 of Cell 1 is exceeded by the configured value in distanceThresh plus 50m.

14.1.8.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.1.8.4.1-1.

Table 14.1.8.4.1-1: Supported test configurations for NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access

| Configuration | Description |
| --- | --- |
| 14.1.8-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.1.8-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 14.1.8.4.1-2

Table 14.1.8.4.1-2: Initial conditions for NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.1.8.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |

1.	The general test parameter settings are set up according to Table 14.1.8.4.1-3.

2.	Message contents are defined in clause 14.1.8.4.3.

3.	Cell 1 and Cell 2 are NR cells with the power level set according to clauses C.1.2 and C.1.3 for this test. Both Cell 1 and Cell 2 are satellite access cells.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.1.8.4.1-3: General test parameters for NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2 | Cell2 |  |
|  | Neighbour cells |  | 1, 2 | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1, 2 | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 102 | The detailed configuration is specified in TS 38.211 [7] clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2 | 40 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

14.1.8.4.2	Test procedure

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility.

For NGSO configurations, the number of LEO satellites that a UE can measure in parallel within an SMTC is a capability the UE reports via maxNumber-NGSO-SatellitesWithinOneSMTC-r17. This value will determine the requirement to be used in the test, as well as the duration of T2.

1.	Ensure the UE is in [state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is an active cell and Cell 2 is powered off.

2.	Set the parameters according to T1 in Table 14.1.8.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts.

3.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for this iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.1.8.5-1. T2 starts.

5.	After 4 seconds from the start of T2, the UE location is changed such that the distance to the reference location broadcasted in SIB19 of Cell 1 is exceeded by the configured value in distanceThresh plus 50m, according to Table 14.1.8.4.3-FFS.

6.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

7.	If the UE responds on the newly detectable cell, Cell 2 during time duration T2 within [36] seconds for GSO configurations or for NGSO configurations for UEs supporting measurement of only one LEO satellite in parallel within an SMTC, or within FFS seconds for NGSO configurations for UEs supporting measurement of two or more LEO satellites in parallel within an SMTC, from the beginning of time period T2, then count a success for the test.

8.	If the UE has re-selected to Cell 2 within T2, after the re-selection or when T2 expires, the SS shall send an RRCRelease message to the UE and then proceed with step 1 for the next iteration. Otherwise proceed with step 8a.

8a.	The SS shall switch off and on the UE, and skip to step 1 for the next iteration.

9.	Repeat step 2-8 until a test verdict has been achieved.

14.1.8.4.3	Message contents

FFS

14.1.8.5	Test requirements

Table 14.1.8.5-1 defines the primary level settings including test tolerances for NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access.

Table 14.1.8.5-1: Cell specific parameters for NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 |  |  | T2 |
| Satellite information |  | SSC.1 for Config 1 SSC.2 for Config 2 |  | NSC.1 for Config 1 NSC.2 for Config 2 |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  | SR.1.1 FDD |  |
| RMSI CORESET configuration |  | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET configuration |  | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
| OCNG Pattern |  | OP.1 |  | OP.1 |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| SSB configuration |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration |  | #1: SMTC.1 for Cell 1 and Cell 2 |  | #1: SMTC.1 for Cell 1 and Cell 2 |  |
| RLM-RS |  | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | -130 |  | -130 |  |
| Pcompensation | dB | 0 |  | 0 |  |
| Qhysts | dB | 0 |  | 0 |  |
| Qoffsets, n | dB | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  | SS-RSRP |  |
|  | dB | 16 | 13 | -infinity |  |  | 16 |
| Note2 | dBm/SCS | -98 |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |
|  | dB | 16 | 13 | -infinity |  |  | 16 |
| SS-RSRP Note3 | dBm/SCS | -82 | -85 | -infinity |  |  | -82 |
| Io | dBm/9.36 MHz | -53.94 | -56.84 | -70.05 | -53.94 |  |  |
| Treselection | s | 0 | 0 | 0 |  |  | 0 |
| SnonIntrasearchP | dB | 50 |  | 50 |  |
| Threshx, highP | dB | 48 |  | 48 |  |
| Threshserving, lowP | dB | 54 |  | 44 |  |
| Threshx, lowP | dB | 50 |  | 40 |  |
| Propagation Condition |  | AWGN |  |  |  |

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a lower priority cell shall be less than [36] s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR,

Where:

Tevaluate, NR_ inter	See Table 4.2C.2.4-1 in 38.133 [6] clause 4.2C.2.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 80ms period.

[In this test, SMTC.1 (period 20ms, duration 1ms, offset 0ms) is configured on both cells. Since only one SMTC is configured there is no chance for overlap, therefore according to clause 14.1.0.1, Kmulti_SMTC = 1 for GEO, Kmulti_SMTC = 1 for LEO for UEs supporting measurement of only one LEO satellite in parallel within an SMTC (NLEO,simul=1), and Kmulti_SMTC = 2 for LEO for UEs supporting measurement of two or more LEO satellites in parallel within an SMTC (NLEO,simul=2,3,4).]

<< End of Changes >>