3GPP TSG-RAN5 Meeting #107	R5-252880

# St Julian, Malta, 19th – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3511 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update IE SL-ConfigDedicatedNR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To align the IE SL-ConfigDedicatedNR with non used extension groups in default information elements contents. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The non used extension group has been removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IE SL-ConfigDedicatedNR remains inconsistent and includes non used ASN.1 fields. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

#### –	SL-ConfigDedicatedNR

Table 4.6.6-7: SL-ConfigDedicatedNR

| Derivation Path: TS 38.331 [6], clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SL-ConfigDedicatedNR-r16 ::= SEQUENCE { |  |  |  |  |
| sl-PHY-MAC-RLC-Config-r16 |  | Not present |  |  |
| sl-PHY-MAC-RLC-Config-r16 SEQUENCE { |  |  |  | SCHEDULING, SELECTED |
| sl-ScheduledConfig-r16 |  | Not present |  |  |
| sl-ScheduledConfig-r16 CHOICE { |  |  |  | SCHEDULING |
| setup |  | SL-ScheduledConfig |  |  |
| } |  |  |  |  |
| sl-UE-SelectedConfig-r16 |  | Not present |  |  |
| sl-UE-SelectedConfig-r16 CHOICE { |  |  |  | SELECTED |
| setup |  | SL-UE-SelectedConfig |  |  |
| } |  |  |  |  |
| sl-FreqInfoToReleaseList-r16 |  | Not present |  |  |
| sl-FreqInfoToAddModList-r16 |  | Not present |  |  |
| sl-FreqInfoToAddModList-r16 SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-FreqConfig-r16 { |  | 1 entry |  | SCHEDULING, SELECTED |
| SL-FreqConfig-r16[1] |  | SL-FreqConfig | entry 1 |  |
| } |  |  |  |  |
| sl-RLC-BearerToReleaseList-r16 |  | Not present |  |  |
| sl-RLC-BearerToAddModList-r16 |  | Not present |  |  |
| sl-RLC-BearerToAddModList-r16 SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-BearerConfig-r16 { |  | 1 entry |  | SL_DRB |
| SL-RLC-BearerConfig-r16[1] |  | SL-RLC-BearerConfig | entry 1 |  |
| } |  |  |  |  |
| sl-MaxNumConsecutiveDTX-r16 |  | Not present |  |  |
| sl-CSI-Acquisition-r16 |  | Not present |  |  |
|  |  | enabled |  | SL_CSI_REPORT |
| sl-CSI-SchedulingRequestId-r16 |  | Not present |  |  |
| sl-SSB-PriorityNR-r16 |  | 1 |  |  |
| networkControlledSyncTx-r16 |  | off | UE doesn't send SL SSB by default |  |
| } |  |  |  |  |
| sl-RadioBearerToReleaseList-r16 |  | Not present |  |  |
| sl-RadioBearerToAddModList-r16 |  | Not present |  |  |
| sl-RadioBearerToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SL-RadioBearerConfig-r16 { |  | 1 entry |  | SL_DRB |
| SL-RadioBearerConfig-r16[1] |  | SL-RadioBearerConfig |  |  |
| } |  |  |  |  |
| sl-MeasConfigInfoToReleaseList-r16 |  | Not present |  |  |
| sl-MeasConfigInfoToAddModList-r16 |  | Not present |  |  |
| sl-MeasConfigInfoToAddModList-r16 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-MeasConfigInfo-r16 { |  | 1 entry |  | SL_MEAS |
| SL-MeasConfigInfo-r16[1] |  | SL-MeasConfigInfo |  |  |
| } |  |  |  |  |
| t400-r16 |  | ms2000 |  |  |
| } |  |  |  |  |

<End of modified section>