3GPP TSG-RAN5 Meeting #107	R5-252930

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.905 | CR | 0281 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Inclination correction in orbital format GSO ephemeris files |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18_Test, LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | It is observed that some of the values corresponding to inclination-r17 don’t have correct values (FALSE value instead). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Inclination-r17 incorrect values are corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Failure in the configuration of the IoT NTN ephemeris if wrong inclination-r17 values are used within the satellite orbit. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << START OF CHANGES 1>>>

## 5.4	Satellite Ephemeris generated files

This section contains information about the satellite orbits generated according to the procedure defined in clause 5.2.

Table 5.4-1: Satellite ephemeris files generated for IoT NTN

| Satellite Type | Satellite parameters | UE location | Applicable to Serving cell / Neighbour cell | File can be found in attachment | Comments |
| --- | --- | --- | --- | --- | --- |
| GSO | 7º inclination Satellite longitude: 121.56076999 Satellite altitude: 35786 km | Longitude: 121.56076999 Latitude: 55.0 Altitude: 0 | Serving cell | “ReportFile2_GSO-Inc7deg-Lat55-Long121.56076999-Step2.56sec-v4.zip” | Added in RAN5#107 |
| GSO | 7º inclination Satellite longitude: 101.56076999 Satellite altitude: 35786 km | Longitude: 121.56076999 Latitude: 55.0 Altitude: 0 | Neighbour cell | “ReportFile2_GSO-SATLong101.56076999-UELong121.56076999_Lat55-Inc7_v4.zip” | Added in RAN5#107 |
| GSO | 7º inclination Satellite longitude: 141.56076999 Satellite altitude: 35786 km | Longitude: 121.56076999 Latitude: 55.0 Altitude: 0 | Neighbour cell | “ReportFile2_GSO-SATLong141.56076999-UELong121.56076999_Lat55-Inc7_v4.zip” | Added in RAN5#107 |
| NGSO | Satellite altitude 600 km | Longitude: 121.56076999 Latitude: 25.08439333 Altitude: 0 | Serving cell | “ReportFile-NGSO-LEO600km-MinElev0-Lat25.08439333-Long121.56076999-Step0.640sec.zip” | Added in RAN5#102 |
| NGSO | Satellite altitude 1200 km | Longitude: 121.56076999 Latitude: 25.08439333 Altitude: 0 | Serving cell | “ReportFile-NGSO-LEO1200km-MinElev0-Lat25.08439333-Long121.56076999-Step0.640sec.zip” | Added in RAN5#102 |

## <<< END OF CHANGES 1>>>

Implementation notes:

- Files to be removed:

- “ReportFile2_GSO-Inc7deg-Lat55-Long121.56076999-Step2.56sec-v3.zip”

- “ReportFile2_GSO-SATLong101.56076999-UELong121.56076999_Lat55-Inc7_v3.zip”

- “ReportFile2_GSO-SATLong141.56076999-UELong121.56076999_Lat55-Inc7_v3.zip”

- Files to be added:

- “ReportFile2_GSO-Inc7deg-Lat55-Long121.56076999-Step2.56sec-v4.zip”

- “ReportFile2_GSO-SATLong101.56076999-UELong121.56076999_Lat55-Inc7_v4.zip”

- “ReportFile2_GSO-SATLong141.56076999-UELong121.56076999_Lat55-Inc7_v4.zip”