3GPP TSG-RAN5 Meeting #107 	R5-252953

# St Julian's, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3997 | rev | - | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Annex H common message contents for NR-NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Following changes are needed in NR-NTN handover test cases: 1) Target cell NTN-Config parameters need to be signaled to the UE via RRC Reconfiguration. 2) In NGSO scenario, UL sync validity timer might expire before test conditions for conditional handover are configured (i.e. ntn-UlSyncValidityDuration-r17 < T1 and/or T2, depending on the scenario). If that happens, the UE would be forced to read SIB19 of the target cell after being triggered, increasing the total CHO delay beyond requirements. Both changes can be updated in Annex H, since they apply to multiple test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added new section H.3.13 with NR-NTN specific message contents - Added new ServingCellConfigCommon and NTN-Config tables |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases would remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.3.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

## H.3.13	RRC messages and information elements contents exceptions for Satellite Access

Table H.3.13-1: ServingCellConfigCommon for Handover for Satellite Access

| Derivation Path: TS 38.508-1, Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 | NTN-Config-r17 | Table H.3.13-2 |  |
| } |  |  |  |

Table H.3.13-2: NTN-Config for Handover for Satellite Access

| Derivation Path: TS 38.508-1 [14], clause 4.6.3-84C |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark |  |  | Comment |  |  | Condition |
| NTN-Config-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |  |
| epochTime-r17 | Not present |  |  |
| ntn-UlSyncValidityDuration-r17 |  | s900 |  |  |  |  | GSO |  |
|  |  | s5 |  |  |  |  | NGSO |  |
|  |  | s10 |  |  |  |  | NGSO_T_CHO |  |
|  |  | s20 |  |  |  |  | NGSO_D_CHO |  |
| cellSpecificKoffset-r17 | 239 |  |  |  |  |  | GSO |  |
|  |  | 4 |  |  |  |  |  | NGSO or NGSO_T_CHO or NGSO_D_CHO |
| kmac-r17 |  | 0 |  |  |  |  |  |  |
| ta-Info-r17 SEQUENCE { |  |  |  |  |  |  |  |  |
| ta-Common-r17 |  | 0 |  |  |  |  |  |  |
| ta-CommonDrift-r17 |  | 0 |  |  |  |  |  |  |
| ta-CommonDriftVariant-r17 |  | 0 |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| ntn-PolarizationDL-r17 |  | linear |  |  |  |  |  |  |
| ntn-PolarizationUL-r17 |  | linear |  |  |  |  |  |  |
| ephemerisInfo-r17 |  | EphemerisInfo |  |  | Ephemeris info in TS 38.508-1 [14] section 7.6.2 |  |  |  |
| ta-Report-r17 |  | Not present |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |
| NGSO_T_CHO | Time-based Conditional Handover for NGSO scenario |
| NGSO_D_CHO | Distance-based Conditional Handover for NGSO scenario |

<< End of Changes >>