3GPP TSG-RAN5 Meeting #107	R5-253016

# St. Julian's, Malta, 19th May 2025 – 23rd May 2025

Agenda item:	5.5.1.13

Source: 	EchoStar

Title: 	Support of Variable Tx-Rx spacing for IoT NTN bands

Document for:	Discussion and Endorsement

## 1.	Introduction

During RAN plenary meeting TSGR#107, CRs [1] related to support for variable Tx-Rx separation for eMTC and NB-IoT NTN bands were approved and subsequently incorporated in the core specifications [2] and [3].

This paper summarizes updates made to the core specifications related to variable Tx-Rx frequency separation requirements, assesses the impact to various conformance specifications and proposes updates to the affected clauses of the affected specifications.

## 2.	Overview of Variable Tx-Rx spacing related updates to core specifications

Based on the approved CRs, Tables 5.4A.3-1, 5.4B.3-1 in [2] were updated as follows (updates are highlighted in yellow):

Table 5.4A.3-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 256 | 190 MHz1 161.4 to 218.6 MHz2 |
| 255 | -101.5 MHz1 -68.9 to -134.1 MHz2 |
| 254 | 873.5 MHz1 858.4 to 888.6 MHz2 |
| 253 | -150 MHz1 -144.4 to -155.6 MHz2 |
| NOTE 1: Default TX-RX separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3A. |  |

Table 5.4B.3-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 256 | 190 MHz1 160.2 to 219.8 MHz2 |
| 255 | -101.5 MHz1 -67.7 to -135.3 MHz2 |
| 254 | 873.5 MHz1 857.2 to 889.8 MHz2 |
| 253 | -150 MHz1 -143.2 to -156.8 MHz2 |
| NOTE 1: Default Tx-Rx separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3B. |  |

Furthermore, Tables 3A.4-1 and F.1-1 in [3] also note that the while the support for variable Tx-Rx separation is mandatory from Rel-18 onwards, Rel-17 NTN UEs are not required to support the capability.

Observation 1: Support for variable Tx-Rx spacing for eMTC/NTN is mandatory for Rel-18 but not required for Rel-17.

Observation 2: Support for variable Tx-Rx spacing for NB-IoT/NTN is mandatory for Rel-18 but not required for Rel-17.

Additionally, to limit the scope of RF conformance testing resulting from wide range of Tx-Rx separation, Clauses 7.1, 7.3A and 7.3B in [2] specify that only reference sensitivity requirements need to be tested for conformance with Tx-Rx separation values other than the default value. The minimum reference sensitivity requirements for the NTN band remain unchanged from that determined for the default Tx-Rx separation. There are no new demodulation performance and RRM requirements and existing ones remain unaffected resulting from introduction of variable Tx-Rx spacing.

Observation 3: Except for reference sensitivity requirements, all other RF requirements are to be tested with the default Tx-Rx separation.

Observation 4: The minimum reference sensitivity requirements for all Tx-Rx spacings is same as that for the default Tx-Rx spacing.

Observation 5: The demodulation and RRM performance requirements are unaffected by the introduction of variable Tx-Rx spacing.

Finally, Tables 7.3A-4 and 7.3B-4 in [2] identify the additional test points for RF reference sensitivity tests to verify UE’s support of variable Tx-Rx separation.

Table 7.3A-4: TX – RX carrier centre frequency separation for REFSENS verification

| E-UTRA Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| 256 | 1.4 MHz | 161.4 MHz, 218.6 MHz |
| 255 | 1.4 MHz | -68.9 MHz, -134.1 MHz |
| 254 | 1.4 MHz | 858.4 MHz, 888.6 MHz |
| 253 | 1.4 MHz | -144.4 MHz, -155.6 MHz |

Table 7.3B-2: TX – RX carrier centre frequency separation for REFSENS verification

| E-UTRA Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| 256 | 0.2 MHz | 160.2 MHz, 219.8 MHz |
| 255 | 0.2 MHz | -67.7 MHz, -135.3 MHz |
| 254 | 0.2 MHz | 857.2 MHz, 889.8 MHz |
| 253 | 0.2 MHz | -143.2 MHz, -156.8 MHz |

Observation 6: The number of additional test points for reference sensitivity tests to verify support of variable Tx-Rx separation is limited to two (2):

1) smallest Tx-Rx separation and

2) largest Tx-Rx separation

## 3.	RF Conformance Specifications Impact Assessment and Proposals

| TS & Clause | Assessment | Proposals |
| --- | --- | --- |
| TS 36.508 |  |  |
| Clause 4.3.1.1.x, 8.1.3.1.1.x | Except for Band 24, legacy TN bands have used the default Tx-Rx separation to determine the test frequencies. Band 24 used two Tx-Rx separation values with Low & Med Range based on one value and High Range based on another value. This was possible since the two values were defined as mandatory in a release independent way for Band 24. For NTN bands, variable Tx-Rx separation is mandatory only from Rel-18; Rel-17 eMTC/NB-IoT NTN UEs are not required to support any other value than the default value. Furthermore, except for reference sensitivity test cases, all other test cases are to be performed using the default Tx-Rx spacing using the low/mid/high test frequencies currently specified for the NTN bands. | Add new test frequencies for eMTC NTN and NB-IoT NTN bands as new tables for each NTN band (CR has been submitted at this meeting for agreement) |
| TS 36.521-4 |  |  |
| Clauses 5.4.A3, 5.4.B3 | Clause 5.4A.3 and 5.4B.3 have been updated in TS 36.102. Corresponding clauses need to be updated to reflect this change | Update Clauses 5.4A.3 and 5.4B.3 (CR has been submitted at this meeting for agreement) |
| Clauses 7.3A, 7.3B | Existing clauses 7.3A and 7.3B for reference sensitivity test cases for Rel-17 UEs and forward that only support test frequencies specified based on default Tx-Rx spacing. Variable Tx-Rx spacing is mandatory from Rel-18. Rel-17 UEs are not required to support variable Tx-Rx spacing. New clauses for reference sensitivity tests with variable Tx-Rx spacing with pointers to the appropriate test frequencies is one way to introduce conformance aspects for this capability. | Update Clauses 7.1, 7.3A.3, 7.3B.3. Add new test cases 7.3A_1 and 7.3B_2 (CR has been submitted at this meeting for agreement) |
| TS 36.521-2 |  |  |
| Clause 4.3.1 | Applicability of test cases related to variable Tx-Rx spacing for NTN bands need to be updated | Update test applicability of proposed new test cases in Table 4.3.1-1 (CR has been submitted at this meeting for agreement) |
| Clause A.4.3 | A new item needs to be added to an appropriate ICS table | Add a new PICS in Table A.4.3-7 (CR has been submitted at this meeting for agreement) Note: alternatively, it can be added to A.4.5-1 (Submitted CR can be revised based on agreement reached during the meeting) |
| TS 36.521-3: No changes are required as R4 RRM performance requirements and specifications remain unimpacted as a result of the introduction of support for variable Tx-Rx spacing |  |  |
| TR 36.903 | No RRM updates have been specified |  |
| TR 36.904 | TTs are not impacted as result of specification of variable Tx-Rx spacing |  |
| TR 36.905 | Since core specification has already limited the number of test points to two, no additional assessment is required |  |

Proposal 1: Endorse the changes proposed as part of the above assessment.

## 4.	Conclusion

Observation 1: Support for variable Tx-Rx spacing for eMTC/NTN is mandatory for Rel-18 but not required for Rel-17.

Observation 2: Support for variable Tx-Rx spacing for NB-IoT/NTN is mandatory for Rel-18 but not required for Rel-17.

Observation 3: Except for reference sensitivity requirements, all other RF requirements are to be tested with the default Tx-Rx separation.

Observation 4: The minimum reference sensitivity requirements for all Tx-Rx spacings is same as that for the default Tx-Rx spacing.

Observation 5: The demodulation and RRM performance requirements are unaffected by the introduction of variable Tx-Rx spacing.

Observation 6: The number of additional test points for reference sensitivity tests to verify support of variable Tx-Rx separation is limited to two (2):

1) smallest Tx-Rx separation and

2) largest Tx-Rx separation

Proposal 1: Endorse the changes proposed as part of the above assessment.

## 5.		References

[1]	RP-250600, RAN4 CRs to Maintenance for Closed WIs/SIs for REL-18 and earlier – Batch 23

[2]	TS 36.102 v18.8.0

[3]	TS 36.307 v18.8.0