3GPP TSG-RAN5 Meeting #107	R5-253041

# Malta, Malta, 19th May 2025 - 23rd May 2025

# Source:	Qualcomm Incorporated

# Title:	Discussion on impact of protection against improper reselection from E-UTRAN to GERAN/UTRAN

# Document for:	Discussion

## 1.	Introduction

3GPP RAN2 CR [1] modified the E-UTRAN inter-RAT cell reselection criteria for UEs supporting protection against improper reselection to GERAN/UTRAN. RAN2 CR [2] further states that the feature can be early implemented from Rel-15. Multiple test cases involving inter-RAT Cell Reselection from E-UTRAN to GERAN/UTRAN from [3] and [6] would have an impact due to this feature.

## 2.	Discussion

Based on the following reference from [4], except for dedicated signalling that allows GERAN/UTRAN frequencies to be treated as highest priority, in all other cases GERAN/UTRAN frequencies shall be treated as lower priority compared to E-UTRAN. In such cases, when evaluating the conditions of serving E-UTRAN cell for inter-RAT reselection, the UE shall apply values that differ from those configured by the network for IEs ThreshServing, LowP, Q-RxLevMin, Pcompensation and Qoffsettemp.

TS 36.304 [4] Clause 5.2.4.5

If the UE supports the protection against improper reselection to GERAN/UTRAN then:

-	When the UE evaluates serving cell conditions for the purpose of cell reselection to GERAN/UTRAN FDD/TDD by applying the reselection priorities provided in AS security protected dedicated signalling with a GERAN/UTRAN FDD/TDD frequency as lower priority compared to the current E-UTRAN frequency, or when the UE applies GERAN/UTRAN FDD/TDD priorities provided in system information, the UE shall:

-	treat GERAN and/or UTRAN FDD and/or UTRAN TDD frequencies as the lower priority compared to E-UTRAN;

-	set the value of ThreshServing, LowP to 6 dB if the value received in the system information is higher than 6 dB;

-	set the value of Q-RxLevMin to -116 dBm if the value received in SIB1 is higher than -116 dBm;

-	set the values of Pcompensation and Qoffsettemp to 0.

Observation 1

- It is optional for UE to support protection against improper reselection to GERAN/UTRAN feature.

Proposal 1

- Define a new PICS parameter in TS 36.523-2 [5] for Signalling and in TS 36.521-2 [7] for RRM to indicate UE support of this feature.

Observation 2.1

- As inter RAT GERAN/UTRAN frequencies are treated as lower priority than E-UTRAN (except the case of dedicated signalling configured with high priority), majority of the inter RAT reselection test cases from E-UTRAN to high priority GERAN/UTRAN of [3] clause 6 and of [6] clause 4 would become not applicable to UEs supporting this feature.

- Some W2L HO test cases in clause 8 of [3] also include L2W reselection with W set as high priority. Therefore, L2W high priority reselection cannot be achieved.

Observation 2.2

- Reselection from E-UTRAN to low priority GERAN/UTRAN is not possible with the current RSRP values, cell reselection priorities, and thresholds specified in the system information of the test cases, as they do not meet the criteria for low priority cell reselection for a UE that supports this feature.

- For some of the affected W2L HO test cases with L2W reselection, the E-UTRA cell can be made unsuitable for the UE to camp on WCDMA, allowing the test procedure to continue.

Table 2.1

| Spec | Test Case | Title | Comments and proposals |
| --- | --- | --- | --- |
| 36.523-1 | 6.2.3.1 | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to GSM_Idle/GPRS Packet_Idle | High priority and low priority L2G reselection cannot be achieved at steps 2 and 10 respectively. |
| 36.523-1 | 6.2.3.1a | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to GSM_Idle/GPRS Packet_Idle (Squal < ThreshServing, LowQ, Srxlev > ThreshX, LowP and Srxlev > ThreshX, HighP) | High priority and low priority L2G reselection cannot be achieved at steps 2 and 8 respectively. |
| 36.523-1 | 6.2.3.3a | Inter-RAT cell reselection / From UTRA_Idle to E-UTRA RRC_IDLE (QqualminEUTRA, SqualServingCell < Threshserving,low2, SqualnonServingCell,x > Threshx, low2 and SqualnonServingCell,x > Threshx, high2) | L2W high priority reselection cannot be achieved at step 5. Test case to be made not applicable. Proposal – Explore options to test few TPs based on industry’s interest. |
| 36.523-1 | 6.2.3.5 | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle | L2W high priority reselection cannot be achieved at step 2. |
| 36.523-1 | 6.2.3.5a | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle (Squal > ThreshX, HighQ, Squal < ThreshServing, LowQ, Squal > ThreshX, LowQ and SnonIntraSearchQ) | L2W high priority reselection cannot be achieved at step 10. |
| 36.523-1 | 6.2.3.6 | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle according to RAT priority provided by dedicated signalling | L2W low priority reselection cannot be achieved at step 18. Proposal – Explore options to test few TPs based on industry’s interest. |
| 36.523-1 | 6.2.3.32 | Inter-RAT cell re-selection / From E-UTRA RRC_IDLE to UTRA_Idle, Snonintrasearch | L2W high priority reselection cannot be achieved at step 2. |
| 36.523-1 | 6.2.3.33 | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle / Squal based cell reselection parameters are broadcasted in E-UTRAN / UE does not support Squal based cell reselection in UTRAN | L2W high priority reselection cannot be achieved at step 3. |
| 36.523-1 | 8.4.2.2 | Inter-RAT handover / From UTRA PS to E-UTRA / Data | L2W high priority reselection cannot be achieved at step 3 with current power levels. Proposed change – Reduce E-UTRA power level at T1 from -90 to -115dBm. |
| 36.523-1 | 8.4.2.4 | Inter-RAT handover / From UTRA HSPA to E-UTRA / Data | L2W high priority reselection cannot be achieved at step 3 with current power levels. Proposed change – Reduce E-UTRA power level at T1 from -90 to -115dBm. |
| 36.523-1 | 8.4.2.7.1/2/3 | CA / RRC connection reconfiguration / Handover UTRAN to E-UTRAN/ Success / SCell addition / CA | L2W high priority reselection cannot be achieved at step 2 with current power levels. Proposed change – Reduce E-UTRA power level of both E-UTRAN cells at T1 from -90 to -115dBm. |
| 36.523-1 | 8.6.3.4 | Logged MDT / Logging and reporting / Reporting at UTRAN Inter-RAT handover / PLMN list | L2W high priority reselection cannot be achieved at step 3. Proposal – Explore options to test few TPs based on industry’s interest. |
| 36.523-1 | 8.6.7.4 | Handover Failure logging / Reporting at UTRAN Inter-RAT handover / PLMN list | L2W high priority reselection cannot be achieved at step 14. Proposed change – Reduce E-UTRA power level E-UTRAN cell at T2 from -85 to -115dBm. |

Table 2.2

| Spec | Test Case | Title | Comments |
| --- | --- | --- | --- |
| 36.521-3 | 4.3.1.1 | E-UTRA FDD-UTRAN FDD cell reselection: UTRA FDD is of higher priority | High priority L2W reselection cannot be achieved at steps 4. |
| 36.521-3 | 4.3.1.2 | E-UTRA FDD-UTRAN FDD cell reselection: UTRA FDD is of lower priority | Low priority L2W reselection cannot be achieved at steps 6. |
| 36.521-3 | 4.3.1.3 | E-UTRAN FDD – UTRAN FDD cell re-selection in fading propagation conditions: UTRA FDD is of lower priority | Low priority L2W reselection cannot be achieved at steps 7. |
| 36.521-3 | 4.4.1 | E-UTRAN FDD - GSM cell re-selection | Low priority L2G reselection cannot be achieved at steps 6. |
| 36.521-3 | 4.3.3 | E-UTRAN TDD - UTRAN FDD cell re-selection | Low priority L2W reselection cannot be achieved at steps 6. |
| 36.521-3 | 4.4.2 | E-UTRAN TDD - GSM cell re-selection | Low priority L2G reselection cannot be achieved at steps 6. |

Proposal 2

- Update the applicability of impacted test cases listed in in Tables 2.1 (clause 6 and 8.6.3.4), 2.2  to be Not Applicable for UEs supporting protection against improper reselection to GERAN/UTRAN.

- Update test cases from clause 8 in [3] except 8.6.3.4 listed in table 2.1 to cater all the UE behaviors.

## 3.  Proposals/Way Forward

Proposal 1

- Define a new PICS parameter in TS 36.523-2 [5] for Signalling and in TS 36.521-2 [7] for RRM to indicate UE support of this feature.

Proposal 2

- Update the applicability of impacted test cases listed in in Tables 2.1 (clause 6 and 8.6.3.4), 2.2  to be Not Applicable for UEs supporting protection against improper reselection to GERAN/UTRAN.

- Update test cases from clause 8 in [3] except 8.6.3.4 listed in table 2.1 to cater all the UE behaviors.

## 4.	References

[1]	R2-2314061	Protection against improper reselection to GERAN/UTRAN [RESELECTION_TO GSM_AND_UTRAN]

[2]	R2-2313930	Protection against improper reselection to GERAN/UTRAN [RESELECTION_TO GSM_AND_UTRAN]

[3]	TS 36.523-1	Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 1: Protocol conformance specificationNetwork support and clarification of the redir-policy bit

[4]	TS 36.304	Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode

[5]	TS 36.523-2	Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification

[6]	TS 36.521-3	Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Radio Resource Management (RRM) conformance testing

[7]	TS 36.521-2	Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Implementation Conformance Statement (ICS)