3GPP TSG-RAN5 Meeting #107	R5-253051

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5456 | rev | 1 | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Idle mode test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To correct the IE lateNonCriticalExtension in SystemInformationBlockType3 based on TS 36.508 and 36.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specific message content of SystemInformationBlockType3 was updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect IE will exist in this message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.3a.3.3, 6.1.2.17.3.3, 6.1.2.18.3.3, 6.2.3.1a.3.3, 6.2.3.5a.3.3, 6.2.3.8a.3.3, 6.2.3.10a.3.3 and 6.2.3.33.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is the outcome of R5-252633 with one revision. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-252633 -> R5-253051 |  |  |  |  |  |  |  |  |

[Start of change]

6.1.2.3a.3.3	Specific message contents

Table 6.1.2.3a.3.3-0: Conditions for table 6.1.2.3a.3.3-1, 6.1.2.3a.3.3-2

Condition descriptions

Cell 1 This condition applies to system information transmitted on Cell 1.

Cell 2 This condition applies to system information transmitted on Cell 2.

Table 6.1.2.3a.3.3-1: SystemInformationBlockType1 for Cell 1 and 2 (all steps, Table 6.1.2.3a.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -53 (-106 dBm) |  | Cell 1 |
| q-RxLevMin | -46 (-92 dBm) |  | Cell 2 |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -5 dB |  | Cell 1 |
| q-QualMin-r9 | -20 dB |  | Cell 2 |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.3a.3.3-1A: SystemInformationBlockType1-BR-r13 for Cell 1 and 2 (all steps when UE under test is CAT M1, Table 6.1.2.3a.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -53 (-106 dBm) |  | Cell 1 |
| q-RxLevMin | -46 (-92 dBm) |  | Cell 2 |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -5 dB |  | Cell 1 |
| q-QualMin-r9 | -20 dB |  | Cell 2 |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.3a.3.3-2: SystemInformationBlockType3 for Cell 1 and 2 (all steps, table 6.1.2.3a.3.2-2)

| Derivation Path: 36.508 clause 4.4.3.3, Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| q-Hyst | dB0 |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -53 (-106 dBm) |  | Cell 1 |
| q-RxLevMin | -46 (-92 dBm) |  | Cell 2 |
| t-ReselectionEUTRA | 7 |  |  |
| } |  |  |  |
| s-IntraSearch-v920 | Not present |  |  |
| s-NonIntraSearch-v920 | Not present |  |  |
| q-QualMin-r9 | -5 dB |  | Cell 1 |
| q-QualMin-r9 | -20 dB |  | Cell 2 |
| threshServingLowQ-r9 | Not present |  |  |
| } |  |  |  |

[Skip unchanged text]

6.1.2.17.3.3	Specific message contents

Table 6.1.2.17.3.3-0: Conditions for specific message contents
in Table 6.1.2.17.3.3-2A and Table 6.1.2.17.3.3-4

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.17.3.3-1: SystemInformationBlockType1 for Cells 1, 2 and 3 (preamble, Table 6.1.2.17.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -53 (-106 dBm) |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.17.3.3-1A: SystemInformationBlockType1-BR-r13 for Cells 1, 2 and 3 (preamble when UE under test is CAT M1, Table 6.1.2.17.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -53 (-106 dBm) |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.17.3.3-2: SystemInformationBlockType3 for Cells 1, 2 and 3 (preamble, Table 6.1.2.17.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| s-IntraSearch-v920 SEQUENCE { |  |  |  |  |  |
| s-IntraSearchP-r9 |  |  | 0 (0 dB) |  |  |
| s-IntraSearchQ-r9 |  |  | 20 dB |  |  |
| } |  |  |  |  |  |
| s-NonIntraSearch-v920 SEQUENCE { |  |  |  |  |  |
| s-NonIntraSearchP-r9 |  |  | 0 (0 dB) |  |  |
| s-NonIntraSearchQ-r9 |  |  | 20 dB |  |  |
| } |  |  |  |  |  |
| q-QualMin-r9 |  |  | -20 dB |  |  |
| } |  |  |  |

Table 6.1.2.17.3.3-2A: SystemInformationBlockType5 for Cell 2 (preamble, Table 6.1.2.17.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| dl-CarrierFreq[1] | EARFCN of Cell 3 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| t-ReselectionEUTRA | 1 |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | Same downlink EARFCN as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.17.3.3-3: SystemInformationBlockType1 for Cell 3 (step 16, Table 6.1.2.17.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.17.3.3-3AA: SystemInformationBlockType1-BR-r13 for Cell 3 (step 16 when UE under test is CAT M1, Table 6.1.2.17.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| q-QualMinOffset-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.17.3.3A: SystemInformationBlockType3 for Cell 3 (step 16, Table 6.1.2.17.3.2-2)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| s-NonIntraSearch-v920 SEQUENCE { |  |  |  |  |  |
| s-NonIntraSearchP-r9 |  |  | 0 (0 dB) |  |  |
| s-NonIntraSearchQ-r9 |  |  | 2 dB |  |  |
| } |  |  |  |  |  |
| } |  |  |  |

[Skip unchanged text]

6.1.2.18.3.3	Specific message contents

Table 6.1.2.18.3.3-1: Conditions for specific message contents
in Tables 6.1.2.18.3.3-2 and 6.1.2.18.3.3-3

| Condition | Explanation |
| --- | --- |
| Cell 1 | This condition applies to system information transmitted on Cell 1. |
| Cell 3 | This condition applies to system information transmitted on Cell 3. |
| Band > 64 | If band > 64 is selected |

Table 6.1.2.18.3.3-2: SystemInformationBlockType3 for Cell 1 and Cell 3 (preamble, Table 6.1.2.18.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 1 | 2 dB | Cell 3 |
| cellReselectionPriority | 5 |  | Cell 3 |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -50 (-100 dBm) |  |  |
| } |  |  |  |
| s-IntraSearch-v920 | Not present |  |  |
| s-NonIntraSearch-v920 | Not present |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| threshServingLowQ-r9 | 0 dB |  | Cell 1 |
|  | 4 dB |  | Cell 3 |
| } |  |  |  |

Table 6.1.2.18.3.3-3: SystemInformationBlockType5 for Cell 1 (preamble and all steps, Table 6.1.2.18.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | EARFCN of Cell 3 |  |  |
| dl-CarrierFreq[1] | maxEARFCN |  | Band > 64 |
| q-RxLevMin[1] | -50 (-100 dBm) |  |  |
| t-ReselectionEUTRA[1] | 7 |  |  |
| threshX-High[1] | 10 (20 dB) |  |  |
| cellReselectionPriority[1] | 5 |  |  |
| q-QualMin-r9[1] | -20 dB |  |  |
| threshX-Q-r9[1] SEQUENCE { |  |  |  |
| threshX-HighQ-r9[1] | 4 | 4 dB |  |
| threshX-LowQ-r9[1] | 2 | 2 dB |  |
| } |  |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | EARFCN of Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.18.3.3-3a: SystemInformationBlockType5 for Cell 3 (preamble and all steps, Table 6.1.2.18.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType5 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq[1] | EARFCN of Cell 1 |  |  |
|  | maxEARFCN |  | Band > 64 |
| q-RxLevMin[1] | -50 (-100 dBm) |  |  |
| t-ReselectionEUTRA[1] | 7 |  |  |
| cellReselectionPriority[1] | 4 |  |  |
| q-QualMin-r9[1] | -20 dB |  |  |
| threshX-Q-r9[1] SEQUENCE { |  |  |  |
| threshX-HighQ-r9[1] | 4 | 4 dB |  |
| threshX-LowQ-r9[1] | 2 | 2 dB |  |
| } |  |  |  |
| } |  |  |  |
| SystemInformationBlockType5-v8h0-IEs SEQUENCE { |  |  | Band > 64 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  |  |
| dl-CarrierFreq-v9e0[1] | EARFCN of Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.18.3.3-4: SystemInformationBlockType3 for Cell 3 (Step 2A, Table 6.1.2.18.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| s-IntraSearch-v920 | Not present |  |  |
| s-NonIntraSearch-v920 | Not present |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| threshServingLowQ-r9 | 26 dB |  |  |
| } |  |  |  |

[Skip unchanged text]

6.2.3.1a.3.3	Specific message contents

Table 6.2.3.1a.3.3-1: SystemInformationBlockType1 cell 1 (preamble and all steps, Table 6.2.3.1a.3.2-3)

| Derivation path: 36.508 table 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension | Not present |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| ims-EmergencySupport-r9 | Not present |  |  |  |
| cellSelectionInfo-v920 SEQUENCE { |  |  |  |  |
| q-QualMin-r9 | -20 |  |  |  |
| q-QualMinOffset-r9 | 1 |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension | Not present |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |

Table 6.2.3.1a.3.3-2: SystemInformationBlockType3 cell 1 (preamble and all steps, Table 6.2.3.1a.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | Not present |  |  |
| threshServingLow | 0 | Actual value =0 dB |  |
| cellReselectionPriority | 4 | A middle value in the range has been selected. |  |
| } |  |  |  |
| threshServingLowQ-r9 | 26 dB |  |  |
| } |  |  |  |

[Skip unchanged text]

6.2.3.5a.3.3	Specific message contents

Table 6.2.3.5a.3.3-0: Conditions for specific message contents
in Table 6.2.3.5a.3.3-3 and Table 6.2.3.5a.3.3-6

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.5a.3.3-1:SystemInformationBlockType3 for Cell 1 (preamble, Table 6.2.3.5a.3.2-3)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| s-IntraSearch-v920 | Not present |  |  |
| s-NonIntraSearch-v920 | Not present |  |  |
| q-QualMin-r9 | -20 dB |  |  |
| threshServingLowQ-r9 | 26 dB |  |  |
| } |  |  |  |

[Skip unchanged text]

6.2.3.8a.3.3	Specific message contents

Table 6.2.3.8a.3.3-1: SystemInformationBlockType1 to Cell 1 (Preamble and all steps, table 6.2.3.8a.3.2-2)

| Derivation path: 36.508 table 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-Rxlevmin | -55 (-110 dBm) |  |  |
| q-Rxlevminoffset | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.8a.3.3-2: SystemInformationBlockType3 for Cell 1 (Preamble and all steps, table 6.2.3.8a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 5 (10dB) |  |  |
| cellReselectionPriority | 4 |  |  |
| } |  |  |  |
| threshServingLowQ-r9 | 26 |  |  |
| } |  |  |  |

[Skip unchanged text]

6.2.3.10a.3.3	Specific message contents

Table 6.2.3.10a.3.3-1: SystemInformationBlockType1 to Cell 1 (Preamble and all steps, table 6.2.3.10.3.2-2)

| Derivation path: 36.508 table 4.4.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-Rxlevmin | -55 (-110 dBm) |  |  |
| q-Rxlevminoffset | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.3.10a.3.3-2: SystemInformationBlockType3 for Cell 1 (Preamble and all steps, table 6.2.3.10a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 5 (10dB) |  |  |
| cellReselectionPriority | 4 |  |  |
| } |  |  |  |
| threshServingLowQ-r9 | 26 |  |  |
| } |  |  |  |

[Skip unchanged text]

6.2.3.33.3.3	Specific message contents

Table 6.2.3.33.3.3-0: Conditions for specific message contents
in Table 6.2.3.33.3.3-3

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 6.2.3.33.3.3-1: SystemInformationBlockType3 for Cell 1 (preamble and all steps, Table 6.2.3.33.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 10 | 20 dB |  |
| } |  |  |  |
| s-IntraSearch-v920 | Not present | Default of infinity applied. |  |
| s-NonIntraSearch-v920 | Not present | Default 0 dB applied |  |
| q-QualMin-r9 | -20 dB |  |  |
| threshServingLowQ-r9 | 4 |  |  |
| } |  |  |  |

[End of change]