3GPP TSG-RAN5 Meeting #107	R5-253063

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5467 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of inter RAT handover CA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At step 2 of test case 8.4.2.7.1, UE is expected to perform a reselection from E-UTRA to high priority UTRA cell. This cannot be achieved on a UE supporting protection against improper reselection from E-UTRAN to GERAN/UTRAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | E-UTRA power levels at T1 are updated for UE to select UTRAN cell in these test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

##### 8.4.2.7.1	CA / RRC connection reconfiguration / Handover UTRAN to E-UTRAN/ Success / SCell addition / Intra-band Contiguous CA

8.4.2.7.1.1	Test Purpose (TP)

(1)

with { UE in UTRA CELL_DCH(PS-DCCH+DTCH_HS-DSCH) state }

ensure that {

when { UE receives a HANDOVER FROM UTRAN COMMAND message including the eutra-Message with RRCConnectionReconfiguration including the sCellToAddModList }

then { UE transmits an RRCConnectionReconfigurationComplete message and enters E-UTRA RRC_CONNECTED state with both PCell and SCell }

}

8.4.2.7.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: Reference TS 36.331 clause 5.4.2.3, and 5.3.10.3b]

[TS 36.331, clause 5.4.2.3]

If the UE is able to comply with the configuration included in the RRCConnectionReconfiguration message, the UE shall:

1>	apply the default physical channel configuration as specified in 9.2.4;

1>	apply the default semi-persistent scheduling configuration as specified in 9.2.3;

1>	apply the default MAC main configuration as specified in 9.2.2;

1>	start timer T304 with the timer value set to t304, as included in the mobilityControlInfo;

1>	consider the target PCell to be one on the frequency indicated by the carrierFreq with a physical cell identity indicated by the targetPhysCellId;

1>	start synchronising to the DL of the target PCell;

1>	set the C-RNTI to the value of the newUE-Identity;

1>	for the target PCell, apply the downlink bandwidth indicated by the dl-Bandwidth;

1>	for the target PCell, apply the uplink bandwidth indicated by (the absence or presence of) the ul-Bandwidth;

1>	configure lower layers in accordance with the received radioResourceConfigCommon;

1>	configure lower layers in accordance with any additional fields, not covered in the previous, if included in the received mobilityControlInfo;

1>	perform the radio resource configuration procedure as specified in 5.3.10;

1>	forward the nas-SecurityParamToEUTRA to the upper layers;

1>	derive the KeNB key, as specified in TS 33.401 [32];

1>	derive the KRRCint key associated with the integrityProtAlgorithm, as specified in TS 33.401 [32];

1>	derive the KRRCenc key and the KUPenc key associated with the cipheringAlgorithm, as specified in TS 33.401 [32];

1>	configure lower layers to apply the indicated integrity protection algorithm and the KRRCint key immediately, i.e. the indicated integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	configure lower layers to apply the indicated ciphering algorithm, the KRRCenc key and the KUPenc key immediately, i.e. the indicated ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition as specified in 5.3.10.3b;

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	perform the measurement identity autonomous removal as specified in 5.5.2.2a;

1>	if the RRCConnectionReconfiguration message includes the reportProximityConfig:

2>	perform the proximity indication configuration in accordance with the received reportProximityConfig;

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and plmn-Identity stored in VarRLF-Report is equal to the RPLMN:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and plmn-Identity stored in VarLogMeasReport is equal to the RPLMN:

3>	include the logMeasAvailable;

1>	submit the RRCConnectionReconfigurationComplete message to lower layers for transmission using the new configuration;

1>	if the RRCConnectionReconfiguration message does not include rlf-TimersAndConstants set to setup:

2>	use the default values specified in 9.2.5 for timer T310, T311 and constant N310, N311;

1>	if MAC successfully completes the random access procedure:

2>	stop timer T304;

2>	apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the target PCell, if any;

2>	apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the target PCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of the target PCell;

NOTE 1:	Whenever the UE shall setup or reconfigure a configuration in accordance with a field that is received it applies the new configuration, except for the cases addressed by the above statements.

2>	enter E-UTRA RRC_CONNECTED, upon which the procedure ends;

NOTE 2:	The UE is not required to determine the SFN of the target PCell by acquiring system information from that cell before performing RACH access in the target PCell.

[TS 36.331, clause 5.3.10b]

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the cellIdentification, in accordance with the received radioResourceConfigCommonSCell and radioResourceConfigDedicatedSCell;

2>	configure lower layers to consider the SCell to be in deactivated state;

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the received radioResourceConfigDedicatedSCell;

8.4.2.7.1.3	Test description

8.4.2.7.1.3.1	Pre-test conditions

System Simulator:

-	Cell 5 is UTRAN Cell

-	Cell 1 is the PCell, Cell 3 is SCell to be added

-	Cell 3 is an Inactive SCell according to [18] cl. 6.3.4

-	System information combination 9 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

-	None

Preamble:

-	The UE is in state Registered, Idle mode (state 2) on Cell 1 according to [18].

8.4.2.7.1.3.2	Test procedure sequence

Table 8.4.2.7.1.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 8.4.2.7.1.3.2-2.

Table 8.4.2.7.1.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 3 | Cell 5 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -115 | -115 |  | The power level values are such that UE camps on Cell 5. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84 MHz | - | - | -65 |  |
|  | PCCPCH Ec(UTRA LCR TDD) | dBm/1.28 MHz | - | - | -65 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -70 | -70 | - | The power level values are such that entering conditions for event 3a are satisfied. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84 MHz | - | - | -85 |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | - | -85 |  |

Table 8.4.2.7.1.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes Cell 5, Cell 1 and Cell 3 level according to the row "T1" in table 8.4.2.7.1.3.2-1. | - | - | - | - |
| 2 | Generic test procedure in TS 36.508 subclause 6.4.2.8 is performed on Cell 5. NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | - | - |
| - | Exception: Steps 2Aa1 to 2Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 2Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 2Aa1a1-2Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 2Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 2Aa1a5 | SS Stops 6s timer | - | - | - | - |
| 2Aa1b1 | The 6s timer expires | - | - | - | - |
| 3-7 | Step 7 to 11 of test procedure in TS 34.123-1 subclause 12.9.14.4 is performed on Cell 5 using the UTRA reference radio bearer parameters and combination “UTRA HSDPA RB” according to TS 36.508 subclause 4.8.3 and Table 4.8.3-1. NOTE: The UE performs Network initiated RAB re-establishment in a UTRAN cell. | - | - | - | - |
| - | EXCEPTION: Steps 8a1 to 8a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 8a1 | IF pc_FDD AND pc_UTRA_CompressedModeRequired THEN the SS transmits a PHYSICAL CHANNEL RECONFIGURATION message on Cell 5 including the DPCH compressed mode info. | <-- | PHYSICAL CHANNEL RECONFIGURATION | - | - |
| 8a2 | The UE transmits a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on Cell 5. | --> | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | - | - |
| 9 | The SS transmits a MEASUREMENT CONTROL message to setup inter-RAT measurement on Cell 5. | <-- | MEASUREMENT CONTROL | - | - |
| 10 | The SS changes Cell 5, Cell 1 and Cell 3 level according to the row "T2" in table 8.4.2.7.1.3.2-1. | - | - | - | - |
| 11 | The UE transmits a MEASUREMENT REPORT message on Cell 5 including the E-UTRA event results. | --> | MEASUREMENT REPORT | - | - |
| 12 | The SS transmits a HANDOVER FROM UTRAN COMMAND message on Cell 5. | <-- | HANDOVER FROM UTRAN COMMAND | - | - |
| 13 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on Cell 1 to confirm the successful completion of the handover and SCell addition of Cell 3? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 13A – 13H | Step 1 to 8 of test procedure in TS36.508 Table 6.4.2.10-1 is performed on Cell1 | - | - | - | - |
| - | EXCEPTION: Steps 13Ia1 to 13Ia2b1 describe optional behaviour if UE sends IMS deregistration at steps 2Aa1 to 2Aa1b1 | - | - | - | - |
| 13Ia1-13Ia2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| 14 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicates that the UE is in E-UTRA RRC_CONNECTED state on Cell 1? | - | - | 1 | - |

8.4.2.7.1.3.3	Specific message contents

Table 8.4.2.7.1.3.3-0: Conditions for specific message contents
in Tables 8.4.2.7.1.3.3-5 and 8.4.2.7.1.3.3-7

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.4.2.7.1.3.3-1: SystemInformationBlockType6 for Cell 1 (preamble, Table 8.4.2.7.1.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[1] | 0 |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { | The same number of entries as the configured UTRA TDD carriers |  | UTRA-TDD |
| carrierFreq[1] | Same downlink ARFCN as used for Cell 5 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| p-MaxUTRA[n] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

Table 8.4.2.7.1.3.3-2: MEASUREMENT CONTROL (step 9, Table 8.4.2.7.1.3.2-2)

| Derivation Path: 36.508, clause 4.7B.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| - Inter-RAT measurement quantity |  |  |  |
| - Measurement quantity for UTRAN quality estimate |  |  |  |
| - Filter coefficient | 0 |  |  |
| - CHOICE mode | FDD |  |  |
| - Measurement quantity | CPICH RSCP |  |  |
| - CHOICE system | E-UTRA |  |  |
| - Measurement quantity | RSRP |  |  |
| - Filter coefficient | 0 |  |  |
| - Inter-RAT reporting quantity |  |  |  |
| - UTRAN estimated quality | FALSE |  |  |
| - CHOICE system | E-UTRA |  |  |
| - Reporting quantity | both |  |  |
| - Reporting cell status | Not present |  |  |
| - CHOICE report criteria | Inter-RAT measurement reporting criteria |  |  |
| - Parameters required for each event | 1 entry |  |  |
| - Inter-RAT event identity | 3a |  |  |
| - Threshold own system | -66 |  |  |
| - W | 0 |  |  |
| - Threshold other system | -55 |  |  |
| - Hysteresis | 0 |  |  |
| - Time to trigger | 10 ms |  |  |
| - Reporting cell status |  |  |  |
| - CHOICE reported cell | Report cells within active set or within virtual active set or of the other RAT |  |  |
| - Maximum number of reported cells | 2 |  |  |

Table 8.4.2.7.1.3.3-3: HANDOVER FROM UTRAN COMMAND (step 12, Table 8.4.2.7.1.3.2-2)

Derivation Path: 36.508, Table 4.7B.1-2

Table 8.4.2.7.1.3.3-4: RRCConnectionReconfiguration (Table 8.4.2.7.1.3.3-3)

Derivation Path: 36.508, Table 4.6.1-8, condition HO-TO-EUTRA(1,0) and SCell_AddMod

Table 8.4.2.7.1.3.3-5: MobilityControlInfo (Table 8.4.2.7.1.3.3-4)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 1. |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 1. |  |  |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Downlink system bandwidth under test. |  |  |
| ul-Bandwidth | Uplink Bandwidth under test. |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| } |  |  |  |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

Table 8.4.2.7.1.3.3-6: SecurityConfigHO (Table 8.4.2.7.1.3.3-4)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| interRAT SEQUENCE { |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |
| cipheringAlgorithm | Set according to PIXIT parameter for default ciphering protection algorithm |  |  |
| integrityProtAlgorithm | Set according to PIXIT parameter for default integrity algorithm |  |  |
| } |  |  |  |
| nas-SecurityParamToEUTRA | Octets 1 to 4 are arbitrarily selected. Bits 1 to 3 of octet 5 are set according to PIXIT parameter for default integrity protection algorithm. Bits 5 to 7 of octet 5 are set according to PIXIT parameter for default ciphering algorithm. Bits 1 to 3 of octet 6 are arbitrarily selected between '000'B and '110'B, different from the valid NAS key set identifier of the UE if such a value exists. Bit 4 of octet 6 is set to 1. | Octets 1 to 4 include the NonceMME value. Bits 1 to 3 of octet 5 include the Type of integrity protection algorithm Bits 5 to 7 of octet 5 include the Type of ciphering algorithm. Bits 1 to 4 of octet 6 include the NAS key set identifier. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.2.7.1.3.3-7: SCellToAddMod-r10-f2 (Table 8.4.2.7.1.3.3-4)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.3-19D SCellToAddMod-r10-DEFAULT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |  |
| SCellToAddMod-r10 ::= SEQUENCE (SIZE (1..maxSCell-r10)) OF SEQUENCE { | 1 entry |  |  |  |
| sCellIndex-r10 | 1 |  |  |  |
| cellIdentification-r10 SEQUENCE { |  |  |  |  |
| physCellId-r10 | Physical Cell Identity of Cell 3 |  |  |  |
| dl-CarrierFreq-r10 | Same downlink EARFCN as used for Cell 3 |  |  |  |
| dl-CarrierFreq-r10 | maxEARFCN |  |  | Band > 64 |
| } |  |  |  |  |
| radioResourceConfigCommonSCell-r10 | RadioResourceConfigCommonSCell-r10-f2 |  |  |  |
| dl-CarrierFreq-v1090 | Same downlink EARFCN as used for Cell 3 |  |  | Band > 64 |
| } |  |  |  |  |  |

Table 8.4.2.7.1.3.3-8: RadioResourceConfigCommonSCell-r10-f2 (Table 8.4.2.7.1.3.3-7)

| Derivation Path: 36.508 clause 4.6.3 table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| dl-Bandwidth-r10 | Same downlink system bandwidth as used for Cell 3 |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>