3GPP TSG-RAN5 Meeting #107	R5-253064

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5468 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of inter RAT MDT test case 8.6.7.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11_Test |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At step 14 of inter RAT MDT test case 8.6.7.4, UE is expected to perform a reselection from E-UTRA to high priority UTRA cell. This cannot be achieved on a UE supporting protection against improper reselection from E-UTRAN to GERAN/UTRAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated powerl of E-UTRA cell 9 at T2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 8.6.7.4	Handover Failure logging / Reporting at UTRAN Inter-RAT handover / PLMN list

8.6.7.4.1	Test Purpose (TP)

(1)

with { UE selects the UTRAN cell and enters UTRA CELL_DCH(PS-DCCH+DTCH_DCH) after detection of handover failure in an E-UTRAN cell in RRC_CONNECTED }

ensure that {
  when { UE receives a HANDOVER FROM UTRAN COMMAND message including the eutra-Message and UE selects the EPLMN which is not the RPLMN }

then { UE transmits an RRCConnectionReconfigurationComplete message containing rlf-InfoAvailable and enters E-UTRA RRC_CONNECTED state }

}

(2)

with { UE in RRC_CONNECTED having reported that the UE has handover failure information available }

ensure that {
  when { UE receives the UEInformationRequest message containing rlf-ReportReq set to true }

then { UE transmits a UEInformationResponse message including rlf-Report }

}

8.6.7.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.5.6, 5.4.2.3 and 5.6.5.3. Unless otherwise stated these are Rel-11 requirements.

[TS 36.331, clause 5.3.5.6 (TP1, TP2)]

The UE shall:

1>	if T304 expires (handover failure):

NOTE 1:	Following T304 expiry any dedicated preamble, if provided within the rach-ConfigDedicated, is not available for use by the UE anymore.

2>	revert back to the configuration used in the source PCell, excluding the configuration configured by the physicalConfigDedicated, the mac-MainConfig and the sps-Config;

2>	store the following handover failure information in VarRLF-Report by setting its fields as follows:

3>	clear the information included in VarRLF-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

3>	set the measResultLastServCell to include the RSRP and RSRQ, if available, of the PCell based on measurements collected up to the moment the UE detected handover failure;

3>	set the measResultNeighCells to include the best measured cells, other than the PCell, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected handover failure, and set its fields as follows;

4>	if the UE was configured to perform measurements for one or more EUTRA frequencies, include the measResultListEUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring UTRA frequencies, include the measResultListUTRA;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring GERAN frequencies, include the measResultListGERAN;

4>	if the UE was configured to perform measurement reporting for one or more neighbouring CDMA2000 frequencies, include the measResultsCDMA2000;

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported.

3>	if detailed location information is available, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

4>	include the horizontalVelocity, if available;

3>	set the failedPCellId to the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover;

3>	include previousPCellId and set it to the global cell identity of the PCell where the last RRCConnectionReconfiguration message including mobilityControlInfo was received;

3>	set the timeConnFailure to the elapsed time since reception of the last RRCConnectionReconfiguration message including the mobilityControlInfo;

3>	set the connectionFailureType to 'hof';

3>	set the c-RNTI to the C-RNTI used in the source PCell;

2>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the RRC connection reconfiguration procedure ends;

The UE may discard the handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the failure is detected, upon power off or upon detach.

NOTE 3:	E-UTRAN may retrieve the handover failure information using the UE information procedure with rlf-ReportReq set to true, as specified in 5.6.5.3.

[TS 36.331, clause 5.4.2.3 (TP1)]

If the UE is able to comply with the configuration included in the RRCConnectionReconfiguration message, the UE shall:

...

1>	set the content of RRCConnectionReconfigurationComplete message as follows:

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	include rlf-InfoAvailable;

2>	if the UE has logged measurements available for E-UTRA and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

3>	include the logMeasAvailable;

2>	if the UE has connection establishment failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

3>	include connEstFailInfoAvailable;

[TS 36.331, clause 5.6.5.3 (TP2)]

Upon receiving the UEInformationRequest message, the UE shall:

...

1>	if rlf-ReportReq is set to true and the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

2>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link or handover failure in E-UTRA;

2>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

2>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

...

1>	if the logMeasReportReq is present and if the RPLMN is included in plmn-IdentityList stored in VarLogMeasReport:

...

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

8.6.7.4.3	Test description

8.6.7.4.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 9 and Cell 12

-	System information combination 4 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cell.

Table 8.6.7.4.3.1-1: PLMN identifiers

| Cell | PLMN name |
| --- | --- |
| 1 | PLMN1 |
| 9 | PLMN1 |
| 12 | PLMN2 |

UE:

None.

Preamble:

-	The UE is registered on PLMN1 (Cell 1) using the procedure described in TS 36.508[18] clause 4.5.2.3 except that the ATTACH ACCEPT message indicates PLMN2 in the Equivalent PLMN list as described in Table 8.6.7.4.3.3-1

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.7.4.3.2	Test procedure sequence

Table 8.6.7.4.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 8.6.7.4.3.2-2. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.6.7.4.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 9 | Cell12 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -85 | - | -79 | The power level values are such that measurement results for Cell 1 (M1) and Cell 12 (M12) satisfy entry condition for event A3 (M12 > M1). (NOTE 1) |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | "Off" | - |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | "Off" | - |  |
| T1A | Cell-specific RS EPRE | dBm/15kHz | "Off" | - | -79 | Only Cell 12 is available. |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | "Off" | - |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | "Off" | - |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | "Off" | - | -115 |  |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | -65 | - |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -67 | - |  |
| T3 | Cell-specific RS EPRE | dBm/15kHz | “Off” | - | -70 | The power level values are such that entering conditions for event 3a are satisfied. (NOTE 2) |
|  | CPICH Ec (UTRA FDD) | dBm/3.84MHz | - | -85 | - |  |
|  | PCCPCH Ec (UTRA LCR TDD) | dBm/1.28 MHz | - | -85 | - |  |
| NOTE 1: Power level "Off" for UTRA cell is defined in TS 34.108 Table 6.1.4 and Table 6.1.9. NOTE 2: Power level “Off” for E-UTRA cell is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.6.7.4.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message to setup inter-frequency measurement on Cell 1. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | The SS changes Cell 1 and Cell 12 parameters according to the row "T1" in Table 8.6.7.4.3.2-1. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message on Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 to order the UE to perform inter frequency handover to Cell 12. | <-- | RRCConnectionReconfiguration | - | - |
| - | EXCEPTION: In parallel to the events described in step 5A the steps specified in Table 8.6.7.4.3.2-3 should take place. | - | - | - | - |
| 5A | The SS changes Cell 1 and Cell 12 parameter according to the row "T1A" in Table 8.6.7.4.3.2-1. |  |  |  |  |
| 6 | The UE transmits an RRCConnectionReestablishmentRequest message on Cell 12. | --> | RRCConnectionReestablishmentRequest | - | - |
| 7 | The SS transmits an RRCConnectionReestablishment message on Cell 12. | <-- | RRCConnectionReestablishment | - | - |
| 8 | The UE transmits an RRCConnectionReestablishmentComplete message with rlf-InfoAvailable on Cell 12. | --> | RRCConnectionReestablishmentComplete | - | - |
| 9 | The SS transmits an RRCConnectionReconfiguration message to resume existing radio bearer on Cell 12. | <-- | RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 12. | --> | RRCConnectionReconfigurationComplete | - | - |
| 10A | The UE transmits a TRACKING AREA UPDATE REQUEST message on Cell 12. | - | - | - | - |
| 10B | SS responds with a TRACKING AREA UPDATE ACCEPT message. NOTE: The TAU is accepted with PLMN1 listed as an Equivalent PLMN | - | - | - | - |
| 10C | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | - | - | - | - |
| 11 | The SS transmits an RRCConnectionRelease message on Cell 12. | <-- | RRCConnectionRelease | - | - |
| 12 | Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. | - | - | - | - |
| 13 | The SS changes Cell 1, Cell 12 and Cell 9 parameters according to the row "T2" in Table 8.6.7.4.3.2-1. | - | - | - | - |
| 14 | Generic test procedure in TS 36.508 subclause 6.4.2.8 is performed on Cell 9. NOTE: The UE performs an RAU procedure and the RRC connection is released. | - | - | - | - |
| - | Exception: Steps 14Aa1 to 14Aa1b1 describe optional behaviour that depend on the UE capability. | - | - | - | - |
| 14Aa1 | IF pc_IMS = TRUE THEN SS starts 6s timer | - | - | - | - |
| 14Aa1a1-14Aa1a3a6 | The procedure defined in TS 36.523-3 Table 10.1.2.1-2 Steps 1-4a6 is performed | - | - | - | - |
| 14Aa1a4 | SS releases RRC Connection | - | - | - | - |
| 14Aa1a5 | SS Stops 6s timer | - | - | - | - |
| 14Aa1b1 | The 6s timer expires | - | - | - | - |
| 15-19 | Step 15 to 19 of test procedure in TS 34.123-1 subclause 12.9.14.4 is performed on Cell 9 using the UTRA reference radio bearer parameters and combination “UTRA PS RB” according to TS 36.508 subclause 4.8.3 and Table 4.8.3-1. NOTE: The UE performs NW initiated RAB re-establishment in a UTRAN cell. | - | - | - | - |
| - | For UTRAN FDD, EXCEPTION: Steps 20a1 to 20a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. For UTRAN TDD, go to Step 21. | - | - | - | - |
| 20a1 | IF pc_UTRA_CompressedModeRequired THEN the SS transmits a PHYSICAL CHANNEL RECONFIGURATION message on Cell 9 including the DPCH compressed mode info. | <-- | PHYSICAL CHANNEL RECONFIGURATION | - | - |
| 20a2 | The UE transmits a PHYSICAL CHANNEL RECONFIGURATION COMPLETE message on Cell 9. | --> | PHYSICAL CHANNEL RECONFIGURATION COMPLETE | - | - |
| 21 | The SS transmits a MEASUREMENT CONTROL message to setup inter-RAT measurement on Cell 9. | <-- | MEASUREMENT CONTROL | - | - |
| 22 | The SS changes Cell 9 and Cell 12 parameters according to the row "T3" in Table 8.6.7.4.3.2-1. | - | - | - | - |
| 23 | The UE transmits a MEASUREMENT REPORT message on Cell 9 including the E-UTRA event results. | --> | MEASUREMENT REPORT | - | - |
| 24 | The SS transmits a HANDOVER FROM UTRAN COMMAND message on Cell 9. | <-- | HANDOVER FROM UTRAN COMMAND | - | - |
| 25 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message with handover failure information on Cell 12? | --> | RRCConnectionReconfigurationComplete | 1 | P |
| 26 | Generic test procedure in TS 36.508 subclause 6.4.2.10 is performed on Cell 12. NOTE: The UE performs tracking area updating procedure without ISR and security reconfiguration after successful completion of handover from UTRA. The TAU is accepted with PLMN1 listed as an Equivalent PLMN. | - | - | - | - |
| - | EXCEPTION: Steps 26Aa1 to 26Aa2b1 describe optional behaviour if UE sends IMS deregistration at steps 14Aa1 to 14Aa1b1 | - | - | - | - |
| 26Aa1-26Aa2b1 | The procedure defined in TS 36.508 subclause 4.5A.3 Steps 10a1-10a2b1 is performed | - | - | - | - |
| 27 | The SS transmits a UEInformationRequest message on Cell 12. | <-- | UEInformationRequest | - | - |
| 28 | Check: Does the UE transmit a UEInformationResponse message on Cell 12? | --> | UEInformationResponse | 2 | P |
| 29 | Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.3 indicate that the UE is in E-UTRA RRC_CONNECTED state on Cell 12? | - | - | 2 | - |

Table 8.6.7.4.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The steps 1 and 2 below are repeated for the duration of T304. | - | - | - | - |
| 1 | The UE attempts to perform the inter-frequency handover using MAC Random Access Preamble on Cell 12. | - | - | - | - |
| 2 | The SS does not respond. | - | - | - | - |

8.6.7.4.3.3	Specific message contents

Table 8.6.7.4.3.3-0: Conditions for specific message contents
in Tables 8.6.7.4.3.3-4, 8.6.7.4.3.3-7, 8.6.7.4.3.3-14 and Table 8.6.7.4.3.3-18

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |
| Band 24 High range | If Band 24 high frequency range is selected for the target cell |

Table 8.6.7.4.3.3-1: ATTACH ACCEPT for Cell 1 (preamble)

| Derivation path: 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN2 |  |  |

Table 8.6.7.4.3.3-2: SystemInformationBlockType2 for Cell 12 (preamble and all steps, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| rach-ConfigCommon SEQUENCE { |  |  |  |
| ra-SupervisionInfo SEQUENCE { |  |  |  |
| preambleTransMax | n50 |  |  |
| } |  |  |  |
| } |  |  |  |
| uplinkPowerControlCommon-v1020 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| ssac-BarringForMMTEL-Voice-r9 | Not present |  |  |
| ssac-BarringForMMTEL-Video-r9 | Not present |  |  |
| ac-BarringForCSFB-r10 | Not present |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-3: RRCConnectionReconfiguration (step 1, Table 8.6.7.4.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.6.7.4.3.3-4: MeasConfig (Table 8.6.7.4.3.3-3)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f2 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3 |  |  |
| } |  |  |  |
| measIdToAddModListSEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { |  |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f2 |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |
| } |  |  |  |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] ::= SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] ::= SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-5: MeasurementReport (step 4, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultPCell SEQUENCE { |  | Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId | PhysicalCellIdentity of Cell 12 | Cell 12 |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| locationInfo-r10 | Not present |  |  |
| measResultServFreqList-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-6: RRCConnectionReconfiguration (step 5, Table 8.6.7.4.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition HO

Table 8.6.7.4.3.3-7: MobilityControlInfo (Table 8.6.7.4.3.3-6)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 12 |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 12. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 12. |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| … |  |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 12 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-8: RRCConnectionReestablishmentRequest (step 6, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity SEQUENCE { |  |  |  |
| c-RNTI | the C-RNTI used in the Cell 1 |  |  |
| physCellId | PhysicalCellIdentity of Cell 1 |  |  |
| shortMAC-I | The same value as the 16 least significant bits of the XMAC-I value calculated by SS |  |  |
| } |  |  |  |
| reestablishmentCause | handoverFailure |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-9: RRCConnectionReestablishmentComplete (step 8, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r8 = SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r9 | true |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-10: RRCConnectionReconfiguration (step 9, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-HO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-11: MEASUREMENT CONTROL (step 21, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, clause 4.7B.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| - Inter-RAT measurement quantity |  |  |  |
| - Measurement quantity for UTRAN quality estimate |  |  |  |
| - Filter coefficient | 0 |  |  |
| - CHOICE mode | FDD |  |  |
| - Measurement quantity | CPICH RSCP |  |  |
| - CHOICE system | E-UTRA |  |  |
| - Measurement quantity | RSRP |  |  |
| - Filter coefficient | 0 |  |  |
| - Inter-RAT reporting quantity |  |  |  |
| - UTRAN estimated quality | FALSE |  |  |
| - CHOICE system | E-UTRA |  |  |
| - Reporting quantity | both |  |  |
| - Reporting cell status | Not present |  |  |
| - CHOICE report criteria | Inter-RAT measurement reporting criteria |  |  |
| - Parameters required for each event | 1 entry |  |  |
| - Inter-RAT event identity | 3a |  |  |
| - Threshold own system | -66 |  |  |
| - W | 0 |  |  |
| - Threshold other system | -80 |  |  |
| - Hysteresis | 0 |  |  |
| - Time to trigger | 10 ms |  |  |
| - Reporting cell status |  |  |  |
| - CHOICE reported cell | Report cells within active set or within virtual active set or of the other RAT |  |  |
| - Maximum number of reported cells | 2 |  |  |

Table 8.6.7.4.3.3-12: HANDOVER FROM UTRAN COMMAND (step 24, Table 8.6.7.4.3.2-2)

Derivation Path: 36.508, Table 4.7B.1-2

Table 8.6.7.4.3.3-13: RRCConnectionReconfiguration (Table 8.6.7.4.3.3-12)

Derivation Path: 36.508, Table 4.6.1-8, condition HO-TO-EUTRA(1,0)

Table 8.6.7.4.3.3-14: MobilityControlInfo (Table 8.6.7.4.3.3-13)

| Derivation Path: 36.508, Table 4.6.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MobilityControlInfo ::= SEQUENCE { |  |  |  |
| targetPhysCellId | PhysicalCellIdentity of Cell 12. |  |  |
| carrierFreq SEQUENCE { |  |  |  |
| dl-CarrierFreq | Same downlink EARFCN as used for Cell 12. |  |  |
| ul-CarrierFreq | Not present |  |  |
|  | Same uplink EARFCN as used for Cell 12. |  | Band 24 High range |
| } |  |  |  |
| carrierFreq | Not present |  | Band > 64 |
| carrierBandwidth SEQUENCE { |  |  |  |
| dl-Bandwidth | Same downlink system bandwidth as used for Cell 12 |  |  |
| ul-Bandwidth | Same uplink system bandwidth as used for Cell 12 |  | FDD |
| ul-Bandwidth | Not present |  | TDD |
| } |  |  |  |
| additionalSpectrumEmission | 1 |  |  |
| carrierFreq-v9e0 SEQUENCE { |  |  | Band > 64 |
| dl-CarrierFreq-v9e0 | Same downlink EARFCN as used for Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

Table 8.6.7.4.3.3-15: SecurityConfigHO (Table 8.6.7.4.3.3-14)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| interRAT SEQUENCE { |  |  |  |
| securityAlgorithmConfig SEQUENCE { |  |  |  |
| cipheringAlgorithm | Set according to PIXIT parameter for default ciphering protection algorithm |  |  |
| integrityProtAlgorithm | Set according to PIXIT parameter for default integrity algorithm |  |  |
| } |  |  |  |
| nas-SecurityParamToEUTRA | Octets 1 to 4 are arbitrarily selected. Bits 1 to 3 of octet 5 are set according to PIXIT parameter for default integrity protection algorithm. Bits 5 to 7 of octet 5 are set according to PIXIT parameter for default ciphering algorithm. Bits 1 to 3 of octet 6 are arbitrarily selected between '000'B and '110'B, different from the valid NAS key set identifier of the UE if such a value exists. Bit 4 of octet 6 is set to 1. | Octets 1 to 4 include the NonceMME value. Bits 1 to 3 of octet 5 include the Type of integrity protection algorithm Bits 5 to 7 of octet 5 include the Type of ciphering algorithm. Bits 1 to 4 of octet 6 include the NAS key set identifier. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-16: RRCConnectionReconfigurationComplete (step 25, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present or any allowed value |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| rlf-InfoAvailable-r10 | true |  |  |
| logMeasAvailable-r10 | Not present |  |  |
| nonCriticalExtension | Not present or any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-16A: Message TRACKING AREA UPDATE ACCEPT (step 26, 8.6.7.4.3.2-2)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.6.7.4.3.3-17: UEInformationRequest (step 27, Table 8.6.7.4.3.2-2)

Derivation Path: 36.508, Table 4.6.1-23A, condition RLF report

Table 8.6.7.4.3.3-18: UEInformationResponse (step 28, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::=SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |
| rlf-Report-r9 SEQUENCE { |  |  |  |
| measResultLastServCell-r9 SEQUENCE { |  |  |  |
| rsrpResult-r9 | (0..97) | Cell 1 |  |
| rsrqResult-r9 | Not present or (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells-r9 SEQUENCE { |  |  |  |
| measResultListEUTRA-r9 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| carrierFreq-r9[1] | Same downlink EARFCN as used for Cell 12 |  |  |
| carrierFreq-r9[1] | maxEARFCN |  | Band > 64 |
| measResultList-r9[1] SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId[1] | PhysicalCellIdentity of Cell 12 |  |  |
| cgi-Info[1] | Not present |  |  |
| measResult[1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultListUTRA-r9 | Not present |  |  |
| measResultListGERAN-r9 | Not present |  |  |
| measResultsCDMA2000-r9 | Not present |  |  |
| } |  |  |  |
| locationInfo-r10 | Not present or any allowed value |  |  |
| failedPCellId-r10 CHOICE { | cellGlobalId-r10 or pci-arfcn-r10 | If the UE has the global cell identity depending on UE implementation, the UE sets the global cell identity; otherwise the UE sets the physical cell identity and the carrier frequency. |  |
| cellGlobalId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| } |  |  |  |
| pci-arfcn-r10 SEQUENCE { |  |  |  |
| physCellId-r10 | PhysicalCellIdentity of Cell 12 |  |  |
| carrierFreq-r10 | Same downlink EARFCN as used for Cell 12 |  |  |
| carrierFreq-r10 | maxEARFCN |  | Band > 64 |
| } |  |  |  |
| } |  |  |  |
| reestablishmentCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 12 |  |  |
| } |  |  |  |
| timeConnFailure-r10 | Any allowed value | Time from UE receive HO command to the failure |  |
| connectionFailureType-r10 | hof |  |  |
| previousPCellId-r10 SEQUENCE { |  |  |  |
| plmn-Identity | plmn-Identity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| cellIdentity | cellIdentity within SystemInformationBlockType1 broadcasted in Cell 1 |  |  |
| } |  |  |  |
| failedPCellId-v1090 SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v1090 | Same downlink EARFCN as used for Cell 2 |  |  |
| } |  |  |  |
| basicFields-r11 SEQUENCE { |  |  |  |
| c-RNTI-r11 | C-RNTI used in Cell 1 |  |  |
| rlf-Cause-r11 | Any allowed value |  |  |
| timeSinceFailure-r11 | Any value | Time elapsed from connection failure |  |
| } |  |  |  |
| previousUTRA-CellId-r11 | Not present |  |  |
| selectedUTRA-CellId-r11 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  |  |
| rlf-Report-v9e0 SEQUENCE { |  |  |  |
| measResultListEUTRA-v9e0 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { |  |  | Band > 64 |
| carrierFreq-v9e0[1] | Same EARFCN as Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.7.4.3.3-19: TRACKING AREA UPDATE ACCEPT for Cell 12 (step 10B, Table 8.6.7.4.3.2-2))

| Derivation path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Equivalent PLMNs | PLMN1 |  |  |

Table 8.6.7.4.3.3-20: SystemInformationBlockType6 for Cell12 (preamble, and all steps, Table 8.6.7.4.3.2-2)

| Derivation Path: 36.508, Table 4.4.3.3-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType6 ::= SEQUENCE { |  |  |  |
| carrierFreqListUTRA-FDD SEQUENCE (SIZE (1..maxUTRA-FDD-Carrier)) OF SEQUENCE { |  |  | UTRA-FDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 9 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| } |  |  |  |
| carrierFreqListUTRA-TDD SEQUENCE (SIZE (1..maxUTRA-TDD-Carrier)) OF SEQUENCE { |  |  | UTRA-TDD |
| carrierFreq[n] | Same downlink UARFCN as used for Cell 9 |  |  |
| cellReselectionPriority[n] | 5 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UTRA-FDD | UTRA FDD cell environment |
| UTRA-TDD | UTRA TDD cell environment |

<End of modified section>