3GPP TSG-RAN5 Meeting #107	R5-253121

# St Julian’s, Malta, 19th - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1599 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of in 486 Busy Here, 600 Busy Everywhere, 603 Decline and 200 OK for eCall |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Media-range and Info-package-type are case sensitive (RFC 8147, RFC 6086). - The purpose parameter in the Call-Info of responses sent to the UE shall be "EmergencyCallData.Control" (RFC 8147). - The MIME type for control of MSD is “application/EmergencyCallData.Control+xml” (RFC 8147). - The XML element name for eCall control is “EmergencyCallData.Control” (case sensitive). - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Media-range and Info-package-type corrected in 486 Busy Here, 600 Busy Everywhere, 603 Decline and 200 OK. - Purpose parameter in the Call-Info corrected in 486 Busy Here, 600 Busy Everywhere and 603 Decline. - Content-Type corrected in the MIME bodies of 486 Busy Here, 600 Busy Everywhere, 603 Decline and 200 OK. - Element name corrected in MIME bodies of 200 OK. - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.21, A.2.22, A.2.23, A.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 1 revision of R5-252218. |  |  |  |  |  |  |  |  |

<Start of modification>

## A.2.21	486 Busy Here

| Header/param |  | Cond |  | Value/remark |  | Rel |  | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Status-Line |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| SIP-Version |  |  |  | SIP/2.0 |  |  |  |  |  |
| Status-Code |  |  |  | 486 |  |  |  |  |  |
| Reason-Phrase |  |  |  | Busy Here |  |  |  |  |  |
| Via |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| via-parm |  |  |  | same value as received in request |  |  |  |  |  |
| From |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | same value as received in request |  |  |  |  |  |
| tag |  |  |  | same value as received in request |  |  |  |  |  |
| To |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | same value as received in request |  |  |  |  |  |
| tag |  |  |  | any arbitrary tag value added |  |  |  |  |  |
| Call-ID |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| callid |  |  |  | same value as received in request |  |  |  |  |  |
| CSeq |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | same value as received in request |  |  |  |  |  |
| Recv-Info |  | A1 |  |  |  |  |  |  |  |
| Info-package-type |  |  |  | EmergencyCallData.eCall.MSD |  |  |  | RFC 8147 [149] |  |
| Call-Info |  | A1 |  |  |  |  |  |  |  |
| info |  |  |  |  |  |  |  |  |  |
| absoluteURI |  |  |  | cid:test-486@3gpp.org |  |  |  | RFC 8147 [149] |  |
| info-param |  |  |  | purpose=EmergencyCallData.Control |  |  |  | RFC 8147 [149] |  |
| Accept |  | A1 |  |  |  |  |  |  |  |
| media-range |  |  |  | application/sdp, application/pidf+xml, application/EmergencyCallData.Control+xml, application/EmergencyCallData.eCall.MSD |  |  |  | RFC 8147 [149] |  |
| Content-Type |  | A1 |  |  |  |  |  |  |  |
| media-type |  |  |  | multipart/mixed;boundary=boundary1 |  |  |  | RFC 8147 [149] |  |
| Content-Length |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | length of message body |  |  |  |  |  |
| Message-body |  | A1 |  | --boundary1 Content-Type: application/EmergencyCallData.Control+xml Content-ID: <test-486@3gpp.org> Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <EmergencyCallData.Control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack received="true" ref="addr-spec of the Content-ID of MIME body part containing the MSD sent by the UE in INVITE”/> </EmergencyCallData.Control> --boundary1 |  |  |  | RFC 8147 [149] |  |

| Condition | Explanation |
| --- | --- |
| A1 | Response sent by SS for INVITE for eCall over IMS |

<End of modification>

<Start of modification>

## A.2.22	600 Busy Everywhere

| Header/param |  | Cond |  | Value/remark |  | Rel |  | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Status-Line |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| SIP-Version |  |  |  | SIP/2.0 |  |  |  |  |  |
| Status-Code |  |  |  | 600 |  |  |  |  |  |
| Reason-Phrase |  |  |  | Busy Everywhere |  |  |  |  |  |
| Via |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| via-parm |  |  |  | same value as received in request |  |  |  |  |  |
| From |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | same value as received in request |  |  |  |  |  |
| tag |  |  |  | same value as received in request |  |  |  |  |  |
| To |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | same value as received in request |  |  |  |  |  |
| tag |  |  |  | any arbitrary tag value added |  |  |  |  |  |
| Call-ID |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| callid |  |  |  | same value as received in request |  |  |  |  |  |
| CSeq |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | same value as received in request |  |  |  |  |  |
| Recv-Info |  | A1 |  |  |  |  |  |  |  |
| Info-package-type |  |  |  | EmergencyCallData.eCall.MSD |  | Rel-14 |  | RFC 8147 [149] |  |
| Call-Info |  | A1 |  |  |  |  |  |  |  |
| info |  |  |  |  |  |  |  |  |  |
| absoluteURI |  |  |  | cid:test-600@3gpp.org |  |  |  |  |  |
| info-param |  |  |  | purpose=EmergencyCallData.Control |  | Rel-14 |  | RFC 8147 [149] |  |
| Accept |  | A1 |  |  |  |  |  |  |  |
| media-range |  |  |  | application/sdp, application/pidf+xml, application/EmergencyCallData.Control+xml, application/EmergencyCallData.eCall.MSD |  | Rel-14 |  | RFC 8147 [149] |  |
| Content-Type |  | A1 |  |  |  |  |  |  |  |
| media-type |  |  |  | multipart/mixed;boundary=boundary1 |  | Rel-14 |  | RFC 8147 [149] |  |
| Content-Length |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | length of message body |  |  |  |  |  |
| Message-body |  | A1 |  | --boundary1 Content-Type: application/EmergencyCallData.Control+xml Content-ID: <test-600@3gpp.org> Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <EmergencyCallData.Control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack received="true" ref="addr-spec of the Content-ID of MIME body part containing the MSD sent by the UE in INVITE”/> </EmergencyCallData.Control> --boundary1 |  | Rel-14 |  | RFC 8147 [149] |  |

| Condition | Explanation |
| --- | --- |
| A1 | Response sent by SS for INVITE for eCall over IMS |

<End of modification>

<Start of modification>

## A.2.23	603 Decline

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Status-Line |  |  |  | RFC 3261 [15] |
| SIP-Version |  | SIP/2.0 |  |  |
| Status-Code |  | 603 |  |  |
| Reason-Phrase |  | Decline |  |  |
| Via |  |  |  | RFC 3261 [15] |
| via-parm |  | same value as received in request |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in request |  |  |
| tag |  | same value as received in request |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | same value as received in request |  |  |
| tag |  | any arbitrary tag value added |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as received in request |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | same value as received in request |  |  |
| Recv-Info |  |  |  | RFC 6086 [139] |
| Info-package-type | A1 | EmergencyCallData.eCall.MSD | Rel-14 | RFC 8147 [149] |
| Call-Info |  |  |  |  |
| info |  |  |  |  |
| absoluteURI |  | cid:test-603@3gpp.org | Rel-14 |  |
| info-param |  | purpose=EmergencyCallData.Control | Rel-14 | RFC 8147 [149] |
| Accept |  |  |  |  |
| media-range |  | application/sdp, application/pidf+xml, application/EmergencyCallData.Control+xml, application/EmergencyCallData.eCall.MSD | Rel-14 | RFC 8147 [149] |
| Content-Type |  |  |  |  |
| media-type | A1 | multipart/mixed;boundary=boundary1 |  | RFC 8147 [149] |
| Content-Length |  |  |  | RFC 3261 [15] |
| value |  | length of message body |  |  |
| Message-body | A1 | --boundary1 Content-Type: application/EmergencyCallData.Control+xml Content-ID: <test-603@3gpp.org> Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <EmergencyCallData.Control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack received="true" ref="addr-spec of the Content-ID of MIME body part containing the MSD sent by the UE in INVITE”/> </EmergencyCallData.Control> --boundary1 | Rel-14 | RFC 8147 [149] |

| Condition | Explanation |
| --- | --- |
| A1 | Response sent by SS for INVITE for eCall over IMS |

<End of modification>

<Start of modification>

## A.3.1	200 OK for other requests than REGISTER or SUBSCRIBE

| Header/param |  | Cond |  | Value/remark |  | Rel |  | Reference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Status-Line |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| SIP-Version |  |  |  | SIP/2.0 |  |  |  |  |  |
| Status-Code |  |  |  | 200 |  |  |  |  |  |
| Reason-Phrase |  |  |  | OK |  |  |  |  |  |
| Via |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| via-parm |  |  |  | same value as received in request |  |  |  |  |  |
| Record-Route |  |  |  | order of the parameters in this header must be like in the respective rows |  |  |  | RFC 3261 [15] |  |
| rec-route |  | A1 AND NOT A23 |  | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: protected server port of SS;lr> |  |  |  |  |  |
|  |  | A3 AND NOT A23 |  | <sip:pcscf.other.com;lr>, <sip:scscf.other.com;lr>, <sip:orig@scscf.3gpp.org;lr>, <sip:SS P-CSCF address: unprotected server port of SS (optional);lr> |  |  |  |  |  |
|  |  | A20 AND NOT A23 |  | same value as received in the request (initial INVITE from the SS) Note: for requests other than INVITE it is not regulated if and what the UE writes into this header in a response. |  |  |  |  |  |
|  |  | A6 |  | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:protected server port of SS;lr> |  |  |  |  |  |
|  |  | A7 |  | <sip:orig@ecscf.other.com;lr>, <sip:SS P-CSCF address:unprotected server port of SS;lr> |  |  |  |  |  |
| From |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | same value as received in request |  |  |  |  |  |
| tag |  |  |  | same value as received in request |  |  |  |  |  |
| To |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| addr-spec |  |  |  | same value as received in request |  |  |  |  |  |
| tag |  |  |  | same value as received in request or any value added if missing from request |  |  |  |  |  |
| P-Asserted-Identity |  | A6,A7 |  |  |  |  |  | RFC 3325 [89] |  |
| addr-spec |  |  |  | A tel URI that can be recognized as valid emergency numbers if dialled by the user are specified in 3GPP TS 22.101 [39]. The emergency numbers 112 and 911 are stored on the ME, in accordance with 3GPP TS 22.101 [39] |  |  |  |  |  |
| uri-parameter |  |  |  | lr |  |  |  |  |  |
| Contact |  | A1,A2, A3,A4, A6,A7 |  |  |  |  |  | RFC 3261 [15] RFC 5627 [61] |  |
| addr-spec |  | A1,A3, A6,A7 |  | MO call: px_IMS_CalleeContactUri MT call: Contact address provided by SS in dialog creating INVITE |  |  |  |  |  |
|  |  | A2,A4 |  | MO call: Contact address provided by UE in dialog creating INVITE MT call: Contact address provided by UE in 1xx response establishing the dialog according to RFC 3261 [15] clause 12. (NOTE: The 'transport' url parameter may change, url parameters are not checked) If there has been no 1xx response establishing the dialog (i.e. the 200 OK establishes the dialog) then the Contact address is as specified for the 183 Session Progress in A.2.3. |  |  |  |  |  |
| feature-param |  | A10 |  | audio |  |  |  |  |  |
|  |  | A18 AND (A19 OR A20) |  | video |  |  |  | IR.94 [134] |  |
|  |  | A11 AND A15 AND (A16 OR A17 OR A22) |  | audio |  |  |  |  |  |
| Call-ID |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| callid |  |  |  | same value as received in request |  |  |  |  |  |
| Call-Info |  | A12 |  |  |  |  |  | RFC 8147 [149] |  |
| info |  |  |  |  |  |  |  |  |  |
| absoluteURI |  |  |  | cid:psap@3gpp.org |  | Rel-14 |  |  |  |
| info-param |  |  |  | purpose=EmergencyCallData.Control |  |  |  |  |  |
| CSeq |  |  |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | same value as received in request |  |  |  |  |  |
| P-Access-Network-Info |  | A8 |  |  |  |  |  | RFC 7315 [132] RFC 7913 [154] |  |
| access-net-spec |  | A21 |  | access network information and, if applicable, the cell ID |  |  |  |  |  |
|  |  | A22 |  | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID |  |  |  |  |  |
| Accept |  | A12,A24 |  |  |  |  |  | RFC 8147 [149] |  |
| media-range |  | A12 |  | application/sdp, application/pidf+xml, application/EmergencyCallData.Control+xml, application/EmergencyCallData.eCall.MSD |  | Rel-14 |  |  |  |
|  |  | A24 |  | application/EmergencyCallData.Control+xml |  | Rel-19 |  | TS 24.229 [10] |  |
| Recv-Info |  | A12,A24 |  |  |  |  |  | RFC 8147 [149] |  |
| Info-package-type |  |  |  | EmergencyCallData.eCall.MSD |  | Rel-14 |  |  |  |
| Content-Type |  | A12 |  |  |  |  |  | RFC 8147 [149] |  |
| media-type |  |  |  | multipart/mixed; boundary=boundary1 |  | Rel-14 |  |  |  |
| Content-Length |  | A10 |  |  |  |  |  | RFC 3261 [15] |  |
| value |  |  |  | length of message body |  |  |  |  |  |
| Message-body |  | A12 |  |  |  |  |  |  |  |
|  |  | A13 |  | --boundary1 Content-Type: application/EmergencyCallData.Control+xml Content-ID: <psap@3gpp.org> Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <EmergencyCallData.Control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack received="true" ref=" addr-spec of the Content-ID of MIME body part containing the MSD sent by the UE in INVITE"/> </EmergencyCallData.Control> --boundary1 |  |  |  | RFC 8147 [149] |  |
|  |  | A14 |  | --boundary1 Content-Type: application/EmergencyCallData.Control+xml Content-ID: <psap@3gpp.org> Content-Disposition: by-reference <?xml version="1.0" encoding="UTF-8"?> <EmergencyCallData.Control xmlns="urn:ietf:params:xml:ns:EmergencyCallData:control"> <ack received="false" ref=" addr-spec of the Content-ID of MIME body part containing the MSD sent by the UE in INVITE "/> </EmergencyCallData.Control> --boundary1 |  |  |  | RFC 8147 [149] |  |

| Condition | Explanation |
| --- | --- |
| A1 | Response sent by SS for INVITE/UPDATE (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A2 | Response sent by UE for INVITE/UPDATE (IMS security, A.6a/2 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A3 | Response sent by SS for INVITE/UPDATE (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A4 | Response sent by UE for INVITE/UPDATE (GIBA, A.6a/1 3GPP TS 34.229-2 [5]) (NOTE 1) |
| A5 | Void |
| A6 | Response sent by SS for INVITE for emergency call with emergency registration or for INVITE/UPDATE for non-UE detectable emergency call |
| A7 | Response sent by SS for INVITE for emergency call without emergency registration |
| A8 | Any response sent by the UE within a dialog, except for CANCEL requests (NOTE 2) |
| A9 | Void |
| A10 | Response sent by SS |
| A11 | Response sent by UE |
| A12 | Response sent by SS for INVITE for eCall over IMS session with either ACK or NACK |
| A13 | Response sent by SS for INVITE for eCall over IMS session with ACK element = TRUE |
| A14 | Response sent by SS for INVITE for eCall over IMS session with ACK element = FALSE |
| A15 | UE supports audio media feature tag (A.12/56 3GPP TS 34.229-2 [5]) |
| A16 | UE uses E-UTRAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.301 [150], clauses 8.2.1 and 9.9.3.12A |
| A17 | UE uses UTRAN/GERAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.008 [12], clauses 9.4.2 and 10.5.5.23 |
| A18 | UE supports video feature tag (A.12/32 3GPP TS 34.229-2 [5]) |
| A19 | Response sent by SS for INVITE |
| A20 | Response sent by UE for INVITE |
| A21 | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |
| A22 | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |
| A23 | Response sent for re-INVITE within an established dialog |
| A24 | Response sent by UE subjected to national regulations in CEN EN 17184:2024 (A.4.5/67 3GPP TS 34.229-2 [5] or A.4.5/68 3GPP TS 34.229-2 [5]) for INVITE for PSAP Callback |

NOTE 1:	When the UE supports IMS security then for establishment of a normal call condition A1 or A2 is applied unless condition A3 or A4 is specified explicitly by the test case. For emergency call condition A6 or A7 shall be specified by the test case.

NOTE 2:	As the 200 OK for SIP MESSAGE is the only case where a 200 OK for other requests than REGISTER or SUBSCRIBE is not sent within a dialog, in fact condition A8 is always applied except for the 200 OK responses for CANCEL and SIP MESSAGE.

<End of modification>