3GPP TSG-RAN5 Meeting #107	R5-253177

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4965 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new LTM test case 7.1.1.1.30 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New LTM test case 7.1.1.1.30 need to be added according to work plan of the WI NR_Mob_enh2-UEConTest. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new LTM test case 7.1.1.1.30. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_Mob_enh2-UEConTest WP is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.30 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR .38.523-2.. CR .0583.. |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-252210r2 |  |  |  |  |  |  |  |  |

7.1.1.1.30	Random access procedure / Successful / LTM cell switch Command MAC CE / T311 running

7.1.1.1.30.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state and attemptLTM-Switch is configured and T311 timer is running and the cell selection procedure was triggered by a failed LTM switch procedure of the MCG triggered by lower layers }

ensure that {

when { LTM candidate cell associated with the MCG becomes available }

then { UE performs LTM cell switch procedure }

}

7.1.1.1.30.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 38.331, clause 5.3.7.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.331, clause 5.3.7.3]

Upon selecting a suitable NR cell, the UE shall:

1>	ensure having valid and up to date essential system information as specified in clause 5.2.2.2;

1>	stop timer T311;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1>	stop the relay (re)selection procedure, if ongoing;

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG, except for an LTM cell switch procedure following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or mobility from NR failure, and

1>	if attemptCondReconfig is configured; and

1>	if the selected cell is not configured with CondEventT1, or the selected cell is configured with CondEventT1 and leaving condition has not been fulfilled; and

1>	if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig and the condExecutionCondPSCell is not configured for the corresponding condReconfigId in the MCG VarConditionalReconfig:

2>	if the UE supports RLF-Report for conditional handover, set the choCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell;

2>	apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3;

NOTE 1:	It is left to network implementation to how to avoid keystream reuse in case of CHO based recovery after a failed handover without key change.

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and

1>	if attemptLTM-Switch is configured; and

1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG:

2>	perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

NOTE 2:	In case both attemptCondReconfig and attemptLTM-Switch are configured, it is left to the UE implementation which procedure to execute.

1>	else:

2>	if UE is configured with attemptCondReconfig; or

2>	if UE is configured with attemptLTM-Switch:

3>	reset MAC;

3>	release spCellConfig, if configured;

3>	release the MCG SCell(s), if configured;

3>	release delayBudgetReportingConfig, if configured and stop timer T342, if running;

3>	release overheatingAssistanceConfig , if configured and stop timer T345, if running;

3>	if MR-DC is configured:

4>	perform MR-DC release, as specified in clause 5.3.5.10;

3>	release idc-AssistanceConfig, if configured;

3>	release btNameList, if configured;

3>	release wlanNameList, if configured;

3>	release sensorNameList, if configured;

3>	release drx-PreferenceConfig for the MCG, if configured and stop timer T346a associated with the MCG, if running;

3>	release maxBW-PreferenceConfig for the MCG, if configured and stop timer T346b associated with the MCG, if running;

3>	release maxCC-PreferenceConfig for the MCG, if configured and stop timer T346c associated with the MCG, if running;

3>	release maxMIMO-LayerPreferenceConfig for the MCG, if configured and stop timer T346d associated with the MCG, if running;

3>	release minSchedulingOffsetPreferenceConfig for the MCG, if configured and stop timer T346e associated with the MCG, if running;

3>	release rlm-RelaxationReportingConfig for the MCG, if configured and stop timer T346j associated with the MCG, if running;

3>	release bfd-RelaxationReportingConfig for the MCG, if configured and stop timer T346k associated with the MCG, if running;

3>	release releasePreferenceConfig, if configured and stop timer T346f, if running;

3>	release onDemandSIB-Request if configured, and stop timer T350, if running;

3>	release referenceTimePreferenceReporting, if configured;

3>	release sl-AssistanceConfigNR, if configured;

3>	release obtainCommonLocation, if configured;

3>	release scg-DeactivationPreferenceConfig, if configured, and stop timer T346i, if running;

3>	release musim-GapAssistanceConfig, if configured and stop timer T346h, if running;

3>	release musim-GapPriorityAssistanceConfig, if configured;

3>	release musim-LeaveAssistanceConfig, if configured;

3>	release musim-CapabilityRestrictionConfig, if configured and stop timer T346n, if running;

3>	release propDelayDiffReportConfig, if configured;

3>	release ul-GapFR2-PreferenceConfig, if configured;

3>	release rrm-MeasRelaxationReportingConfig, if configured;

3>	release maxBW-PreferenceConfigFR2-2, if configured;

3>	release maxMIMO-LayerPreferenceConfigFR2-2, if configured;

3>	release minSchedulingOffsetPreferenceConfigExt, if configured;

3>	release aerial-FlightPathAvailabilityConfig, if configured;

3>	release ul-TrafficInfoReportingConfig, if configured, and stop all instances of timer T346l, if running;

3>	suspend all RBs, and BH RLC channels for the IAB-MT, except SRB0 and broadcast MRBs;

2>	remove all the entries within the MCG VarConditionalReconfig, if any;

2>	perform the LTM configuration release procedure for the MCG and the SCG as specified in clause 5.3.5.18.7;

2>	for each measId, if the associated reportConfig has a reportType set to condTriggerConfig:

3>	for the associated reportConfigId:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	remove the servingSecurityCellSetId within the VarServingSecurityCellSetID, if any;

2>	release the PC5 RLC entity for SL-RLC0, if any;

2>	start timer T301;

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

2>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4;

NOTE 2a:	This procedure applies also if the UE returns to the source PCell.

NOTE 3:	A L2 U2N Relay UE may re-establish (e.g. via release and establish) the SL-RLC0 and SL-RLC1 of the connected L2 U2N Remote UE(s).

Upon selecting an inter-RAT cell, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

7.1.1.1.30.3	Test description

7.1.1.1.30.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except that System information combination NR-2.

7.1.1.1.30.3.2	Test procedure sequence

Table 7.1.1.1.30.3.2-1 and 7.1.1.1.30.3.2-2 illustrates the downlink power levels to be applied for NR Cell 1, NR Cell 2 and NR Cell 4 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1" and "T2", are applied at the point indicated in the Main behaviour description in Table 7.1.1.1.30.3.2-3.

Table 7.1.1.1.30.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -91 | -91 | Power levels are such that entry condition for event A3 is not satisfied for neighbour NR cells |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -79 | -91 | Power levels are such that entry condition for event A3 is satisfied for NR cell 2 |
| T2 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -91 | -79 | Power levels are such that entry condition for event A3 is satisfied for NR cell 4 |

Table 7.1.1.1.30.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 4 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -100 | -100 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -82 | -100 | Power levels are such that entry condition for event A3 is satisfied for NR cell 2 |
| T2 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -100 | -82 | Power levels are such that entry condition for event A3 is satisfied for NR cell 4 |

Table 7.1.1.1.30.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for inter-frequency event A3 on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 3 | SS re-adjusts the cell-specific reference signal level according to row "T1" in table 7.1.1.1.30.3.2-1/2. | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message to report event A3 with the measured RSRP value for NR Cell 2 in NR Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCReconfiguration message with attemptLTM-Switch-r18 set to true, and LTM-Candidate-r18 set for NR Cell 2 and NR Cell 4 on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 6 | The UE transmits RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 7 | The SS transmits the candidate cell TCI activation MAC CE with Candidate Cell ID set to 0 for NR Cell 2 on NR Cell 1. | <-- | MAC PDU (Candidate Cell TCI Activation/Deactivation MAC CE) | - | - |
| 8 | The SS transmits LTM cell switch command MAC CE with Timing Advance Command value set to FFF and Target Configuration ID value set to 0 on NR Cell 1 to the UE. | <-- | MAC PDU (LTM Cell Switch Command MAC CE) | - | - |
| - | EXCEPTION: The steps 9 and 10 below are repeated for the duration of T304. | - | - | - | - |
| 9 | The UE transmits a preamble on PRACH using the preamble indicated by LTM Cell Switch Command MAC CE in NR Cell 2. | --> | PRACH Preamble | - | - |
| 10 | The SS does not respond. | - | - | - | - |
| 11 | SS re-adjusts the cell-specific reference signal level according to row "T2" in table 7.1.1.1.30.3.2-1/2 right before T304 expire. | - | - | - | - |
| 12 | The UE transmits a MeasurementReport message to report event A3 with the measured RSRP value for NR Cell 4 in NR Cell 1. | --> | MeasurementReport | - | - |
| 13 | The SS transmits the candidate cell TCI activation MAC CE with Candidate Cell ID set to 1 for NR Cell 4 on NR Cell 1. | <-- | MAC PDU (Candidate Cell TCI Activation/Deactivation MAC CE) | - | - |
| 14 | The SS transmits LTM cell switch command MAC CE with Timing Advance Command value set to FFF and Target Configuration ID value set to 1 on NR Cell 1 to the UE. | <-- | MAC PDU (LTM Cell Switch Command MAC CE) | - | - |
| 15 | Check: Does the UE transmits a preamble on PRACH using the preamble indicated by LTM Cell Switch Command MAC CE in NR Cell 4? | --> | PRACH Preamble | 1 | P |

7.1.1.1.30.3.3	Specific message contents

Table 7.1.1.1.30.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.1.30.3.2-3)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition NR_MEAS

Table 7.1.1.1.30.3.3-2: MeasConfig (step 1, Table 7.1.1.1.30.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 7.1.1.1.30.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3 | Table 7.1.1.1.30.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.30.3.3-3: MeasObjectNR (Table 7.1.1.1.30.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 7.1.1.1.30.3.3-4: ReportConfigNR-EventA3 (Table 7.1.1.1.30.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB(2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | infinity |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | true |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.30.3.3-5: MeasurementReport (step 4, step 12, Table 7.1.1.1.30.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 1 |  | Step 4, Step 12 |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 1 |  | Step 4, Step 12 |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not present |  |  |
|  | Not checked |  | pc_ss_SINR_Meas |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Physical layer cell identity of NR Cell 2 |  | Step 4 |
|  | Physical layer cell identity of NR Cell 4 |  | Step 12 |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.30.3.3-6: RRCReconfiguration (step 5, Table 7.1.1.1.30.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 with condition LTM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ltm-Config-r18 CHOICE { |  |  |  |
| setup | LTM-Config-r18 | Table 7.1.1.1.30.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.1.30.3.3-7: LTM-Config-r18(Table 7.1.1.1.30.3.3-6)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Config-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 { | 2 entries |  |  |
| LTM-Candidate-r18[1] | LTM-Candidate-r18 | Table 7.1.1.1.30.3.3-8 |  |
| LTM-Candidate-r18[2] | LTM-Candidate-r18 | Table 7.1.1.1.30.3.3-9 |  |
| } |  |  |  |
| ltm-CSI-ResourceConfig-r18 | Not present |  |  |
| attemptLTM-Switch-r18 | True |  |  |
| } |  |  |  |

Table 7.1.1.1.30.3.3-8: LTM-Candidate-r18(Table 7.1.1.1.30.3.3-7)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Candidate-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateId-r18 | 1 |  |  |
| ltm-CandidatePCI-r18 | Physical layer cell identity of NR Cell 2 |  | Step 5 |
| ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) { |  |  |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig specified in TS 38.508-1 [4] Table 4.6.3-19 with condition PCell_change |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ltm-TCI-Info-r18 | LTM-TCI-Info specified in TS 38.508-1 [4] Table 4.6.3-67I with condition Joint |  |  |
| } |  |  |  |

Table 7.1.1.1.30.3.3-9: LTM-Candidate-r18(Table 7.1.1.1.30.3.3-7)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Candidate-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateId-r18 | 2 |  |  |
| ltm-CandidatePCI-r18 | Physical layer cell identity of NR Cell 4 |  | Step 5 |
| ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) { |  |  |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig specified in TS 38.508-1 [4] Table 4.6.3-19 with condition PCell_change |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ltm-TCI-Info-r18 | LTM-TCI-Info specified in TS 38.508-1 [4] Table 4.6.3-67I with condition Joint |  |  |
| } |  |  |  |