3GPP TSG-RAN5 Meeting #107	R5-253421

# Malta, Malta, 19th May 2025 - 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3396 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to REFSENS power level test for XR UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Need to update missing details of the REFSENS test for XR UEs |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to the test case contents of the REFSENS test for XR UEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing details for the REFSENS test for XR UEs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1 corrects the references to the minimum conformance requirements and test requirements tables in the baseline REFSENS test for 2Rx and 4Rx UEs and also clarifies the applicability. Annex F is also updated. R5-252998r1 -> R5-253421 |  |  |  |  |  |  |  |  |

### <Unchanged sections skipped>

### 7.3.2_1	Reference sensitivity power level for XR

7.3.2_1.1	Test purpose

The test purpose is to verify the ability of the XR UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

7.3.2_1.2	Test applicability

This test case applies to all types of NR UE release 18 and forward XR UEs supporting 4Rx and XR UEs supporting 2Rx that also indicate support for supportOf2RxXR-r18.

7.3.2_1.3	Minimum conformance requirements

For a XR UE equipped with 4 Rx antenna ports, tThe throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2.3-2 for the applicable operating bands.

For two Rx antenna port XR UE(s) indicating UE capability supportOf2RxXR-r18, reference sensitivity for two Rx antenna ports in Table 7.3.2.3-1a and in Table 7.3.2.3-1b  shall be modified by the amount given in ΔRXR,2R in Table 7.3.2_1-1 for the applicable operating bands.

Table 7.3.2_1-1: Two antenna port XR UE reference sensitivity allowance ΔRXR,2R

| Operating band | ΔRXR,2R (dB) |
| --- | --- |
| n7, n38, n41, n48, n77, n78, n79 | -1.0 |

The reference receiver sensitivity (REFSENS) requirement specified in Table 7.3.2-1a, Table 7.3.2-1b along with the modification specified in Table 7.3.2_1-1 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3.

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3.2.

7.3.2_1.4	Test description

7.3.2_1.4.1	Initial conditions

Same as 7.3.2.4.1 with test configuration as listed below in Table 7.3.2_1.4.1-1

Table 7.3.2_1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1 |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB allocation | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: In a band where UE supports 2Rx but not supports 4Rx, the test needs to be repeated with only 2Rx antennas connected and the other antennas terminated. |  |  |  |  |

7.3.2_1.4.2	Test procedure

Same as 7.3.2.4.2

7.3.2_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6 ensuring Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

7.3.2_1.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Tables 7.3.2.5-1a and Tables 7.3.2.5-1b for XR UEs with 2 Rx antenna port where the allowance in Table 7.3.2_1-1 shall be applied and Tables 7.3.2.5-2a and Tables 7.3.2.5-2b for XR UEs with 4 Rx antenna port and parameters specified Table 7.3.2_1.4.1-1, Table 7.3.2.4.1-2 and Table 7.3.2.4.1-3. The Test Tolerances in Table 7.3.2.5-3 shall be applied.

### <Unchanged sections skipped>

Table F.1.3-1: Maximum Test System Uncertainty for receiver tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.3.2 Reference sensitivity power level | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz |  |
| 7.3.2_1 Reference sensitivity power level for XR | Same as 7.3.2 |  |

### <Unchanged sections skipped>

Table F.3.3-1: Derivation of Test Requirements (Receiver tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 7.3.2 Reference sensitivity power level | 0.7 dB, f ≤ 3.0GHz 1.0 dB, 3.0GHz < f ≤ 6.0GHz | Reference sensitivity power level + TT T-put limit unchanged |
| 7.3.2_1 Reference sensitivity power level for XR | Same as 7.3.2 | Same as 7.3.,2 |

### <End of changes>