3GPP TSG-RAN5 Meeting #107	R5-253430

# St Julian’s, Malta, 19 – 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3369 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case minimum output power for Tx Diversity for UE supporting 4Tx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Minimum output power for Tx Diversity for UE supporting 4Tx test case is absent in 6.3G.1 and need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Minimum output power for Tx Diversity for UE supporting 4Tx test case is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Minimum output power for Tx Diversity for UE supporting 4Tx test case can not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3G.1_1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-252671. |  |  |  |  |  |  |  |  |

<Start of Changes>

6.3G.1_1	 Minimum output power for Tx Diversity for UE supporting 4Tx

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- MU and TT are FFS.

6.3G.1_1.1	Test purpose

Same test purpose as in 6.3.1.1.

6.3G.1_1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE release 18 and forward that support 4Tx Tx diversity.

6.3G.1_1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.3G.1.3

6.3G.1_1.4	Test description

Same test description as specified in clause 6.3G.1.4 with following exceptions:

For initial condition in 6.3G.1.4.1:

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2a for TE diagram and section A.3.2 for UE diagram.

Step 3 of Test procedure as in 6.3G.1.4.2 is replaced by:

3.	Measure the sum of mean power at each antenna connector in the associated measurement channel bandwidth specified in Table 6.3G.1_1.5-1 for the specific channel bandwidth under test. The period of measurement shall be at least the continuous duration of one active sub-frame (1ms) and in the uplink symbols. For TDD symbols with transient periods are not under test.

6.3G.1_1.5	Test requirement

The minimum output power, derived in step 3 shall not exceed the values specified in Table 6.3G.1_1.5-1.

Table 6.3G.1_1.5-1: Minimum output power

| Minimum output powerChannel bandwidth(MHz) | Minimum output power (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 3 | -40+TT | 2.715 |
| 5 | -40+TT | 4.515 |
| 10 | -40+TT | 9.375 |
| 15 | -40+TT | 14.235 |
| 20 | -40+TT | 19.095 |
| 25 | -39+TT | 23.955 |
| 30 | -38.2+TT | 28.815 |
| 35 | -37.6+TT | 33.855 |
| 40 | -37+TT | 38.895 |
| 45 | -36.5+TT | 43.575 |
| 50 | -36+TT | 48.615 |
| 60 | -35.2+TT | 58.35 |
| 70 | -34.6+TT | 68.07 |
| 80 | -34+TT | 78.15 |
| 90 | -33.5+TT | 88.23 |
| 100 | -33+TT | 98.31 |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.3G.1_1.5-2 |  |  |

Table 6.3G.1_1.5-2: Test Tolerance (Minimum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.0 dB | 1.3 dB |
| 40MHz < BW ≤ 100MHz | 1.3 dB | 1.3 dB |

< End of Changes >