3GPP TSG-RAN5 Meeting #107	R5-253487

# Malta, May 19 - 23 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0961 | rev | 1 | Current version: | 18.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on enhanced receiver type 2 test cases for FDD 2Rx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh3-UEConTest |  |  |  |  |  | Date: |  |  | 2025-05-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For TC 5.2.2.1.16_2, the required DCI information as captured in Table 5.2.2.1.16.0-4 is missing in the message content part. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Physical layer parameters for DCI format 1_1 for TC 5.2.2.1.16_2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The message content for this TC will remain uncompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1.16_2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision: Add editor note that ‘The minimum test time analysis is pending’ as per company comment. |  |  |  |  |  |  |  |  |

/// Start of change

5.2.2.1.16_2	2Rx FDD FR1 for PDSCH with intra cell inter user interference performance for Enhanced Receiver Type 2 – 2x2 MIMO for both NSA and SA

Editor’s Note: 	This test case is incomplete. Following aspects are either missing or TBD

-	TC applicability is FFS in TS38.522

-	The minimum test time analysis is pending

5.2.2.1.16_2.1	Test purpose

Test 2-1 is to verify PDSCH performance under 2 receive antenna conditions, when the PDSCH transmission of target UE is interfered by co-scheduled UE with Enhanced Receiver Type 2-1 when modulation order for co-scheduled UE is explicitly signalled by DCI.

Test 2-2 is to verify PDSCH performance under 2 receive antenna conditions, when the PDSCH transmission of target UE is interfered by co-scheduled UE with Enhanced Receiver Type 2-2a or Enhanced Receiver Type 2-2b when modulation order for co-scheduled UE is detected.

5.2.2.1.16_2.2	Test applicability

Test 2-1 applies to all types of NR UEs and E-UTRAN UEs supporting EN-DC release 18 and forward supporting capability IE advReceiver-MU-MIMO-r18.

Test 2-2 applies to all types of NR UEs and E-UTRAN UEs supporting EN-DC release 18 and forward which declare supporting ‘MU-MIMO Interference Mitigation advanced receiver with modulation order detection’ or ‘MU-MIMO Interference Mitigation advanced receiver with modulation order detection Enh’ in TS38.306 [14].

5.2.2.1.16_2.3	Test description

5.2.2.1.16_2.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1 and Table 5.2.2.1.16.0-2 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.2.1.16_2.3.3.

5.2.2.1.16_2.3.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.2.1.16_2.4-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.1.16_2.4-1 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

4.	Repeat steps from 1 to 3 for each subtest in Tables 5.2.2.1.16_2.4-1 as appropriate.

5.2.2.1.16_2.3.3	Message contents

5.2.2.1.16_2.3.3_1	Message exceptions for SA

Same as 5.2.2.1.16_1.3.3_1 with the following exceptions:

Table 5.2.2.1.16_2.3.3_1-1: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.1-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { | 1 entry |  |  |
| AdvancedReceiver-MU-MIMO-r18 ::= SEQUENCE { |  |  |  |
| precodingAndResourceAllocation-r18 | true |  |  |
| pdsch-TimeDomainAllocation-r18 | true |  |  |
| mcs-Table-r18 | qam256 |  |  |
| advReceiver-MU-MIMO-DCI-1-1-r18 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.1.16_2.3.3_1-2: Physical layer parameters for DCI format 1_1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| Co-scheduled UE information | ‘In all the PRBs allocated to the UE, all the co-scheduled UE(s), if any, which have the same root DMRS sequence as the UE, are scheduled with modulation scheme QPSK’ as per Table 7.3.1.2.2-12 in TS 38.212 [10] | ‘001’ B | Test 2-1 |
|  | ‘In each individual PRB allocated to the UE, all the co-scheduled UE(s), which have the same root DMRS sequence as the UE, are scheduled with the same modulation scheme, except the cases corresponding to index 0~5’ as per Table 7.3.1.2.2-12 in TS 38.212 [10] | ‘110’ B | Test 2-2 |

5.2.2.1.16_2.3.3_2	Message exceptions for NSA

Same as 5.2.2.1.16_2.3.3_1.

5.2.2.1.16_2.4	Test requirement

Table 5.2.2.1.16_2.4-1: Test requirement for PDSCH of target UE with Rank 1 with Enhanced Receiver Type 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | Random QPSK symbols | TDLC300-100 | 2x2, ULA Medium | 70 | 17.1 |
| 2-2 | R.PDSCH.5-1.3 FDD | 10 / 15 | 64QAM, 0.43 | Random 16QAM symbols | TDLC300-100 | 2x2, ULA Medium | 70 | 25.4 |

/// End of change