3GPP TSG-RAN5 Meeting #107	R5-253527

# Malta, Malta, 19th May 2025 - 23rd May 2025

Agenda item:	5.4.20

Source:	Keysight Technologies

Title:	FR2 MU - PC3 analysis for OBW UL MIMO test

Document for:	Discussion and Endorsement

# Introduction

This contribution aims to continue the MU analysis for PC3 in OBW UL MIMO test initiated in [1] and [2] by leveraging MU analysis for PC3 in OBW UL SISO test made for FR2c in [3] and [4].

# Discussion

Currently the MU is undefined for the following cases in the OBW UL MIMO test.

| Frequency sub-range | CBW | UL level | Noise Floor assumption | SNR | OBW MU | MBW/CBW |
| --- | --- | --- | --- | --- | --- | --- |
| FR2a | 400 MHz | 13.4 dBm | -7.6dBm | 21 dB | TBD | TBD |
| FR2b | 400 MHz | 11.6 dBm | -5.5dBm | 17.1 dB | TBD | TBD |
| FR2c | 200 MHz | 11.7 dBm | -7.5dBm | 19.2 dB | TBD | TBD |
|  | 400 MHz | 9.7 dBm | -4.5dBm | 14.2 dB | TBD | TBD |

Table 2.1. Test points with undefined MU in OBW UL MIMO test for PC3.

In [3], the following graph was presented to estimate the OBW MU. The MU depends on the MBW/CBW ratio and the available SNR so values were presented for MBW 1.5CBW (default FR2 OBW measurement configuration), 1.3CBW and 1.1CBW and for SNR ≥12dB.

Figure 2.1. OBW MU vs SNR vs MBW/CBW ratio [3].

In case of FR2c OBW SISO test for PC3, the UL level to estimate the MU is 14.7dBm for 200MHz CBW and 13.2dBm for 400MHz CBW what results into a SNR of 22.2dB and 17.7dB for 200MHz and 400MHz respectively. In that situation, it was clear that, for 200MHz, MBW=1.5CBW is valid while for 400MHz it is needed to reduce the MBW down to 1.3*CBW driving into an MU of about 1.5% as proposed in [4].

Considering the 97.5-tile curves depicted in Figure 2.1, and the SNR estimated in Table 2.1, the proposed OBW MU and corresponding MBW/CBW ratio is captured in Table 2.2.

| Frequency sub-range | CBW | UL level | Noise Floor assumption | SNR | OBW MU | MBW/CBW |
| --- | --- | --- | --- | --- | --- | --- |
| FR2a | 400 MHz | 13.4 dBm | -7.6dBm | 21 dB | 1.3% | 1.5 |
| FR2b | 400 MHz | 11.6 dBm | -5.5dBm | 17.1 dB | 1.7% | 1.3 |
| FR2c | 200 MHz | 11.7 dBm | -7.5dBm | 19.2 dB | 1.2% | 1.3 |
|  | 400 MHz | 9.7 dBm | -4.5dBm | 14.2 dB | 1.2% | 1.1 |

Table 2.2. Proposed MU and required MBW/CBW ratio for PC3 in UL MIMO test.

For OBW SISO test, for FR2c it has already been agreed to reduce MBW down to 1.3*CBW so proposals above for OBW UL MIMO test for FR2b 400MHz and FR2c 200MHz should be agreeable because same MBW reduction is proposed.

However, in case of FR2c 400MHz, in order to keep MU under control, the required MBW should b reduced significantly, down to 1.1*CBW. Despite there is no explicit core requirement about the MBW to consider to perform the OBW measurement, 2*CBW has been the typical value in FR1 that later needed to be reduced in FR2 down to 1.5*CBW in order to keep MU under control. The smaller is the MBW/CBW ratio, the worse will be the validity of the measurement because the less the UE power transmissions at the adjacent channels are taking into account to estimate the BW at which the UE concentrate 99% of its total power. For that reason, for FR2c 400MHz, we propose to declare it as not testable under the assumption that MBW = 1.1*CBW is too small. The spectrum emission mask requirements permit the UE a relatively high power level up to ΔfOOB = 10% of the CBW. Therefore, a correct OBW measurement should imply MBW ≥ 1.2*CBW so at least power at ΔfOOB ≥ 10% is computed at each edge of the channel to do the OBW measurement.

Table 6.5.2.1-1: General NR spectrum emission mask for frequency range 2 (TS 38.101-2). Spectrum emission limit (dBm) / Channel bandwidth. ΔfOOB (MHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz; Measurement bandwidth. 0-5; -5; -5; -5; -5; 1 MHz. 5-10; -13; -5; -5; -5; 1 MHz. 10-20; -13; -13; -5; -5; 1 MHz. 20-40; -13; -13; -13; -5; 1 MHz. 40-80; -13; -13; -13; -13; 1 MHz. 80-100; -13; -13; -13; -13; 1 MHz. 100-160; -13; -13; -13; 1 MHz. 160-200; -13; -13; -13; 1 MHz. 200-400; -13; -13; 1 MHz. 400-800; -13; 1 MHz.

Observation 1. The spectrum emission mask requirements allow UE high power up to ΔfOOB = 10% of CBW.

Observation 2. If it is intended measuring OBW including UE power at least up to ΔfOOB = 10% of CBW at each edge of the channel, then, MBW ≥ 1.2*CBW is needed.

Observation 3. To measure OBW UL MIMO test in FR2c, for CBW = 400MHz, to achieve a reasonable MTSU, MBW = 1.1*CBW is required.

Proposal 1. For OBW UL MIMO test in FR2c, for CBW = 400MHz, declare the test as not testable.

Proposal 2. For OBW UL MIMO test in FR2a, for CBW = 400MHz, apply MBW = 1.5*CBW and define MTSU as +/- 1.3 %CBW.

Proposal 3. For OBW UL MIMO test in FR2b, for CBW = 400MHz, apply MBW = [1.3]*CBW and define MTSU as +/- [1.7] %CBW.

Proposal 4. For OBW UL MIMO test in FR2c, for CBW = 200MHz, apply MBW = [1.3]*CBW and define MTSU as +/- [1.2] %CBW.

Proposal 5. For the channel bandwidths and frequency ranges with defined MU in Proposals 2, 3 and 4, define TT = 0 kHz.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1. The spectrum emission mask requirements allow UE high power up to ΔfOOB = 10% of CBW.

Observation 2. If it is intended measuring OBW including UE power at least up to ΔfOOB = 10% of CBW at each edge of the channel, then, MBW ≥ 1.2*CBW is needed.

Observation 3. To measure OBW UL MIMO test in FR2c, for CBW = 400MHz, to achieve a reasonable MTSU, MBW = 1.1*CBW is required.

Proposal 1. For OBW UL MIMO test in FR2c, for CBW = 400MHz, declare the test as not testable.

Proposal 2. For OBW UL MIMO test in FR2a, for CBW = 400MHz, apply MBW = 1.5*CBW and define MTSU as +/- 1.3 %CBW.

Proposal 3. For OBW UL MIMO test in FR2b, for CBW = 400MHz, apply MBW = [1.3]*CBW and define MTSU as +/- [1.7] %CBW.

Proposal 4. For OBW UL MIMO test in FR2c, for CBW = 200MHz, apply MBW = [1.3]*CBW and define MTSU as +/- [1.2] %CBW.

Proposal 5. For the channel bandwidths and frequency ranges with defined MU in Proposals 2, 3 and 4, define TT = 0 kHz.

# References

- R5-247165, “MU discussion on OBW for FR2 UL MIMO”, Anritsu, RAN5#105

- R5-247337, “On the MU and TT for SISO and MIMO test cases”, R&S, RAN5#105

- R5-242998, “On testability issue of FR2c OBW”, Anritsu, RAN5#103

- R5-244167, “Discussion on MU and TT analysis for FR2c and FR2b”, Keysight, RAN5#104