Title:	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA

Source:	Rohde & Schwarz

# 1. Introduction

This document relates to UE Rx-Tx time difference measurement accuracy test case for single positioning frequency layer in FR1 SA in TS 37.571-1. The test case is:

#### 15.5.2, UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA with reduced number of samples for RRC_INACTIVE

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Test cases in TS 38.133

The test conditions are defined in the following extract from TS 38.133 v17.17.0.

#### A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples

##### A.6.9.3.2.1	Test purpose and environment

The purpose of the test is to verify that the UE Rx-Tx time difference measurement accuracy with reduced number of samples in RRC_INACTIVE state is within the specified limits. This test will verify the requirements in clause 10.1.25.2. The test is conducted in AWGN propagation condition in FR1 in standalone scenario when single positioning frequency layer is configured.

The supported test configurations in listed in Table A.6.9.3.2.1-1.

Table A.6.9.3.2.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). Both cells are on the same RF channel in FR1.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355 [34, clause 6.5.12.1], shall be provided to the UE before the start of the test. The UE is configured to measure UE Rx-Tx time difference using reduced number of samples via requestedDL-PRS-ProcessingSamples in NR-Multi-RTT-RequestLocationInformation.

UE shall be configured to enter into RRC_INACTIVE state before the start of the test.

The UE is configured to transmit positioning SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

##### A.6.9.3.2.2	Test parameters

The UE Rx-Tx time difference accuracy test parameters are given in Table A.6.9.3.2.2-1.

Table A.6.9.3.2.2-1: UE Rx-Tx time difference measurement accuracy test parameters

| Parameter | Unit | Test configuration | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 |
| RF Channel Number |  | 1,2,3 | 1 | 1 |
| DRX cycle |  | 1,2,3 | 1.28s |  |
| Time offset with Cell 1 | s | 1, 2, 3 | N/A | 3 |
| TDD configuration |  | 1 | N/A | N/A |
|  |  | 2 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 | TDDConf.2.1 | TDDConf.2.1 |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD | N/A |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD | N/A |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD | N/A |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 | OP.1 |
| EPRE ratio of PSS to SSS | dB | 1, 2, 3 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| EPRE ratio of PRS to SSS |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD | N/A |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 | N/A |
| PRS configuration |  | 1 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  |  | 2 | PRS.1.1 FR1 | PRS.1.1 FR1 |
|  |  | 3 | PRS.2.1 FR1 | PRS.2.1 FR1 |
| PRS BW |  | 1 | 52 PRBs | 52 PRBs |
|  |  | 2 | 52 PRBs | 52 PRBs |
|  |  | 3 | 48 PRBs | 48 PRBs |
| PRS Resource slot offset | slot | 1, 2, 3 | 0 | 4 |
| SRS configuration |  | 1 | POS-SRS.1 | N/A |
|  |  | 2 | POS-SRS.1 | N/A |
|  |  | 3 | POS-SRS.2 | N/A |
| Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98 |  |
|  |  | 3 | -95 |  |
| Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | 0 | -6 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS | dB | 1 | 2.23 | -1.73 |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRP Note 3 | dBm/SCS kHz | 1 | -95.77 | -99.73 |
|  |  | 2 | -95.77 | -99.73 |
|  |  | 3 | -92.77 | -96.73 |
| Io | dBm/19.08 MHz | 1 | -61.71 | -61.71 |
|  | dBm/19.08 MHz | 2 | -61.71 | -61.71 |
|  | dBm/47.88 MHz | 3 | -57.73 | -57.73 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: PRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

##### A.6.9.3.2.3	Test requirements

The UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE state fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 10.1.25.2 for both Cell 1 and Cell 2.

For UE Rx-Tx time difference, the core requirements for UE Rx-Tx Time Difference Measurements are specified in TS 38.133 clause 10.1.25, with side conditions in Annex B.2.14.

# 3. Discussion

There are two cells in the test: PCell (Cell 1) and a neighbour cell (Cell 2). All cells are on the same RF channel in FR1.

The NR-Multi-RTT-ProvideAssistanceData and nr-Multi-RTT-RequestLocationInformation as defined in TS 37.355, shall be provided to the UE before the start of the test.

The UE is configured to transmit SRS on Cell 1 during the test.

The test equipment measures the transmit timing of the UE using the transmitted SRS and measures the receive timing using the PRS. The test equipment then compares the difference of these two timings to the UE Rx-Tx measurement reported by the UE for each cell.

The rate of the correct events for each neighbour cell observed during repeated tests shall be at least 90%, where the reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause TS 38.133 clause 10.1.25.3.1.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are selectively copied from Table A.6.9.3.1.1.1-1in TS 38.133, which are subcarrier spacing, channel bandwidth, RBs in channel bandwidth, Noc, Cell 1 PRS Es/Noc and Cell 2 PRS Es/Noc. All other parameters such as Io, PRS Es/Iot and PRS-RSRP are derived, and not independently settable by the Test Equipment.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

c) Uncertainties

The SS provides AWGN and 2 intra-frequency cells to the UE under AWGN conditions in FR1 in standalone scenario. We propose to control the following parameters:

- AWGN absolute power, Noc ±1.5 dB averaged over BWConfig

- Ratio of cell 1 PRS signal / AWGN, PRS Ês1 / Noc ±0.3 dB averaged over BWConfig

- Ratio of cell 2 PRS signal / AWGN, PRS Ês2 / Noc ±0.3 dB averaged over BWConfig

- LPP Response Time, 0.3 seconds

The uncertainties appear in section c) of the accompanying spreadsheet.

The absolute level uncertainties of Noc are chosen to ensure the analysis and Test Requirements are valid for frequency bands up to 6GHz.

The LPP Response Time uncertainty has been chosen to be in-line with all other LPP test cases defined in TS 37.571-1 and there is no necessity to derive the test tolerance in the spreadsheet. LPP Response Time is as defined in TS 37.571-1 clause 4.14.3.

d) Controlled parameters critical to verdict

In many RRM test cases, and particularly intra-frequency multi-cell tests, there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. In this test case there are two aspects to consider:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE reporting delay (UE Rx-Tx time difference measurements)

The stimulus set by the Test system should be within the constraints (side conditions) for the UE measurement. For Cell 1, they are the PRS Es/Iot range and the PRP power range for the reference cell over which the UE is expected to provide UE Rx-Tx time difference measurements. For Cell 2, they are the PRS Es/Iot range and the PRP power range for the neighbour cell over which the UE is expected to provide UE Rx-Tx time difference measurements. The Io power shall also be within the range over which the UE is expected to provide UE Rx-Tx time difference measurements. The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given later in the “Comment” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the Cell 1 PRP, so the sensitivity factor is 1.000 for all tests. However, the same error of 1dB in the absolute AWGN level Noc would cause no change to Cell 2 Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

In some cases, the sensitivity factor is an intermediate value. For example, the Cell 2 Es/Noc has an effect on Cell 1 Es/Iot which depends on ratios of the powers making up the total. In such cases the sensitivity factor value between 0 and 1 result. It is important to calculate these correctly to obtain the overall uncertainty.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903. Uncertainties are calculated separately for T1 and T2.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

e) Determine parameters to offset

During T2, the uncertainties take Cell 1 PRS Es/Iot outside the allowed ranges.

We observe that for the test configuration that the test equipment uncertainties could take the UE outside the allowed range, so the Es/Noc of Cell 1 is therefore increased by an amount sufficient to achieve Es/Iot >=0dB.

The PRS Es/Noc of Cell 2 is increased by an amount sufficient to achieve Cell 1 Es/Iot >=-6dB.

The RSRP and Io values for both cells remain inside the allowed range, and are checked in step g).

The UE Rx-Tx time differnece measurement reporting delay will be offset by the LPP Response Time uncertainty as specified in other LPP test cases defined in TS 37.571-1.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances.

The PRS Es/Noc of Cell 1 is increased by an amount sufficient to achieve PRS Es/Iot >=0dB. The actual offset required is +0.7dB for cell 1.

The PRS Es/Noc of Cell 2 is increased by an amount sufficient to achieve PRS Es/Iot >=-6dB. The actual offset required is +0.83dB for cell 2.

These values are found empirically by observing the minimum/maximum parameter values in step g).

g) Check controlled parameters Min/Max

For the Test System downlink signal, the nominal value of each controlled parameter is equal to the original since there has been no application of the Test Tolerances in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (PRS Es/Iot range, PRP power range, and Io power range). It is not necessary to calculate all parameters during each test, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all the stimulus requirements are met, then the chosen stimulus offsets are acceptable.

The stimulus requirements defined in TS 38.133 clause 10.1.25 are reproduced below. The stimulus requirements defined in TS 38.133 clause 10.1.25 indicate that the UE Rx-Tx Time Difference measurement accuracy for all measured cells and the Io range.

### 10.1.25	UE Rx-Tx Time Difference Measurements

<< Some text skipped >>

The accuracy requirements in Table 10.1.25.2-1a for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band.

AWGN propagation condition.

Table 10.1.25.2-1a: UE Rx-Tx time difference measurement accuracy in FR1 in AWGN with reduced measurement samples

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition (TrepPRS*LPRS/KcombPRSNote 3 | NR operating band groupsNote 2 | IoNote 4 range |  |
|  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  |  | dBm / SCSPRS | dBm/BW |
| ± 59+ | 0 | ≥52 | 15 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -126.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -126 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -50 |
| ± 30+ |  | >104 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 30+ |  | ≥48 | 30 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -123.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -123 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -121.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -120.5 | -50 |
| ± 15+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 15+ |  | ≥64 | 60 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
| ± 7+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 75+ | -6 | ≥52 | 15 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 37+ |  | >104 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 39+ |  | ≥48 | 30 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 16+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 16+ |  | ≥64 | 60 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 8+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.25.2-5. |  |  |  |  |  |  |  |

<< Some text skipped >>

Table 10.1.25.2-5: Margin for UE Rx-Tx time difference measurement accuracy in FR1

| Min(PRS BW, SRS BW) (RB) |  |  | Margin (Tc Note 1) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 24 | N/A | N/A | 160 |
| ≥ 52 | ≥ 24 | N/A | 80 |
| ≥ 104 | ≥ 48 | ≥ 24 | 56 |
| N/A | ≥ 132 | ≥ 64 | 24 |
| N/A | N/A | ≥ 132 | 24 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |  |

The side conditions are defined in TS 38.133 Annex B.2.14 and are reproduced below.

## B.2.14	Conditions for NR PRS-based measurements

This clause defines the following conditions for NR PRS-based measurements and corresponding procedures performed based on PRS: PRP and PRS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.14-1 for FR1 NR cells.

The conditions are defined in Table B.2.14-2 for FR2 NR cells.

Table B.2.14-1: Conditions for NR PRS-based measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum PRP1,2 |  |  | PRS Ês/Iot |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSPRS |  |  | dB |  |
|  |  | SCSPRS = 15 kHz | SCSPRS = 30 kHz | SCSPRS = 60 kHz |  |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -121 | -6 Note2 -13 Note3 -3 Note4 | -6 Note5 -3 Note6 0 Note7 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 | -120.5 |  |  |
|  | NR_TDD_FR1_C | -126 | -123 | -120 |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 | -119.5 |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 | -119 |  |  |
|  | NR_FDD_FR1_F | -124.5 | -121.5 | -118.5 |  |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 | -121 | -118 |  |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 | -117.5 |  |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. NOTE 2: PRS Ês/Iot for RSTD measurement reference cell PRS resource. NOTE 3: PRS Ês/Iot for RSTD measurement neighbor cell PRS resource, PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement. NOTE 4: PRS Ês/Iot for PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement. NOTE 5: PRS Ês/Iot for RSTD measurement neighbor cell PRS resource, PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement when performed with reduced number of samples. NOTE 6: PRS Ês/Iot for RSTD measurement reference cell PRS resource when performed with reduced number of samples. NOTE 7: PRS Ês/Iot for PRS-RSRP measurement, PRS-RSRPP measurement and UE Rx-Tx time difference measurement when performed with reduced number of samples. |  |  |  |  |  |  |

We note that for the 20MHz Channel BW test cases, the tightest applicable requirements are given by Band n25 in NR_FDD_FR1_H. The highest PRP side condition is -123.5dBm/15kHz. the tightest applicable requirements are given in NR_TDD_FR1_G. The highest PRP side condition is -121dBm/30kHz.

It can be seen that with the uncertainty values and Test Tolerances proposed, the stimulus requirements are met.

# Calculation of UE Rx-Tx Time difference:

As per TS38.133 10.1.25.2 we need to calculate accuracy and measure uncertainties by adding the values from Table 10.1.25.2-1a and Table 10.1.25.2-5

Furthermore, we need to consider the test equipment uncertainty for the Cell Timing Difference. We propose to define a test system uncertainty of ±32 Tc.

The reporting limits, based on given uncertainties and the UE reporting limits are summarized in the following tables:

| Test Configuration | Subtest | Reported limits calculations |
| --- | --- | --- |
| 1&2 (15kHz) | cell 1 | ±59 ± 80 ± 32 = ±171 |
|  | cell 2 | ±75 ± 80 ± 32 = ±187 |
| 30kHz | cell 1 | ±30 ± 56 ± 32 = ±118 |
|  | cell 2 | ±39 ± 56 ± 32 = ±127 |

UE Rx – Tx time difference measurement accuracy requirements

| Test Configuration | Subtest | Lowest reported value | Highest reported value |
| --- | --- | --- | --- |
| 1&2 | cell 1 | (Measured value from step 12 - 171) Tc converted to UE Rx-Tx time difference measurement according to clause 4.14.6 | (Measured value from step 12 + 171) Tc converted to UE Rx-Tx time difference measurement according to clause 4.14.6 |
|  | cell 2 | (Measured value from step 12 - 187) Tc converted to UE Rx-Tx time difference measurement according to clause 4.14.6 | (Measured value from step 12 + 187) Tc converted to UE Rx-Tx time difference measurement according to clause 4.14.6 |
| 3 | cell 1 | (Measured value from step 12 – 118) Tc converted to UE Rx-Tx time difference measurement according to clause 4.14.6 | (Measured value from step 12 + 118) Tc converted to UE Rx-Tx time difference measurement according to clause 4.14.6 |
|  | cell 2 | (Measured value from step 12 - 127) Tc converted to UE Rx-Tx time difference measurement according to clause 4.14.6 | (Measured value from step 12 + 127) Tc converted to UE Rx-Tx time difference measurement according to clause 4.14.6 |

# 5. Treatment of other configurations within the same test case

TS 37.571-1 test case 15.5.2 also contains other sub-test with different PRS configurations. The calculations use the same values, a separate spreadsheet tab is not provided.